USR168_2

By Kurt Heilbron

Sysop, Dirty Joe's Sweatshop (808)536-3600

WHAT IT DOES:

-------------

USR168 is a utility program written with Turbo Pascal 5.0 using

routines from Pro kit by Samuel Smith.

USR168 is released as a free program.  No donations are requested for

it's use.  The authors are not responsible for any damage that might be

caused by the use of this program.  Please use this program at your own

risk.

The purpose of USR168 is to aid owners of US Robotics Courier Dual Standard

modems either the new 16.8K OR the 14.4k in setting up for optimum speed

under LOCKED PORT AT 19200!

This program is designed for callers only.  Although I run my BBS with

the settings from USR168, I would recommend that Sysops follow the

instructions for their specific BBS package on modem settings.

This program will lock your serial port at 19200.  This setting may

only be effective on 286 and 386 systems.  Some multi-tasking software

may not be able to handle this setting.  If you find that you are

having trouble with aborted downloads after running this program then

you should probably go into your terminal program and type AT&F

followed by the ENTER key to reset the modem to it's factory settings.

Using these settings along with protocols designed for error correcting

modems such as DSZ's Ymodem-G and Xmodem-1K-G or Qmodem's version of

Ymodem-G I have achieved file transfer speeds in the 1800 - 2000 cps

range on a regular basis using both 286 and 386 systems.

HOW IT DOES IT:

---------------

USR168 starts by giving the suggested dip switch settings, which are

as follows:

1   2   3   4   5   6   7   8   9   10

--  --  --  --  --  --  --  --  --  --

UP  UP  DN  DN  UP  UP  UP  DN  DN  UP

These settings will do the following to your modem:

1 UP - DTR is controlled by software

2 UP - Send verbal result codes

3 DN - Result codes are displayed

4 DN - Local echo of commands is suppressed

5 UP - Auto Answer Suppressed

6 UP - Do not force carrier detect

7 UP - Single phone line connection (RJ-11)

8 DN - AT command set enabled

9 DN - Hang up when escape code is sent

10 UP- Load from NRAM in power up

It then will reset your modems internal NRAM settings to the original

factory defaults.

Next it sends the following settings to the modem's non-volatile

memory:

ATS11=40E0M0X7

S11=40 - Sets dialing speed to max!!

E0 - Local echo off

M0 - Speaker Off (can be overridden by putting M1 in your modem program's

initialization string)

X7 - Result code option (OK, CONNECT, RING, NO CARRIER, ERROR, CONNECT,

NO DIAL TONE, BUSY, NO ANSWER, RINGING, VOICE)

Once this string is received it sends the following additional

settings:

AT&H1&R1&S0&B1&A3&W

&H1 - Hardware control

&R1 - Ignore RTS

&S0 - DSR overrides

&B1 - Lock port at speed setting (19200)

&K1 - Auto Enable data compression

&W -  Write these settings to non-volatile RAM (NRAM).

After running this program, you should go into the setup portion of

your modem program (Qmodem, Procomm or whatever) and change the modem

initialization string to ATZ followed by whatever character that

program uses to send a carriage return (Qmodem uses a { Procomm uses a

!).

DATA COMPRESSION?

-----------------

The USR DS has a setting for "Data Compression".  USR168 turns this on

because I've found that transfer rates are faster even with files that

are already compressed!  Some say that it is just the opposite, but I know

from experience that this is totally false!

If your application finds you down loading mostly compressed files

then you may want to put &K1 in your modem initialization string.

Otherwise, leave it alone and you will see a marked increase in

performance. You may want to disable MNP5 as this form of DATA compression

may slow transfer rates.

TIPS 'N' TRICKS:

----------------

If you want to override any of the NRAM settings that I have written in

the program, you can either do by placing the replacement commands in

the modem initialization string, or by going into the terminal mode of

your modem software and typing those commands followed by AT&W.

It is highly recommended that you use a high performance protocol

driver such as DSZ by Chuck Foresberg.  The registered version of DSZ

includes YMODEM-G and Xmodem-1K-G protocols which should yield the

highest possible CPS ratings under this setup.  The current versions of

many popular modem programs have built-in versions of Xmodem-1K-G

(called Ymodem-G in some versions) which will yield excellent speed,

but won't allow batch mode down loading that true Ymodem-G from the DSZ

program features.

Zmodem will give 1800-2000 cps on properly configured systems. Zmodem

is featured in non-registered versions of DSZ that are available on

most BBS systems.

Avoid using Xmodem, Ymodem, WXmodem, Kermit, Jmodem or other protocols

that rely on software error checking when you are connected to another

USR at the fixed 19200 bps rate.  These protocols will give you between

400-900 cps.  One reason I wrote this program was that I see people

night after night calling with USR DS modems improperly configured and

getting transfer speeds of 400 cps.

ADDITIONS:

---------

This program will program your modem to run just fine.  But if you want to

see ALL my current settings, well here they are.  Remember, some of the

default settings should be left alone.  That is why the USR168 programs

only what is necessary.

US Robotics Courier 16800 HST Dual Standard Fax Settings...

B0  C1  E0  F1  M0  Q0  V1  X7

BAUD=19200  PARITY=N  WORDLEN=8

DIAL=HUNT   ON HOOK   TIMER

&A3  &B1  &C1  &D2  &G0  &H1  &I0  &K1  &L0

&M4  &N0  &P0  &R2  &S0  &T5  &X0  &Y1  %R0

S00=001  S01=000  S02=043  S03=013  S04=010  S05=008

S06=002  S07=045  S08=002  S09=006  S10=015  S11=040

S12=050  S13=000  S14=001  S15=008  S16=000  S17=000

S18=000  S19=000  S20=000  S21=010  S22=017  S23=019

S24=150  S25=005  S26=001  S27=000  S28=000  S29=020

S30=000  S31=000  S32=006  S33=000  S34=000  S35=000

S36=000  S37=000  S38=000  S39=000  S40=000  S41=000

S42=126  S43=200  S44=015

ATI4         Will display your current MODEM settings

ATI5         Will display your NVRAM settings

ATI6         Will display what your LAST connect stats.

ATI7         Will show the production date etc.

ATZ ENTER    Will set the modem back to factory defaults.

ATE1         Allows you to view what you are typing.

ATH          To command modem.

AT&B1        To start inputting the NRAM settings.

Finish with a &W    Ex: ATHS1=043S03=013&W  <-Then hit enter

AT&F&W       Will reset the NRAM settings.

GT POWER Modem int string AND Host mode init string

--------------------------------------------------

Also, here are the MODEM AND the HOST mode init strings I use with running

GT POWER.  IMPORTANT IMPORTANT!!!  Do NOT forget in the GT setup (Alt-I) to

select number 5 (baud rate set up) and change it to match your locked port

configuration!  AND furthermore select number 30 in the ALT-I setup, go to

number 14 (under 30 misc. options) and make all three of them TRUE!

MODEM INIT STRING= ATV1Q0X7S11=40S0=1S2=43|

-----------------

HOST MODE INIT STRING= ATV1Q0E0S0=1M0S2=43X7|

---------------------

ALT-I SETUP

-----------

#5      DEFAULT BAUD RATE= 19200

MINIMUM BAUD RATE= 2400  (OR WHATEVER YOU WANT)

MAXIMUM BAUD RATE= 19200

MINIMUM HOST RATE  2400  (OR WHATEVER YOU WANT)

#30 (Select #14)

USE CTS FLOW CONTROL?= TRUE

USE RTS FLOW CONTROL?= TRUE

FIXED DTE RATE?      = TRUE

SUPPORT:

--------

As stated above, this is a free program.  You will probably use it

once, then throw it away and forget all about me.  That's fine, but at

least when you call my BBS at 19200 you won't be getting 400 cps!

Updates to USR168 are always available on my BBS, Dirty Joe's Sweatshop

which is listed at the top of this file.

BBS Phone 808-536-3600