Changing IOBASE Settings With NRMOVE

---------------------------------------

(C) Copyright 1993 Artisoft, Inc.

(Edition 1)

ABOUT NRMOVE

============

The NRMOVE utility lets you temporarily force the NodeRunner adapter to

use an I/O Base address that you specify at the NRMOVE command line.

You would typically need to use NRMOVE if you installed your NodeRunner

adapter into a PC that contained another device that was already using the

NodeRunner's default I/O Base address of 300h. NRMOVE allows you to

easily resolve the I/O Base conflict, as explained in the section called

"Using NRMOVE" below.

After making the temporary I/O Base change using NRMOVE, you must run

NRSETUP's Manual Configuration option to update the I/O Base address field

to the same value you specified with NRMOVE.  Doing so makes the I/O Base

change permanent.

NOTE:  An inability to load the NR driver or run NRSETUP is not always

caused by an I/O Base conflict. However I/O Base conflicts are the

most common problems encountered when installing a new network

adapter.

USING NRMOVE

============

If you cannot load the NR driver or run NRSETUP, assume that you have an

I/O Base conflict. Follow the steps below to attempt to solve the problem.

If your PC is locked up:

------------------------

1.  Turn off your PC using the power switch.  Using Ctrl+Alt+Del

or the reset button to reboot will not work.

2.  Turn your PC back on, then perform the steps in the next section.

NOTE: If the NR command is contained in a startup batch

file, such as AUTOEXEC.BAT or STARTNET.BAT, use the

control+C (^C) command to interrupt the startup sequence

BEFORE the NR driver loads.

If your PC is not locked up:

----------------------------

1.  At the DOS prompt, type: NRMOVE <Enter>. This command

specifies the default I/O Base of 320h. To specify a different

I/O Base, type: NRMOVE IOBASE=[?] where [?] is the I/O Base

you want to use.

Example:  NRMOVE IOBASE=340

2.  At the DOS prompt, type: NRSETUP <Enter>.

NOTE:  If NRSETUP does not run, turn off your PC (using the

power switch), then turn it back on.  Repeat steps 1

and 2 in this section, specifying a different I/O Base.

You may need to repeat this sequence up to two more

times before you find an I/O Base that works. Once you

find a suitable I/O Base, continue to step 3.

3.  Set the NRSETUP I/O Base address field to the I/O Base you

specified with NRMOVE.

NOTE:  If NRSETUP does not run with any of the I/O Base

addresses, you may have multiple devices installed

that are using all of the NodeRunner 's I/O Base

options. You may also have a problem that is unrelated

to the I/O Base address.

4.  If you want to continue trying to solve the problem on your

own, take out any other installed devices one at a time,

repeating steps 1 - 3 above after each removal.

5.  If you would like assistance, contact the Artisoft Technical

Support Department at (602) 670-7000.