C A L C O M P

907/PCI

WINDOWS DRIVER

VECTOR ONLY

SEPTEMBER 1993

TABLE OF CONTENTS

PAGE

NOTICES	                                                               i

CHAPTER 1 INTRODUCTION                                                 1

CHAPTER 2 INSTALLATION WITH MICROSOFT WINDOWS VER 3.0                  3

CHAPTER 3 INSTALLATION WITH MICROSOFT WINDOWS VER 3.1                  8

CHAPTER 4 TROUBLESHOOTING                                             13

APPENDIX A  USING THE TEST FILES				      15

For Use with Microsoft Windows

Installation and User's Guide

September 1993

Revision 3.5

NOTICES

While every precaution has been taken in the preparation of this

publication, CalComp assumes no responsibility for errors or omissions.

Nor is any liability assumed for damages resulting from the use of the

information contained herein. Further, this publication and features

described herein are subject to change without further notice.

TRADEMARKS

Each of the following products is a trademark or a registered trademark

of the company listed after the product name.

DESIGNMATE, DRAWINGMASTER,PLOTMASTER, ARTISAN, PLOT MANAGER,

PACESETTER, CLASSIC, COLORMASTER, COLORVIEW;  CALCOMP

MICROSOFT, WINDOWS;  MICROSOFT CORP.

AUTOCAD;  AUTODESK INC.

IBM;  INTERNATIONAL BUSINESS MACHINES CORP.

CENTRONICS;  CENTRONICS DATA COMPUTER CORP.

COPYRIGHT = COPYRIGHT 1992 CalComp Inc.

All Rights Reserved

i

C H A P T E R   1

INTRODUCTION

ABOUT THE 907/PCI DEVICE DRIVER

The CalComp 907/PCI device driver is used with Microsoft Windows

Versions 3.0 and 3.1. The 907/PCI device driver provides high-quality,

high-speed capability to drive CalComp printers and plotters through

a serial or parallel communications port.

The device driver allows high-speed plotting of complex vector graphics.

It fits seamlessly into the Windows environment, so it can be accessed

through any Windows-based application for direct printing to a CalComp

device.

VECTOR BASED DEVICE DRIVER

This driver (named CALCOMP.DRV) allows you to print or plot files from

within an application that supports Microsoft Windows version 3.0 or 3.1.

You can send files to a CalComp pen plotter, direct thermal plotter,

electrostatic plotter, and a desktop color thermal printer.

You can use the device driver as long as the software application can use

the Windows Control Panel printer drivers.

Note: The printing/plotting of raster data is NOT supported by the driver.

NETWORK CONFIGURATION

The CalComp 907/PCI device driver runs transparently to a network configuration.

No special set-up procedures are needed for installation and configuration.

If you want to connect to a network printer, complete the network

connections dialog boxes as listed in the Microsoft Windows User's Guide

"Connecting to a Network Printer" section.

AutoCAD Configuration

The CalComp 907/PCI device driver operates with AutoDesk AutoCAD Release 11

or Release 12C2 Windows Version. You must specify a Systems printer for the

plotter configuration. This passes control of the printing function to the

Windows Control Panel printer.

Please refer to the AutoDesk AutoCAD Release 11 or Release 12 for Windows

User's Guide for specific setup procedures for both the Windows environment

and plotter selection/setup.

CalComp Device Control Language (CDCL)

The CalComp 907/PCI device driver automatically uses CDCL if the selected

device (from Config window) is CDCL compatible. CDCL is a set of software

instructions that automatically sends control commands to CalComp

printers and plotters. Using CDCL, you can override the printer or

plotter configuration and set-up parameters. The 907/PCI device driver

automatically generates this information in the CDCL format and places

it in the configuration file (named CDCLINFO.INF) in the root directory

on the drive where Windows is installed. In this way, you can enter

configuration and set-up parameters for the 907/PCI device driver, not for

the individual printer or plotter.

-1-

Not all CalComp devices accept CDCL. See the following table for

information about your printer or plotter. For the non-supported

CDCL printers and plotters, you must first enter the set-up parameters for

the 907/PCI device driver. Then configure and set up each printer or plotter

to match the CalComp 907/PCI device driver configuration.

Devices Supported by the 907/PCI Device Driver

The 907/PCI device driver supports the following CalComp plotters and printers.

VECTOR PLOTTERS                                   SUPPORTS CDCL:

1020 Artisan Series                                     No

1040 Series                                             No

1070 Series (requires Plot Manager Interface)           No

PaceSetter Series (Models 2024 & 2036)                 Yes

DesignMate Series (Models 3024 & 3036)                 Yes

Classic           (Model 4036)			       Yes

DIRECT IMAGING PLOTTERS:

DrawingMaster 52224                                    Yes

DrawingMaster 52236                                    Yes

DrawingMaster Plus 52224                               Yes

DrawingMaster Plus 52236                               Yes

DrawingMaster Plus 52424                               Yes

DrawingMaster Plus 52436                               Yes

ELECTROSTATIC PLOTTERS:

Model 5700 Series                                       No

Model 57000 Series                                      No

Model 5800 Series                                       No

Model 58000 Series                                      No

Model 67436                                            Yes

Model 68436                                            Yes

Model 68444                                            Yes

PRINTER MODELS, SUPPORTS CDCL:

PlotMaster Model 5902A* & 5902G                         No

ColorView Model 5912 & 5913                             No

ColorMaster Plus Model 6603VRC & 6613VRC               Yes

*Note: When using the PlotMaster Models 5902A or 5902G verify that the PCI

button is depressed and the LED is illuminated before using this driver.

-2-

C H A P T E R   2

The 907/PCI Device Driver with Windows 3.0

This chapter provides instructions for installing the CalComp 907/PCI device

driver in Microsoft Windows Version 3.0. Go to Chapter 3 if you are running

Windows 3.1. If you are not an experienced Windows user, please refer to

your Microsoft Windows User's Guide during this installation procedure.

Installing the 907/PCI Device Driver

Install the CalComp 907/PCI device driver by following the steps below.

Change to the Windows Program Manager window.

Single-click on the Main Program group in the Program Manager window.

Double-click on the Control Panel icon in the Main window.

Double-click on the Printers icon from the Control Panel window.

Single-click on the Add Printer button, scroll through the list of

printers until you see the option Unlisted Printer, and single-click it.

Single-click the Install button to reveal an Add unlisted printer window.

Insert the CalComp 907/PCI device driver disk into your computer's disk drive.

The default drive and directory are your current Windows 3.0 drive and

directory. Scroll through the directories list to select the drive,

such as [-a-] or [-b-], by double-clicking the drive letter.

The CALCOMP.DRV file displays in the Driver Files list.

Double-click on the CalComp 907/PCI Printer/Plotter device driver.

The driver is installed and you are returned to the Printers window.

At the Printers window, you can verify the active or inactive status of

an installed printer. If the CalComp 907/PCI Printer/Plotter shows an inactive

status, the port configuration has not been assigned. Assign a serial or

parallel port to the installed device driver and then select active status.

You may need to configure a port for the device driver, then assign the

port to the device driver. See the procedure "Configuring a Port for the

907/PCI Device Driver" below.

You can also set the CalComp 907/PCI Printer/Plotter to be the default printer.

Double click the 907/PCI item to set it as the default printer.

Configuring a Port for the 907/PCI Device Driver

Once the CalComp 907/PCI device driver is installed, you need to select a port

or connect to a network.

For a network connection, no special set-up procedures are needed.

From the Printers window, click on the Network button to connect

to a network printer port and complete the dialog boxes.

For a direct connection to your computer, follow the steps below.

Verify that the CalComp 907/PCI Printer/Plotter driver is highlighted in the

Installed Printers List of the Printers window.

Single-click the Configure button. The Printers-Configure window displays.

Highlight the desired Port in the Ports dialog box and single-click it.

Note: Some CalComp pen plotters do not support Centronics parallel data

transmission.

-3-

At this point, you can assign the timeout categories. These categories are:

Device Not Selected - Specifies the amount of time in seconds that the

Print Manager continues to check the printer or plotter to see if it is

online and ready to print. If the printer or plotter is not ready to print

after the specified time has elapsed, a message appears notifying you

that it is offline.

Transmission Retry - Specifies the amount of time in seconds the Print

Manager waits before notifying you that the printer or plotter cannot

accept any more print information.

Note: If a warning message is displayed by Windows indicating that

a transmission timeout has occurred (with options for Retry or Cancel),

you need to specify a larger value, such as 180. If that does not solve

the problem, use NOTEPAD or any other available text editor to edit the

WIN.INI file. Change the line stating "TransmissionRetryTimeout=45"

to reflect a larger value. (A value of 300 would yeild 5 minutes).

Once you have completed the configuration information in the Printers-Port

window, single-click OK. You are returned to the Printers window, where you

can close and save the Printers window by single-clicking OK. You are

returned to the Control Panel.

You may exit the Control Panel normally. If you installed the CalComp 907/PCI

device driver for parallel Centronics port (LPTn), no further settings

are needed. Proceed to the section titled "Configuring the 907/PCI Device

Driver" later in this chapter. If you installed the CalComp 907/PCI device

driver for serial communications ports (COMn), then you may need to

establish communications settings for the port(s). Click on OK in the

Printers window to return to the Control Panel window and perform the

"Serial Communications Settings" procedure.

Serial Communications Settings

To define the settings for a serial communications port, follow the

steps below.

Single-click the Ports icon in the Control Panel window.

The Ports window will be displayed.

Single-click the COMn port for the port settings you wish to specify.

Select "Settings" to set up the communication parameters.

Baud Rate - Single-click the baud rate drop-down menu and then

single-click the needed setting (9600 *).

Data Bits - Single-click the data bits button setting needed (8*).

Parity - Single-click the parity button setting needed (None *).

Stop Bits - Single-click the stop bits button setting needed (1 *).

Flow Control - Single-click the flow control button setting needed (H/W *).

* These are the recommended values, but check to make sure that

they match your configuration.

Verify that the parameters you set up for the COMn port match exactly

what has been set up for your CalComp printer or plotter.

THESE PARAMETERS MUST MATCH EXACTLY.

Single-click OK. This brings you back to the Ports window.

Single-click OK. This brings you back to the Control Panel window.

Double Click on the Printers icon. This brings you to the Printers window.

-4-

Configuring the 907/PCI Device Driver

You are now ready to configure the CalComp 907/PCI device driver to

match your printer or plotter settings.

From the Printers window control panel, single-click on the Configure

button, then single-click the Setup button. The Setup window displays.

Setup Window Configuration Settings

To set up the configuration parameters for your printer or plotter,

you need to identify the following:

Select a printer or plotter

Choose a paper size

Orient the plot

Select the plot type

Assign pens

Some CalComp plotters, such as the color electrostatics, can accept

pen and width assignments at the plotter and override the pen and width

assignments specified in the 907/PCI device driver setup. CalComp plotters

must match the pen color and width assignments of the 907/PCI device driver

setup windows. Make sure this assignment information is identical to the

plotter setup.  This assures you that the correct pen widths and colors are

assigned properly.

NOTE: Pen assignments are only relative if the application software supports

pen width and color assignments.  (for instance ACAD & Cadvance)

You may cancel any time during the procedure by positioning the mouse

cursor over the Cancel dialog box and single-clicking the left mouse

button. This returns you to the Printers-Configure window.

Begin configuring your printer or plotter.

Select a Printer or Plotter.  Highlight the printer or plotter in

the Select Printers list and single-click it.

Choose a Paper Size.  You have several options for paper size.

You may select a pre-defined option or you may choose a custom

paper size. If you need more information about specifying units

of measure, please refer to your Windows manual.  To select a

pre-defined paper size, highlight the paper size and single-click

your choice.  To specify a custom paper size, highlight the

Custom option in the Paper Size list and single-click it. The

Custom (Paper Size) window displays.

NOTE: Actual output plottable area is controlled by the Windows

application software, NOT by the driver or device.

-5-

You can now enter custom sizes. If the width and length dimensions are

different, enter the smaller dimension as the width.  To save the new

parameters and return to the Setup window, single-click OK.

Orient the Plot.  Choose between portrait and landscape in the Setup

window. Choose the landscape orientation if your image is wider than

it is tall. Single-click your choice in the Orientation group box.

Select the Plot Type.  Choose between normal plot output and

mirror plot output in the Setup window. Single-click your choice

in the Plot Type group box.

Assign Pens.  Double-click on the Assign Pens button.

Position the mouse cursor over the pen you want to assign color

and line width, and single-click the left mouse button.

Assign a color for the pen and its line width. Once you have

assigned pens, you can verify your settings by single-clicking

the Summary button. Single-click OK to return to the Pens window.

To save the new parameters and return to the Setup window, single-click OK.

You are now ready to test the configuration of the CalComp 907/PCI

device driver.

Open your software application under Windows and send a file to a

printer or plotter. Refer to Chapter 4 to resolve any problems.

Optional Configuration Settings

At the Setup window, you may change the number of copies (CDCL-compatible

devices only).

To change the number of copies, double-click the Copies box and enter the

number. Valid numbers are 1 through 99. The entered number overwrites the

displayed number.

Advanced Configuration

To set up the advanced configuration parameters for your printer or plotter,

you need to identify the following:

Sync character

Number of sync characters

Enable/disable checksum

End-of-buffer code characters

Job timeout in seconds

Steps per inch

If you need detailed definitions of these parameters, refer to

the plotter or printer user's guide.

Single-click the Configure button to set up the advanced parameters.

Note: The sync character and end-of-buffer character cannot be the same.

Sync Character.  The default is X02, where X denotes the HEX (or

the number in parentheses is the decimal equivalent).

Single-click and then scroll to and enter the sync character HEX number.

Number of sync characters.  The default is one sync character.

Single-click either the one sync character or two sync characters.

Enable/Disable Checksum.  Single-click either the Enable Checksum

or Disable Checksum.

-6-

End-of-Buffer Character.  The default is X03. Scroll and then enter

the correct HEX character. This character cannot  be the same

as the sync character.

Steps per Inch.  This option specifies the number of units per

inch for the specific printer or plotter. If your printer or

plotter does not have different resolutions, this option will be

gray. Single-click your selection.

To save these new parameters and return to the Setup window,

single-click OK.

Once you have selected the parameters in the Setup window, save

and exit the window by single-clicking OK. You are returned to

the Printers-Configure window.

Single-click on OK from the Printers-Configure window. You are

returned to the Printers window.

At the Printers window, single-click the OK button. This

returns you to the Windows Control Panel.

You are now ready to test the configuration of the CalComp 907/PCI

device driver.

Open your software application under Windows and send a file to a

printer or plotter. If your printer or plotter does not generate

output, refer to Chapter 4 to resolve any problems.

Operating the 907/PCI Device Driver

When you are in a Windows application, you can select File/Print

to generate output on your CalComp device. If the "Use Print

Manager" box is selected in the Printers Dialog Box, the data

will be spooled to the Print Manager. It can be monitored and

manipulated through the Print Manger icon in the Main Program Group.

Otherwise, the data will be sent directly to your printer or plotter,

and your application will be unavailable while your image is being

printed or plotted.

-7-

C H A P T E R   3

The 907/PCI Device Driver with Windows 3.1

This chapter provides instructions for installing the CalComp 907/PCI

device driver in Microsoft Windows Version 3.1. Go to Chapter 2

if you are running Windows 3.0. If you are not an experienced

Windows user, please refer to your Microsoft Windows User's Guide

during this installation procedure.

Installing the 907/PCI Driver

Install the CalComp 907/PCI device driver by following the steps below.

Change to the Windows Program Manager window.

Single-click on the Main Program group in the Program Manager window.

Double-click on the Control Panel icon in the Main Panel window.

Double-click on the Printers icon from the Control Panel window.

Single-click on the Add Printer button. The Unlisted Printer is

highlighted.

Single-click the Install button. The Install Driver window

displays.

Insert the CalComp 907/PCI device driver disk into your computer's

disk drive.

The default drive and directory are A:. If you inserted your

diskette into your A: drive, click on OK.   Otherwise, enter the

alternate drive name (such as B:) and click OK.

The CalComp 907/PCI Printer/Plotter driver name appears in the Add

Unlisted or Updated Printer window.

Single-click OK to install the driver. The driver is now installed and

you are returned to the Printers window.

At the Printers window, click on the Connect button to assign an

output port. Assign a serial or parallel port to the installed printer.

You can set the CalComp 907/PCI Printer/Plotter to be the default

printer. To set it as the default printer, highlight the driver

and single-click the Set As Default Printer button.

Configuring a Port for the 907/PCI Device Driver

Once the CalComp 907/PCI device driver is installed, you need to select a

port or connect to a network. The other alternative is to print or plot

to a file.  Print to a file by selecting the file entry.

For a network configuration, no special set-up procedures are

needed for the 907/PCI device driver.

Follow these steps for the 907/PCI configuration.

Single-click the Connect dialog box.

Choose a port and single-click it.

Note: Some CalComp pen plotters do not support Centronics parallel

data transmission.

-8-

At this point, you can assign the timeout categories. These

categories are:

Device Not Selected - Specifies the amount of time in seconds

that the Print Manager continues to check the printer or plotter

to see if it is online and ready to print. If the printer or

plotter is not ready to print after the specified time has

elapsed, a message appears notifying you that it is offline.

Note: You should use the default set-up for the timeout values that

correspond to the port.

Transmission Retry - Specifies the amount of time in seconds the

Print Manager waits before notifying you that the printer or

plotter cannot accept any more print information.

Note: If a warning message is displayed by Windows indicating that a

transmission timeout has occurred (with options for Retry or

Cancel), you need to specify a larger value, such as 180. If that

does not solve the problem, use NOTEPAD or any other available

text editor to edit the WIN.INI file. Change the line stating

TransmissionRetryTimeout=45 to reflect a larger value. (A value of

300 would yeild 5 minutes).

Fast Printing Direct To Port - Allows Windows to bypass the standard DOS

I/O capabilities and access the output ports directly for improved

performance.

If you did not select Fast Printing Direct to Port, no further

settings are needed. Click on OK to return to the Printers window.

If you selected "Fast Printing Direct to Port" and are using a

serial port, then you must provide communications settings.

Serial Communication Settings

To define the settings for a serial communications port, follow

the steps below:

Single-click the Settings dialog box in the Ports window. This

displays the settings for the selected port. You can add or

change any of the settings.

Baud Rate - Single-click the baud rate box and then single-click

the needed setting (9600 *).

Data Bits - Single-click the data bits box and then single-click

the needed setting (8 *).

Parity - Single-click the parity box and then single-click the

needed setting (None *).

Stop Bits -  Single-click the stop bits box and then single-click

the needed setting (1 *).

Flow Control - Single-click the flow control box and then

single-click the needed setting (H/W *).

*  These are the recommended values, but check to make sure that

they match your configuration.

Note: You should not have to use the Advanced features dialog box

unless you need to set up a special Base I/O Port Address or

Interrupt Request Line (IRQ).

Single-click OK. This brings you back to the Connect window.

-9-

Configuring the 907/PCI Device Driver

You are now ready to configure the CalComp 907/PCI device driver to

match your printer or plotter settings.

From the Printers window, single-click on the Configure button.

Setup Window Configuration Settings

To set up the configuration parameters for your printer or

plotter, you need to identify the following:

Select a printer or plotter

Choose a paper size

Orient the plot

Select the plot type

Assign pens

Some CalComp plotters, such as the color electrostatics, can

accept pen and width assignments at the plotter and override the

pen and width assignments specified in the 907/PCI device driver

setup. CalComp plotters must match the pen color and width

assignments of the 907/PCI device driver setup windows. Make sure

this assignment information is identical to the plotter setup.

This assures you that the correct pen widths and colors are

assigned properly.

NOTE: Pen assignments are only relative if the application software supports

pen width and color assignments.  (for instance ACAD & Cadvance)

You may cancel any time during the procedure by positioning the

mouse cursor over the Cancel dialog box and single-click the left

mouse button. This will return you to the Printers window.

Begin configuring your printer or plotter.

Select a Printer or Plotter.  Highlight the printer or plotter in

the Select Printers list and single-click it.

Choose a Paper Size.  You have several options for paper size.

You may select a pre-defined option or you may choose a custom

paper size. If you need more information about specifying units

of measure, please refer to your Windows manual.  To select a

pre-defined paper size, highlight the paper size and single-click

your choice.  To specify a custom paper size, highlight the

Custom option in the Paper Size list and single-click it. The

Custom (Paper Size) window displays.

NOTE: Actual output plottable area is controlled by the Windows

application software, NOT by the driver or device.

You can now enter custom sizes. If the width and length

dimensions are different, enter the smaller dimension as the

width.

To save the new parameters and return to the Setup window,

single-click OK.

-10-

Orient the Plot.  Choose between portrait and landscape in the

Setup window. Use landscape orientation if your image is wider

than it is tall. Single-click your choice in the Orientation

group box.

Select the Plot Type.  Choose between normal plot output and

mirror plot output in the Setup window. Single-click your choice

in the Plot Type group box.

Assign Pens.  Double-click on the Assign Pens button. The Pens

window displays.

Position the mouse cursor over the pen you want to assign color

and line width, and single-click the left mouse button.

Assign a color for the pen and its line width.

Once you have assigned pens, you can verify your settings by

single-clicking the Summary button.

Single-click OK to return to the Pens window.  To save the

new parameters and return to the Setup window, single-click OK.

You are now ready to test the configuration of the CalComp 907/PCI

device driver.

Open your software application under Windows and send a file to a

printer or plotter. If your printer or plotter does not generate

output, refer to Chapter 4 to resolve any problems.

Optional Configuration Settings

At the Setup window, you may change the number of copies

(CDCL-compatible devices only).

To change the number of copies, double-click the Copies box and

enter the number. Valid numbers are 1 through 99. The entered

number overwrites the displayed number.

Advanced Configuration

To set up the advanced configuration parameters for your printer

or plotter, you need to identify the following:

Sync character

Number of sync characters

Enable/disable checksum

End-of-buffer code characters

Job timeout in seconds

Steps per inch

If you need detailed definitions of these parameters refer to the

printer or plotter user's guide.

Single-click the Configure button to set up the advanced

parameters.

-11-

The Configure Window displays.

Note: The sync character and end-of-buffer character cannot be the

same.

Sync Character.  The default is X02, where X denotes the HEX (or

the number in parentheses is the decimal equivalent).

Scroll through, select and then enter the character HEX number.

Number of sync characters.  The default is one sync character.

Single-click either the one sync character or two sync character.

Enable/Disable Checksum.  Single-click either the Enable Checksum

or Disable Checksum.

End-of-Buffer Character.  The default is X03. Single-click and

then enter the HEX character. The end-of-buffer character cannot

be the same as the sync character.

Steps per Inch.  This option specifies the number of units per

inch for the specific printer or plotter. If your printer or

plotter does not have different resolutions, this option will be

gray. Single-click your selection.

Job Timeout in Seconds.  This option sets a time-out value in

seconds for the printer or plotter. Double-click the option and

then enter the number of seconds you want.

NOTE: This feature has not been implemented in Windows, although

it is a planned feature.  Therefore, the Job Timeout is not adjustable.

To save these new parameters and return to the Setup window,

single-click OK.

Once you have selected the parameters in the Setup window, save

and exit the window by single-clicking OK. You are returned to

the Printers window.

At the Printers window, single-click the Close button. This

returns you to the Windows Control Panel.

You are now ready to test the configuration of the CalComp 907/PCI

device driver.

Open your software application under Windows and send a file to a

printer or plotter. Refer to Chapter 4 to resolve any problems.

Operating the 907/PCI Device Driver

When you are in a Windows application, you can select File/Print

to generate output on your CalComp device. If the "Use Print

Manager" box is selected in the Printers dialog box, the data

will be spooled to the Print Manager. It can be monitored and

manipulated through the Print Manager icon in the Main Program

Group.

PRINT MANAGER IS NOT RECOMMENDED BECAUSE OF SPEED CONSIDERATIONS

Otherwise, the data will be sent directly to your printer or

plotter, and your application will be unavailable while your

image is being printed or plotted.

-12-

C H A P T E R   4

Troubleshooting

Common Problems

If you are having trouble using the CalComp 907/PCI device driver,

check the list below for assistance. The most common problems

are:

The setup and configuration parameters between the CalComp 907/PCI

device driver and the CalComp printer or plotter are not matched

exactly. All parameters, such as baud rate, parity, data

bits, etc., must be the same.

A cabling problem exists between the computer and the printer or

plotter. If you are using serial communications, install a

null modem adapter into the cabling loop. All CalComp printers and

plotters use either a straight through RS-232C cable or one with a null

modem adapter attached. From the computer, use a 25-pin to 25-pin male

to female cable, or on the computer side, a 9-pin to 25-pin male to female

cable. See your printer or plotter user's guide for cabling information.

The configuration has incorrect setup parameters. Check to see

that the correct port (such as COM1 or LPT1) has been selected.

Verify that your Windows application supports the media size you

selected.

Download the sample file provided on this installation diskette. If

you are having trouble generating your plots or images, use the

solutions listed below.

Problems:

No output is generated.

Check that both CalComp 907/PCI device driver and printer/plotter have

identical setup parameters. Check your cabling. Refer to the information

on page 4-1 and your CalComp printer/plotter user's guide. Check that

both CalComp 907/PCI device driver and the printer/plotter are connected to

the correct port.

Output is not what you expected.

Compare the paper size selected in the CalComp 907/PCI device driver to the

size specified in your application.

-13-

Cable Information

Below is 907/PCI Windows driver cable table to assist you in identifying your

port and cable options.

CalComp Hardware             Port           Cable Type

Artisan series               RS-232         25-25 straight, 9-25 standard IBM

1040/1070 series             RS-232         25-25 straight, 9-25 standard IBM

Pacesetter/DesignMate        RS232          25-25 straight, 9-25 standard IBM

parallel       IBM Centronics

Classic                      RS232          25-25 straight, 9-25 standard IBM

parallel       IBM Centronics

DrawingMaster                RS-232         25-25 straight, 9-25 standard IBM

parallel       IBM Centronics

Electrostatic plotters       RS-232         25-25 straight, 9-25 standard IBM

parallel       IBM Centronics

All CalComp Printers, RS-232 - 25-25 null modem<R>9-25 standard

IBM and null modem, Centronics parallel

CalComp Customer Service

Please call CalComp Customer Service if you need installation and

usage clarifications. See your CalComp printer or plotter user's

guide for additional information and telephone numbers.

Before calling, please list your complete system (computer,

operating system, printer or plotter model, etc.), the port you

are using, your cable type (including pinouts and connectors,

whether 9-pin or 25-pin, etc.), and the application where the

problem occurred.

CalComp customer service may be reached by calling 1-800-451-7568

Outside of the United States, please contact your local CalComp

Sales office or Distributor.

-14-

APPENDIX A

HOW TO USE THE WINDOWS 907/PCI VECTOR TEST FILES

There are two test files included with the CalComp 907/PCI Vector driver for

Windows. They are;

1) WIN907.TST

2) WIN907.PLT

The purpose of these files is to help you to quickly and easily verify that

your newly installed CalComp Windows Vector driver is working correctly.

Both files produce exactly the same output.

The WIN907.TST file

===================

You must initially install the driver before you can run this test

file. (Refer to the installation information in the Windows 907/PCI driver

documentation supplied by CalComp.)

WIN907/PCI.TST was created from within Microsoft Windows version 3.1.

The application used was Freelance Graphics version 1.0 for Windows.

After being created in Freelance Graphics, the file was "cut" and

"pasted" into Windows Write. Windows Write is supplied as a standard

part of the Windows 3.1 package. It is accessed through the "Accessories"

Program Group.

The WIN907.TST file is to be called up into the Windows Write application

and printed or plotted on your particular CalComp device using the CalComp

Windows Vector driver. The WIN907.TST file enables you to see, on the

screen, from within a standard Windows 3.1 application, exactly what you

will be printing.

Configure the driver as follows to plot or print this file:

1) Select your CalComp model

2) Select paper size of "A" (letter size, 8.5 x 11, portrait)

3) Click-on "Configuration" and select the following parameters:

- Sync Code Value = 02

- Number of Syncs =  1

- End of Buffer   = 03

- Checksum        = disabled

- Stepsize        = Match with your selected model;

- If ColorMaster Plus VRC or ColorView (5913)

use 300 steps and 1/1 numerator and denominator

- If PlotMaster or ColorView (5912) use 200 steps

and 1/1 numerator and denominator

- Electrostatics and DrawingMasters may be used at

either 400 or 2032

- All pen plotters automatically set for 2032

- 15 -

4) Click-on "OK", then "OK" again from the first window and then

click-on "Close".

Be sure that your CalComp printer or plotter matches the above configuration

parameters. For information regarding the configuration of your CalComp

device please refer to your User's Guide.

Now, with the WIN907.TST file "opened" in Windows Write, click-on "File" and

then "Print". Finally, click-on "OK" from the "Print" window.

The WIN907.PLT file

===================

WIN907.PLT is a plotfile created with the CalComp Windows 907 Vector driver.

It was created from the WIN907.TST file using the CalComp driver configured

to print or plot to a file.

To run this test file, your CalComp printer or plotter must be configured

with the parameters listed above (in the first section which covered the

WIN907.TST file). The stepsize for this file is 2032. To test this file

on printers, numerator and denominator should be set as follows:

Numerator     Denominator

=========     ===========

ColorMaster Plus VRC and ColorView 5913  -       2430         16460

PlotMaster and ColorView 5912  - - - - - -        125          1397

The WIN907.PLT plotfile does include the appropriate CDCL configuration

commands as part its header. If your CalComp device supports CDCL, it will

be automatically configured with the parameters listed above (including

stepsize). Otherwise, refer to the instructions for CalComp device port

configuration found in your User's Guide.

Although this file does not test the CalComp Windows Vector driver itself,

it does serve as a verification of the printer or plotter setup as well

as to the correct hardware connection. It also provides a ready sample or

yardstick of output quality and speed from a plot created in a Windows 3.1

application using the CalComp 907/PCI Windows Vector driver. (i.e., how long

does it take to plot the file and what does it look like)

To print or plot this file, proceed with following steps:

1) At the DOS prompt, configure your computer's port with the DOS

MODE command for the specific computer port you have connected

to your CalComp device. (i.e. parallel or serial).

examples:  C:\>MODE LPT1:,,P  <RETURN>

C:\>MODE COM1:9600,N,8,1,P  <RETURN>

Substitute the "P" with a "B" if your DOS version is 4.x.

- 16 -

If you are going to use the serial port, be sure the CalComp

device is set for 9600,N,8,1 and hardware handshake (also called

direct control or CTS).

2) Use the DOS COPY command to send this file down to the CalComp

device.

examples:  C:\>COPY WIN907.PLT COM1  <RETURN>

C:\>COPY WIN907.PLT LPT1  <RETURN>

- 17 -