Yamaha TG100

Serial Device Driver

Version 1.0

Overview:

---------

The Yamaha Serial Device Driver allows Windows 3.1 users to attach the

Yamaha TG100 to their computer's serial port.  Follow the direction in the

manual concerning cabling and the appropriate switch setting on the rear panel.

Note:

-----

This device driver is only compatible with Windows 3.1.

Hardware Installation:

---------------------

Connect the TG100 to the computer using the cable recommended in the

Reference Manual and set the switch on the back of the TG100 as described

in the Reference Manual.

Note that the driver will not load if the TG100 is not connected to the

specified COM port and/or is not powered on when Windows loads.  The driver

verifies the existence of the TG100 before fully initializing.

Standard mode users should also be aware that due to the way the Windows

COM drivers are written, it is impossible to place the TG100 on a shared

interrupt port.  In ISA bus machines, if there are more than two COM ports,

ports 1 and 3 share interrupt 4 and ports 2 and 4 share interrupt 3.  On MCA

machines, port 1 uses interrupt 4 and all other ports share interrupt 3.

On EISA machines either sharing scheme is possible depending on the type

of COM boards used.

Therefore, standard mode users should set the TG100 on COM port 1 to assure

compatibility and, on ISA or some EISA machines (depending on the type of

COM board) disable COM port 3.

Software Installation:

---------------------

To install the device driver, simply bring up the Windows Control Panel

application (typically found in Main group) and select the Drivers icon.

Press the Add button and select the Unlisted or Updated Driver entry from

the list box and press OK.  Respond to Windows prompt concerning the location

of the driver and press OK.  The driver will then display a list of COM ports

available for the TG100.  Select the appropriate COM port and press OK.

Windows will then ask if you wish to restart Windows.  You must restart

Windows for the installation to be completed.

If you install the driver while running in Standard mode on a 386 and later

want to run in Enhanced mode, it will be necessary to re-install the driver

to cause Windows to load the 386 driver files.  If you install the driver in

Enhanced mode first and later want to run in Standard mode no actions are

necessary:  Windows will have already loaded all of the necessary files.

Enhanced versus Standard Mode:

------------------------------

This driver is capable of support Windows running in both Enhanced mode and

Standard mode.  The Standard mode capability is primarily provided for users

of 80286 based systems and/or users that have applications that only run in

Standard mode.

This driver has been tested in Standard mode on 80286 machines with clock

speeds starting at 12MHz and in Enhanced Mode on 80386SX machines with

clock speeds starting at 16 MHz.

SYSTEM.INI Settings:

--------------------

The following values can be defined under the [TG100.DRV] section of your

SYSTEM.INI file:

[SysExWait=nn]

Because of the interrupt latency in Windows, it is impossible for slow

machines to keep up with a SysEx dump a byte at a time.  For this reason,

when the device driver recognizes that the TG100 is sending a SysEx dump, it

will hold on to the COM port until the completion of all pieces of a SysEx

dump.  Upon recognizing the completion of the SysEx dump the driver will pass

the SysEx data to the MIDI application.  Note that the entire SysEx dump must

first be received  by the driver before data is passed to the MIDI

application.

By default the device driver will wait 40 milliseconds to determine that a

SysEx dump is complete.  If no more data is received in this time interval

the data received will then be forwarded to the MIDI application.

You can change this interval by setting the SysExWait setting in the

TG100.DRV section of the SYSTEM.INI file.  The value given is in

milliseconds: i.e. SysExWait=100 would cause the driver to wait 100

milliseconds instead of the default 40 milliseconds.

Users should consider adjusting this value if they are having problems

getting complete SysEx dumps.

[COMPort=n]

This specifies which COM port the TG100 is attached to and must appear

in the TG100.DRV section of the SYSTEM.INI file.

[BufferSize=nnnn]

This specifies the number of bytes that should be made available to handling

SysEx dumps.  This can be any value from 1 to 65535: the default value is

10240.

Users should consider adjusting this value if they plan on receiving large

SysEx dumps from MIDI devices attached to the TG100.

[MIDIINPersistence=nn]

This specifies the number of bytes the driver will read from the TG100 before

returning control to Windows (except in the case of SysEx dumps with are read

until the dump time interval specified by the SysExWait expires).  The default

value is 50 which should be adequate for most installations.