Overview

This standard represents the specification of the digitalvideo device class for the Media Control Interface (MCI), or DV MCI. It applies to hardware and software manufacturers of digital video products as well as ISVs creating applications using the digital video products. This specification was developed in collaboration with OEMs to ensure that it reflects the inherent characteristics of digital video hardware. However, this specification is not limited to a single manufacturer’s hardware and is expected to be applicable to a wide range of hardware implementations.

This document is not self-contained. In particular, it assumes you have a general familiarity with the documentation for Windows with multimedia extensions, as well as a general familiarity with Windows programming. This document contains the following sections:

·	The overview summarizes the documentation and lists available resources.

·	Chapter 1, “Digital Video Commands for MCI,” provides an overview of the characteristics of a digital-video device and introduces the digital video command set.

·	Chapter 2, “Digital Video Command Strings,” lists the MCI command strings available for digital video devices.

·	Chapter 3, “Digital Video Command Messages,” lists the MCI command messages available for digital video devices.

·	Appendix A, “Archaic String Commands,” lists archaic string commands.

·	Appendix B, “Digital Video MCI Command String Syntax,” summarizes the digital-video commands and their proper usage for digital video devices that use files.

·	Appendix C, “Video Overlay MCI Command String Syntax,” summarizes the digital-video commands and their proper usage for video overlay devices.

The major purpose of this document is to provide clear guidelines to any developer of a digital-video device driver as to how to implement specific features. Because a feature is described herein does not imply that the developer of a compliant digital-video device driver must support that feature. All digital-video device drivers should use the following guidelines:

(a) A digital-video device driver that does not support a specific aspect of the specification should return an error code of MCIERR_UNSUPPORTED_FUNCTION.

(b) A digital-video device driver that supports functionality described by this specification should use those aspects of the specification relevant to performing that functionality.

(c) All command strings and MCI messages should support the test flag. This lets an application check whether a device driver supports a proposed action.

This standard presents new and updated information for the MCI command set. This document relies on the information previously published for MCI commands and programming. The following sources contain this information:

1.	The Multimedia Development Kit (MDK) 1.0 Programmer’s Reference

2.	The Windows 3.1 Software Development Kit (SDK) Multimedia Programmer’s Reference

3.	The Windows 3.1 Software Development Kit (SDK) Multimedia Programmer’s Guide

4.	The Multimedia Programmer’s Reference from Microsoft Press

5.	The Multimedia Programmer’s Workbook from Microsoft Press

6.	The Windows 3.1 Device Development Kit (DDK) Multimedia Device Adaptation Guide (this guide is needed only by device driver developers)

# Where to Look for Information

Current versions of this document as well as other technical updates and technical notes and sample code are available from the following:

1.	Compuserve WINSDK forum.

2.	Microsoft Multimedia BBS at (206) 936-4082 in the files library in the specs section in the DVMCI1.ZIP file. Technical notes are available in the technote section. BBS modem settings are 9600 baud, no parity, 8 data bits, 1 stop bit.

3.	Via anonymous FTP on ftp.uu.net in the vendors\microsoft\multimedia directory. Sample code is in the samples directory and technical notes are in the technote directory.

4.	Current versions of any document can be ordered (currently free) by calling (206) 936-8644.

# Versions of this Document

This document is continually being updated and expanded. Eventually the information presented in this document will be included in the standard references from Microsoft, such as the Multimedia Programmer’s Reference from Microsoft Press.

When referring to standards defined in this document, please refer to the data and version number printed on the cover page.

Versions of this document with the same version number are just expanded editions of the same information.

# If You Have Questions

If you have questions, requests, or problems with this technical update, you should send your question to the following address:

Microsoft Corp.
Multimedia Product Management
One Microsoft Way
Redmond, WA 98052-6399

FAX (206) 93MSFAX

Chapter 1
Digital Video Commands for MCI

# General and Syntactic Comments

In the following text, a bold font indicates ASCII strings that must be typed precisely as shown (except case is ignored). An italic font indicates information supplied by the user.

# Device Driver Instances

Like other MCI devices, a digital-video device is opened with the open command. If necessary, this loads the device driver into memory and locates any media resource (file) specified. Each open command creates an instance of the device driver. The device name acts as a handle to the instance of the device driver created. A device driver instance (or more simply, device instance) is simply the collection of status information that describes the net result of all the operations that have been performed using a single device name. Commands use the device name so that they can be issued in a specific context.

The digitalvideo device class lets applications open multiple device instances simultaneously. Each instance has its own context. For example, an application might have two files playing simultaneously in two different windows. Alternatively, an application can open the same file twice and position it differently in two device instances. While the same file can be used by two different instances, each instance must have been opened with a unique device name. (MCI returns an error when an application attempts a second open with the same device name.)

MCI also supports shared device instances. When the same file is opened with two or more different device names, and the shareable flag is used, then a single file context is shared by these device instances. That is, changing the position of the file affects its position in all the shared device instances. While a digital video device must check for the shareable flag when it is opened, it might refuse to accept such an open. Devices that support shareable could allow the same presentation source to be displayed in several different windows in different positions and sizes on the display.

Hardware or software constraints of digital-video devices can limit the number of simultaneously open device instances, or what they can do simultaneously. Application programmers can determine the device constraints by using the capability, info version, and info product commands, and by using the test flag with a command.

# Data Flow Model

The data flow model illustration indicates the principal paths for data flow within a single instance of a digital-video device driver.

The data sources are on the left, and the data destinations are on the right. The input data might be digital data coming from a computer file that is decompressed and presented to the user with the monitor and speakers. Alternately, the input data might be analog data that is digitized, and then compressed and stored in a computer file, and routed for display on a monitor and speakers.

The blocks in the diagram are controlled with the digital-video MCI commands. The following table lists some of the commands that control the functions represented by the blocks:

| Block | Typical commands that control it |
| --- | --- |
| Input Switching and Effects | setaudio input volume setvideo source |
| Compress | setvideo algorithm |
| File I/O | save record |
| Workspace | play stop |
| Clipboard | cut copy |
| Monitor Switch | monitor |
| Audio Output Effects | setaudio output treble |
| Video Output | setvideo output brightness |
| Frame Buffer | put frame freeze |
| Capture | capture setvideo still algorithm. |

The Decompress block is drawn twice to illustrate the monitor input post and monitor file paths but is not necessarily implemented as two distinct hardware components.

# Multimedia Data Types

The digitalvideo device class handles three types of data: audio, video, and still images. Many commands are specified to be data type neutral. For example, when you play or pause a file, you use the same command whether the file contains both audio and video data, audio-only data, or video-only data.

To maintain a uniform conceptual model, the device class treats a still image as a limiting case of a motion video file with only a single image. When you play a motion video file to its end, it automatically pauses or stops once the last frame is displayed. So, when you play a still image file, it pauses or stops after showing its last and only image. You can, for example, seek to the start of a still image file, and play or step it again.

# Video Overlay Devices

Much of the digital-video command set also applies to video overlay devices. The command set enhances the video overlay command set included in the Windows 3.1 Software Development Kit. The enhancements include the ability to control audio and video effects. Video overlay devices are distinguished from digital video devices in that they have no workspace, they cannot play files, and they cannot record audio or video.

A separate syntax summary for overlay devices is provided in an appendex. Only the commands listed in this summary are recognized by overlay devices. Other comments in this document indicate slight differences between overlay and digital-video devices for individual commands.

Although not required, a single driver can provide both overlay and digital video functionality. A combination overlay and digital video driver must be installed with two separate entries in the [mci] section of the system.ini file. One entry uses the identifier “overlay” with an optional digit suffix. This lets the device detect that it is being opened as an overlay-only device. The other entry lets the device be opened as a digital-video device. As a digital-video device, it should use a documented identifier that allows applications to select the device by name.

For example, if a device supports a file format called “mympeg,” it might document the digital-video identifier as “mympeg_video” and install the following entries in the [mci] section of the system.ini file:

[mci]
overlay=mympeg.drv
mympeg_video=mympeg.drv

An application uses the device for overlay functions by opening it as an overlay device: open overlay

An application uses the device for digital-video functions by opening it as a digital-video device. For example, it could use the command open name.dat type mympeg_video

The driver detects whether the device is opened for overlay-only functions or for digital-video functions by checking the wType field of the data structure sent with the MCI_OPEN_DRIVER message. If wType is set to MCI_DEVTYPE_OVERLAY, the device should only respond to overlay commands and reject an attempt to open the device as a compound device.

If wType is set to MCI_DEVTYPE_DIGITAL_VIDEO or MCI_DEVTYPE_OTHER, the device should function as a digital-video device. If opened as a simple device, the device should respond only to capability and info commands. If opened as a compound device, the device responds to the entire command set.

# Compression Formats

Digital-video devices can compress audio and video data in real time. In general, compression involves tradeoffs between the data rate and different kinds of compression artifacts. For a given compression algorithm, there are typically many parameters that reflect the different tradeoffs possible. Unfortunately, the myriad of algorithmic possibilities makes the direct control of all the tunable parameters of an algorithm impractical. (This is especially true for the MCI string-command interface.) As an extreme example, the JPEG algorithm has 192 tunable parameters, in the form of three 8-by-8 quantization matrices.

Parameter descriptors deal with this algorithmic richness in a reasonably device-independent manner. At the procedural level, the parameter descriptor uses a data structure that contains a specific set of values for all the tunable parameters needed by a specific compression algorithm. This data structure is given a symbolic name so that the associated descriptor can be conveniently referenced at the application level.

Every digital-video device that supports recording supports descriptors associated with the descriptor names low, medium, and high. For example, the set device name video quality low command is valid for all digital-video devices. The precise definition of what constitutes low quality video is device specific. Individual devices can also create other predefined descriptor names. (Devices with unique descriptor names provide the documentation for these descriptors.) Also, new descriptors can be created with the quality command.

Some devices support several different compression algorithms. For these devices, applications can select the audio or video algorithm with the MCI commands. These commands can also describe major variations. For example, a program capable of compressing into CD-XA audio formats might accept a command such as setaudio device name algorithm cdxa-b. The acceptable parameters for the argument of the algorithm flag would be device-specific.

# The Frame Directory

Variable-length encoding and interframe encoding make it difficult to seek to an arbitrary frame in a digital video file. Variable-length encoding means the number of bits in each image of a sequence is not constant. This makes it impossible for a device to algorithmically locate the compressed data for a specified frame in the file. Interframe encoding means the decompression of an image might require prior decompression of one or more nearby images. This implies that, even if you know where data for a specified frame begins, decompression of the specified frame can take a long time.

For algorithms and file definitions that support uniquely recognizable image start codes (such as the emerging MPEG standard), a device might deal with variable length data by estimating the proper seek position, and then scanning the file until it finds recognizable data. In general, however, having auxiliary data available that points directly to important places in the file is a more useful way of locating frames in a file.

The auxiliary data forms the frame directory. For example, a device might build a frame directory after the data is compressed. By analyzing the data in the file, the device could create a table listing the exact location of the beginning of every frame. Alternately, the frame table could at least include the location of the beginning of every key frame image. (Key frame images are those that do not require data from a previous frame to be decompressed.) Frame directories are device specific and each device might have a unique definition. Regardless of when the frame directory is generated and how it is associated with the motion video file, the frame directory enables relatively rapid access to a specified frame.

When real-time motion video data is captured, it might not have a frame directory. Such a frame directory might be added after the data is safely stored on disk. This method is reasonable, especially for a very large file, since generating a frame directory could significantly increase the processing time while storing real-time motion video to disk. Different devices might choose different approaches in building a frame directory.

# Finding Precise Frames

A frame directory can help locate data in a compressed file because it can indicate a suitable key frame image to act as a starting point. However, there still might be a significant delay until a requested image is displayed. For example, a file might have a large number of interframe encoded images the device must decode before it can display them. In an extreme case, a motion video file might be entirely interframe encoded, with the exception of its initial image.

To handle delays associated with interframe encoding, the digital video command set includes the seek exactly flag for the set command (for example, set device name seek exactly on). With seek exactly enabled, a seek command, or a play command with the from flag, will go to the precise frame specified by the command. This can cause a significant delay as the device locates and decompresses the frame. With seek exactly disabled, the file will be positioned to a (optimally) nearby, previous key frame image. An application can determine the approximate location of the nearest preceding key frame image by using the status device name reference command. This is only approximate, because a frame directory might be required to efficiently determine the precise location of this image. The exact location can be determined by using the status device name position command after using the seek command. Because implementation of the seek exactly mode is tricky, especially with out of order algorithms such as MPEG, the default is device dependent.

# Frame Numbering and Time Formats

All digital-video devices must support the frames time format. A frame is a complete image, typically intended for playback at 30 frames per second. Visual material originating on film will have a natural frame rate of 24 frames per second. PAL-originated material will have a frame rate of 25 frames per second.

With a time format of frames, there is a potential “off-by-one” ambiguity. This section provides guidelines for avoiding such ambiguities. To simplify the discussion, assume that seek exactly is enabled. The following guidelines define frame numbering:

·	Frames are numbered from an arbitrary origin. (An application can determine the origin with status start position.)

·	The initial state of the workspace with new contents is stopped, positioned at status start position. This occurs after an open or load operation, and when a monitor file is sent after recording is completed.

·	A paused device instance returns the number of the frame currently in the frame buffer in response to status position.

·	When the seek to and play to commands complete, the current position and image shown is specified in the to parameter. Some devices will be stopped, some devices will be paused.

·	For the play to command, all frames up to and including the to position are shown but no audio associated with the to position is played. For example, the command play from 3 to 5 will show three frames of video but the duration of audio played is equivalent to two frames.

·	A frame number one greater than the last frame in the file is a valid argument for cue output to, play to, or seek to. This frame is referred to as the “final valid frame”. Values larger than this cause a range error. When the device reaches this position, it displays the same image associated with the last frame in the file. This final valid frame number equals status start position plus status length.

For example, if a file contains ten frames and the first is frame one, frames one through eleven are valid frames. Frame eleven shows the same image as frame ten.

·	The default final position for a play command without a specified end position, or for a seek to end command, is the final valid frame.

·	A command specifying a frame prior to the status start position causes a range error.

·	On a stop command, the status position of a device instance does not change.

·	When seek exactly is off, a play command without a from position will not necessarily start from the current position, because such a play has an implied initial seek, which might find an earlier key frame image.

·	When seek exactly is off, and a file contains interframe encoded images, if a seek command specifies an interframe encoded image, the actual frame reached by the device will be an key frame image prior to the specified frame. The status position command will return the actual frame reached.

# Copy, Cut, Delete, Paste, and Undo

The copy, cut, delete, paste, and undo commands let the user edit digital video data. A device can use the Windows Clipboard or an unrelated private clipboard to perform these functions.

Regardless of how the device implements these functions, the copy and cut operations should put a device-independent bitmap (DIB) representing the first frame of the selected sequence onto the Windows Clipboard.

The copy, cut, delete, and paste commands are all influenced by the state set with seek exactly. When seek exactly is off, these commands might not use the exact frames specified for them. For example, the actual sequence of frames deleted by cut or delete might not be the exact frames specified for the command. Similarly, frames copied to the Clipboard by copy or cut might not be exactly those specified. For the paste command, the position frame used for inserting data might not be exactly what is specified.

When seek exactly is on, a device might be unable to use the specified positions if they are not key frame images. In this case, the driver should return MCIERR_DGV_BAD_CLIPBOARD_RANGE as the error code.

# The Frame Buffer

Some digital-video devices support video-digitizer hardware. The digitalvideo device class creates an abstract model of this hardware as a “frame buffer” and a “lock mask.” The frame buffer is a region of device memory that can hold a single full-screen image. You can determine the size of the frame buffer with the where frame max command.

The lock mask is an array with one bit for each pixel of the frame buffer. You can modify the lock mask with the freeze and unfreeze commands. These commands let you specify rectangular regions where the lock mask bits should be turned on or off. A sequence of such commands can yield arbitrarily complex patterns of locked bits. Devices that do not have hardware support for a lock mask should reject commands that specify a rectangle on a freeze or unfreeze command. (You can use capability can lock to determine if a device supports a lock mask.)

The following illustration shows the model used for digital-video devices:

µ §

In the digital-video model, the frame buffer is off screen. You specify what is displayed with the put source and put destination commands. The put source command specifies a source rectangle in the frame buffer. The put destination command specifies a destination rectangle relative to the client window. For display, the device scales the visual content of the source rectangle to fit the destination rectangle. By default, the source rectangle is the entire image in the frame buffer, and the destination rectangle is the entire client area of the current window. That is, if you have never used a put command, the device scales the entire image to fit in the current window.

The put command also lets you control how the device places the digitized information into the frame buffer. The put video rectangle specifies the region of the external input’s active scan area used when the device digitizes the image. The video rectangle defaults to the entire external input (that is, full screen). The put frame rectangle specifies the region within the frame buffer where the device places the digitized image. This rectangle defaults to the full size of the frame buffer. If you use the default regions for both the video and frame rectangles, the digitizer stores the full-screen video in the full-screen frame buffer. Full-size video and frame rectangles are commonly used when the hardware can resize images while copying from the frame buffer to the destination rectangle. (Devices that can resize images return true to the capability can stretch command.)

The hardware might also support stretching while digitizing. (These devices return true for the capability can stretch input command.) When using such hardware, an application might use the put frame rectangle to have the hardware scale the image to the right size as it places the image in the frame buffer.

Some devices might be incapable of supporting an abstract frame buffer for each device-driver instance. These devices should treat the digitizer hardware as a resource that is typically only accessed by a single device-driver instance. Multiple instances can access a common frame buffer, but only by specifying the shareable flag with the open command.

The frame buffer concept originated in support of the overlay device class. Hence, its description uses the assumption that pixels stored in the frame buffer come from the external input. However, the concept of a frame buffer and a lock mask is also useful during playback of motion video and images. Therefore, video playback supports commands such as put, freeze, and unfreeze.

# Wait, Notify, and Test

MCI supports the command flags wait and notify on every command. While these flags are not explicitly listed with the commands, they are always accepted. Wait and notify implement two different strategies for handling with delays associated with time consuming commands.

The digital-video MCI command set adds the test flag that you can use with all commands except open and close. When this flag is used, the device checks if it can execute the associated command. If the device driver determines that it can execute the command, it exits without processing the command and without returning an error. If an application supplies a return buffer for a command that returns data, the device returns 0 for integer return values and an empty string for string return values. If the device cannot handle the command, it returns a suitable error.

If a device cannot execute the command because the requested function is not supported, the device should return MCIERR_UNSUPPORTED_FUNCTION for the error. If the device supports a command but it cannot execute it because the device is not in a suitable state, the device should return the error MCIERR_NONAPPLICABLE_FUNCTION.

Applications can use the test flag to determine and adapt to the subtle differences among the digital-video devices. For example, some devices do not disable audio with setaudio off when recording while other devices can disable it. An application might also use test to determine which capabilities are temporarily not available and dim buttons and menu entries accordingly.

To improve the transition to future versions of MCI that include more extensive use of test, the digitalvideo device class also adds the capability can test command.

# Glossary of Terms Used for Digital Video Devices

The following terms are used in describing the MCI commands for digital video devices:

Client Rectangle: The rectangular area used by the window to render motion video or still images.

Device Name: The name used to identify the device after it is opened. Typically it is the filename corresponding to a digital-video resource or an alias.

Device Specific: An aspect of the command set left to the discretion of the device developer. When a feature is outside the definition of the command set, it is referred to as device specific.

Device Instance: Each open command creates a device-driver instance and associates a device name with it. The device name is used in subsequent commands to reference the device-driver instance. The state of a device-driver instance is the net effect of all the commands sent to a device with a given device name. The device driver instance, and all state information associated with it, ceases to exist after the device is closed.

External Input: A digital-video device might be connected to a source of analog video and/or audio data. The result of digitizing this data is called the external input.

Frame Buffer: The frame buffer is an off-screen area containing the pixels that can be copied to the display. Commands in the digital video command set affect which pixels are updated as video is played or digitized, and how pixels are copied from the frame buffer to the visible display.

Frame Directory: Digital video encoding uses techniques such as interframe encoding and variable length encoding that can make starting playback at an arbitrary frame difficult. For algorithms and file structures that do not support uniquely identifiable image-start codes, auxiliary information associated with the file can make random access more efficient. When such auxiliary information is provided, it is called a frame directory. A possible method of generating the frame directory might involve post processing the media file. The format of the frame directory and how it is associated with the media file is device dependent. Whether or not a file has an associated frame directory affects the semantics of the various commands. These effects are described in the command overview and with the affected commands.

Presentation Source: The sound and pictures presented to the user can come from either the workspace or the external input. The presentation source is the selected origin for the sound and pictures. The user presentation is also conditioned by whether audio and/or video output have been enabled, which pixels of the frame buffer are enabled for update (if any), and what (if anything) is currently in the workspace or connected to the external input.

Track: The media can be divided into a series of tracks. These function like chapters in a book to break up a long file into sections. Most digital-video devices do not support tracks. For these drivers, the result of status queries about track information assume the media consists of exactly one track.

Unsaved Data: After recording, newly created data exists in the workspace. This data is discarded if commands such as close, load, record, and reserve are used before the data is saved. In some circumstances it is appropriate to warn the user before performing these commands. The device driver should keep track of whether there is unsaved data in the workspace and let the application detect this situation by using the status device name unsaved command.

Window: Video or images are displayed within the client area of a window. If the application does not specify a window, or specifies the default window, the device driver selects the window appearance and how it responds to user actions. Alternatively, an application might create its own window, and pass its handle to the device driver. In this case the application controls the appearance of the window and how it responds to user actions. The device driver can subclass this window and provide additional or alternative actions to those specified by the application’s window procedure.

Workspace: Every device instance has an associated workspace containing a working copy of the multimedia data. Commands such as record, load, and paste place new multimedia data in the workspace. (Record compresses analog video and/or audio data into the workspace.) Commands such as play present the data in the workspace to the user.