NATIONAL SEMICONDUCTOR CORPORATION

"TROPIC"  TokenRing  NDIS Device Driver

The NSC Tropic Tokenring adapter is a standard ISA BUS 16 bit network card

compatible with Tokenring topology networks. The Driver is designed to operate

under Microsoft LANMAN 2.1 for OS2 1.3 and DOS 5.0 operating systems.

IBM Lan Server running under OS2 2.0 as well as Microsoft Windows for Workgroups

is also supported. These notes cover certain details of operation for building the

source code as well as installing and using the driver software on the above

mentioned systems.

Source build notes:

-------------------

Source code for the driver is included for those developers interested in

modification for their own purposes or to see how a driver is structured

for the Tropic Tokenring architecture.

BAT and CMD files are included to build the NSCTOK driver.  These command

files set the environment variables used by the MAKEFILE. The standard

Microsoft Toolkit files are assumed for building the driver.

ENVIRONMENT VARIABLES:

INCLUDE  - Include file path

LIB      - Library file path

TARGETOS - Target Operating System (DOS | OS2)

VERSION  - Version being built (PRODUCT | DEV)

LISTINGS - Listing control (YES | NO)

(Note, the DEV version conditional assembly statements have been removed,

as development has been completed, PRODUCT is now the only version

produced during the make.)

COMMAND FILES

MKALL    - Makes both OS2 and DOS versions for both DEV and PRODUCT

MKDOS    - Makes DOS (accepts DEV or PRODUCT as command line argument)

MKOS2    - Makes OS2 (accepts DEV or PRODUCT as command line argument)

* All builds should be run from this directory.

Installation Disk Notes:

------------------------

OS2 and DOS DIRECTORY CONTENTS

There are three different directory structures on the disk with the contents

needed for each of the operating systems Lan manager products. The autosetup

features of each Lan management product will work with the installation disk.

When the Lan Management installation procedure prompts for the user setup

disk, insert the NSC Tropic Installation disk and proceed as requested.

Lanman.dos = The directory for DOS 5.0.

Lanman.os2 = The directory for Microsoft OS2 1.3

IBMLAN = The directory for IBM OS2 2.0 Lan Server.

All these directorys contain the special NIF Files and a sample of

the Protocol.ini files needed to install the driver for each system.

Windows for Workgroups is supported by the OEMSETUP.INF file in the root

of the install disk, as well as the PROTOCOL.INI fragment and the actual

driver itself; NSCTOK.DOS.

OPERATIONAL COMMENTS: DOS

The NSC Tropic Token Ring adapter and driver have been tested with

several EMS Memory management programs. QEMM works fine in that

it will detect if a Token Ring style adapter of any type is in the system

and will not relocate the shared memory segment. (Nice feature...)

Microsoft's own EMM386 needs the shared memory area excluded with

the x = parameter set accordingly or else the adapter and driver will not

operate. Qualitas likewise requires an exclusion parameter set. These

last two memory managers do not detect automatically as does QEMM. See

the user manual for the proper procedure to exclude the shared memory

space of the driver. Default shared memory area is D800 Hex.

OPERATIONAL COMMENTS: OS2

The OS2 2.0 Lan Server product from IBM does not currently support the

automatic installation feature for an external vendor device driver disk.

For OS2 2.0 IBM Lan Server installations, the user should manually move the

files needed to the proper locations. Additionally, the config.sys and

protocol.ini files will need to be edited to reflect the correct information

for the NSC driver. The procedure to install is as follows:

1. Install any driver from the selection list (IBMTOK works fine).

2. Reboot after the Lan Server is installed under OS2.

3. Copy the NSCTOK.OS2 driver from the IBMLAN directory of the Tropic

installation disk to the IBMCOM\MACS directory that was created

during the IBM Lan Server installation process.

4. Copy the NSCTOK.NIF file from the IBMLAN directory of the Tropic

installation disk to the IBMCOM\MACS directory.

5. Edit the Config.sys file to reflect the name of the NSC TROPIC

device driver in the section that calls out the lan adapter

driver in the Lan Server declaration statements. For example,

if the driver installed initially was the IBM Token Ring driver

the following line should be changed:

DEVICE=C:\IBMCOM\MACS\IBMTOK.OS2           (original line)

DEVICE=C:\IBMCOM\MACS\NSCTOK.OS2 	      (modified line)

6. Edit the Protocol.ini file in the IBMCOM directory to include the

information for NSCTOK as listed in the IBMLAN directory of the

NSC Tropic TokenRing installation disk. The Protocol.ini sample

there has the typical default information.

7. Reboot the system with the new Tropic Tokenring card installed

and everything should bind up and run correctly with the NSC card.