Below was via TCB BBS:

Sum Kunt writes:

Today I was busy calling BBS and trying out the 0014 number, when i tried to

Box it didn't work.

So I had a word to my dad and asked him how it would work he didn't know.

But he came up with a story.. When I was a boy my friends used to quickly

tap out there number with the on/off buttun. He said he could never get it to

work. That was with the hold fashion phones with the lever.

But with a curious mind I tried it, well it should work because they are

basically the same thing just a diferent casing and a few other things.

It does work on all phones. But you must be really quick with the fingers.

Its kinda like morse code but you just tap out the numbers in full one by one

Its best to experiment do not do a set out number straight away. Get to

understand it.

A bit of a warning It's HARD to do it by the phone so make up a device to aid

you along.

You will probablly get a lot of Telescum anoucments saying that the number

is not availible atc.................

HAVE FUN..

Actually it only works on Touch phones to my knowlege at the momoment

---------- Addition by Merlin, 30-05-94

don't know who wrote this, but...  what's going on is the tapping of

the hook switch is simulating pulse dialing. Pulse dialing is done by

very quickly and precisely breaking and re-making the connection

between the phone and the exchange. The exchange detects this as a

change in the current flow on the line and converts it to the number.

The hookswitch does exactly the same thing, but the exchange is

clever eonugh to know wether you're hanging up or dialing, by the

timing - that's indirectly why you have to hang up for about 1/2 a

sec or more before picking up again in order to get a dialtone again.

The accuracy of the timing is obviously critical, otherwise the

exchange gets the number wrong. The old dials were mechancly

engineered with weights and such to be quite accurate.

By tapping the hookswitch you can simulate the dialpulses, and if

you're very good, you'll get the right number (!). Personally, I

suggest testing with 199 or something so as to not piss people off

too much. Oh, and it works on ALL phones...   simply because all

phones have a hookswitch and all exchanges support pulse dialing

(well, in Oz, anyway..!)

------------------------------------------------------------------------------

This file bought to you via the Communications Barrier BBS

"Knowledge is our business"

+61-3-585-1112, +61-3-583-6119. -> 14.4k, 24hrs, all welcome.

------------------------------------------------------------------------------