DOCUMENT FILE FOR PADS2PCB.EXE  Version 1.00

(c) 1991  PROTEL TECHNOLOGY PTY LTD

1. How to Run Pads2Pcb

----------------------

This program translates PADS-PCB ASCII format files to Autotrax format files.

The following files are included with this program:

PADS2PCB.EXE   the file translator program

PADS2PCB.MNU   menu/strings file for PADS2PCB

PADS2PCB.DAT   data file for PADS2PCB

It can be run from the command line :

pads2pcb <PADS-PCB File> <Autotrax File> [/q]

where /q puts the program into quiet mode, ie. no feedback.

If no filenames are provided the program will prompt for them.

2. The Default File

-------------------

A default file with level correspondence is optional. If one exists it should

be in the following format :

<PADS-PCB Level>   <Autotrax Level>

The Pads-Pcb level is an integer 0-30.

The Autotrax Level is a string. It can be one of the following :

TopLayer

MidLayer1

MidLayer2

MidLayer3

MidLayer4

BottomLayer

TopOverlay

BottomOverlay

GroundLayer

PowerLayer

BoardLayer

KeepOutLayer

MultiLayer

3. The Conversion Process

-------------------------

In the conversion process, the following should be noted :

(1) As yet the Netlist conversion has not been incorporated into this program.

It is still in 2net.exe.

(2) OddShape and Annular Pads are converted to Round Pads of  the  same  size.

(3) Arcs in Autotrax can only be quadrants, so arcs  in  PADS-PCB  files  are

rounded to nearest 90 degrees.

(rev 3.91)

(end)