--------------------------------- Copyright ----------------------------------

Copyright (c) 1993 by Michael Lee Finney.  All rights reserved.

--------------------------------- Trademarks ---------------------------------

Many names used in this document are trademarked.  I apologize for the

omission of any trademark which I have used and failed to mention as

trademarked.  If notified, I will gladly place the appropriate notice in

the next revision of this file.  The trademarks of which I am aware are...

Trademark   ...................Owner...................

Altra       Altra

Felix       Altra

IBM         International Business Machines Corporation

Inport      Microsoft Corporation

Logitech    Logitech

Microsoft   Microsoft Corporation

Mouseman    Logitech

OS/2        International Business Machines Corporation

PC Mouse    MSC Technologies

PS/2        International Business Machines Corporation

Trackman    Logitech

WIN-OS/2    International Business Machines Corporation

Windows     Microsoft Corporation

--------------------------------- Shareware ----------------------------------

This program is shareware.  If you find this program useful you must

register your copy.  To register a private copy, send $15.00 (US funds)

for each copy to:

Michael Lee Finney

114 Old Wiggington Road

Lynchburg, Va. 24502-4669

Please make remittance payable to: Michael Lee Finney, and include your

company name (for a corporate license), name, address and phone number.

I would also appreciate the options used for your particular mouse.  This

information will allow me to better tune the options in the next release

and possibly provide an installation program.  The registration cost for

corporate site licenses is...

1..4   copies -- $25 each

5..10  copies -- $20 each

11..20  copies -- $15 each

21..40  copies -- $10 each

41...   copies --  $5 each

These prices are accumulative, so the first four copies cost $25 each,

the next six cost $20 each, and so on.  Thus, for example, a corporate

license for 23 copies would cost...

4 @ $25 ... $100

6 @ $20 ... $120

10 @ $15 ... $150

3 @ $10 ...  $60

------

$430

To obtain the latest version of this program when you register, send

$10 for shipping and handling to the above address.  This $10 does NOT

apply to the registration cost and only one copy will be sent regardless

of the number of copies registered (the author is NOT in the business of

diskette duplication).  However, the $10 is a flat fee and not a per

registered copy charge.  If you send me $25 without indicating that a

private license with requested diskette or a commercial license without

a requested diskette is desired, I will take my cue from the method of

payment and request -- namely a check that appears to be a company check

or the use of a company letterhead or the mention of a company name will

be assumed to indicate a commercial registration whereas a private check

and a simple note without the mention of any company will be assumed to

be a private registration with a request for a diskette.

Note: Registration is not required for beta releases of this program.

However, the license to use the beta release expires when the

corresponding general availability release is issued and with this

release the license to use all previous beta releases has expired.

---------------------------------- License -----------------------------------

Unregistered users (personal or corporate) are permitted to use this

program for a trial period of one month.  By the end of this period you

must either register the program or remove it from your system(s).

Registered users are permitted to use this program on one system at a

time and may make as many archived copies for backup purposes as needed.

It may be used on multiple systems (for a single license) only under the

condition that only one of the systems is in use at a time.  Since this

program is an operating system device driver, the assumption is that it is

in continual use when the system in use.  Therefore a registered copy

is required for each system on which this program is used simultaneously.

Registered users are entitled to support.  If after registration a

program error is found, the author will make every attempt possible to

repair the program.  If the failure cannot be repaired and makes the

product unusable the author will refund the registration fee.  Otherwise,

registered users are eligible for the "bug" reward (see details below).

Additionally, registered users will also be sent notices of new versions

of this program, and of other shareware releases by the author.  In

particular a registered beta user will be notified of the general

availability release.  Your name will NOT be sold to a mailing list.

This program is not intentionally crippled in any manner, therefore a

new copy of the program will NOT be sent when the program is registered

unless explicitly requested.

Note: The license to use previous beta releases has expired with this

release of the general availability version of this software.

---------------------------------- Contents ----------------------------------

The distribution file RODNT100.ZIP contains the following files...

LICENSE.DOC  - Licensing information

ORDER.DOC    - Ordering information

RELEASE.DOC  - Release information

README.1ST   - Quick installation hints

RODENT.DOC   - This file

RODENT.SYS   - Mouse driver

RODENT.EXE   - Presentation Manager test program

BUSS.DDP     - Device driver installation file

CALCMP2X.DDP - Device driver installation file

CALCMP3X.DDP - Device driver installation file

CALCMP9X.DDP - Device driver installation file

CALCMPWZ.DDP - Device driver installation file

FELIX.DDP    - Device driver installation file

INPORT.DDP   - Device driver installation file

LOGITECH.DDP - Device driver installation file

MSOFT.DDP    - Device driver installation file

MSYSTEMS.DDP - Device driver installation file

PS2.DDP      - Device driver installation file

SGRAPH.DDP   - Device driver installation file

If this device driver is placed on a bulletin board, all files must

be distributed -- preferably in the original RODNT100.ZIP file.

---------------------------------- Summary -----------------------------------

RODENT.SYS is a mouse driver which supports most mice.  It provides

three button support.  The advantages of RODENT.SYS over the IBM mouse

drivers (assuming that they even work for your mouse) are...

1. Up to 7 buttons are supported.

2. The mouse buttons can be arbitrarily reassigned (reordered).

3. The middle button on most 3 button mouse can be programmed as a

chord of the left and right buttons.

4. The interrupt handlers have been carefully tuned resulting in a

lower system load (compared to the IBM mouse drivers) when the

mouse is active.

5. The FIFO buffer of an 16550AFN or 16552 can be enabled, further

lowering the system load under the appropriate conditions.

6. More serial mice are supported using more protocols than the IBM

mouse drivers support.  Some digitizer tablets are supported.

All mice supported by the IBM mouse drivers are supported.

7. Auto-detects the type of mouse, where possible.

8. Auto-detects the number of mouse buttons, where possible.

9. Auto-detects the type of uart (for serial mice).

10. Supports additional baud rates over the basic 1200 provided by the

IBM mouse drivers.  Baud rates of 150, 300, 600, 1200, 2400, 4800,

9600 and 19200 are supported for those mice which allow the baud

rate to be set.

11. Supports additional reporting rates over the default provided by

the IBM mouse drivers.  Reporting rates of 10, 20, 30, 40, 50,

60, 80, 100 and 200 are supported for those mice which allow the

reporting rate to be set.

12. Any 8250 compatible uart can be used at any port address and any irq.

The IBM drivers only support COM1..COM2.

13. Should not "lose" the mouse during a reboot via Ctrl-Alt-Del.

14. Mouse deinstalls correctly on ABIOS systems (a bug in some of the IBM

mouse drivers).

15. Low battery detection for serial Logitech radio mice is provided

during system boot.

16. The actual mouse resolution can be specified.

17. The mouse sensitivity can be increased or reduced by a factor of

1, 2, 4 or 8.  This is useful for very low or high resolution mice

and for handicapped individuals who have problems with fine motor

control.

---------------------------------- Support -----------------------------------

This is a shareware product.  Support is only available to registered

users.  The registration cost is only $15 so PLEASE register if you find

this product useful.  Problems can be reported via email (in order of

preference at:

IBM NTS:    Mike Finney

Compuserve: 71573,1075

Genie:      M.FINNEY

Commercial usage of the Internet is prohibited so it may not be used

to provide general support.  However, I will respond to a personal note

from any source whatsoever including an Internet email message.  Please

refrain from using the Internet for support issues unless you have no

alternative.  My Internet email addresses are:

Internet:   71573.1075@COMPUSERVE.COM

Internet:   M.FINNEY@GENIE.GEIS.COM

Note: Delays and/or failures in sending/receiving messages via this

address have been reported.  If you send a message and fail

to get a response please try again, possibly with another

address.  I respond to EVERY message, but have had one or two

returned and without a repost have no way to deliver my

response.

and by U.S. Mail at:

Michael Lee Finney

114 Old Wiggington Road

Lynchburg, Va. 24502-4669

Please include all pertinent information regarding the problem, including

hardware configuration, operating system release and corrective service

level.  Include your name, address, email address, phone number, etc. so that

I can contact you.  Please include time zone, and the best hours to reach you

by phone and the hours you would rather NOT be reached by phone (shareware

authors aren't permitted to sleep).  However, a binary copy of another mouse

driver is useless (several people have actually sent me one).

I will attempt to resolve any reported problems.  However, if the problem

is caused by a hardware incompatibility then resolution may be difficult if

I am unable to locate equivalent hardware.  I will work with you as far as

possible.  I am willing to support incompatible mice, but I must be able to

get the necessary hardware.  If you have such a mouse send me a note with a

return address or phone number and I will work with you.

----------------------------------- Reward -----------------------------------

In order to promote software quality, and as an assurance of the existing

software quality of this product, a reward of 25% of the registration fee

with a minimum of $5.00 and maximum of $25.00 is offered to any registered

user for each report of a previously unknown program error.

RULES:

1. The reward is only offered to registered users who have not requested

a refund (however, I will accept bug reports from any user).

2. The maximum reward paid to a single user is limited to twice the

registration cost paid by the user (if you register 10 copies then

your maximum possible reward is greater than if you register 1 copy).

3. I am the final arbiter with regards to precedence between users

reporting the same program error.

4. I am the final arbiter in determining that a reported program error

does, in fact, exist.

5. Failure to support incompatible hardware is NOT considered an error.

I cannot be responsible for every device some twisted hardware

designer creates (even IBM cannot do that).  However, I will work

with users to resolve any problem to the best of my ability.  To

date, all but one reported problem (for the beta release) has fallen

into this category.

6. This reward is not offered for beta releases and experimental

features.

7. The reward is only offered to end users.  A distributor that purchases

copies for their users is NOT considered an end user.  A company that

purchases copies for their internal use IS considered an end user.

-------------------------------- Installation --------------------------------

This mouse driver is an OS/2 2.x device driver ONLY, it does NOT

function under OS/2 1.x or under any version of DOS, Windows or Unix.

It is not possible to install this driver under OS/2 1.x, DOS, Windows

or Unix.

Installing RODENT.SYS during OS/2 installation:

1. Unzip RODNT100.ZIP and copy the files to the root directory on

a diskette prior to installing OS/2 (unless using a diskette

received from the author, in which case this step has been done

for you).

2. Proceed with installing OS/2.

3. At the "Select Mouse" screen, select "No pointing device support".

This prevents OS/2 from placing incorrect statements in CONFIG.SYS

during installation.

4. At the "Advanced Options" screen, select "Install Device Support

Diskette".

5. At the "OS/2 2.1 Device Driver Installation" screen, place the

diskette from step 1 into the diskette drive and select "Install".

6. At the "Select Device Drivers" screen, select the appropriate mouse

driver (down arrow to change current highlighted entry and space bar

to select the current highlighted entry).  Press <tab> to move to

the "Ok" button and then press <Enter>.  OS/2 will copy RODENT.SYS

into the \OS2 directory and update your CONFIG.SYS file.  It will

then present the "OS/2 2.1 Device Driver Installation" screen again

because you might have other device drivers.  If so, install them

now.  After all of your device drivers have been installed use the

right arrow to select "Exit" and press <Enter>.  At the "Exit the

Program" dialog, select "Yes".  OS/2 will then present you with a

message that your system configuration has been changed.  Press

<Enter> to continue with the installation of OS/2.

7. At the "Advanced Options" screen, press <Enter> to continue.  You

may now complete the installation of OS/2.

Warning: OS/2 will add the statements to CONFIG.SYS in the incorrect

order.  If you have installed a serial mouse you may need

to edit your CONFIG.SYS file before your mouse can be used.

The mouse statements in your CONFIG.SYS file must occur in

the following order...

DEVICE=...POINTDD.SYS...

DEVICE=...RODENT.SYS...

DEVICE=...MOUSE.SYS...

DEVICE=...VMOUSE.SYS...

DEVICE=...COM.SYS...

DEVICE=...VCOM.SYS...

If this is the case, then when OS/2 is rebooted it may tell

you that it failed to install some or all of RODENT.SYS,

MOUSE.SYS and VMOUSE.SYS.  Press <Enter> to continue booting

OS/2, start an editor (the system editor is always available)

and change the CONFIG.SYS file so that the mouse statements

are in the correct order.  This should be done when installing

a serial mouse even if OS/2 boots without error messages to

avoid conflicts with the mouse driver and the serial port

driver.

Installing RODENT.SYS after OS/2 installation:

The file RODENT.SYS should be copied to a directory of your

choice -- the C:\OS2 directory is the usual choice, but any directory

can be used.  If OS/2 does not support your mouse at all then remember

that the mouse will not function correctly until RODENT.SYS has been

installed.  It is necessary to modify the CONFIG.SYS files to contain

the following lines...

DEVICE=x:\OS2\POINTDD.SYS

DEVICE=x:\path\RODENT.SYS [options...]

DEVICE=x:\OS2\MOUSE.SYS TYPE=RODENT$

DEVICE=x:\OS2\MDOS\VMOUSE.SYS

and any other existing lines relating to the mouse driver should be

removed.  These lines should be placed in the order given and for

serial mice must precede the "DEVICE=...COM.SYS..." line.  For

example, assuming that OS/2 is located on the C: drive and that

RODENT.SYS has been placed in the C:\OS2 directory:

DEVICE=C:\OS2\POINTDD.SYS

DEVICE=C:\OS2\RODENT.SYS

DEVICE=C:\OS2\MOUSE.SYS TYPE=RODENT$

DEVICE=C:\OS2\MDOS\VMOUSE.SYS

or:

DEVICE=C:\OS2\POINTDD.SYS

DEVICE=C:\OS2\RODENT.SYS BAUD=19200 BUFFERED DPI=400

DEVICE=C:\OS2\MOUSE.SYS TYPE=RODENT$

DEVICE=C:\OS2\MDOS\VMOUSE.SYS

or:

DEVICE=C:\OS2\POINTDD.SYS

DEVICE=C:\OS2\RODENT.SYS PORT=73D8 IRQ=11 BAUD=2400 BUFFERED MOUSE=C PROTOCOL=MM BUTTONS=2 DPI=300

DEVICE=C:\OS2\MOUSE.SYS TYPE=RODENT$

DEVICE=C:\OS2\MDOS\VMOUSE.SYS

The first DEVICE line in the last example is longer than 80 characters.

In most cases an installation similar to the first two examples would

be used.  If you have multiple mice attached to your system, then you

may need to specify the mouse type or the serial port for the desired

mouse.  Only one mouse can be used at a time.  Lines in your CONFIG.SYS

which are mouse related are...

DEVICE=C:\OS2\POINTDD.SYS

DEVICE=C:\OS2\VISION.SYS...

DEVICE=C:\OS2\PCLOGIC.SYS...

DEVICE=C:\OS2\MOUSE.SYS...

DEVICE=C:\OS2\MDOS\VMOUSE.SYS...

all such lines should be removed and replaced as shown above.  There may

be other mouse related lines for different releases of OS/2 and where

mouse vendors have provided an OS/2 mouse driver.  Further, the

statements in CONFIG.SYS should occur in the following order...

DEVICE=...POINTDD.SYS...

DEVICE=...RODENT.SYS...

DEVICE=...MOUSE.SYS...

DEVICE=...VMOUSE.SYS...

DEVICE=...COM.SYS...

DEVICE=...VCOM.SYS...

failure to observe this ordering may result in failure of the mouse to

load properly or of the serial ports to operate correctly.  This mouse

driver has been tested under OS/2 2.0 GA, 2.0 GA+SP, 2.1 March beta and

2.1 GA.  It has been tested on a 50Mhz 486DX AT-clone and on an IBM

PS/2 model 70 (25Mhz 386).  The following mice have been tested...

CalComp Wiz digitizer

CalComp DrawingPad digitizer

Felix mouse

Honeywell Mouse

Keen Mouse

Logitech C9 Mouse

Logitech First Mouse (serial)

Logitech MouseMan Cordless Mouse (serial)

Logitech MouseMan Mouse (buss)

Logitech TrackMan Mouse (serial -- Mouse Systems compatible)

Microsoft Mouse (Inport buss)

Microsoft Mouse (serial)

SummaSketch III

Only these mice have been tested by the author with this mouse

driver.  However, the driver is MUCH more capable because special

provisions have been made to support "partially" compatible mice

and most common mice use one of the available support protocols.

In some cases a mouse might function with the driver, but require

some of the more exotic options (see the option list below).  For

example, you could have a mouse which recognized most of the protocols

associated with a Logitech type C mouse, but could not be automagically

recognized.  In this case you would use the CONFIG.SYS entries:

DEVICE=C:\OS2\POINTDD.SYS

DEVICE=C:\OS2\RODENT.SYS COM=1 MOUSE=C

DEVICE=C:\OS2\MOUSE.SYS TYPE=RODENT$

DEVICE=C:\OS2\MDOS\VMOUSE.SYS

(assuming the mouse is attached to COM1) which forces the mouse

driver to accept the mouse as a Logitech type C mouse, but avoids

the recognition check used for the type C mice.  However, all

programming codes are still sent to the mouse.  Or as a more severe

example, you could have a mouse which supports the Mouse System's 5B

communication protocol (default with Mouse System's mice) but nothing

else works.  In that case you would use the CONFIG.SYS entries:

DEVICE=C:\OS2\POINTDD.SYS

DEVICE=C:\OS2\RODENT.SYS COM=1 MOUSE=* PROTOCOL=5B

DEVICE=C:\OS2\MOUSE.SYS TYPE=RODENT$

DEVICE=C:\OS2\MDOS\VMOUSE.SYS

which forces the mouse driver to use the correct communication protocol,

but with NO attempt made at sending programming codes to the mouse.

The mouse must default to the appropriate state to use the specified

protocol.  Many mice can be setup this way, however other mice either

cannot be automagically recognized or intrinsically REQUIRE programming.

------------------------- Known Setup Configurations -------------------------

Felix mouse

Honeywell Mouse (as shipped)

Keen Mouse (switch in MS mode, provides two buttons)

Logitech C7 Mouse (serial)

Logitech C9 Mouse (serial or buss)

Logitech First Mouse (serial -- model: M-MD15L-9F)

Logitech MouseMan Cordless Mouse (serial -- model: M-RA12)

Logitech MouseMan Mouse (buss -- model: M-PD13-9MD)

Logitech TrackMan Mouse (serial -- models: T-CA1- and T-CA1-9F)

Microsoft Mouse (serial or Inport buss)

PS/2 port mice

DEVICE=C:\OS2\POINTDD.SYS

DEVICE=C:\OS2\RODENT\RODENT.SYS

DEVICE=C:\OS2\MOUSE.SYS TYPE=RODENT$

DEVICE=C:\OS2\MDOS\VMOUSE.SYS

Honeywell Mouse (with jumper J3 cut)

Keen Mouse (switch in PC mode, provides three buttons)

Mouse Systems PC Mouse III

DEVICE=C:\OS2\POINTDD.SYS

DEVICE=C:\OS2\RODENT.SYS COM=# MOUSE=MS

DEVICE=C:\OS2\MOUSE.SYS TYPE=RODENT$

DEVICE=C:\OS2\MDOS\VMOUSE.SYS

CalComp Wiz and 2x00 digitizers

DEVICE=C:\OS2\POINTDD.SYS

DEVICE=C:\OS2\RODENT.SYS MOUSE=DW

DEVICE=C:\OS2\MOUSE.SYS TYPE=RODENT$

DEVICE=C:\OS2\MDOS\VMOUSE.SYS

CalComp 3300 (and newer) digitizers

DEVICE=C:\OS2\POINTDD.SYS

DEVICE=C:\OS2\RODENT.SYS MOUSE=D3

DEVICE=C:\OS2\MOUSE.SYS TYPE=RODENT$

DEVICE=C:\OS2\MDOS\VMOUSE.SYS

CalComp 9x00 digitizers

DEVICE=C:\OS2\POINTDD.SYS

DEVICE=C:\OS2\RODENT.SYS MOUSE=D9

DEVICE=C:\OS2\MOUSE.SYS TYPE=RODENT$

DEVICE=C:\OS2\MDOS\VMOUSE.SYS

Summagraphics digitizers

DEVICE=C:\OS2\POINTDD.SYS

DEVICE=C:\OS2\RODENT.SYS MOUSE=SG

DEVICE=C:\OS2\MOUSE.SYS TYPE=RODENT$

DEVICE=C:\OS2\MDOS\VMOUSE.SYS

----------------------------------- Hints ------------------------------------

Absolute pointing device:

When using the Felix mouse, or any other absolute positioning device

whose maximum resolution is less than the screen dimensions the mouse

pointer will not traverse every screen point.  For example, on a 640x480

screen and an absolute device that has 320 points, only every other

horizontal screen point can be pointed at using the mouse cursor.

This does not cause a problem for most applications.  However, when

attempting to size a window it may not be possible to "grab" the

window's border.  There are two ways to alleviate this problem: first,

change the default border width using the Scheme Palette and second

use the window menu to select the "resize" option.  After the window

has been resized once using the mouse there will be no future

difficulties unless the screen resolution or the mouse resolution

is subsequently changed.

Button ordering:

When using a digitizer or mouse whose buttons aren't arranged in a good

default order, use the ORDER option to rearrange the buttons.

If the button order (i.e. right hand vs left hand) option has been

changed using the Presentation Manager mouse setup options the button

ordering is not maintained correctly for all sessions, and may even

reorder the wrong buttons.  Do not use the PM mouse setup option for

button ordering.  Use the "ORDER=..." option for RODENT.SYS instead.

This allows button reordering to apply to EVERY session and is

independent of PM.

CalComp DrawingPad

I have seen one occasion that the DrawingPad reversed the vertical

direction (down became up and vice-versa).  Admittedly, this WAS after

plugging in the wrong power supply <g> (but operating with the correct

supply).  Powering down the system and rebooting corrected the problem.

Clone mice:

Many of these mice (the Honeywell and Keen mice, for example) emulate

Microsoft mice and Mouse System's mice.  However, frequently these mice

have three buttons and when emulating Microsoft mice only allow the

outermost two buttons to be used.  Sometimes the middle button may

produce a chord click (the Honeywell mouse, for example) or may simply

ignore the middle button (the Keen mouse, for example).  RODENT.SYS

cannot do anything about this situation because that is a mouse hardware

limitation.  However, when emulating Mouse System's mice all three

buttons work properly.  This is the recommended mode for such mice.

This mode can frequently be selected by a slide switch.  The Honeywell

mouse requires disassembly of the mouse and cutting jumper J3 (see the

note below about Honeywell mice).

COM4 Installation:

There's a problem in VVGA.SYS and VSVGA.SYS that can kill COM4 when you

enter (or exit) a DOS full-screen session, even if that session isn't

using that port.  The drivers which handle that screen switching,

mistakenly think that there's an 8514/A video card also in the system.

and write to two registers on that card.  Of course, if you really DO

have an 8514/A or S3-based video card then you CAN'T use COM4 at 0x02E8

for this very reason.

You can patch VVGA.SYS and VSVGA.SYS to eliminate the problem; look for

the

MOV DX,4AE8

instruction and NOP out the next two OUT DX,AL instructions which write

to 0x4AE8 and 0x4AE9 (which map to 0x02E8 and 0x02E9 on most com boards).

This appears to be fixed in the March beta of OS/2 2.1 and in the GA

release of OS/2 2.1.

Thanks to Jens Schlatter for relaying this information from Bill Hinkle.

DOS session support:

All DOS support is provided by the IBM virtual mouse driver VMOUSE.SYS.

If you do not have DOS support, i.e. your CONFIG.SYS contains the line:

PROTECTONLY=YES

then none of the lines:

DEVICE=...VMOUSE.SYS...

DEVICE=...VCOM.SYS...

should be included in your CONFIG.SYS file.

DOS Virtual Machine support:

When booting a version of DOS in a virtual machine (VMB) then a native

mouse driver should NOT be used, but instead use the "stub" driver that

OS/2 provides.  This driver is called MOUSE.COM and is located in the

OS2\MDOS directory.  It can be loaded in your AUTOEXEC.BAT if you so

desire.

Dual boot or boot manager:

The Logitech type C mice frequently use the MM protocol when in DOS and

so the option "PROTOCOL=MM" may be necessary.  This allows OS/2 to use the

same protocol as used under DOS so the mouse won't be lost when switching

between DOS and OS/2.  In general, if using dual boot or boot manager and

the mouse uses a particular protocol under DOS the appropriate option may

need to be used under OS/2 to allow switching without losing the mouse.

However, under normal circumstances programmable mice such as the Logitech

type C mice are reset by RODENT.SYS to avoid such problems.  But (see the

note below for Logitech mice) there are some older type C mice which

appear to have initialization problems.

Felix mouse:

When this mouse is first powered on, it is necessary to move the mouse

stick to each edge of the mouse for the hardware to correctly recognize

the range of motion for the mouse stick.  If the mouse stick is moved

to one side and the cursor stops partway across the screen then this

resetting action needs to be performed.  The Felix mouse is an absolute

pointing device which has lower resolution than the screen.  Therefore

it cannot point to every pixel on the screen.  Normally this is not a

problem.  See the hints section for more information on how to handle

this situation.

Honeywell mice:

These mice emulate Microsoft mice, but the middle button generates a

chord click.  However, they can also emulate Mouse System's mice if

jumper J3 is cut.  This requires disassembly of the mouse.  Please

call Honeywell at 800/445-6939 to obtain instructions for making this

modification.  I take no responsibility for any such hardware

modification.  Please note that any hardware alteration may affect

your warranty.  Honeywell is releasing a mouse with a slide switch to

overcome this problem 4Q '93.  Call Honeywell for availability and

pricing.

Hot plugging:

Unplugging the mouse and then plugging it back in while the system

is active is not supported in this release.  This is normally not

a problem unless you are using a mouse switcher between multiple

systems.  However, many protocols have no problem with this because

they automatically resynchronize.  Usually the mouse must be present

for the mouse driver to load, however using the "MOUSE=* PROTOCOL=xx"

options (where xx is the desired protocol may allow OS/2 to be booted

without the mouse being present.  The mouse must default to the

specified protocol or must be attached to a computer which sets the

protocol properly when booted.  PS/2 port mice disable themselves

when unplugged and replugged so they may not be used in this type of

application.

IBM 8516 Touch Screen

This is not supported.  IBM has a special driver for this device, see

the OS/2 documentation for installation instructions.

IBM PS/2 mice:

Some IBM PS/2 mice with a black roller ball have hardware problems.

IBM's recommendation for these mice is to replace them with IBM PS/2

mice with a gray roller ball (or any OTHER mouse for that matter).

Keyboard operation of OS/2:

When installing RODENT.SYS, especially if experimenting with different

features make sure that you have an editor easily accessible from the

desktop and make sure that you know the various keyboard commands

needed to navigate the desktop during times that the system is booted

without mouse support.  Some keyboard commands that are especially

useful are...

<Alt> + <Tab>

Move to the next window

<Up arrow> or <down arrow>

Use to highlight an object in a window or option in a menu

<Enter>

Use to select the highlighted object

<Alt> + F O

Use to open a file once the system editor (E.EXE) has been started

<Alt> + F S

Use to save a file once altered when using the system editor

<Alt> + <F4>

Use to close a window (for example the system editor)

<Alt> + <Shift> + <Tab>

Use to select the desktop

<Ctrl> + "\"

Use to deselect all objects on the desktop once the desktop has

been selected

<Shift> + <F10>

Use to pop-up the desktop menu that contains the shutdown option

Note: these last three commands are usually used in order to shutdown

OS/2 when the mouse is not available.

Logitech mice:

There are some older Logitech mice which seem to have initialization

problems.  These mice typically hang up or completely refuse to

install.  Using the "UART=..." option and possibly the "MOUSE=C" or the

"MOUSE=*" and "PROTOCOL=5B" options may help.  If the mouse installs but

hangs up and unplugging the mouse and then plugging it back in makes the

mouse come "alive" then try the above options.

Microsoft PS/2 mice:

Microsoft PS/2 mice are not fully compatible with IBM PS/2 mice and

Logitech PS/2 mice.  This causes some difficulties in the interrupt

handler because the protocol is dependent upon the number of physical

buttons.  If using any three button PS/2 mouse with the options:

PORT=0060 IRQ=12 MOUSE=* PROTOCOL=PS BUTTONS=2

then the middle button will not be recognized.  However, this problem

does not exist when using any of:

...\RODENT.SYS

...\RODENT.SYS BUTTONS=2

...\RODENT.SYS BUTTONS=3

...\RODENT.SYS MOUSE=PS

...\RODENT.SYS MOUSE=PS BUTTONS=2

...\RODENT.SYS MOUSE=PS BUTTONS=3

because these allow the actual number of physical buttons to be

determined.  Also, the option:

...\RODENT.SYS PORT=0060 IRQ=12 MOUSE=* PROTOCOL=PS BUTTONS=3

can be used as well because it correctly specified the number of

physical buttons.  There are no problems when using two button

PS/2 mice.

Mouse sensitivity:

When the mouse is too sensitive or too slow, use the RESPONSE option

to change the mouse responsiveness.  Remember that the BAUD and RATE

options interact with the RESPONSE option.  If the reporting frequency

is increased then the average distance moved by the mouse between

reports is smaller.  Since there is a gradual acceleration threshold

to allow pixel by pixel control at slow speeds this means that the

mouse must be moved faster before acceleration becomes effective.

Conversely reducing the reporting frequency will effectively make the

mouse more "active" because the acceleration threshold is more likely

to be reached.

If the mouse sensitivity has been changed using the Presentation

Manager mouse setup options it may be found that the sensitivity in

other sessions is not appropriate.  Use the "RESPONSE=..." option

for RODENT.SYS instead.  This allows the mouse sensitivity to apply

to EVERY session and is independent of PM.  Also the Presentation

Manager acceleration occurs in addition to the acceleration provided

by RODENT.SYS.

Multiple mice:

While multiple mice may be present in your computer system, only one mouse

can be used at a time.  The mouse to be used is selected when OS/2 is

booted and cannot be changed without rebooting OS/2.  When searching for

a mouse, RODENT.SYS uses the search order:

Inport mice

Buss mice

PS/2 mice

Serial mice

Logitech type C mice

Microsoft compatible mice (also Logitech type M, MW, V or VW mice)

Felix mice

Mouse System mice                 (MOUSE=MS required)

CalComp 3300 and newer digitizers (MOUSE=D3 required)

CalComp 9x00 digitizers           (MOUSE=D9 required)

CalComp 2x00 and WIZ digitizers   (MOUSE=DW required)

SummaGraphics digitizers          (MOUSE=SG required)

Therefore Buss mice, Inport mice and PS/2 mice are recognized before

serial mice.  If you have one of these mice in your system but also

have a serial mouse attached you can force only serial mice to be

recognized by using the "MOUSE=S" option.  You can also force a

particular mouse to be recognized by using the appropriate "MOUSE=xx"

option.  You can prevent recognition of any mouse by using the "MOUSE=*"

option with the appropriate "PROTOCOL=xx" option.

PS/2 Installation:

Some PS/2 systems will require you to tell the hardware that you have "no

mouse" when installing a serial mouse instead of a PS/2 mouse.

PS/2 mice:

Some motherboards have incompatible keyboard/mouse controllers.  Normally

an 8042 chip is used, but some clones have used the 8242.  PS/2 mice will

not function on such systems.

RODENT.SYS not found

Check the drive and path used in your CONFIG.SYS file.

RODENT.SYS not installed

Check the options used for RODENT.SYS in your CONFIG.SYS file.

Serial port support:

If you are using a serial mouse and only have a single serial port then

none of the lines:

DEVICE=...COM.SYS...

DEVICE=...VCOM.SYS...

should be placed in your CONFIG.SYS file.

Summagraphics digitizers

This release does not support changing the puck while OS/2 is running.

Shutdown OS/2, power off the digitizer, change the puck and then reboot

the system.

SummaSketch III

If the SummaSketch III is attached to a PS/2 model 70 and is powered on

before the model 70 then the model 70 may signal a hardware failure

during the power on hardware check.  This is a hardware issue and cannot

be alleviated by RODENT.SYS because the problem occurs before RODENT.SYS

is loaded into memory.  To alleviate this problem simply power up the

SummaSketch III after the model 70 has completed its hardware checks.

VMOUSE.SYS not installed, but RODENT.SYS and MOUSE.SYS installed

This is frequently due to the failure to specify either the COM option

or the PORT and IRQ options when using MOUSE=* PROTOCOL=xx.  This

generally happens when RODENT.SYS installed but the mouse is not

actually present.  This can happen for mice which cannot be checked

and when checking is disabled.

---------------------------------- Testing -----------------------------------

After booting OS/2 preliminary testing can be accomplished simply by

moving the mouse and making sure that the mouse cursor follows the movement

of the mouse.  Execute RODENT.EXE to further test this device driver.  This

test program will display a Presentation Manager window.  It will display

the number of buttons as perceived by OS/2 and the mouse resolution as

perceived by OS/2.  Further, if you click the mouse buttons, the associated

OS/2 mouse event is displayed.  The most recent mouse event is flagged by

an asterisk.  If the mouse pointer follows mouse movement correctly and

all buttons operate correctly then you will probably not have any trouble

with the mouse driver.  Other signs of trouble might be consistently missing

mouse events (such as clicks) or events occurring without any associated

physical mouse action.

When evaluating missing or extra events reasonable care should be taken.

It has been observed that occasionally OS/2 will "lose" a mouse click during

rapid mouse activity -- this is related to the fact that OS/2 only maintains

a small mouse event queue to avoid "piling" up an extremely large number of

events.  Sometimes losing a button release has the appearance of additional

clicks if an application auto-repeats while a mouse button is pressed.  The

loss, by OS/2, of mouse events is VERY rare except under very heavy mouse

activity.  However, it appears to be slightly more common when using higher

baud rates because the internal OS/2 queue is more likely to overflow.  This

has been seen using the Logitech radio mouse at 9600 baud.  Also this loss

might not actually be the fault of OS/2, but is occurring in the applications

instead (actually more probable).  Nevertheless, this behavior has sometimes

been seen, both with these mouse drivers AND with the OS/2 mouse drivers.

It has been seen most often using Logitech's radio mouse and Microsoft's PWB

in a full screen session.  Sometimes if this happens when scrolling releasing

the mouse button will not stop the scrolling.  Simply click the mouse button

again to stop scrolling.

This is a low-level mouse driver.  OS/2 handles all of the higher level

functions, including routing mouse events to DOS sessions, OS/2 sessions,

PM sessions, WIN-OS2 sessions and VMB sessions.  OS/2 reports all mouse

events (in the appropriate form) to the currently active session.  Therefore

DOS sessions CAN take advantage of all three buttons.  Since this mouse

driver is not involved in these actions, failure of OS/2 in that manner

is probably not caused by this device driver.

Note: The resolution reported by OS/2 is in "mickeys per centimeter".

Therefore, the dots per inch provided to the device driver is divided by

2.54 and rounded when reporting to OS/2.  This device driver reports the

closest value possible.  For example, 400 dpi reports as 157 mickeys.

Since the value reported to OS/2 is a fixed point number, the result if

multiplied by 2.54 is only 399 (after rounding) instead of the original 400.

---------------------------------- Options -----------------------------------

BAUD=#

This option allows the baud rate to be specified.  It may be one of

150, 300, 600, 1200, 2400, 4800, 9600 or 19200

All mice do not support all baud rates.  To use the lowest baud rate

supported by the mouse specify:

BAUD=150

and to use the highest baud rate supported by the mouse specify:

BAUD=19200.

The lowest baud rate which gives good performance should be used to

reduce system load.  If this option is not specified then it defaults

to a value determined by the protocol in use.  Each protocol has a

default baud rate based on the default mouse baud rate.  If a baud

rate is specified which is not supported by the protocol the highest

baud rate which is less than the specified baud rate will be used, if

any, and otherwise the lowest baud rate which is higher than the

specified baud rate will be used.

BUFFERED

This option enables the FIFO buffer if a 16550 uart is detected.  The

FIFO buffer is NOT enabled by default because some 16550 uarts do not

function correctly.  All 16550AFN uarts should function correctly, if

you have a 16550, 16550A or 16550AF uart then it should be replaced by

a 16550AFN uart.  RODENT.SYS will function correctly if the uart is

not replaced, but this option can only be used with a 16550AFN or

equivalent uart (such as 16c552 or 16c554).

BUTTONS=#

This option indicates the number of buttons to be supported.  Many

protocols support both 2 button and 3 button mice.  This option may

be specified as:

BUTTONS=2

BUTTONS=3

If BUTTONS=3 is specified for a 2 button mouse the mouse driver will

function normally, but will report to OS/2 that there are two buttons

if RODENT.SYS can obtain the number of buttons from the mouse and

will otherwise report to OS/2 that there are three buttons.  However,

events for the third button will simply never occur.  If BUTTONS=3 is

specified for a 3 button mouse (the default) then all three buttons

will be returned to OS/2.  OS/2 treats clicking any two or three)

mouse buttons together as a "chord" click.  It also handles single or

double clicks and single click and drag from all three buttons.

If BUTTONS=2 is specified for a 3 button mouse then the mouse driver

will convert a press of the middle button into a press of both left

and right buttons.  This means that clicking the middle button is

equivalent to a "chord" click.  However, if PROTOCOL=MI is specified

for a three button mouse (the default is PROTOCOL=MP) this cannot be

done because the Microsoft-compatible 2 button protocol is not aware

of events for the middle button.  In this case the middle button will

simply be ignored.  The included PM test program will report the number

of mouse buttons as seen by OS/2.

COM=#

This option allows the serial port to explicitly specified.  This

option may be specified as:

COM=1

COM=2

COM=3

COM=4

where 1..4 is one of the standard com ports.  So,

COM=1

is normally equivalent to:

PORT=03f8 IRQ=4

If both this option and the PORT option are omitted then the mouse

driver will scan COM1..COM4 (in order) to find the mouse.  This

procedure works and is reasonably reliable, however it is impossible

to completely restore the uart state so automatic scanning could

potentially cause problems with other devices attached to serial

ports.  This mouse driver will restore more of the serial port state

than the IBM mouse drivers.  Specifying either this option or the

PORT option also reduces the time required to initialize the mouse

driver -- by as much as a couple of seconds.  This option can ONLY

be specified for serial mice.  It is recommended that either this

option or the PORT and IRQ options be used for all serial mice.

When the MOUSE option is used either this option or the PORT and IRQ

options may be necessary.

DPI=#

This option allows the mouse resolution to be specified.  This must

be in the range 100..640 (inclusive).  The default is approximately

DPI=200 (actually the default is 80 mickeys per centimeter).  Most

newer mice are higher resolution (and even some of the older mice).

The C9 Logitech Buss Mouse has a resolution of 320 dpi and the newer

Logitech mice have a resolution of 400 dpi.  The value used should be

the HARDWARE resolution, many DOS mouse drivers allow the resolution

to be "set" in the software -- this does not affect the actual mouse

events but only their reporting by the DOS mouse driver.  This value

is reported to OS/2, but at this time, OS/2 does not appear to make

a significant use of the value due to a bug in MOUSE.SYS.  The

included test program will report the value as set by RODENT.SYS

and as seen by OS/2 (but which is ignored by MOUSE.SYS).

IRQ=#

This option allows the interrupt number to explicitly specified.  Any

interrupt from 2..15 may be used, unlike the IBM drivers which are

restricted to interrupts 2..7.  If omitted, the interrupt level to

be used will be determined from the port address.  This procedure is

only reliable for the standard COM1 and COM2 port addresses 3f8 and

2f8.  It may work for COM3 and COM4 (depending on the actual port

addresses).  The IBM mouse driver has the same (undocumented) problem,

but does not provide a mechanism to specify the interrupt number.

MARGIN=#

This option allows the margin on an absolute pointing device to be

specified.  The default is MARGIN=0 since some digitizer tablets have

a hardware margin.  Units are in terms of the device resolution.  Most

digitizers are programmed for 1000lpi so MARGIN=500 gives a half inch

margin on each side of the tablet.  When the cursor is in the margin

area button presses will be recognized and the mouse cursor will move

in two dimensions but will be "pegged" to the appropriate side and so

will not move in that dimension.  Use of a margin prevents a "cliff"

effect.  The hardware margins provided by some devices will ONLY

recognize button presses.

MOUSE=xx

This option allows the mouse type to be specified.  This is useful to

reduce the time required to load the mouse driver, or if the mouse

used responds correctly but the mouse type cannot be auto-determined.

If an mouse type is specified that fails to match the hardware in use

then the mouse driver may or may not load, and if loads may fail to

operate properly.  This option may be specified as:

MOUSE=B

Buss mouse, can be two button or three button.  Note that an

Inport mouse is NOT the same as a buss mouse.

Protocols: BS

default: BS

Rate: 30, 60

note: set by hardware jumper

MOUSE=C

Logitech Mouse System's compatible mouse.  DOS mouse drivers

frequently operate this mouse using the MM protocol.  When

using dual boot or Boot Manager it may be desirable to use

the MM protocol.

Protocols: 5B, MM, RE

default: 5B

better:  MM

Baud Rate: 1200, 2400, 4800, 9600

default: 1200

MOUSE=D3

CalComp 3300 and newer digitizers -- this is an "absolute"

pointing device.  Each point on the digitizer table corresponds

exactly to a point on the screen.  This is similar to "touch"

screens in operation.  This feature can be awkward to those not

used to it, but may be preferred by those who use a digitizer

tablet on a regular basis.  This option is required for these

digitizers, they cannot be automagically recognized.

Protocols: HA, HR

default: HA

Baud Rate: 150, 300, 600, 1200, 2400, 4800, 9600, 19200

default: 9600

Note: This type mouse cannot be automagically recognized, so

this parameter is required for such mice.  CalComp has

many different styli and pucks available so the use of

the ORDER option is almost essential.  For the 4-button

puck arranged in a diamond, ORDER=2143 is recommended.

For the styli with a tip and double-action button, the

default of ORDER=123 has the tip has the "left" button.

This is much more subject to personal preference so no

recommendation will be made.  There are many other pucks

available from CalComp and trial and error is the only

guide to the best ordering for those.

MOUSE=D9

CalComp 9x00 digitizers -- this is an "absolute" pointing device.

Each point on the digitizer table corresponds exactly to a point

on the screen.  This is similar to "touch" screens in operation.

This feature can be awkward to those not used to it, but may be

preferred by those who use a digitizer tablet on a regular basis.

The 9x00 digitizers are older products which do not support full

software initialization.  These tablets MUST be set for 9600

baud.  The 9100 model tablets must have the hardware switches

configured for format 5.  If reconfigured by other applications

the tablet must be reset before this driver will function.  This

option is required for these digitizers, they cannot be

automagically recognized.

Protocols: HA, HR

default: HA

Baud Rate: 9600

default: 9600

Note: This type mouse cannot be automagically recognized, so

this parameter is required for such mice.  CalComp has

many different styli and pucks available so the use of

the ORDER option is almost essential.

MOUSE=DW

CalComp WIZ and 2x00 digitizers -- these are "absolute" pointing

devices.  Each point on the digitizer table corresponds exactly

to a point on the screen.  This is similar to "touch" screens in

operation.  This feature can be awkward to those not used to it,

but may be preferred by those who use a digitizer tablet on a

regular basis.  The WIZ and 2x00 digitizers are older products

which do not support full software initialization.  These tablets

MUST be set for 9600 baud.  If reconfigured by other applications

the tablet must be reset to 9600 baud before this driver will

function.  This option is required for these digitizers, they

cannot be automagically recognized.

Protocols: HA, HR

default: HA

Baud Rate: 9600

default: 9600

Note: This type mouse cannot be automagically recognized, so

this parameter is required for such mice.  CalComp has

many different styli and pucks available so the use of

the ORDER option is almost essential.  For the 4-button

puck arranged in a row, ORDER=1234 is recommended.

MOUSE=FX

Altra's Felix mouse.  This is an "absolute" pointing device.

Each position of the mouse "handle" corresponds exactly to a

point on the screen.

Protocols: FA, FR

default: FA

Baud Rate: 9600

default: 9600

Note: This type of mouse allows different baud rates to be set

but the commands used are inconsistent between different

mice with the same internal version number depending on

the cystal used during manufacture of the mouse.  Since

this mouse can function in "delta" mode only the hardware

default baud rate is allowed because this can be set

consistently.

MOUSE=I

Inport buss mouse, can be two button or three button.  Note

that an Inport mouse is NOT the same as a buss mouse.

Protocols: IN

default: IN

Rate: 30, 50, 100, 200

default: 100

MOUSE=M

Microsoft compatible serial 2 button mouse, non-programmable

Protocols: MI

default: MI

Baud Rate: 1200

default: 1200

MOUSE=MS

Mouse System's compatible serial 3 button mouse

Protocols: 5B

default: 5B

Baud Rate: 1200

default: 1200

Note: This type mouse cannot be automagically recognized, so

this parameter is required for such mice.

MOUSE=MW

Logitech's Microsoft/Mouse System compatible serial 2 button

mouse

Protocols: MI, MP, 5B

default: MP

Baud Rate: 1200, 9600

default: 1200

MOUSE=PS

PS/2 port compatible mouse

Protocols: PS

Rate: 10, 20, 40, 60, 80, 100, 200

default: 100

MOUSE=S

This causes RODENT.SYS to only check for serial mice.  Any

other mice are ignored.  This could be useful if a buss mouse

interface is present, but the mouse itself is not present.

This option is supplied because some motherboards and video

adapters have a buss or Inport mouse interface built-in which

should be ignored.

MOUSE=SG

Summagraphics digitizers -- this is an "absolute" pointing

device.  Each point on the digitizer table corresponds exactly

to a point on the screen.  This is similar to "touch" screens

in operation.  This feature can be awkward to those not used

to it, but may be preferred by those who use a digitizer

tablet on a regular basis.  This option is required for these

digitizers, they cannot be automagically recognized.

Protocols: MA, MM, MR

default: MA

Baud Rate: 9600

default: 9600

Note: This type mouse cannot be automagically recognized, so

this parameter is required for such mice.  Further, if

the stylus is used then "ORDER=132" is recommended as

well.  If the 16-button puck is used then "ORDER=213" is

recommended.  The default "ORDER=123" is suggested for

the 4-button puck.

MOUSE=V

Logitech compatible serial 3 button mouse, non-programmable.

If the MI protocol is used then the middle mouse button cannot

be programmed as a chord.

Protocols: MI, MP

default: MP

Baud Rate: 1200

default: 1200

MOUSE=VW

Logitech's Microsoft/Mouse System compatible serial 3 button

mouse

Protocols: MI, MP, 5B

default: MP

Baud Rate: 1200, 9600

default: 1200

MOUSE=*

Unknown mouse.  This option would be used when the mouse is

known to communicate with a given protocol, but the mouse type

cannot be recognized and the mouse cannot be programmed.  In

all cases, the PROTOCOL=xx and either COM=# or PORT=# IRQ=#

must also be present.  This is NOT checked for and failure

to provide these options will prevent the mouse from being

properly initialized.  In some cases it can also cause the

system to lockup requiring a reboot.  If an absolute protocol

is in use then the X=# and Y=# options must also be used.

ORDER=#

This option specifies the button ordering.  The default button ordering

is usually adequate for all mice.  However, digitizer pucks tend to be

inconsistent between logical button numbering and physical button

placement and this option is almost always required for a digitizer.

Further, while OS/2 allows the left and right buttons to be swapped

for left hand users, it doesn't do so correctly for three button mice

and the swapping does not function at all for DOS and OS/2 full screen

sessions.  Trailing digits may be omitted if they are in order.  At

least one digit must be specified.  This mouse driver supports from 2

to 7 physical buttons. Any buttons above button 7 will be ignored.

This option may be specified as:

ORDER=1234567

where each digit is the button to be used for the corresponding button

position.  Omitted digit positions which be assigned their position

index.  It is an error to specify the same button twice, or to fail to

specify a button.  Thus:

ORDER=1

ORDER=12

ORDER=123

ORDER=1234

ORDER=12345

ORDER=123456

ORDER=1234567

all result in the default button ordering.  But:

ORDER=122

ORDER=124

are both errors.  And:

ORDER=7654321

reverses the button ordering.  So button 1 will be treated the same as

button 7, button 2 the same as button 6, and so on.  The most common

button orderings for mice are:

ORDER=321

which reverses the left and right buttons, leaving the middle button

unaffected.  Also:

ORDER=132

which reverses the middle and right buttons.  Thus OS/2 button 1

is physical button 1, OS/2 button 2 is physical button 2 and OS/2

button 3 is physical button 3.

One useful ordering for some digitizer pucks arranged in a "diamond"

ordering is:

ORDER=2143

For mice, the left mouse button is always physical button 1, the right

mouse button always physical button 3 and the middle mouse button (if

present) is always physical button 2.

The general button assignment strategy for two button mice is...

Button 1 -- OS/2 button 1 (left button down)

Button 2 -- OS/2 button 1+2 (all buttons down)

Button 3 -- OS/2 button 2 (right button down)

Button 4 -- OS/2 button 1+2 (all buttons down)

Button 5 -- OS/2 button 1+2 (all buttons down)

Button 6 -- OS/2 button 1+2 (all buttons down)

Button 7 -- OS/2 button 1+2 (all buttons down)

Notice that for two button mice no more than 3 buttons can be

profitably used for OS/2.  The general button assignment strategy for

three button mice is...

Button 1 -- OS/2 button 1 (left button down)

Button 2 -- OS/2 button 3 (middle button down)

Button 3 -- OS/2 button 2 (right button down)

Button 4 -- OS/2 button 1+2 chord (left + right buttons down)

Button 5 -- OS/2 button 1+3 chord (left + middle buttons down)

Button 6 -- OS/2 button 2+3 chord (middle + right buttons down)

Button 7 -- OS/2 button 1+2+3 chord (all buttons down)

While different chord clicks are generated, OS/2 recognizes all

multiple button presses as a chord click so any program that wishes

to distinguish between chord clicks must examine the primitive button

event messages.

Notice that for three button mice no more than 7 buttons can be

profitably used for OS/2.  Also, many digitizer pucks have the

limitation that only one button can be pressed at a time and there

are both mice and digitizer pucks which have buttons hard-wired to

generate chord clicks of other buttons and where multiple presses

of buttons may result in the appearance of some other button being

pressed.  These problems cannot be resolved by this driver, but the

ORDER option can allow the best setup for a particular mouse or puck.

To obtain the best ordering some experimentation may be necessary.

PRESSURE=#

This option is only effective for digitizer pens which are pressure

sensitive.  This option sets the pressure threshold at which a pen

tip recognizes a button press.  The pressure must be in the range of

1 to 255 inclusive.  The default, and value recommended by CalComp for

their pressure pens is 4.

PORT=#

This option allows the serial port address to explicitly specified,

and may be specified as:

PORT=xxxx

where "xxxx" is the hex address of the port to be used.  If the COM

option is also specified, this option takes precedence.

Any serial port which is compatible with the 8250 uart may be used.

This mouse driver is not restricted to COM1..COM2, unlike the IBM

mouse drivers.  If both this option and the COM option are omitted

then the mouse driver will scan COM1..COM4 (in order) to find the

mouse.  This procedure works and is reasonably reliable, however it

is impossible to completely restore the uart state so automatic

scanning could potentially cause problems with other devices attached

to serial ports.  This mouse driver will restore more of the serial

port state than the IBM mouse drivers.  Specifying this option also

reduces the time required to initialize the mouse driver -- by as

much as a couple of seconds.

PROTOCOL=xx

This option allows the communications protocol to be specified.  This

is useful to select a non-default protocol and when using the MOUSE=*

option.  If the mouse type is known and an unsupported protocol is

selected the default protocol for that mouse type will be used.  This

option may be specified as:

PROTOCOL=3B

Three byte binary protocol -- this is an experimental protocol

which MAY work with some older mice.  If used then MOUSE=* must

also be used.  This protocol will work instead of the 5B protocol

but the mouse will "slip".  It also supports either a 2 button

or 3 button mouse.  Since this is an experimental protocol, the

reward does NOT apply to its use.

PROTOCOL=5B

Five byte binary protocol.  It is the default for Mouse System's

compatible mice.  This protocol supports either a 2 button or 3

button mouse.

PROTOCOL=BS

Buss mouse protocol, supports either 2 or 3 buttons.  This

protocol is used for buss mice.  Normally using MOUSE=B is

preferred to this option, but MOUSE=* PROTOCOL=BS is allowed.

PROTOCOL=FA

Felix mouse protocol.  This is an "absolute" protocol, i.e. each

position of the mouse corresponds exactly to a point on the

screen.

PROTOCOL=FR

Felix mouse protocol.

PROTOCOL=HA

CalComp high resolution binary digitizer protocol, supports

either the 4 button puck (which may actually have 6 buttons, but

two of the buttons are hardware-wired as chords of the remaining

buttons) or the 16 button puck.  Pens are also supported, but

obtaining a "chord" click can be difficult if the middle button

is not programmed to generate a chord.  Pressure pens may also

have difficultly generating a double click for the tip.  OS/2

only supports three buttons so extra buttons are mapped to chord

clicks.  This protocol is normally used with digitizers.  This

is an "absolute" protocol, i.e. each position on the digitizer

tablet corresponds exactly to a point on the screen.

PROTOCOL=HR

CalComp high resolution binary digitizer protocol, supports

either the 4 button puck (which may actually have 6 buttons, but

two of the buttons are hardware-wired as chords of the remaining

buttons) or the 16 button puck.  Pens are also supported, but

obtaining a "chord" click can be difficult if the middle button

is not programmed to generate a chord.  Pressure pens may also

have difficultly generating a double click for the tip.  OS/2

only supports three buttons so extra buttons are mapped to chord

clicks.  This protocol is normally used with digitizers.

PROTOCOL=IN

Inport buss mouse protocol, supports either 2 or 3 buttons.

This protocol is used for Inport buss mice.  Normally using

MOUSE=I is preferred to this option, but MOUSE=* PROTOCOL=IN

is allowed.

PROTOCOL=MA

Summagraphics digitizer protocol.  OS/2 only supports three

buttons so extra buttons are mapped to chord clicks.  This

protocol is normally used with digitizers.  This is an

"absolute" protocol, i.e. each position on the digitizer

tablet corresponds exactly to a point on the screen.

PROTOCOL=MI

Microsoft compatible 2 button protocol.  Can be used with a 3

button mouse, but completely ignores the middle button.

PROTOCOL=MM

Mouse System's compatible protocol, supports either 2 or 3

buttons.  Also available on Summagraphics digitizer tablets.

PROTOCOL=MP

Extension to MI protocol which supports either 2 or 3 buttons.

If only two buttons are present the MI protocol is more

efficient.

PROTOCOL=MR

Summagraphics digitizer protocol.  OS/2 only supports three

buttons so extra buttons are mapped to chord clicks.  This

protocol is normally used with digitizers.

PROTOCOL=RE

Relative Bit Pad One protocol, supports either 2 or 3 buttons.

PROTOCOL=PS

PS/2 port protocol, supports either 2 or 3 buttons.

RATE=#

This option allows the reporting rate to be specified for those mice

which accept this option (currently only Inport mice and PS/2 mice).

It may be one of:

10, 20, 30, 40, 50, 60, 80, 100 or 200

All mice do not support all reporting rates.  To use the lowest

reporting rate supported by the mouse specify:

RATE=10

and to use the highest reporting rate supported by the mouse specify:

RATE=200.

The lowest reporting rate which gives good performance should be used

to reduce system load.  If this option is not specified then it

defaults to a value determined by the protocol in use.  Each protocol

has a default reporting rate based on the default mouse reporting rate.

If a reporting rate is specified which is not supported by the protocol

the highest baud rate which is less than the specified rate will be

used, if any, and otherwise the lowest rate which is higher than the

specified rate will be used.  For some mice the reporting rate is set

via hardware jumper.

RESPONSE=#

This option specifies a divisor to increase or reduce the mouse

responsiveness.  If omitted it defaults to 0.  It may be specified as:

RESPONSE=-8

RESPONSE=-4

RESPONSE=-2

RESPONSE=-1

RESPONSE=0

RESPONSE=1

RESPONSE=2

RESPONSE=4

RESPONSE=8

and only these values may be used.  If the value is negative then the

responsiveness will be reduced by the given factor, and if positive

increased by the given factor.  If zero then there will be no change.

Since increasing or decreasing the responsiveness by a factor of one

has no effect these values are equivalent to zero.

This can be useful for very low or very high resolution mice (some

digitizers are 1000 dpi which is FAR too sensitive for most people).

Further, if a user has difficultly holding the mouse steady (such as

Parkinson's victims) this allows the mouse to ignore small movements.

Caution: with this option it is possible to slowly move the mouse

without moving the mouse cursor at all.  This effect does NOT happen

for RESPONSE=-1 or any non-negative value and would be most noticeable

for RESPONSE=-8.  Thus, RESPONSE=-8 would effectively make a 200 dpi

mouse into a 25 dpi mouse.  As for the ORDER option, OS/2 DOES provide

a mouse sensitivity control, but that control is not effective for DOS

or OS/2 full screen sessions and frequently the available range is

insufficient.  This option does not have effect on absolute protocols

since those protocols map mouse positions directly to screen positions.

UART=#

This option allows the uart type to be specified.  It may be specified

as...

UART=8250

UART=16450

UART=16550

UART=16550A

UART=16552

If this option is omitted, the uart type will be determined by testing

the uart.  All uart testing will be skipped if this option is present.

This option is provided because some mice are sensitive to testing for

the uart type.  This sensitivity is very rare and probably also

depends on the design of the serial port.  This option should not be

specified unless RODENT.SYS fails to work properly.  The symptoms of

the only known occurrence of this problem was that RODENT.SYS would

load but the mouse pointer would not move.  If this option is specified

then either the COM option or the PORT and IRQ options must also be

specified.

X=[#..]#

This option specifies the active horizontal range for a digitizer

tablet.  It may be specified as:

X=#

X=#..#

In the first case, the active range is assumed to be 0..#-1 and in the

second case the active range is #1..#2-1.  Thus, X=7500 would result

in an active range of 0 to 7499, inclusive.  And X=500..7000 would

result in an active range of 500 to 6999, inclusive.  If "..#" is

present, spaces may NOT surround "..".  However, spaces MAY surround

"=" as in all other options.

This option is only necessary when it is not possible to determine

the digitizer size automatically or when it is not desired to use the

entire digitizer surface as the active mouse area.  This option can be

used to create nonsymmetrical margins.  Normally the MARGIN parameter

is used to specify symmetrical margins.  The MARGIN value is added to

the minimum and subtracted from the maximum horizontal range.  However,

if the digitizer tablet's physically active area was X=0..7500 then

specifying X=1000..4000 would give a left (or right) margin of 1000

and a right (or left) margin of 3500 (some tablets may physically

reverse left and right).

Y=[#..]#

This option specifies the active vertical range for a digitizer

tablet.  It may be specified as:

Y=#

Y=#..#

In the first case, the active range is assumed to be 0..#-1 and in the

second case the active range is #1..#2-1.  Thus, Y=7500 would result

in an active range of 0 to 7499, inclusive.  And Y=500..7000 would

result in an active range of 500 to 6999, inclusive.  If "..#" is

present, spaces may NOT surround "..".  However, spaces MAY surround

"=" as in all other options.

This option is only necessary when it is not possible to determine

the digitizer size automatically or when it is not desired to use the

entire digitizer surface as the active mouse area.  This option can be

used to create nonsymmetrical margins.  Normally the MARGIN parameter

is used to specify symmetrical margins.  The MARGIN value is added to

the minimum and subtracted from the maximum horizontal range.  However,

if the digitizer tablet's physically active area was Y=0..7500 then

specifying Y=1000..4000 would give a bottom (or top) margin of 1000

and a top (or bottom) margin of 3500 (some tablets may physically

reverse up and down).

-------------------------------- Testimonials --------------------------------

RODENT.SYS (with a Logitech at 400 DPI) is a real pleasure to use.  Due

to a bad SIMM for the last few days I have gone from 20Megs RAM to 4Megs.

2.1 is limping, no, crawling along, but is very visibly more responsive

than with the IBM driver.  You may quote me. -- Allen Koppelman

--------------------------------- What's New ---------------------------------

Release  ..Date..  ..........................Changes..........................

1.00   93/07/12  First general availability release.  Support added

for PS/2 mice, Inport mice, Felix mice, Summagraphics

digitizers and CalComp digitizers.  Many new PROTOCOL

options added.  The PORT, IRQ, RESPONSE, ORDER, UART,

RATE, X, Y, MARGIN and PRESSURE options were added.

The COM option was split into the COM and PORT/IRQ

options to allow a consistent format for each option.

The MOUSE=X option was replaced by the MOUSE=* option

to avoid a conflict with the X option.

Initialization error fixed which caused the ordering of

statements in CONFIG.SYS to be important for systems

using Lan Server or NetWare.

Uart testing affected some sensitive mice.  This was

not a bug but caused some sensitive (older) mice to

lock up.  The uart testing was changed to reduce the

chances of this problem and the UART option was added

to assist with these mice by completely avoiding uart

testing.

0.90   93/04/07  Beta of initial release with digitizer support disabled.

-------------------------------- Coming Soon ---------------------------------

No explicit plans have been made at this time.  However, some new features

under consideration are...

An installation program (sending in the options you use will facilitate

this feature by providing the necessary information)

A Presentation Manager program to allow changing options "on the fly"

Possible support for more than three mouse buttons to "kick off" a program

or for a button to automatically generate a single or double click (these

features might require a Presentation Manager program to be active at all

times)

Support for additional mice

Better "hot plug" support

However, none of these new features will be possible unless RODENT.SYS is

used by a sufficient number of people who register their copies.  So PLEASE

register!  If you have features you would like to see please send in your

suggestions when you register (or at any time, for that matter).

------------------------------------------------------------------------------