QUADRIVE

Copyright 1993 Ronald Q. Smith

Shareware $10 (Source license $25)

Quad-density diskette device driver for IBM PC/AT and compatible computers.

QUADRIVE supports 720KB and 1.44MB 3 1/2" diskette drives without requiring a

new controller or BIOS.  Use your 1.2MB 5 1/4" diskette controller and

existing BIOS.  Just add your new diskette drive.

QUADRIVE also adds support for a 720KB format using standard 360KB floppy

disks in your 1.2MB drive.  QUADRIVE allows you to read, write, format, and

use most DOS compatible utilities as if your 1.2MB drive was a 3 1/2" 720KB

drive.  This is the diskette format used by the Tandy model 2000 and a

few other vendors, so you can interchange diskettes between those systems and

your new system.  It may also be useful for making exact copies of 720KB

3 1/2" diskettes when you have one 3 1/2" drive and the 1.2MB drive.  If you

have a system with a true 720KB 5 1/4" drive (e.g., from a Tandy system),

QUADRIVE will allow you to use that drive with standard DOS functions.

DEFINITION OF SHAREWARE

Shareware distribution gives users a chance to try software before buying it. If

you try a Shareware program and continue using it, you are expected to register.

Individual programs differ on details -- some request registration while others

require it, some specify a maximum trial period. With registration, you get

anything from the simple right to continue using the software to an updated

program with printed manual.

Copyright laws apply to both Shareware and commercial software, and the copy-

right holder retains all rights, with a few specific exceptions as stated below.

Shareware authors are accomplished programmers, just like commercial authors,

and the programs are of comparable quality. (In both cases, there are good

programs and bad ones!) The main difference is in the method of distribution.

The author specifically grants the right to copy and distribute the software,

either to all and sundry or to a specific group. For example, some authors

require written permission before a commercial disk vendor may copy their

Shareware.

Shareware is a distribution method, not a type of software. You should find

software that suits your needs and pocketbook, whether it's commercial or

Shareware. The Shareware system makes fitting your needs easier, because you can

try before you buy.  And because the overhead is low, prices are low also.

Shareware has the ultimate money-back guarantee -- if you don't use the product,

you don't pay for it.

DISCLAIMER - AGREEMENT

Users of QUADRIVE must accept this disclaimer of warranty:

"QUADRIVE is supplied as is.  The author disclaims all warranties, expressed or

implied, including, without limitation, the warranties of merchantability and of

fitness for any purpose.  The author assumes no liability for damages, direct or

consequential, which may result from the use of QUADRIVE."

QUADRIVE is a "shareware program" and is provided at no charge to the user for

evaluation.  Feel free to share it with your friends, but please do not give it

away altered or as part of another system.  The essence of "user-supported"

software is to provide personal computer users with quality software without

high prices, and yet to provide incentive for programmers to continue to develop

new products.

If you find this program useful and find that you are using QUADRIVE and

continue to use QUADRIVE after a reasonable trial period, you must make a

registration payment of $10 to Ronald Q. Smith at the address below.  The

$10 registration fee will license one copy for use on any one computer at any

one time.  For one year after receipt of the this registration fee, you will

recieve notification of the availability of any updates.  If you report a bug

within 3 months of receipt of the registration fee, you will receive a

correction to the problem or the fee will be refunded to you.  For a $25

registration fee you will receive the latest version including a copy of the

source code.  You will also automatically receive any updates for one year.

If you license the source code, you may use it to maintain your copy of

QUADRIVE and as a source of information.  You may not use any or all of it in

your own products without first obtaining authorization from Ronald Q. Smith.

You must treat this software just like a book.  An example is that this

software may be used by any number of people and may be freely moved from one

computer location to another, so long as there is no possibility of it being

used at one location while it's being used at another.  Just as a book cannot

be read by two different persons at the same time.

Commercial users of QUADRIVE must register and pay for their copies of

QUADRIVE within 30 days of first use or their license is withdrawn.

Site-License arrangements and volume discounts may be made by contacting

Ronald Q. Smith.

Anyone distributing QUADRIVE for any kind of remuneration must first contact

Ronald Q. Smith at the address below for authorization.

Send fees and any inquiries to:

Ronald Q. Smith

11 Black Oak Road

St. Paul, MN 55127-6204

You may also contact me via CompuServ mail at userid 71620,514.  I will be

happy to respond to any problems and suggestions for future capabilities.

You are encouraged to pass a copy of QUADRIVE along to your friends for

evaluation.  Please encourage them to register their copy if they find that

they can use it.

USING QUADRIVE

QUADRIVE supports 3 1/2" drives exactly as a computer with a modern BIOS

would.  Anything that DOS and Windows does is fully supported.  Programs that

bypass DOS and use the BIOS directly, as some vendors' backup and  format

programs do, will probably not work correctly.

QUADRIVE allows you to use any mixture of 160KB, 180KB, 320KB, 360KB, 720KB,

and 1.2MB formats on a 5 1/4" 1.2 MB drive.  The new 720KB format is identical

in all respects except pysical size to the 3 1/2" 720KB format.  In fact, DOS

utilities such as FORMAT and DISKCOPY will believe that it is a 3 1/2" 720KB

media.  QUADRIVE will also allow you to read and write your Tandy and DEC

720KB diskettes that your new system doesn't support.  These vendors and

probably several others supported a 720KB capacity on 5 1/4" diskettes.  That

is the capacity that QUADRIVE supports.

Unfortunately, limitations in the DOS FORMAT command mean that you can only

format in the 720KB mode when the drive is configured so that FORMAT will not

allow formatting in any of the other modes.  Reading and writing are not

affected, only formatting.  The DOS FORMAT command "knows" what media formats

are possible for each drive type and will not believe that the 720KB format

is possible on a drive that also supports the other formats.

Note that a DOS device driver is only effective when all access to the device

is via DOS.  This includes both files and the drive oriented interfaces such

as INT 25H and INT 26H.  Any application that uses the BIOS directly, such as

many backup utilities, will not be able to use QUADRIVE.  DOS BACKUP and

RESTORE will, however, work correctly.  Non-DOS formatters such as Central

Point Software's PCFORMAT will not correctly support this capacity.  They

make direct calls to the BIOS and so don't take advantage of QUADRIVE's BIOS

helper that tells the BIOS how to handle the 720KB format.

NOTE:  If you use a disk cache or other resident program that intercepts calls

to the BIOS, you must instruct them to not cache or otherwise process the

QUADRIVE devices.  Tell the cache to exclude the QUADRIVE device just as you

would tell it to exclude a RAM disk.  The cache manager will not know how to

correctly tell the BIOS to access the QUAD drive.

The 720KB capacity is inherent in the 1.2MB disk controller and in all 360KB

floppy media.  No drive or media specifications are being exceeded.  This is

just a format that Microsoft and IBM chose not to support.

The driver is configured in CONFIG.SYS with the statement:

DEVICE=QUADRIVE.SYS /D:d1 [/F:t1] [/D:d2] [/F:t2] [/D:d3] [/F:t3] [/D:d4]

[/F:t4]

QUADRIVE will support up to four devices concurrently.  The parameters d1

through d4 are physical device numbers for which you want support.

They must represent drives attached to a 1.2MB controller.  These are the

physical drive numbers 0 and 1 and not DOS drive letters A:, B:, etc.  Thus

if you have a single floppy drive, you would specify DEVICE=QUADRIVE.SYS /D:0.

The parameters t1 through t4 are the drive types that you want DOS to see.

These are the same choices that you would use on a DRIVPARM or

DEVICE=DRIVER.SYS statement in CONFIG.SYS.  That is, "1" or "1200" defines a

1.2MB drive that also supports reading and writing 720KB diskettes using

360KB media.  "2" or "720" may be used to specify either a real 720KB drive

or a 1.2MB drive on which you want to be able to format 720KB capacities.

"7" or "1440" may be used to specify a 1.44MB drive that supports the 720KB

and 1.44MB capacity diskettes.

You may repeat the same physical device number two or more times.  This will

assign multiple drive letters to the same drive just as DOS assigns both A:

and B: to drive 0 if you don't have a drive 1.  You will be prompted to change

diskettes with the familiar DOS message when you change the drive letter.

Be sure that you have LASTDRIVE set large enough to accommodate the new drive

letters that will be created.  If you have one or two floppy disks, a hard disk,

and a RAM disk, the default value for LASTDRIVE of E will allow you to add

QUADRIVE for one of the floppy drives.

Once QUADRIVE is installed there will be some additional floppy drive letters

available to you (e.g., D: through G:).  QUADRIVE will display messages at

boot to tell you what drive types it is supporting for each drive letter.

EXAMPLES:

DEVICE=QUADRIVE.SYS /d:1 /f:7 /d:0 /f:2

This installs QUADRIVE with two new drive letters.  If you have the two

floppy drives plus a hard disk but no other drives or RAM disks installed,

QUADRIVE will respond with:

Physical unit 1, type 7, supported as drive D:

Physical unit 0, type 2, supported as drive E:

Physical unit 1 must be a 1.44MB 3 1/2" drive (or something compatible).  It

will be attached to your system in the spot normally reserved for the B:

drive.  Both the 1.44MB and 720KB media will be fully supported and both may

be formatted as well as read and written.  You will have to use:

FORMAT D: /F:720

to format a 720KB diskette as FORMAT will otherwise default to the 1.44MB.

You SHOULD NOT configure your system SETUP (CMOS) for a second floppy disk.

This will only confuse DOS (and probably yourself at some point).

Physical unit 0 is (we assume) a 1.2MB 5 1/4" diskette drive.  When you use

drive letter E: to refer to it, it will read and write all of the previously

supported 5 1/4" diskette formats (160KB, 180KB, 320KB, 360KB, 1.2MB) as well

as the new 720KB format.  It will only format at 720KB.  When you use drive

letter A: or B: to refer to it, it will only read and write the previously

supported 5 1/4" diskette formats, but it will be able to format them.  Be

careful when switching back and forth between drive letter E: and drive

letters A: and B:.  DOS will not realize they refer to the same drive and

will not display a message and give you time to change diskettes.

DEVICE=QUADRIVE.SYS /d:1 /f:2

Physical Device 1, type 2, is supported as drive D:

Your second floppy disk drive is either a 1.2MB 5 1/4" drive or is a 720KB

3 1/2" diskette.  In the first case you may want to configure it via SETUP as

well so that DOS assigns drive letter B: to it.  If it is a 1.2MB drive, the

same rules apply as for the first example.  If it is a 720KB 3 1/2" drive,

you will not want to configure it in SETUP.  You will only be able to read

and write in the 720KB format (that's all the drive supports).

Since QUADRIVE will let you read and write all of the existing 5 1/4" formats

as well as the new 720KB format, it may occur to you to wonder why it doesn't

simply add that capability to drive letter A: (or B:).  There are actually

two reasons.  One is that DOS does not define any way for a device driver to

redefine any of the standard drives (usually A: through C:).  It is possible

only by using undocumented interfaces and we want to avoid that in order to

retain the greatest level of compatibility.  The more important reason is that

FORMAT.COM is just too smart for its own good.  FORMAT "knows" that you can't

mix those capacities on the same drive.  If it thinks the drive is a 1.2MB

drive, it will not let you select the 720KB capacity and vice versa.  This is

built right into FORMAT and we really don't want to require a non-standard

FORMAT program when simply using an additional drive letter avoids it.

So, you can read, write, and perform any other function that doesn't require

formatting using the QUADRIVE drive letter, and you can do it using all of

supported formats.  If you use a function that requires formatting, you must

use the QUADRIVE letter for 720KB diskettes and the DOS A: or B: drive letter

for other formats.  Functions that may require formatting are FORMAT, DISKCOPY,

and BACKUP.  DISKCOPY and BACKUP only require formatting if the target disk is

not already formatted.

You can use DISKCOPY, DISKCOMP, and many other copying programs with your

QUADRIVE drive.  Just use DISKCOPY E: E: or DISKCOPY E: F: to copy from one

floppy to another.  DISKCOPY and others include their own prompting

capability and will notify you when to change diskettes if you use the same

drive letter for both the source and the target.  Due to limitations in

DISKCOPY you may not be able to copy from a 720KB diskette to a previously

formatted 360KB diskette without reformatting the target first.  While

DISKCOPY will normally format the target as it writes if it discovers that the

target is not formatted, it will see the first track on the previously formatted

360KB diskette as correctly formatted.  When it tries to write the second track

and gets an error, DISKCOPY assumes that the diskette is actually damaged and

not just unformatted.  That is, DISCOPY will only format if it is unable to

write the first track, and a 360KB floppy has a correctly appearing first track

in 720KB mode.  If this happens to you, simply format the target floppy with

FORMAT E: (or whatever the QUADRIVE drive letter is) and then repeat your

DISKCOPY.  The same rule applies to BACKUP.

TECHNICAL STUFF

The earliest floppy drives supported one or two sides, 40 tracks, and 8 or 9

sectors per track for capacities of 160KB, 180KB, 320KB, and 360KB.  The

read/write heads in the drives required a wide data area and a wide gap (by

today's standards) for each track.  As the technology for heads and media

improved, the width of the data area and gap were reduced to half the earlier

value.  When a modern 360KB drive reads or writes a floppy disk, it leaves a

very wide gap between the tracks.  It is basically using every other track on

the device even though it calls them track (cylinder) 0, 1, etc.

By the mid-1980s several PC vendors were using the 720KB format.  These

included some DEC and Tandy systems.  They simply made small changes to the

drive controller and the BIOS to use every track.  When they had to handle an

"old" 360KB floppy, the BIOS simply doubled the track (cylinder) number

requested.  This format never caught on generally, but QUADRIVE will

successfully handle most of those old diskettes.

When the 1.2MB drive was introduced, it supported the use of all the tracks

on the drive and higher density media that permitted 15 sectors per track.

Like the other implementations, when presented with a 360KB diskette,

the BIOS simply doubles the track number for all operations.  This then

provides the opportunity for QUADRIVE to use those intervening tracks.

The IBM PC/AT BIOS defined a state machine for its floppy BIOS.  As part of

the implementation, the BIOS attempts to sense the format of the media and

respond properly without you having to tell it in advance what capacity you

are using. The BIOS keeps a value that tells it what kind of media it is

currently handling.  If you insert a new diskette, the BIOS sets the media

type to unknown.  Whenever the BIOS handles a request for an "unknown" media,

it first tries to use the 360KB format.  If that gets an I/O error, it tries

the 1.2MB format.  Whichever one succeeds causes the BIOS's state for the

drive to be set to "determined" and the media type.  The media type state is

actually a bit mask which tells the BIOS what transfer rate to use, whether

to double track numbers, and how many sectors per track to permit.

The BIOS only has two values that it places in the state.  These are a 1.2 MB

media type, 15 sectors, and no doubling or 360KB media type, 9 sectors, and

doubling.  QUADRIVE takes advantage of this design to create another state

value that looks identical to the BIOS 360KB value except that the doubling

bit is not set.  As long as QUADRIVE sets this value in the state before

every I/O operation, it all works.

The reason that you need a full device driver rather than just a little TSR

that sets the state is that you want to be able to continue to use the 360KB

and 1.2MB formats.  With a device driver, DOS tells QUADRIVE what format it

is expecting and QUADRIVE knows whether to set the state or not.  This is the

technique that allows you to use drive letter A: for 1.2MB and 360KB formats

and drive letter E: for 720KB formats.

Actually, QUADRIVE will automatically adjust for reading and writing all

media formats from 160KB, 180KB, 320KB, and 360KB to 720KB and 1.2MB.

However, you cannot FORMAT anything other than the 720KB with the QUADRIVE

drive letter.  Unfortunately, while DOS would happily allow QUADRIVE to

handle all of these and does, the FORMAT program itself has an error check

which says that the 720KB format cannot be used on the same drive as the

other formats.  It is trying to keep you from accidentally asking to format a

true 3 1/2" diskette at 360KB or 1.2 MB or vice versa.  Thus it will display

an error message and not even try.

Basically, a few programs like FORMAT ask the device driver what physical type

of drive they support and will only allow the capacity options that they know

about for those physical types.  In order for QUADRIVE to get FORMAT to work

at all at 720KB on a 5 1/4" drive, it must always respond that this is a

3 1/2" drive type.  FORMAT then refuses to accept a request for any of the

other capacities.

QUADRIVE will only work with those BIOS implementations (IBM, Mitsubishi,

Phoenix, etc.) that compatibly implement the multi-media state machine

described in the IBM PC/AT Technical Reference manual.

When you use QUADRIVE to handle a real 3 1/2" diskette drive, you don't have

these problems with FORMAT because you aren't asking it to do anything it

thinks it shouldn't.  The only limitation is that you must tell FORMAT what

capacity diskette you have installed if it isn't the default.  That is, you

must tell it to format 720KB on a 1.44MB drive in order to do so.  Most real

3 1/2" capable controllers can automatically determine the diskette media

type and the BIOS can then tell DOS what type of diskette is inserted.  Since

we are using a 5 1/4" controller that doesn't have this capability, we have

to tell FORMAT what diskette type is inserted.  Otherwise, it will try to

format at 1.44MB and you will get an error.

INSIDE QUADRIVE

If you choose to license the source code, you will see that it is fully

annotated with an explanation of what DOS expects from a device driver and

the mechanics of the BIOS state machine.  In writing QUADRIVE, I discovered

that the DOS documentation is often incomplete or misleading and is actually

wrong in a few places.  It takes a lot of trial and error and time spent with

a debugger to determine what DOS actually wants in all cases.

QUADRIVE is written in Microsoft MASM 6.00.  You would need that version of

the assembler to use the source code, but you can use it as a source of

information without anything more than an understanding of assembler code and

a copy of a DOS technical reference manual such as the "Microsoft MS-DOS

Programmer's Reference".  In spite of my comment about its incompleteness, it

is an invaluable source of information for anyone writing system-level

programs.  I also recommend "Undocumented DOS" by Andrew Schulman, Raymond J.

Michels, Jim Kyle, Tim Paterson, David Maxey, and Ralf Brown.  It does not

have as much information on device drivers but the insight into the inner

workings of DOS were very valuable in trying to figure out what DOS really

wants.

IBM is a registered trademark of International Business Machines Corporation.

MS-DOS and Microsoft are registered trademarks of Microsoft Corporation.

REVISION HISTORY

931128  -       Version 3.00:  Added support for real 3 1/2" diskette drives.

Also fixed a bug in handling device parameters that sometimes

led to FORMAT reporting an error or other programs not

being able to read or write the diskettes following a FORMAT.

911006  -       Version 2.10:  Added support for up to four drives.  Added

support for multiple drive letters for a drive including

generating diskette change messages.

910927  -       Version 2.00:  Updated for DOS 3.3 through 5.0.  Use 1.2MB

drive to support 720KB.

850612  -       Version 1.10:  Added support for 720KB drives on Sperry PC-1

and PC/HT.

840819  -       Version 1.00:  Original version.  Support 720KB drive on IBM

PC-1 under DOS 2.10.