�����������������������������������������������������������������������������Ŀ

�                       �����ۿ �ۿ  �ۿ  ���ۿ  �����ۿ                      �

�                       ������� ��ۿ��۳ �����ۿ �������                      �

�                       ���ۿ   ������۳ �����۳   �۳                        �

�                       �����   ������۳ �����۳   �۳                        �

�                       �۳     �۳���۳ �۳ �۳   �۳                        �

�                       ���     ���  ��� ��� ���   ���                        �

�                     FMAT V6.5, A Simple Disk Initializer                    �

�                     Copyright (C) 1991-94 by Chris Buijs                    �

�������������������������������������������������������������������������������

I had the need for a SMALL and FAST format utility which could be included

in some archives i use for my BBS without increasing the size of the archive

to much, so i created one, after making a couple of versions it is grown to

a very usefull utility with more capability's i first planned....

FMAT is FREEWARE, it may be freely copied as a complete package.

�����������������������������������������������������������������������������Ŀ

� ���� Disclaimer ����������������������������������������������������������� �

�������������������������������������������������������������������������������

FMAT is supplied as is. The author disclaims all warranties, expressed

or implied, including, without limitation, the warranties of

merchantability and of fitness for any purpose.

The author assumes no liability for damages, direct or consequential, which

may result from the use of FMAT.

Feel free to share FMAT with your friends, but please do not give it away

altered or as part of another system.

In commercial environments FMAT may only be used when the author has

given his explicit permission.

�����������������������������������������������������������������������������Ŀ

� ���� Requires ������������������������������������������������������������� �

�������������������������������������������������������������������������������

FMAT requires the following:   XT or AT computer.

128 KB free memory.

MS/PC-DOS V4.0 or higher.

A standard floppy-drive.

�����������������������������������������������������������������������������Ŀ

� ���� Usage ���������������������������������������������������������������� �

�������������������������������������������������������������������������������

FMAT is simple to use, it uses the same parameters as the DOS format

utility, FMAT only formats DISKETTES, all formats on different drives

or diskettes are allowed, so you can format a 360 kb 3.5-Inch floppy in

a 1.44 MB drive if you want, this can very handy when you have a 1.2 MB

drive and must convert a disk-file to disk, and the disk-file represents

a 720 KB disk.

Syntax:  FMAT <Drive> [Parameters]

Parameters:  /1         - Format a single side of a disk.

/2         - Format both sides of a disk.

/4         - Format a 5.25-inch 360 KB disk.

/8         - Format 8 sectors per track.

/C:Sects   - Specifies the number of sectors per cluster.

/E         - Erase disk, all tracks will be cleared.

/F:Size    - Specifies the size of the disk to format.

/I:Interl  - Specifys sector-interleave.

/M         - Multiple disk formatting.

/N:Sects   - Specifies the number of sectors per track.

/Q         - Perform a quick-format.

/R:Entrys  - Specifies the number of entrys in root-directory.

/S         - Create a bootable disk.

/T:Tracks  - Specifies the number of tracks per side.

/U         - Unconditional format (Batch).

/V[:Label] - Specifies the volume label (Max 11 chars).

/W         - Write Verify, verify every track after format.

/X[:Sects] - Enable sector-shifting (enables drives to

read the formatted disk faster).

More info about some parameters:

/E - This parameter will erase all tracks on the disk by writing zero's

to it, the boot-sector, fat-tables and root-directory will be

re-initialized.

/I - Specifies the sector-interleave, the number of sectors the leave

between following sectors, FMAT calculates automatically which

interleave is the best to use.

/N - Specifies the number of sector per track, by testing we discovered

that the maximum value of 22 can be used before FMAT gets any errors,

this depends on what kind of disk-drive you use.

/Q - Like DOS 5.0 this will do a initialization of the boot-sector, FATS

and root-directory.

/R - Specifies the number of root-entrys, the maximum setting DOS allowes

is 224 entrys.

/T - Specifies the number of tracks per side, by testing we discovered that

the maximum value of 84 can be used before FMAT gets any errors, this

depends on what kind of disk-drive you use.

/W - Turns on write-verify, when you use this, FMAT will write to the

track that was formatted a test-patern and will read the track in,

FMAT will then compare the data read with the data that was written,

when the data doesn't match, a verify error will be displayed.

/X - Turns on sector-shifting, when you enable this, FMAT will shift the

starting sector each track it formats, this will speed up

read-operations of the formatted disk, some explaination:

When a drive wants to read some tracks, it first wants to find sector

one of the first track, when it is found the track is read in and the

heads are moved to the second track, because the disk is still

spinning, the head is not positioned at sector one of the second

track, but on the third or fourth (depends what kind of drive you

have and how fast it is), so the first sector must be found again.

FMAT can shift the sectors so that when the head is positioned

onto the next track it can find sector one immidiate, this will

speedup disk-reading at the most 50% !!!

By tests we discovered that a sector-shift of 3 or 4 sectors will

speedup the most drives (both 3.5 and 5.25 inch drives), we recommed

to use a sector-shift of 3 for 5.25 inch drives, and a sector-shift

of 4 for 3.5 inch drives.

NOTE: on some drives sector-shifting will SLOW-DOWN your drive, you

are on your own to test this.

�����������������������������������������������������������������������������Ŀ

� ���� FMAT Environment variable �������������������������������������������� �

�������������������������������������������������������������������������������

You can use the environment variable FMAT to define the default-settings

at startup, for example, is you always format drive A: at 720 KB and want

to do it quick, you can include for example in your AUTOEXEC.BAT file:

SET FMAT=A: /F:720 /Q

The only thing you'll ever have to do is typing in FMAT followed by an

ENTER and FMAT will format drive A: quickly at 720 KB.

NOTE: The parameters you use at the command-line ALWAYS overrule the

parameters you use in the FMAT environment variable.

�����������������������������������������������������������������������������Ŀ

� ���� NON-Bootable Disks ��������������������������������������������������� �

�������������������������������������������������������������������������������

When you create a NON-Bootable disk, FMAT will create a special boot-sector

on it, when you boot from a non-bootable disk created by FMAT, it will

automatically boot from drive C: if possible, when this is not possible,

the standard message is displayed.

�����������������������������������������������������������������������������Ŀ

� ���� Formats �������������������������������������������������������������� �

�������������������������������������������������������������������������������

The following table shows the combination of parameters that can be used

to format diskettes with a higher capacity, not all combinations are shown,

but this ones most certainly work.

FMAT automatically adjust the gap-size between sectors so that all sectors

can fit into a track, the sector values in this table are not the maximum

values but the reliable ones, use the trial/error method to find out your

maximum settings.

Size    Tracks  Sectors  Drive   DiskType  Parameters

�������  ������  �������  �����   ��������  �����������������������������

360  KB    40       9     12345   5� DS/DD  /F:360

369  KB    41       9     12345   5� DS/DD  /F:369

400  KB    40      10     12345   5� DS/DD  /F:400

410  KB    41      10     12345   5� DS/DD  /F:410

720  KB    80       9     .2345   3� DS/DD  /F:720

738  KB    82       9     .2345   3� DS/DD  /F:738

800  KB    80      10     .2345   3� DS/DD  /F:800

820  KB    82      10     .2345   3� DS/DD  /F:820

1200 KB    80      15     ..345   5� DS/HD  /F:1200

1230 KB    82      15     ..345   5� DS/HD  /F:1230

1280 KB    80      16     ..345   5� DS/HD  /F:1280

1312 KB    82      16     ..345   5� DS/HD  /F:1312

1440 KB    80      18     ...45   3� DS/HD  /F:1440

1476 KB    82      18     ...45   3� DS/HD  /F:1476

1600 KB    80      20     ...45   3� DS/HD  /F:1600

1640 KB    82      20     ...45   3� DS/HD  /F:1640

1680 KB    80      21     ...45   3� DS/HD  /F:1680

1722 KB    82      21     ...45   3� DS/HD  /F:1722

2880 KB    80      36     ....5   3� DS/EHD /F:2880

Drive:  1 = 360 KB Drive

2 = 720 KB Drive

3 = 1.2 MB Drive

4 = 1.44 MB Drive

5 = 2.88 MB Drive

�����������������������������������������������������������������������������Ŀ

� ���� Error Codes ���������������������������������������������������������� �

�������������������������������������������������������������������������������

When FMAT is running, it can happen that FMAT aborts with the message:

==== (Type) ERROR: nnn

This can happen when a bad diskette is used or something like that, the

message wil return a number, following is a list with explaination of

most common diskette-drive codes:

Code Explaination

���� ����������������������������������������������������������������������

1  Bad command, FMAT works only on DISKETTE drives.

2  Address mark not found, a bad diskette is used.

3  Diskette is write-protected.

4  Sector not found.

8  DMA Overrun, Check memory manager or cache.

9  DMA Across 64K Boundery, Check memory manager or cache.

13  Unsupported track (strange floppy formats like Amiga, atari etc.).

16  Bad CRC/ECC, bad diskette.

32  Controller failure, something is wrong with your controller.

64  Seek failed, FMAT couldn't verify a track or sector.

128  Time-Out, controller or drive doesn't respond fast enough.

-127  No diskette present in drive.

When an error occurrs, FMAT wil also exit with an errorlevel, the

following list explains the errorlevels:

Code Explaination

���� ����������������������������������������������������������������������

0   No error.

1   Incorrect DOS-Version (4.0 or higher needed).

2   Invalid drive-specification.

3   Drive-Type not supported.

4   Invalid parameter used or wrong syntax.

5   Error on system-files.

6   Error copying system-files.

7   General disk-error.

�����������������������������������������������������������������������������Ŀ

� ���� Support �������������������������������������������������������������� �

�������������������������������������������������������������������������������

For bug-reports, suggestions or other things concerning FMAT, you can

write me a letter at the followin address:

C.Buijs

De Spil 22

1507 CX Zaandam

Netherlands

Or phone to my BBS:

Action Board Zaandam

Phone: +31-(0)75-704890

Speeds: 1200,2400 HST MNP V42 V42Bis 14.400

Or send me Email, FidoNet   : 2:280/407.16

InterNet  : 100117.3242@compuserve.com

Compuserve: 100117,3242

Have Fun...