**********                      ***********

***   ********   ********     ***     ********  *******   ***    ****

***    ***        ***          ***     ***       ***  ***  ****  *****

***     ***        ***          ***     ***       ***  ***  *** *******

***      ******     ******       ***     ******    *******   ***  ** ***

***       ***        ***          ***     ***       ******    ***     ***

***        ***        ***          ***     ***       *** ***   ***     ***

**********  ********   ********     ***     ********  ***  ***  ***     ***

Version 0.92

Written by Per Hansen, 1994

Copyright (C) 1994 by Tribune Channel Software

A telecommunucations tool for the Amiga personal computer.

DISCLAIMER

--------------

No  warranties  are  implied  or  expressed with regard to the fitness or

merchantability  of  ZeeTerm  for  any  particular  purpose.   All  risks and

damages,  incidental  or  otherwise,  arising  through  the  use or misuse of

ZeeTerm are entirely at the responsibility of the user.

While  considerable  effort  has been made to provide you with a reliable

product,  there  is  no  guarantee that this program is 100% "bug-free".  Any

problems  that  do arise will be corrected when and if feasible.  Maintenance

releases  will  be  made  available in order to correct potentially hazardous

malfunctions within the program.  Fixes for benign problems, usually cosmetic

in  nature,  will  most  likely  be  deferred until the next major release of

ZeeTerm.  This disclaimer does not guarantee future versions of this product,

only  a  prescribes  method  of  dealing  with  possible  errors,  and  their

corrections, to this program.

LICENCE

-----------

ZeeTerm  is a copyrighted product made available to you under the concept

of  Shareware.   It  has  never  been,  and will never be, a Public Domain or

otherwise  "free"  program,  therefore, you must obtain a license in order to

rightfully use it.

You  are  free  to  use  ZeeTerm  in it's unregistered mode for a limited

period  of thirty days after initial usage, but will not be able to make full

use  of  all the functions within the program until it is registered.  All of

the   save   functions  and  "Read  configuration",  doesn't  work  prior  to

registration.   Please  consult  the  registration  form  included  for  more

information.

*Please*  don't  try to crack ZeeTerm.  Considerable effort has been made

developing this program.  The registration fee isn't that expensive, is it?

DISTRIBUTION

----------------

Distribution  is  limited  to the original shareware distribution archive

file  only.  None of the member files may be omitted.  Furthermore, no profit

or other material gain may be realized for distributing ZeeTerm with the sole

exception   of  recovery  for  media  and  postage.   Bundling  ZeeTerm,  the

distribution  archive  or  any  member  files, with any commercial product is

expressly prohibited, without the authors written permission.

ACKNOWLEDGEMENTS

--------------------

Ward Christensen for creating the Xmodem protocol.

Chuck Forsberg for his X/Ymodem and Zmodem protocol specifications and

for his public domain rz.c, sz.c & zm.c source files.

Andrew Milner, Chris Irwin and Joaquim H. Homrighausen for the EMSI/IEMSI

protocol definitions.

Thomas Brobech, Jorn Bang-Andersen and Jesper Kehlet for betatesting and

moral support.

Lars Dam for help during debugging.

Anyone else forgotten.

CONTENTS

------------

The distributed archive should contain the following files:

ANSITest.TXT        - An ascii file containing ansi control sequences.

Used in the Test.zt arexx script.

Changes.TXT         - Text file containing information on changes from

previous releases of ZeeTerm.

DEVS/ZT             - Dosdriver for use with WorkBench 2.10 and above.

DEVS/ZT.info        - Workbench icon for ZT.

DEVS/Mountlist      - Mountlist entry for the ZTHandler.

Devs.info           - Workbench icon for the DEVS drawer.

FILE_ID.DIZ         - Description file for the archive, meant to be

used for BBS's.

FONTS/ZeeTerm.font  - Provided font for use with the IBM ANSI and

VT-100 terminal emulations.

FONTS/ZeeTerm/8     - Data file for ZeeTerm.font.

L/ZTHandler         - Device handler for the ZT: device.

Registration.form   - Text file that should be filled out and sent

to me, when you have evaluated ZeeTerm, and

wish to register the program.

REXX/Host.zt        - Example arexx script, that shows how ZeeTerm

easily can be set up to run as a "host".

REXX/Test.zt        - Shows most of the arexx functions in ZeeTerm.

ZeeTerm             - The program itself.

ZeeTerm.CFG         - Configuration file for ZeeTerm. Please note

that this file is kept in pure ascii, and that

you are able to edit this to reflect your own

needs, before starting ZeeTerm the first time.

ZeeTerm.doc         - This file.

ZeeTerm.guide       - AmigaGuide format documentation.

ZeeTerm.guide.info  - Workbench icon for ZeeTerm.guide

ZeeTerm.info        - WorkBench icon for ZeeTerm.

ZeeTerm.keys        - "F-key" file. Contains definitions of the

function keys. Please note that this file is

kept in pure ascii too.

ZT_EMSI.dat         - Example data file for doing an EMSI session with

ZeeTerm.

INSTALLATION

----------------

If you wish to take advantage of the ZTHandler, you should make an

entry in your DEVS:Mountlist, that looks like the one provided in

the DEVS/Mountlist file in this archive. The files named "ZT" and

"ZT.info" are for use with the WorkBench 2.10, and should be copied

to your "dosdrivers" directory.

The "ZTHandler" file in the L drawer, should be copied into the L:

drawer on your system disk.

Please consult section 13 for further information on the DOS HANDLER.

The files in the FONTS drawer should be copied into your FONTS: drawer

on your system disk. The font provided is for use with the IBM ANSI and

VT-100 terminal emulations, and isn't required for the program to work.

The REXX directory contains some sample Arexx scripts, provided as a

guide for you to see how to create Arexx scripts for ZeeTerm. These

files should be copied into your REXX: directory.

Please consult section 11 for more information on AREXX.

Copy the other files to whereever you wish. ZeeTerm doesn't require

an assigned pathname, but reads the default configuration file from the

current directory. See section 2.3 for information about this.

ZeeTerm 0.92 Users Manual                               Electronic Edition

1 OVERVIEW

1.1 Display features

-   ZeeTerm automatically adjusts itself to either NTSC or PAL

displays. Interlace and overscan is supported as well.

-   Terminal emulations for TTY (dumb terminal), IBM ANSI

and VT-100.

-   An optional status line with display of current time and the

time ZeeTerm has sensed a carrier.

-   40 user definable function key macros per macro file. Each

macro can contain a maximum of 100 characters. Some macro

primitives are available.

-   Optimized scrolling for boosting throughput of multiple

bit-plane displays.

-   Optionable "Screen-memory", so that you can select characters

simply by pointing at them with the mouse pointer and clicking.

If you then move the mouse in the X-direction, the characters

appearing under the mouse pointer will be sent to the modem.

-   User definable font available.

-   User definable keymap available.

1.2 File transfer protocols

-   Xmodem CRC, Xmodem Checksum, Xmodem-1k, Ymodem, Ymodem-g,

Zmodem and ASCII file transfer protocols are present.

All implementations adhere strictly to their respective

specifications.

-   Zmodem supports auto-downloads and auto-uploads as well as

other of the Zmodem standard specifications.

1.3 Phonebook

-   Ability to generate a unique password for each directory

entry.

-   An intelligent dialer that monitors the result of each attempt

and is capable of multiple entry dialing.

-   Optional use of traplist.library to find telephone numbers in

the FidoNet node list.

-   IEMSI implementation for fast logons on systems that supports

this feature.

1.4 Miscellaneous

-   A powerful FILE requester that maintains several lists of

files, devices and directories rather than hardcoded selections

that may not exist on your system, or even worse,

may not include devices/volumes unique to your environment.

-   A batch selection facility which enables you to select multiple

files for a batch file transfer.

-   ZeeTerm detach itself from the CLI/Shell it is started from.

-   User adjustable task priority.

-   User definable serial device for third party multi-port boards.

-   EMSI implementation for use together with FidoNet systems.

The EMSI implementation supports the special FidoNet file

transfer protocols ZedZip and ZedZap.

-   Special dos-handler which enables you to send ascii text

directly from a background CLI/Shell via ZeeTerm to the

serial port.

-   Arexx implementation with more than 50 different commands.

In fact there are so many, that you might set up an entire

bulletin board system (although it would operate quite slow).

-   Special IEMSI Split-screen chatmode with "word-wrap", that is

accessible from the pulldown menus as well.

-   Possibility to specify another configuration file than the

default, by giving ZeeTerm the configuration file as an

argument, or by adding "CONFIG=NewConfig.CFG" in the

Workbench icon.

2 SETUP

2.1 System requirements

Version 1.3 Workbench or later is required.

To fully evaluate all of the features ZeeTerm provides, version

2.0 Workbench or later is recommended.

Although ZeeTerm can be run in a 512k system, 1Meg is highly

recommended to take advantage of several features ZeeTerm

incorporates.

A Hayes compatible modem is recommended if you plan to utilize

all of the advanced capabilities within ZeeTerm.

Your modem must be able to return extended result codes, such as

"CONNECT 2400", in order to take advantage of the intelligent

dialer. Finally, your modem must have the ability to sense the

Data Terminal Ready (DTR) signal so that ZeeTerm can hangup the

modem very quickly.

ZeeTerm was written with high-speed modems in mind. Tests

have been conducted locally during development with a 14.4kbps US

Robotics HST/V42bis and with nullmodem cables between two Amigas.

The discussion that follows assumes a certain level of

familiarity with the AmigaDOS CLI. "The AmigaDOS Manual", 2nd.

edition, from Bantam Computer Books is a valuable book to have

and is highly recommended.

2.2 ZeeTerm data files

All datafiles except currently the phonebook datafile, are kept

in pure ASCII format. You may edit the default files, or use the

'Save' options from within ZeeTerm.

Although the phonebook datafile are saved in binary format, ZeeTerm

is capable of reading an ASCII phonebook file, if you so desire.

Everywhere in the datafiles, the "\" character stands for a carriage

return, and the "|" character stands for a line feed.

The only file that has a preset name, is the ZeeTerm main

configuration data file. - Within this file, you may specify all

the names of the other data files used in ZeeTerm.

2.3 File search logic

ZeeTerm will first attempt to load its initial default parameters

from a file which is hardcoded with the filename "ZeeTerm.CFG".

This file must reside in ZeeTerm's current directory. This is the

ONLY defaults filename that ZeeTerm will look for when it is started.

You can change this when starting ZeeTerm from CLI only, by

specifying your alternative configuration file as the first

argument, e.g.: "ZeeTerm MyCfg.CFG".

An alternative configuration file may be specified when starting

ZeeTerm from Workbench too. Add the "CONFIG" keyword to the list

of tooltypes in the Workbench icon, and specify the filename of

your alternative configuration file, e.g.: "CONFIG=MyCfg.CFG".

The current directory that ZeeTerm was started from will be

searched first for the "ZeeTerm.CFG" file. ZeeTerm will then

attempt to find the files defined in this file for the phonebook,

macros, keymap and a diskfont (if a diskfont is chosen).

2.4 The ZeeTerm.CFG configuration file

The ZeeTerm.CFG defaults parameters file is kept in pure Ascii, and

may therefore be edited in your favorite text editor.

It doesn't need to be there when you start ZeeTerm, in which case

it will use its own internal defaults.

All path definitions must be ended with a "/" unless otherwise

stated.

Any parameter may be omitted, if you prefer the internal defaults

instead of your own defined defaults.

The parameters are discussed below.

2.4.1 Macrofile ZeeTerm.keys

This defines which filename to search for when ZeeTerm search

for the macro-keys definitions.

2.4.2 Phonebook ZeeTerm.phone

This defines which filename to search for when ZeeTerm search

for the telephone phonebook.

2.4.3 Sendpath RAM:

Default send path directory.

2.4.4 Recvpapth RAM:

Default receive path directory.

2.4.5 Captfile ZeeTerm.capt

Default capture filename.

2.4.6 Nodelist ZeeTerm_nodes/

Default nodelist directory.

2.4.7 PhoneBook ZeeTerm.phone

Default phonebook filename.

2.4.8 Priority 0

Priority the program should use when running, defaults to 0.

2.4.9 LogFile ZeeTerm.log

Default log filename. If this is specified, ZeeTerm will

start with the log activated.

2.4.10 Protocol Zmodem

Default protocol, currently available are:

ASCII, Xmodem, Xmodem_CRC, Xmodem1K, Ymodem, Ymodem_G and

Zmodem.

2.4.11 ZMprotect

Zmodem file receive options. Should be one of the following:

ZMAPPEND    Appends to file if exists, elwhile create new file.

ZMPROTECT   Creates new filename: "filename1" becomes "filename2".

ZMREPLACE   Deletes old file, create new file instead of old one.

2.4.12 ZMCRC32

Zmodem CRC option. Should be one of the following:

ZMCRC16     Use 16 bit CRC error correction.

ZMCRC32     Use 32 bit CRC error correction.

2.4.13 ZMAutoDwl YES

Should Zmodem do automatic file transfers? Defaults to NO.

2.4.14 ZMSend_Pathname NO

Should Zmodem send the full pathname? Defaults to NO.

2.4.15 ZMRecv_Pathname NO

Should Zmodem use the received full pathname? Defaults to NO.

2.4.16 IgnoreCD NO

Should ZeeTerm ignore carrier detect when performing file

transfers? Defaults to NO.

2.4.17 Ascii_Sendpath RAM:

Default special Ascii sendpath directory.

2.4.18 Ascii_Recvpath RAM:

Default special Ascii receivepath directory.

2.4.19 Interlace OFF

Specify whether or not ZeeTerm should use interlace mode.

2.4.20 Bitplanes 3

Default number of bitplanes ZeeTerm should use. 1 to 4 are valid.

2.4.21 ScreenWidth=0

Default screen width. If set to 0, or if this is not stated,

it defaults to 640.

On systems running Kickstart v37 or later, you might wish to

specify a value of -1 which means that ZeeTerm will open the

screen with the highest screenwidth possible (regarding overscan).

2.4.22 ScreenHeight=0

Default screen height. If set to 0, or if it is not stated,

ZeeTerm will detect the system it is running under (i.e NTSC or

PAL), and set the screen height according to that.

On systems running Kickstart v37 or later, you might wish to

specify a value of -1 which means that ZeeTerm will open the

screen with the highest screenheight possible (regarding overscan).

2.4.23 Palette

Format: RGB RGB RGB ... etc. up to a total of 16 definitions,

where "R" stands for RED, "G" stands for GREEN and "B" stands for

the BLUE palette.

Any definition may consist of the hexadecimal numbers from zero

through fifteen; in hexadecimal : 0123456789ABCDEF.

For the numbers 10 to 15, may be used both lower and upper case

letters for the hexadecimal representations.

The default palette consists of:

Color 0: Black (background)

Color 1: White (default pen-color)

Color 2: Green

Color 3: Yellow

Color 4: Blue

Color 5: Purple

Color 6: Cyan

Color 7: Red

Color 8 through 15 should be the same as 0 through 7, but with

increased intensity, e.g. brighter.

If this option is not stated, ZeeTerm will use its own internal

default colors for 1, 2, 3 or 4 bitplanes. These default colors

will ofcourse be the standard VT-100 and ANSI emulation colors.

To take advantage of the correct VT-100 emulation colors,

following colors (or equivalent) should be used:

Palette=000 bbb 0a0 a60 008 a0a 0aa a00 666 fff 0f0 ff0 00f f0f 0ff f00

2.4.24 FontName topaz.font

Default font name.

2.4.25 FontSize 8

Default font size.

2.4.26 Keymap IBM

Default keymap.

Should be entered without leading pathnames, as ZeeTerm will

look in "DEVS:Keymaps/" for this file.

2.4.27 Device serial.device

Default serial device name for communication with the serial

port. Be aware, that the device name is indeed case sensitive,

and should always be entered the way it appears in your DEVS:

drawer on your system disk.

2.4.28 DeviceUnit 0

Default serial device unit number.

2.4.29 DeviceShared NO

Specify whether or not you wish the serial port to open in

"shared" mode. Defaults to YES.

2.4.30 Baudrate 38400

Default baudrate. Valid baudrates are: 300, 1200, 2400, 4800,

9600, 19200, 38400, 57600, 76800 and 115200.

If none of these matches, a default baudrate of 19200 will be

chosen.

2.4.31 Parity 8N1

Parity. Should be entered as "8N1", "7N2" etc.

First parameter stands for databits, second for parity and

the third parameter stands for stopbits.

Valid entries are: 8N1, 8N2, 8E1, 8O1, 7N2, 7E1, 7O1, 7E2, 7O2.

Defaults to "8N1".

Be aware, that the Xmodem, Ymodem and Zmodem requires that the

parity is set to "8N1" and will automatically switch to this

before a file transfer is performed.

2.4.32 Handshake CTS_RTS

Default handshake that should be used. Currently, following are

supported: CTS_RTS, XON_XOFF or NONE.

2.4.33 Duplex FULL

Default duplex. When set to half, all characters typed on the

keyboard will be printed on the local screen right after sending

them to the serial port.

Should be enteres as "Full" or "Half".

Most bulletin board systems send the characters back, so default

is "FULL".

But there are some restrictions regarding the IEMSI Split-screen

chatmode. See section 12.2.1.

2.4.34 DTRDrop YES

If this option is set to YES, ZeeTerm will drop the DTR signal

in order to hang up. Elsewhile it will use the hangup string.

2.4.35 Emulation IBM_ANSI

Default terminal emulation. Valid are: "TTY", "VT100" and

"IBM_ANSI".

2.4.36 Translate_TX_CR CR

Convert outgoing carriage returns to:

CRLF -> Carriage Return + Line Feed.

CR   -> Carriage Return.

LF   -> Line Feed.

2.4.37 Translate_RX_CR CR

Convert incoming carriage returns to:

CRLF -> Carriage Return + Line Feed.

CR   -> Carriage Return.

LF   -> Line Feed.

2.4.38 SwapDEL_BS NO

Swap the functionality of the Delete and the Backspace keys?

Defaults to "NO".

2.4.39 DestructiveBS NO

Use destructive Backspace? Defaults to NO.

2.4.40 Redial_Delay 10

Dial delay between retries used in the intelligend dialer.

2.4.41 Dial_Delay 30

Delay while waiting for a carrier after dialing the number.

2.4.42 Max_Tries 10

Maximum attempts to dial before aborting.

2.4.43 DialAbort \

Abort string. This string or character will be sent to the

modem to abort an unsuccessfull dial attempt. The "\" stands

for a carriage return.

2.4.44 Initstring ATZ\

This is the string that will be send to the modem immediately

after a startup.

2.4.45 Exitstring \

This string will be sent to the modem right before it exits.

2.4.46 ATTstring +++

This is the ATTention string, that will be sent to the modem

before the hangup string, when the hangup option is telling

ZeeTerm to use the hangup string to hangup.

2.4.47 DialPrefix ATDT

2.4.48 DialSuffix \

2.4.49 HangupString ATH0\

These are the modem response codes.

You should not put a "|" character in the end (line feed), although

the modem actually generates this, as this character is ignored by

ZeeTerm, and it will always check for the strings written here.

If you do so anyway, the strings will never match, and the dialer

will not work properly.

See the discussion about the items below, in section 5.4.

2.4.50 ConnectString CONNECT

Default CONNECT string.

2.4.51 DisconnectString NO CARRIER\

Not currently used in ZeeTerm.

2.4.52 BusyString BUSY\

Default BUSY string.

2.4.53 VoiceString VOICE\

Default VOICE string.

2.4.54 NodialtoneString NO DIAL TONE\

Default NO DIAL TONE string.

2.4.55 ErrorString ERROR\

Default ERROR string.

2.4.56 RingingString RING\

Default RING string.

2.4.57 NoanswerString NO CARRIER\

Default NO CARRIER string.

2.4.58 Remotering RINGING\

Default RINGING string.

2.5 Starting ZeeTerm for the first time

If the "ZeeTerm.CFG" is not present in the current directory,

ZeeTerm will use its internal defaults the first time you start

it. You can save any changes you make so that ZeeTerm will

configure itself to these settings every time it is used

afterwards by writing the defaults to the file "ZeeTerm.CFG".

As this file is kept in pure Ascii, you are able to edit this

file with your favorite text editor.

The internal defaults are aimed at giving the new user the

most common configuration to "plug-n-play" with a minimum of hand

holding, but you may want to browse over the various options that

ZeeTerm has to offer (it is *very* configurable!) after having

used it a few times so that you can personalize the way it

operates.

2.6 Modem setup

Hayes compatible modems are preferred for use with ZeeTerm.

Fortunately, almost evey modem made today is Hayes compatible.

The specific features the modem should have are discussed below.

2.6.1 Carrier detect signal

The modem should have a functioning carrier detect (DCD)

signal on pin 8 of the serial port. The DCD signal goes

true once your modem has established a connection with the

remote modem that you have called. It is not required to have

this signal be functional, but it is recommended.

Most Hayes compatible modem manufacturers default the

modem to permanently set this signal true. You will be able

to determine if DCD is active if the red clock to the left

of the system clock in the status pane is counting, although

you are not online anywhere. If so, please refer to the users

manual for your modem in order to set the modem so that it

allows the DCD signal to work properly.

2.6.2 Data terminal ready signal

This signal (DTR) on pin 20 of the serial port tells the

modem when the data terminal, (ZeeTerm) is ready. ZeeTerm

also uses this signal, if the modem recognizes it, to force

the modem to hang up or go on hook. As with the DCD signal,

many manufacturers will assert this signal permanently true.

Check the modem's users manual for the proper command which

will cause the modem to go "on hook" (hangup) and also

return to its command state whenever the DTR signal is

dropped and then raised again. If the modem is able to do

this you can activate the pulldown menu that is labeled

"Hangup option", submenu "Drop DTR" so that ZeeTerm can use

it to hangup the modem.

2.6.3 Additional information for MNP modems

MNP stands for Microcom Networking Protocol. It is a

hardware based method of error correction built into the

modem itself. There are several levels of MNP with 3, 4 and

5 being the most commonly used modes.  Levels 3 and 4

provide error correction, with level 4 being more efficient

of the two. A level 4 MNP connection can give you as much as

19% gain in throughput under optimal (noise free) conditions.

Level 5 adds adaptive data compression "on the fly". It is

of little value (and can degrade performance) if you plan on

doing a mejority of file transfers with already compressed

files (.ARC, .ZOO, .ZIP and .LZH files). But, it can boost

text based transmissions by incredible amounts depending on

the profile of text being sent.

In order to use an MNP modem properly, and to maximize

the throughput it is capable of delivering, you need to set

ZeeTerm and the modem to a fixed baud rate that is usually 2

or 4 times greater than the connection rate between the

local and remote modem.

Since ZeeTerm is capable of sending data at a baud rate

higher than that at which the two modems are connected, it

requires some sort of handshaking so that no data losses

occur. The kind of handshaking most often used in this type

of situation is hardware or CTS/RTS handshaking. CTS refers

to the Clear To Send signal while RTS is for the Ready To

Send signal. This pair of signals is used by the modem and

the computer to stop and start the flow of data between them.

If you plan to download to a floppy disk and are going

to use Zmodem, you must have XON/XOFF handshake active in

order for the serial.device to stop the flow of data while

writing to disk. Otherwise, you run the risk of buffer

overrun errors. Although the transfer won't be corrupted or

lose data due to error correction, it will increase transfer

time because of the sender having to resynchronize with the

receiver.

Ymodem-G transfers to floppy are not possible when

using the internal serial.device due to it not dropping

RTS when the serial input buffer is almost full. XON/XOFF

handshake is disabled for Ymodem-G because it needs full 256

byte transparency for successful transfers. If you are using

an alternate serial device that supports RTS handshake then

downloading to floppy using Ymodem-G is possible.

The default serial device for the internal port in the

Amiga is odd in that it requires the Data Set Ready (DSR)

signal (pin 6) to be active when it is first opened or it

will not activate the CTS/RTS handshake mode. Worse yet,

your modem will suddenly appear to not communicate with

ZeeTerm since the serial device thinks that the modem is not

"ready" to receive data. Most MNP modem manufacturers default

the modem to having the DSR signal follow the carrier detect

signal. Fortunately, they always have a command and/or switch

to permanently set DSR active. SO, in order to utilize the

CTS/RTS handshake mode, you will have to set your modem so

that it keeps the DSR signal permanently active.

In addition to the DSR signal having to be permanently

asserted, your modem must have CTS true when the modem is

not online. Check the manual for your modem to insure that

these two signals are set up correctly.

3 SYSTEM BASICS

3.1 The display

The default display used by ZeeTerm when it is first started

and it cannot find a configuration file, is a three bitplanes

custom screen with a borderless window.

Optional displays include borderless windows on a 2, 4, 8

or 16 color custom screen.

The Palette requester allows you to modify the colors for

your display, but the standard colors used by VT-100 and ANSI

are the colors that should normally be used, and therefore are

hardcoded within ZeeTerm.

ZeeTerm uses a custom display driver which is much faster

than the internal console device used by the Amiga for the CLI

and other text based displays.

External font is provided for use with the IBM ANSI and

VT-100 terminal emulations.

The ability to change the default keymap is also provided

for users who may need to use more than one keymap and/or font

due to differences between countries.

3.2 The status line

The status line in ZeeTerm is normally displayed like this:

+---------------------------------------------------------------+

| xx 8N1 Full No Hshak ZM/P/CRC32 Capt:xx Prt:xx IEMSI:xx xx xx |

+---------------------------------------------------------------+

| | | |  | |      | |        | |     | |    | |      | || ||

+ +-+ +--+ +------+ +--------+ +-----+ +----+ +------+ ++ ++

1  2    3      4         5        6       7       8     9 10

From left to right, this means:

1: Baudrate.

2: Data bits, parity, stop bits.

3: Duplex mode: Full or Half.

4: Handshake: None, Xon/Xoff or CTS/RTS.

5: Protocol and protocol options.

6: Capture status: On or Off.

7: Printer status: On or Off.

8: IEMSI status: On or Off.

9: Connect clock.

10: System clock.

3.2.1 System clock and Connect clock

The system clock and connect clock, which is displayed in

the rightmost side of the status pane, is running as a

seperate process.

This process does two things, updating the display of the

system clock and connect clock, and keeping an eye of the

serial port for the sense of a carrier detect.

If a carrier is detected, the screen title is changed to

something like: "Connected to: Name".

In order for you to spot the difference of the system clock

and the connect clock, the system clock is displayed in

white, and the connect clock is displayed in red (on a 3 or

4 bitplanes screen, that is).

The connect clock starts counting, when ZeeTerm sense that

there is a carrier. This works independantly from the normal

carrier detect routines, and the "Ignore carrier" function

described in section 8.4.4 will have no effect on this.

3.3 Terminal emulations

3.3.1 TTY

The TTY emulation is a basic, line oriented, "dumb"

terminal. There are no graphics characters possible when

using this emulation. The acronym TTY stands for teletype.

3.3.2 VT-100

The VT-100 emulation is an almost complete VT-100 terminal,

which provides you a with VT-100 ansi control sequence

handling, also known as ANSI.

This emulation does not accept the ascii 0x9b (155) as

a control sequence introducer (CSI), as this ascii code

is used as a letter in i.e. Denmark.

The acronym VT stands for virtual terminal, and 100 stands

for the version number.

3.3.3 IBM Ansi

Almost the same as the VT-100 emulation, with some minor

differences in the Clear-Screen and linefeed routines.

This emulation will work on almost any system you connects

at, but if the screen looks weird you should try switching

to VT-100 mode.

3.4 Log file

This option allows ZeeTerm to log transfers and session

activity. The following actions are presently recorded:

-   ZeeTerm session start and end.

-   Connection to a system via the dialer.

-   Modem hangup command.

-   Carrier detect, if the "Ignore CD" option is not active.

-   Carrier loss, if the "Ignore CD" option is not active.

-   File transfers in both directions.

-   EMSI session progression.

(default: OFF)

3.5 Capture file

The capture file feature allows you to save data received by

ZeeTerm.

3.6 File transfers

When you initiate a file upload or download you are first

greeted with the FILE requester. Select the desired file(s)

for an upload or enter the new filename for a download when

using a protocol other than Ymodem(-G) or Zmodem. These protocols

send the filename in a special header block, so the FILE

requester isn't opened when downloading via these protocols.

The transfer status window is then opened. The information

this window displays is as follows:

Protocol:   The file transfer protocol used for

this file transfer.

File path:   The file path for the current file being

transferred.

File name:   File name being transferred.

File length:   Size of file being transferred.

Error check:   Whether or not there are error check

on this transfer. Error check may be

Checksum (Xmodem), CRC-16 (Ymodem, Zmodem)

and CRC-32 (Zmodem).

Bytes sent/rcvd:   How many bytes of file transferred.

Blocks sent/rcvd:   How many blocks of file transferred.

ZM status:   Used only in Zmodem transfers, and replace

the 'Blocks sent/rcvd' as Zmodem does not

use blocks. - It shows the current Zmodem

status.

Started:   Actual time file transfer was started.

Elapsed:   Elapsed time since start of file transfer.

Chars/sec:   Throughput figure.

Status:   Error/status line.

Selecting the close window gadget or pressing the ESC key while

a transfer is in progress will abort the transfer. Please be

patient if you do abort it, as ZeeTerm attempts to purge all data

before returning to the terminal in an effort to prevent garbage

from being displayed.

During the transfer ZeeTerm will update the elapsed time

and the current chars/sec figures. Errors or other delays can

result in the chars/sec drops. The cps indicator will fluctuate

a good bit during the early stages of the transfer. It require

upwards of 20 kilobytes to settle down, more for high speed

modems.

At baud rates above 2400bps you should be aware that file

transfer throughput begins to be affected by the method used for

transmitting data by the sending program. If a byte-by-byte

method is used, efficiency is going to drop off rapidly. If,

instead, the sender uses several different optimized methods,

the rate will stay somewhat the same as baud rate increases.

3.7 Gadget types

ZeeTerm uses several types of gadgets. Their functions are

described as follows.

3.7.1 String gadgets

These gadgets use the familiar Amiga rectangle with a label

to the immediate left. ZeeTerm enhances the string gadgets

by range checking integer input for validity.

3.7.2 Cycle gadgets

These gadgets are likened to a button or switch. If the

gadget is selected, the inside text will change.

3.7.3 Control gadgets

These gadgets select functions within a requester. Control

gadgets perform an immediate action and/or open an

additional sub-requester.

3.7.4 Intuition gadgets

The default Intuition close window, depth and drag gadgets

work as expected. ZeeTerm retains the position of some

requesters for future use if it is dragged to a new location

from the default centered position.

4 SPECIAL REQUESTERS

4.1 File requester

YAFR (Yet Another File Requester), why?

The ZeeTerm filerequester has some abilities that doesn't

come with any of the other librarybased filerequesters on

the market. It was made mainly to be able to read/write

on the serial port while it is opened.

4.1.1 Functional description

The FILE requester opens up in one of three modes.

One mode is "Single File Request", which will allow you

to select only one file at a time. Another mode is

"Batch File Request", which allows you to select any number

of files. The third mode is "Directory Request", which

allows you to select a directory.

In all modes the FILE requester is dynamic. This means

that it can process gadget selections while building the

current directory files list.

The large rectangle is the familiar file selection

array, which displays up to 15 files and/or directories at

a time. Directories are prefixed with the string "DRAWER".

The righthand side of the list shows the size in bytes and

creation date for each file.

In the title bar of the FILE requester there is a

size indicator labeled "Disk free:", which shows the amount

of free space left on the current device for the current

directory.

To the right of the selection rectangle is the

proportional scroll gadget which enables the user to scroll

through a list of filenames. The knob, or smaller rectangle

inside the scroll gadget indicates what percentage of the

list is displayed at any time.

4.1.2   Rename

This gadget will pop up a string requester containing

the name of the file contained in the Filename string

gadget. It will allow you to rename this file to another

filename of your desire.

4.1.3   Okay

This gadget instructs ZeeTerm to accept your file

selection(s). It is the same as pressing return after

entering a filename in the string gadget, or by

double-clicking on one of the files.

4.1.4   Parent

This gadget will change the current directory to the

parent of the current directory. The pathname string

gadget will also be updated to reflect this change.

4.1.5   Devices

Will fill the selection rectangle with a list of all

filing system devices on your system, enabling you to

change the current directory to another device.

4.1.6   Read dir

Selecting this gadget, will cause ZeeTerm to scan the

current directory (displayed in the Pathname string

gadget) again. Useful, if you have pressed the Devices

gadget and don't want to change the current directory.

4.1.7   Make dir

This gadget will pop up a string requester enabling

you to create a new subdirectory with a name of your

desire, in the current directory.

4.1.8   Select All

This gadget is functional only while the FILE requester

was opened in batch file mode. It will select every file

in the current directory.

4.1.9   Clear All

This gadget is functional only while the FILE requester

was opened in batch file mode. All selected files will

be deselected.

4.1.10  Delete

This gadget will pop up a string requester containing

the name of the file contained in the Filename string

gadget. It allows you to remove this file from existance.

4.1.11  Cancel

This gadget will clear the selected files and abort the

function that opened the FILE requester. The close window

gadget for the requester produces the same result as the

Cancel gadget.

4.1.12  Filename

The FILE requester will open with this string gadget

set to a suggested filename for phonebook, macro, capture

and EMSI files. It also displays the last filename

selected. You may enter a filename of your choosing

instead if you so desire.

4.1.13  Pathname

This string gadget contains the pathname of the current

directory.

5 PHONEBOOK

5.1 Phonebook List requester

The PHONEBOOK LIST requester is a control panel for adding,

editing and deleting phonebook entries. It also allows you to

select one or more entries for use by the intelligent dialer.

5.1.1   Dial

ZeeTerm is capable of multi-dialing any number of directory

entries. Just select the entries you want dialed first.

You then start the dialer by either double-clicking the last

selection or by selecting this gadget. You can also dial one

entry alone by double-clicking it.

5.1.2   Add

Add a new phonebook entry by selecting this gadget.

See the discussion about the PHONEBOOK EDITOR requester

below.

5.1.3   Sort

Alphabetically sort the entry names.

5.1.4   Edit

Selecting this gadget without having marked any entries in

the phonebook will have no effect.

Select the entries you wish to edit, then select this gadget.

See section 5.2 for a discussion of the PHONEBOOK EDITOR

requester.

5.1.5   Toggle

Selecting this gadget will cause all selected phonebook

entries to become unselected, and all unselected phonebook

entries to become selected.

5.1.6   All

Selecting this gadget will cause all phonebook entries to

become selected.

5.1.7   Print

Selecting this gadget will cause the FILE REQUESTER to pop

up in order for you to select the name of the device you

wish to write the phonebook entries to.

5.1.8   None

Selecting this gadget will cause all phonebook entries to

become unselected.

5.1.9   Load

Loads a phonebook file via the FILE requester.

5.1.10  Save

Save the current phonebook via the FILE requester.

5.1.11  Delete

Selecting this gadget will cause the selected phonebook

entries to be deleted. In order to save your deletions,

you must save the phonebook again.

5.1.12  Cancel

Aborts the phonebook selection, and closes the PHONEBOOK LIST

requester window.

5.2 Phone entry editor requester

The PHONEBOOK EDITOR requester is accessed whenever you select

Add or an existing phonebook entry when selecting Edit from

the PHONEBOOK LIST requester.

5.2.1   BBS Name

The phonebook entry name is entered here. If this is left

blank the entry will be discarded regardless of any other

information entered into the requester. You MUST enter some

sort of name in order to have the new entry entered into the

phonebook. You must then save the phonebook itself in order

for this entry to be available for future use.

5.2.2   Comment

Enter a comment for the phonebook entry here.

5.2.3   Phone number

Enter the phone number here. Up to 40 digits are allowed.

5.2.4   Password

The password for the current entry can be entered here.

The length of the password is limited to 20 characters.

5.2.5   F-key file

The macros file that will be loaded on connect with the

phone number entered above. - Optional, if you do not

write a filename here, no macros file will be loaded.

5.2.6   Arexx Script

Another optional entry. If you enter a filename here it

will be assumed as an Arexx script, and will be started

when a connection is established.

5.2.7   Baud rate

Selecting this cycle gadget will cause it to change to

another baud rate. - It starts at 300 baud and ends at

115200 baud. When reaching the highest possible baud rate,

it will cycle to start all over again at 300 baud.

5.2.8   Data Length

Selecting this cycle gadget will cause it to switch between

7 and 8 data bits.

5.2.9   Duplex

Will switch between half and full duplex.

5.2.10  Parity

Will switch between No parity, Odd parity and Even parity.

5.2.11  Protocol

Selecting this cycle gadget will cause the protocol desired

for this phone entry to change to what is written.

5.2.12  Stop Bits

Will switch between 1 and 2 stop bits.

5.2.13  Use IEMSI

If this gadget shows "No", selecting this will inwoke the

IEMSI Data editor.

If it shows "Yes", it will disable IEMSI on this phonebook

entry.

See section 5.3 for details on the IEMSI Data editor.

5.2.14  Swap DEL&BS

Switch the functionality on this phonebook entry.

5.2.15  Okay

This gadget will end the current edit and return to the

phonebook with whatever change you have made. If the

PHONEBOOK EDITOR was opened via the Add gadget, the

new entry will be added to the list.

5.2.16  Cancel

This gadget will abort the current edit and return to the

phonebook. If the PHONEBOOK EDITOR was opened via the Add

gadget the entry will be discarded.

5.3 IEMSI Data editor requester

The IEMSI DATA EDITOR requester is an editor for allowing ZeeTerm

to use IEMSI on a given phonebook entry. It is inwoked whenever

you select the "Use IEMSI" from the PHONEBOOK EDITOR.

The Name, Alias, Location, Dataphone and Voicephone are your

information to the other end of the line.

The News, New mail, New files, Hot keys, Screen CLR, Quiet mode,

More and FS Editor are the request fields.

The request fields specifies what the user wishes to do once

the initial IEMSI negotiation has been successfully completed.

5.3.1   Name

Enter your name here. This is the name ZeeTerm will use

when logging on a system supporting IEMSI.

You must enter a name here, as an IEMSI session will not

be successfull if you don't.

5.3.2   Alias

If you are using an alias, enter this here.

5.3.3   Location

The geographically location of the user,

ie. Stockholm, Sweden.

5.3.4   Dataphone

Unformatted Data telephone number of the user.

Unformatted is defined as the full telephone number,

including country and local area code. Eg. 46-8-90510 is a

telephone number in Stockhold, Sweden.

5.3.5   Voicephone

Unformatted Voice telephone number of the user.

Unformatted is defined as discussed above in section 5.3.4.

5.3.6   News

Show bulletins, announcements, etc.

5.3.7   New mail

Check for new mail to you.

5.3.8   New files

Check for new files.

5.3.9   Hot keys

States whether or not you wish to use hot-keys.

5.3.10  Screen CLR

States whether or not you wish to use the screen clearing

option. Normally this means that screen clearing codes will

be sent right before e.g. a menu is drawn on the screen, so

that it will appear at the top of the screen instead of

having to scroll the text up all the time.

5.3.11  Quiet mode

Do not disturb.

5.3.12  More

Page pausing, often referred to as "More".

5.3.13  FS Editor

Use the full-screen editor (If available).

5.3.14  Okay

This gadget instructs ZeeTerm to accept your IEMSI data

editions. It will close the requester window and enable

IEMSI on the current phonebook entry.

5.3.15  Cancel

Aborts the IEMSI DATA EDITOR, closes the requester window,

and disables IEMSI on the current phonebook entry.

Selecting the close window gadget will perform the same

action as selecting the Cancel gadget.

5.4 Dialer operation

The dialer is controlled by three parameters in the main

configuration file; number of retries, dial timeout and redial

delay. The number of retries applies to each entry that is

selected when the dialer is started. The dial timeout refers

to how many seconds the dialer will wait for a connection

after dialing. The redial delay is also in seconds, and is

the amount of wait time at the end of the list before restarting

again.

When the dialer establishes a connection it will reconfigure

ZeeTerm and load a macros file if one exists for that entry and

start an Arexx script if one exists for that entry.

A menu item is also provided to re-start the dialer from

where it left off after making the last connection. This is not

an auto-start type feature; you must restart the dialer yourself

via the menu item or <RAMIGA><R> key sequence.

5.4.1 Response codes

The dialer has a degree of intelligence by responding to the

different codes that the modem is capable of returning. The

codes and their effect on dialer operation is as follows:

NO DIALTONE     Dialer skips to next entry.

VOICE           Entry is de-selected.

BUSY            Dialer skips to next entry.

RINGING         No special effect.

RING            An incoming call is being made, the

dialer will terminate immediately.

NO CARRIER      Dialer skips to next entry.

CONNECT         Entry is de-selected and dialer exits

after re-configuring all ZeeTerm

parameters to the settings in the

phonebook entry.

ERROR           Causes dialer to exit immediately.

5.4.2 Dialer window

The dialer will open the DIALER requester while dialing.

There are two keys that are assigned functions to control

the operation of the dialer. They are:

<ESC>   Exits the dialer immediately.

<SPACE> Skips to next entry or re-dial delay, as

necessary.

The same functions are achieved by activating the Abort

and Skip gadgets.

5.5 The Phonebook data file

ZeeTerm is capable of reading Ascii phonebooks, although it

saves in a binary data format. This Ascii file works in the

same manner that the ZeeTerm.CFG default parameters file, by

using some keywords, as mentioned below.

5.5.1 Name SAGA AmigaBBS

This defines the name of the system in the current entry.

5.5.2 Number 31163217

Defines the telephone number of the current entry.

5.5.3 Comment Home of ZeeTerm

Enter a comment for the phonebook entry here.

5.5.4 FKeyfile ZeeTerm.keys

The macros file that will be loaded on connect with the

phone number entered above. - Optional, if you do not

write a filename here, no macros file will be loaded.

5.5.5 ArexxScript Script.zt

Another optional entry. If you enter a filename here it

will be assumed as an Arexx script, and will be started

when a connection is established.

5.5.6 Password password

The password for the current entry can be entered here.

The length of the password is limited to 20 characters.

5.5.7 Baudrate 38400

Select the baudrate to switch to before attempting to dial

the current phonebook entry.

5.5.8 Datalength 8

The datalength to use on the current phonebook entry.

Valid are 7 or 8.

5.5.9 Parity None

The parity to use on the current phonebook entry.

Valid are None, Even or Odd.

5.5.10 Stopbits 1

The stopbits to use on the current phonebook entry.

Valid are 1 or 2.

5.5.11 Duplex Full

The duplex mode to use on the current phonebook entry.

Valid are Full or Half.

5.5.12 SwapDelBS NO

Swap functionality of the DEL and BS keys on the current

phonebook entry? Defaults to NO.

5.5.13 Protocol ZMODEM

Which protocol to use on the current phonebook entry.

Valid entries are: ASCII, XMODEM, XMODEM_CRC, XMODEM1K,

YMODEM, YMODEM_G and ZMODEM.

5.5.15 #NEWENTRY

This indicates that the current phonebook entry definition

is done, and that there will be new definitions below this

keyword.

5.6 Passwords

The idea behind this feature is to make it simple to create and

maintain unique passwords for each system you have an account on.

Common sense tells you that having unique password for each

system is safer than using the same password on many of them.

Changing them every so often is also another good habit to

develop in order to prevent unauthorized access of your accounts.

ZeeTerm makes the maintenance of passwords easy by keeping

the password in the phonebook entry. Having the password placed

in the phonebook entry itself eliminates the possibility of

losing it as long as the phonebook file isn't destroyed. Backup

your phonebook often if you plan to use this feature.

6 FUNCTION KEY MACROS

Up to 40 macros of 100 characters can be defined for each macros file.

Select the appropriate modifier gadget in order to edit the desired

set of macros for function keys <F1> to <F10>.

6.1 Macro escape codes

A simple set of escape codes are included. They are listed as

follows.

\   Carriage return.

\\  The '\' character itself.

|   Line feed.

||  The '|' character itself.

6.2 The ZeeTerm.keys file

This datafile is, like all others, kept in pure ascii.

When editing this file manually, you must know how ZeeTerm

evaluates the entries.

Every entry must be entered on one single line, with the

initiater immediately in front. The initiater is defined as

one character followed by the number of the F-key the text is

supposed to be attached to. The text follows immediately after,

without any seperation.

The initiater has the following format.

F1 = The key F1

F2 = The key F2

F0 = The key F10

A1 = The key combination ALT + F1

S1 = The key combination Shift + F1

C1 = The key combination Ctrl + F1

Enter the text you wish to be attached to the desired function

key, immediately after the initiater, like this.

F1This is the F1 key.

This would perform "This is the F1 key." to be written, when you

press the F1 key.

7 PALETTE REQUESTER

Works like most any color palette requester found in other Amiga

applications.

7.1 Red Green and Blue proportional gadgets

These sliders adjust the associated color component of the

selected color.

7.2 Color selection gadgets

Depending on the screen type, there can be from 2 to 16 color

selection gadgets available on the upper part of the PALETTE

requester. Select the color you wish to adjust.

7.3 Swap

Will swap the current color selection with another selection

made immediately after selecting the Swap gadget.

7.4 Okay

Selecting this gadget will close the COLOR requester, and

cause ZeeTerm to use this new set of colors.

7.5 Copy

Will copy the current color selection into another selection

made immediately after selecting the Copy gadget.

7.6 Reset

This gadget will reset the palette to the colors that were

active when you activated the COLOR requester.

7.7 Cancel

Selecting this gadget will close the COLOR requester, and

will reset the palette to the colors that were active when

you activated the COLOR requester.

7.8 Default palette

ZeeTerm has a default color palette built-in for use with

the terminal emulations.

This palette is the custom default palette. When ZeeTerm

is started without a defaults file present or when the

defaults palette is not defined in the defaults file, this

palette is set to the same values as the IBM color palette.

8 TRANSFER PARAMETERS

8.1 Protocol

All the file transfer protocols in use by ZeeTerm strictly adhere

to the specifications outlined in the various documents that are

available for each protocoll. The following is a short

description of the protocols available.

(default: Zmodem)

Xmodem      The "grandfather" of most every protocol in use

by personal computers today. The Xmodem checksum

protocol was created by Ward Christensen back

in 1977.

Xmodem-CRC  The original Xmodem protocol modified with a

16 bit CRC polynomial for better error detection.

ZeeTerm will "step down" to Xmodem checksum if

the sender does not recognize the initial CRC-16

handshake.

Xmodem-1K   A variant of Xmodem-CRC that uses 1024 byte

blocks which gives you higher throughput than

the original Xmodem protocol. Some systems

incorrectly call this Ymodem. As with Xmodem-CRC,

ZeeTerm will "step down" to Xmodem checksum if

necessary.

Ymodem      This is TRUE Ymodem(tm) except that it will

force uploads to 1024 byte blocks. It is a batch

transfer protocol. The filename, date and file

size are transferred in a separate header

block at the beginning of each file that is

transferred. This protocol also preserves the

correct file size; the Xmodem and derivative

protocols described above pad characters at the

end of the file.

Ymodem-G    This variation of Ymodem is intended for error

correcting modems and hard-wired connections.

Because this protocol does no error correcting,

it can dispense with the need for an

acknowledgement of each transmitted block of data.

The file is transferred with little delay and can

achieve some of the highest throughput of any of

the protocols listed here. If an error is

detected, the transfer will be aborted, hence

the need for an error-free connection in order

to use this protocol with confidence.

Zmodem      This is one of the newest and most powerful

protocols available. It is capable of batch

transfers, automatically initiated uploads and

downloads, 32 bit CRC error checking, and a

unique ability to resume a transfer that had been

partially completed. The Zmodem protocol was

created by Chuck Forsberg.

8.1.1 Some notes on flow control

Zmodem and Ymodem-G are known as  full-flow protocols due to

their ability to send data without interruption.

For Zmodem transfers, all that is necessary is to activate

the Xon/Xoff handshake in the Serial menu.

Ymodem-G is a different problem altogether. If you're

downloading to a floppy disk and are using the internal

serial port you cannot use this protocol. Otherwise, you

WILL lose data and, therefore, abort the transfer as this

protocol only does error checking, not error correction. It

also requires full 256 byte transparency so local Xon/Xoff

handshaking is disabled when using it.

If you're using a multi-port add-on board that supports

the dropping of the RTS signal when the internal serial

buffer is almost full (you must have CTS/RTS handshake

enabled for this to work) then you will be able to use

Ymodem-G to floppy disk.

It goes without saying that if you have ample memory to

spare, you would do much better transferring files to and

from ram disk.

8.2 Zmodem send pathname.

This mode is applicable to Zmodem uploads only. It will have no

effect on Zmodem downloads.

It will send the FULL path together with the filename, when

selected.

8.3 Zmodem download options.

These options are specific to Zmodem downloads only. They will

have no effect on Zmodem uploads.

8.3.1 Protect file

If the requested file already exists, ZeeTerm will change

the filename of the new file, so that it will no longer

conflict with the existing file. This is done by adding

one to the last character in the filename. If this file

also exists, ZeeTerm will continue until it can create

a completely new file.

(Eg. Filename0 will become Filename1 etc)

8.3.2 Append file

If the requested file already exists, ZeeTerm will make

attempt to resume the file transfer. A check is made by the

receiving system to determine if the file that is about to

be transferred is shorter than the one the sender has. The

file will be skipped otherwise.

8.3.3 Replace file

If the requested file already exists, ZeeTerm will simply

overwrite the file with the new one.

8.3.4 16 or 32 bit CRC

Instructs ZeeTerm whether or not to use the newer 32 bit CRC

mode of error detection if possible. A 16 bit CRC will be

used if the remote system does not support 32 bit CRC. File

transfers using 32 bit CRC have a much higher degree of

catching hard to detect errors at the expence of slightly

slower throughput.

8.3.5 Auto Up/Download

This feature, when selected, allows ZeeTerm to start a

Zmodem up- or download automatically when a Zmodem auto-

d/l or u/l header is received.

8.3.6 Receive pathname

This mode is applicable to Zmodem downloads only. It will have

no effect on Zmodem uploads.

It will use the FULL path as received from remote, when

opening the receiving file.

8.4 ASCII send options

8.4.1 Expand blanks

When this feature is selected, ZeeTerm will send one space

character for lines which contain only an end-of-line

character. Use this option when the on-line text editor

you're using exits its input mode whenever a carriage return

is received without any text preceding it.

(default: ON).

8.4.2 Receive path

The ASCII protocol uses its own unique receive path for

doing file transfers. This file path can be selected here.

8.4.3 Send path

Like the ASCII receive path, the send path is unique too.

8.4.4 Ignore Carrier Detect

Activate this if your modem does not have a functional

carrier detect signal.

9 MODEM PARAMETERS

These parameters enables ZeeTerm's intelligent dialer to interact with

your modem properly. All strings default to standard Hayes modem commands

and responses.

9.1 Modem commands

Modem commands are sent to the modem for initialization, hanging

up, and dial prefixes/suffixes.

9.1.1 Init command

Sent to the modem when ZeeTerm is first started.

Can be found and edited in the ZeeTerm.CFG file.

9.1.2 Hangup command

Used to hangup the modem if the hangup option is set to

use the hangup string.

9.1.3 Dial-up prefix

This is the default dialing prefix that the dialer sends to

the modem before the phone number for the phonebook entry is

sent.

(default: "ATDT")

9.1.4 Dial-up suffix

This is sent after the dialer has transmitted the phone

number.

(default: "\")

9.2 Modem response codes

The eight response codes and their Hayes defaults are:

NO DIALTONE     Sent if no dialtone signal was present.

VOICE           Not all modems are capable of sending this.

BUSY            Sent when a busy signal is detected.

RINGING         Sent when the phone rings in the other end.

RING            Sent when the modem detects a ring signal.

NO CARRIER      The remote system did not respond.

CONNECT         The modem has connected to another modem.

ERROR           Sent when the modem detects an error.

9.3 Intelligent dialer parameters

The following parameters control how the intelligent dialer

operates. See section 6.3 for details of its operation.

9.3.1 Redial delay

Indicates how many seconds ZeeTerm will wait before re-

starting after it has dialed the last selected entry in the

list. Values between 0 and 999 will be accepted.

9.3.2 # of retries

Controls how many times ZeeTerm will loop through the list

of selected entries before exiting the dialer. Valid values

range from 1 to 999.

9.3.3 Dial timeout

How many seconds ZeeTerm will wait for a carrier after

dialing the phone number for the current entry. Values

between 1 and 999 are allowed.

CAUTION!    Your modem may have an internal timeout for this

as well, make sure that it is set to a value in

seconds that is shorter than the value entered

for this option. If the modem is set to a longer

value you will have disabled this feature of the

dialer.

9.4 Hangup options

9.4.1 Drop DTR to hangup

Select this option if your modem is capable of disconnecting

when the DTR signal is dropped.

9.4.2 Send hangup string

Select this option if your modem is not capable of

disconnecting when the DTR signal is dropped.

10 MENUS

The following are descriptions of the menu items available.

10.1 Project menu

10.1.1 About

Displays information about ZeeTerm.

10.1.2 Capture file

Select a capture file via the FILE requester, to which events

happening on the screen will be saved.

10.1.3 Printer

Select printer device (usually PRT:) whereto events happening

on the screen will be printed.

10.1.4 Read config

Will via the FILE requester read a default parameters file.

10.1.5 Save config

Save a default parameters file via the FILE requester.

10.1.6 Quit

Exits the program.

10.2 Phone menu

10.2.1 Phonebook

Opens the phonebook. (See details discussed in section 5).

10.2.2 Dial number

Prompts you with a string requester asking for a number to

dial, then dials (if anything entered) the number via the

intelligent dialer.

10.2.3 Re-dial

Will dial any marked entries in the phonebook.

NOTE! You cannot re-dial a number that already has been

de-selected (if you already have been online), unless you once

again open the Phonebook and select the number.

10.2.4 Send Password

Sends the password for the active phonebook entry.

The password is edited in the PHONEBOOK EDITOR.

10.2.5 Nodelist dir

Change the directory via the FILE requester, for traplist

library node list files.

This is OPTIONAL, and if you have traplist.library node list

files in this directory, you will be able to use node numbers

instead of telephone numbers in the phonebook.

10.2.6 Hangup option

Enables you to select if you want to hangup by dropping DTR,

or by sending the hangup string.

10.2.7 Hangup

Hang up the phone.

When ZeeTerm is set to send the hangup string, ZeeTerm will

make a short delay, send the ATTention string, make another

short delay, and then send the hangup string.

10.2.8 Response codes

Enables you to edit the various modem response codes.

See the discussion in section 5.4.

10.2.9 Dial-up prefix

Edit the default dialing prefix that the dialer sends to the

modem before the phone number for the phonebook entry is

sent.

10.2.10 Dial-up suffix

Edit the suffix that is sent after the dialer has transmitted

the phone number.

10.2.11 ATT string

Edit the ATTention string, that is sent to the modem when the

hangup option is set to send the hangup string.

10.2.12 Hangup string

Edit the hangup string, that is sent to the modem when the

hangup option is set to send the hangup string.

10.2.13 Init string

Edit the initialization string, that is used to setup the

modem when ZeeTerm is started.

10.2.14 Exit string

Edit the exit string, that is sent to the modem right before

ZeeTerm exits.

10.3 Transfers menu

10.3.1 Send file

Initiate a file upload (from your system).

See the discussion in section 3.6 and section 9 on file

transfers.

10.3.2 Receive file

Initiate a file download (to your system).

10.3.3 Protocol

Select another file transfer protocol.

See the discussion in section 8.

10.3.4 Zmodem options

Tells Zmodem how to handle files during a file transfer.

See the discussion in section 8.

10.3.5 Ascii options

Tells the Ascii protocol how to handle files during file

transfer.

See the discussion in section 8.

10.3.6 File paths

Change default send/receive directories.

ZeeTerm uses two different file receive paths and two different

send paths. One for the Ascii protocol and one for all the

other protocols. This is very handy, as you probably don't have

your text files in the same directory where you keep your other

binary files.

10.3.7 Ignore CD

Ignore carrier detect during transfers.

If you are using a null-modem cable between two amigas (or

possibly other machines), you will need to set this to on.

Otherwise you will not be able to do file transfers, as a

null modem cable do not have a carrier detect signal.

10.4 Serial menu

10.4.1 Baudrate

Change ZeeTerm to use another baudrate.

Baud rates from 300 to 115,200bps are supported.

10.4.2 Half duplex

On older systems, or if you call a friend that also use a

software terminal program, you will probably have to turn this

on in order to see what you are writing (and not only the text

your friend is writing).

There is one exception to this, read the section about the

Split-screen chatmode in section 12.2.1.

10.4.3 Parity

Change the parity mode with which ZeeTerm will communicate with

your modem.

10.4.4 Handshake

Handshake (Xon/Xoff, CTS/RTS or None).

See the discussion on handshake in section 2.

10.4.5 Send break

Sends a break signal to the modem.

10.4.6 Device name

Change the default serial device name via the FILE requester.

10.4.7 Device unit

Change the default serial device unit number.

10.5 Terminal menu

10.5.1 Emulation

Change the terminal emulation.

See the discussion in section 3.

10.5.2 Font

Enables you to change the currently used font parameters.

10.5.3 Load Keymap

Loads a standard keymap from the DEVS:Keymaps/ directory via

the FILE requester.

10.5.4 Load F-keys

Load a pre-set of macro-key definitions via the FILE requester.

10.5.5 Edit F-keys

Edit the function key macros.

See the discussion in section 6 on this subject.

10.5.6 Save F-keys

Save the current macro-key definitions via the FILE requester.

10.5.7 Translate Tx CR

Change the way ZeeTerm translate transmitted CR's (Carriage

return).

There are three ways this can be translated,

CR -> CR    No translation, CR transmitted.

CR -> LF    CR is translated into LF (Line feed).

CR -> CR+LF CR is translated into a CR+LF pair.

10.5.8 Translate Rx CR

Change the way ZeeTerm translate received CR's.

Like 10.5.7, this can be done in three ways.

10.5.9 Swap DEL & BS

Swap the functionality of the delete and backspace keys.

10.5.10 Destructive BS

Switch On/Off whether or not the backspace should be destructive.

10.5.11 Screen Mem

Switch On/Off the use of "screen memory", so that ZeeTerm, when

turned off, does not use time on scrolling and writing to this

memory, making the screen output faster.

The screen memory routines use the 68010 "loop-mode", so if you

are using a system with this kind of CPU, you will find ZeeTerm

taking advantage of it.

If you, however, have a system with a standard 68000 CPU, the

scrolling will go considerable faster with the screen memory

turned off, disabling the "point & click" operations of the

characters on the screen.

10.6 Miscellaneous menu

10.6.1 Colors

Change default number of colors (bitplanes).

10.6.2 Palette

Opens the palette requester, if it is not already active.

See the discussion in section 7.

10.6.3 Screen size

Enables you to define how big (small) a screen you wish to use.

10.6.4 Interlace toggle

Toggles the display between interlace and not-interlace.

10.6.5 Close Workbench

Enables you to close the Workbench screen.

When the Workbench is closed, this item will change contents

so that it now reads "Open Workbench".

If, for some strange reason, the Workbench screen could not be

closed, the display will flash indicating that the action could

not be performed.

10.6.6 StatusPane On/Off

Toggles the statuspane on and off to give you an extra line of

text (when using fonts smaller than the height of the statuspane).

Selecting this menuitem when the Split-screen chatmode is active

will do nothing.

10.6.7 Start Arexx script

Starts an Arexx script. Arexx scripts for ZeeTerm has a filename

ending with ".zt".

10.6.8 Execute DOS command

Opens a window on the Workbench (or default public screen, if

you are running under Kickstart v37 or later), and uses this as

output for the command you specify here.

CAUTION!    Do not attempt to redirect input or output from

commands executed from within ZeeTerm to the

ZTHandler (">ZT:" or "<ZT:"), unless you start the

commands with "Run"!

Avoiding to do so will cause ZeeTerm to lock up,

as ZeeTerm will wait for the command to finish before

doing anything else. The command, however, will wait

for the redirection file to finish, which is supposed

to be done by ZeeTerm which is otherwise busy, by

waiting for the command..

10.6.9 IEMSI enable

Enables check for an IEMSI connection.

Please be aware, that if you are making a dial by yourself,

without using the intelligent dialer, ZeeTerm will use the

IEMSI data from a previous connection, or none if you haven't

used the dialer yet. This could cause some problems somewhere,

because your passwords probably not are the same everywhere.

Therefore, if you wish to use IEMSI you should use the phonebook

and the intelligent dialer each time.

See section 12 for a discussion about IEMSI.

10.6.10 EMSI poll

EMSI poll a FidoNet system.

See section 12 for a discussion about EMSI.

10.6.11 Split-screen toggle

Toggles the Split-screen chatmode on and off.

See section 12.2.1 for a discussion about the IEMSI Split-screen

chatmode.

11 AREXX

This section assumes a certain level of familiarity with the Arexx

script language.

ZeeTerm supports a lot of Arexx commands which are listed below

together with a description of what they do.

The functions does not give a return value. You must tell Arexx that

you want results by specifying "options results", and then use the

Arexx auto-variable "result" after the use of the function.

The Arexx port of ZeeTerm is named "ZT_REXX_x", where x stands for the

x'th running copy of ZeeTerm, starting with 0, so that the first

running copy has a port named "ZT_REXX_0", and the next "ZT_REXX_1"

and so on.

All Arexx commands are case-insensitive, unless otherwise stated.

11.1 About

Opens/closes the About window, result will be 1 if the window is

open, and 0 if it is closed.

11.2 'Addprintfile "string" "Filetoshow"'

As ZeeTerm MUST have the double quotes ("), it is necessary, when

using this command, to enclose the whole line within single quotes

('). Otherwise Arexx will remove the double quotes, and the command

will fail.

This commands adds a "string" to the list of strings that ZeeTerm is

supposed to show textfiles on.

When ZeeTerm meet the textstring "string", the textfile "Filetoshow"

will be shown on the local screen only (no serial).

The command can be called as many times you want (and has got memory

for), and when you don't need this check no more, you can free the

list with the command ClearPrintFiles.

Remember, the fewer checks ZeeTerm should do, the faster the update

of the screen will work.

This function is very usable for users with slow modems, and can be

used as some kind of a "fastmenu" option. It provides, ofcourse, that

the sysop on the system you are calling, has made some special

"fast-menu" menus.

11.3 Bitplanes <planes>

Ex: Bitplanes 3

Sets the number of bitplanes to 3.

The number of colors available follows the number of bitplanes in

the power of 2, so that 1 bitplane gives 2 colors, 2 bitplanes gives

4 colors, 3 bitplanes gives 8 colors, and 4 bitplanes gives 16 colors.

11.4 Carrier

Calling this command gives three different results, they are:

0   No carrier

1   Carrier present

2   Ignore CD is chosen

11.5 Clearprintfiles

Frees ZeeTerm from making check for strings with accompagnying

textfiles (See 11.2), and free all memory used for that.

It is not necessary to call this command before you exits an Arexx-

script and the list allocated by AddPrintFile will automatically be

freed when ZeeTerm exits.

11.6 Conprint "String"

Writes the string "String" to the screen, and adds <CR>+<LF>.

11.7 Conputc <asciivalue>

Writes <asciivalue> as a character on the screen.

11.8 ConShowFile "Filename"

Write the contents of the (text-) file "Filename" to the screen.

11.9 Conwrite "String"

Writes the string "String" to the screen.

11.10 DropDTR

Drops DTR.

11.11 Dialnumber "Number"

Calls the phonenumber "Number", and checks in the nodelist.

11.12 EmsiPoll "EMSIdata"

Tells ZeeTerm to use the EMSIdata file, and make a poll. Just as

if you selected the "Poll EMSI" from the pulldown menus.

When called from Arexx, ZeeTerm will not open the FILE requester

in order to input a file name. This means that you can start

ZeeTerm by a script, tell it to do an EMSI poll, and exit again

right after.

11.13 GetAscRecvPath

Tells which receivepath used at the present for the Ascii protocol.

11.14 GetAscSendPath

Tells which sendpath used at the present for the Ascii protocol.

11.15 GetBaudrate

Tells which baudrate used at the present.

This result can also be used in conjunction with SetBaudrate.

11.16 GetChar

Because of a lot of overhead, this function will not be usable under

other circumstances than when running in "host"-mode.

It causes ZeeTerm to wait for any character, and put it into the

result.

11.17 GetDialdelay

Tells the delay ZeeTerm will wait after dialing a number, and

until it will abort the current dial.

11.18 GetDuplex

Tells if you are running half or full duplex.

11.19 GetMaxTries

Tells how many times the dialer will attempt to dial before aborting.

11.20 GetProtocol

Tells which protocol used at the present.

The string can also be used in the SetProtocol function.

11.21 GetRecvPath

Tells which receivepath used at the present.

Note: The Ascii protocol has its own paths.

11.22 GetRedialDelay

Tells how many seconds ZeeTerm will wait between dialing two numbers.

11.23 GetSendPath

Tells which sendpath used at the present.

Note: The Ascii protocol has its own paths.

11.24 GetString

Some sort of "input" routine. Takes all characters until an <CR> is

met, and puts this in the result.

11.25 Hangup

Looks like "DropDTR", but checks if the user wants "DTR-drop", and

sends the hangup-string if not.

11.26 Iemsi "On"|"Off"

Enables/Disables check for an IEMSI connection.

11.27 IngoreCD "Yes"|"No"

Very useful in "host"-mode, because ZeeTerm will not send/receive

files if there isn't a carrier present. Therefore, if you are using a

null modem cable this must be turned on (IgnoreCD "Yes").

11.28 Print "String"

Writes "String" to both the serial port and the screen, and adds a

<CR>+<LF> on the end.

11.29 Printer "On"|"Off"

Starts/stops output to the printer.

11.30 Putc <asciivalue>

Writes <asciivalue> as a character to both the serial port and on

the screen.

11.31 Quit

Exit's ZeeTerm.

11.32 ReceiveFile "Filename"

Receives a file named "Filename" using the present transfer protocol.

If you are using a batch protocol, the filename from remote will be

used. If you are not using a batch protocol, "Filename" will be used

as the filename, and the FILE requester will not be opened.

11.33 SendFile "Filename"

Sends a file named "Filename" using the present protocol.

Full path must be specified, or ZeeTerm will look in its current

directory.

11.34 SerPrint "String"

Writes the string "String" to the serial port, and adds <CR>+<LF>.

11.35 SerPutc <asciivalue>

Writes <asciivalue> as a character to the serial port.

11.36 SerShowFile "Filename"

Writes the contents of the (text-) file "Filename" to the serial

port.

11.37 SerWrite "String"

Writes the string "String" to the serial port.

11.38 SetAscRecvPath "FilePath"

Sets the Ascii protocol receive path to "FilePath".

11.39 SetAscSendPath "FilePath"

Sets the Ascii protocol send path to "FilePath".

11.40 Setbaud <baudrate>

Sets the baudrate to <baudrate>.

11.41 SetDialdelay <seconds>

Sets the delay ZeeTerm will wait after dialing a number, and until

it will abort the current dial.

11.42 SetDuplex "FULL"|"HALF"

Sets duplex mode to full or half.

11.43 SetHostMode "ON"|"OFF"

Only available from Arexx.

If you are running in "host"-mode, ZeeTerm will normally not send the

characters back to the remote. If you are setting "SetHostmode ON",

all characters that is received will be sent back to the sender.

Be sure that you switch this off *before* an Arexx script is exited!

11.44 SetMaxTries <tries>

Set how many times the dialer will attempt to dial before aborting.

11.45 SetProtocol "Protocol"

Sets the protocol to "Protocol".

Currently, following protocols are available:

"ASCII","XMODEM","XMODEM_CRC","XMODEM_1K","YMODEM","YMODEM_G"

and "ZMODEM".

11.46 SetRecvPath "Filepath"

Sets the default receive path to "Filepath".

11.47 SetRedialDelay <seconds>

Set how many seconds ZeeTerm will wait between dialing two numbers.

11.48 SetSendPath "Filepath"

Sets the default send path to "Filepath".

11.49 ShowFile "Filename"

Writes the contents of the (text-) file "Filename" to both the serial

port and the screen.

11.50 SplitScreen "On"|"Off"

Enters or terminates the IEMSI Split-screen chatmode.

See section 12.2.1 for a discussion on this.

11.51 Version

Tells you the $VER: informations in the result.

11.52 WaitChar "C"

Waits for the character "C" before proceeding.

The character specified is CASE SENSITIVE!

You can also wait for a more specific asciivalue, so that

e.g. WaitChar 27 will wait for a press on <ESC>.

11.53 WaitString "String"

Wait for the string "String" before proceeding.

The string specified will be treated CASE SENSITIVE!

11.54 Wait <secs>

Waits for <secs> seconds.

11.55 Write "String"

Writes "String" to both the serial port and the screen.

11.56 ZMAutotrans "ON"|"OFF"

Enable/disable  autotransfer mode in Zmodem. Handy if running in

"host"-mode.

11.57 ZMcrc 16|32

Sets Zmodem error-correction for either 16- or 32-bit CRC.

11.58 ZMFileOption "PROTECT"|"APPEND"|"REPLACE"

Sets Zmodem file option as discussed in section 8.3.

11.59 ZTDelay <ticks>

Calls dos.library Delay() with <ticks> as an argument.

The <ticks> is in 1/50 of a second. If <ticks> is lesser than

2 (2/50 second), no delay is performed.

12 EMSI and IEMSI

What is EMSI/IEMSI?

The EMSI/IEMSI handshake protocol allows for maximum flexibility in

E-Mail session start-up and control. EMSI/IEMSI should provide for

virtually unlimited growth and expansion of its own scope. By

providing variable-length packets, EMSI/IEMSI is capable of being

as simple or as complex as necessary and entirely backwards compatible

when new features and/or protocols are added.

The EMSI/IEMSI implementation adhere strictly to the definitions in

the FSC-0056 document.

12.1 EMSI

EMSI requires a special datafile for each remote system you wish to

poll. The remote system must be able to make a FidoNet EMSI session

(which in fact almost every system today is able to) in order for

ZeeTerm to correctly perform a mail session.

This datafile must contain data about your system and some data

on the system you wish to poll. As ZeeTerm asks for a datafile

every time you choose the "EMSI poll" from the pulldown menus,

you can have as many EMSI datafiles as you wish, doing mail

sessions with as many systems as you have datafiles for.

12.1.1 The EMSI datafile

The EMSI datafile is in pure Ascii, and should be edited with

your favorite text editor to reflect the correct information

during the mail session. This is done by some keywords, like

in the ZeeTerm.CFG, and all of the following keywords must

be present (and defined) in this datafile.

SYSTEM >>> Your System <<<

The name of your system. This would normally be a compagny

name, BBS name or other identifying text.

The length of the name is limited to 80 characters.

AKA 2:231/84.0 2:30110/0.0

These are your Zone:Net/Node.Point numbers (seperated by

spaces!). As the EMSI implementation is based on FidoNet-

technology, it is required that Zone:Net/Node be presented,

and .Point be omitted if zero. Zone and Net must not be zero.

Note that the Zone:Net/Node.Point numbers MUST be seperated

by spaces, and not tabs or anything else!

PASSWORD password

For systems using a session level password, it would be

passed in this field. Note that the same password is used

for all presented addresses and that it must be treated

case insensitive.

If you are using the traplist library, the password is

usually taken from the node list files, however if you

write a password here, ZeeTerm will use this instead.

If you do not have a password to the system you are

calling, you should not specify a password or comment

it out.

The length of the password is limited to 20 characters.

SYSOP Per Hansen

The name of the person primarily responsible for your

system. Change this to your own name.

The length of the name is limited to 50 characters.

PHONE 45-31163217

The telephone number of your system, or "-Unpublished-" if

your telephone number is unpublished. This MUST be in the

standard format COUNTRY-CITY-NUMBER. Leading zeros should be

stripped from the city code, ie. Stockholm (Sweden) has a

city code of 08, included in an EMSI packet, it would read

46-8-<number>.

CITY Valby, Denmark

The geographically location of the system.

The length of the location is limited to 50 characters.

BAUDRATE 38400

The baudrate ZeeTerm should use during mailsessions using

this datafile.

BOSSNAME >> Your Boss <<

The name of your "Boss" (the place where you pick up mail).

This is only used in the dialer, and only if you do not use

the traplist library. Otherwise the name for your boss will

be taken from the node list data files.

BOSSNUMBER 12345678

The telephone number for the "Boss". If you are using

traplist library, this can be a node number as well in

which case the name (as mentioned above) will be taken from

the node list.

MAILPREFIX <string>

This string indicates that ZeeTerm is supposed to delete

the files after a successfull filetransfer during an EMSI

session.

The string appears in the #?.FLO file before the filenames,

and is normally created by the mailer to indicate which files

that is supposed to be deleted after a successfull transfer.

Examples could be a "#" or a pathname like "OUTBOUND:".

The specified string will be removed before it is processed

further, so if you specify a pathname, be sure that the

files can be found by ZeeTerm after removing it. As ZeeTerm

adds the outbound directory after removing the MAILPREFIX,

you could specify the same directory that you use as the

outbound directory, in which case all files found in the

outbound directory will be deleted after a successfull

transfer.

That means that if e.g. a filename of "OUT:mailpkt.MO0" is

found in the #?.FLO file, and you specify the MAILPREFIX as

"OUT:" and the outbound directory is "OUT:", ZeeTerm will

first remove the "OUT:" (that is specifyed as the MAILPREFIX)

and thereafter add the outbound directory name "OUT:".

If you don't specify a MAILPREFIX, ZeeTerm will make a

(lucky) guess, which files to delete or not to delete.

INBOUND MAIL:Inbound/

The inbound directory, for incoming files. The pathname must

end with a "/", if it is not an assigned directory or a

volume.

OUTBOUND MAIL:Outbound/

The outbound directory, for outgoing files.

12.1.2 What happens when doing an EMSI mail session?

After reading the parameters from the EMSI datafile, ZeeTerm

will dial the BOSSNUMBER, using the intelligent dialer, until

there is a connection.

After sensing the connection, the dialer exits and leave control

to the EMSI routines.

When the EMSI handshake is complete, ZeeTerm will look for

files to the "Boss" by looking for outbound files with names

that match the "Boss" AKA string (as received by EMSI).

These files are created by your mailer, and contains the

filenames of the files that is going to be sent to the other

end.

The filenames of the outbound filelist will normally be the

zone.net.node.point.ext, where zone, net, node and point is

that of your boss, and ext be either .FLO, .CUT or .OUT.

When ZeeTerm has read the outbound filelist, it will delete

the file, and proceed to the transmission of the files.

After completion of the transmission, ZeeTerm will observerve

if any files was not sent, and create a new outbound filelist

where appropriate.

ZeeTerm will then go into receiver mode, and will receive any

incoming files your "Boss" might want to send to you.

When this is done, the EMSI session is complete, and ZeeTerm

will go back to its normal functions.

12.2 IEMSI

The IEMSI protocol enables you to do fast logons on systems that

supports this feature.

The IEMSI has a built-in split-screen chatmode, that has some

abilities you will not normally use when using ZeeTerm on standard

systems without IEMSI.

The various abilities are discussed below.

12.2.1 The IEMSI Split-screen chatmode.

When ZeeTerm sense that the other end of the line wants to chat

to you, it will go into Split-screen chatmode.

This means that the screen splits into two parts, showing incoming

characters at the upper half, and your typing at the lower half.

The Split-screen chatmode works not quite the same way as running

normally, in that any outgoing carriage returns are converted into

carriage return and a line feed. ZeeTerm also does "word-wrapping"

which means that if you are entering a line of text, and exceeds

the screen-width, it will go backwards at the line you just

entered, until a space character is met, whereon the word is

"wrapped" (deleted from the current line, and automatically

moved into the next line).

The Split-screen chatmode can be entered manually by selecting

the "Split-screen toggle" menuitem from the Miscellaneous pulldown

menu.

When the Split-screen chatmode is active, ZeeTerm will regardless

of the Duplex mode (section 2.4.32) always use "Half" duplex.

13 THE DOS HANDLER

The ZTHandler, which must be placed in your L: directory on your system

disk, has got the ability of sending/receiving characters directly from/

to ZeeTerm. Very handy, if you need to send an "Info" (or something else)

from the CLI to the other end of the line. This way you don't have to

redirect to a temporary file and send this file with ascii later on.

Place the handler in your L: directory, and make an entry in your

mountlist (in the DEVS: drawer) that looks like this.

ZT:

Handler = L:ZTHandler

StackSize = 3000

Priority = 5

GlobVec = -1

#

The handler has got some options, to make life easier for you when logged

onto systems that can't do these things.

In order to address the right running copy of ZeeTerm, you will have to

specify which one of them you want to access.

This is done by writing a "filename" of the number of the running copy.

The first running copy will have number zero, the next number one etc.

If you do not specify a number, ZTHandler will default to 0 (the first).

ZTHandler supports up to ten running copies of ZeeTerm.

When logged on a BBS, you cannot count on remote handling linefeeds in

the same manner that yourself. Therefore you can switch the output line

terminators (<CR> and <LF>) around, so that a <LF> will be sent as a

<CR>, or the other way around. Most bulletin boards will work alright

when receiving a <CR>, but not when receiving a <LF> that is default

in the CLI.

Using the ZTHandler in conjunction with the Split-screen chatmode, works

in a similar way, ie. you will have to convert line feeds into carriage

returns.

This is done by writing a "sub-directoryname" *after* the number of the

running copy of ZeeTerm. Note that if you want one of these conversions,

you *must* specify the number of the running copy.

Examples:

Info >ZT:

Causes an "Info" to be redirected to ZTHandler, that sends the

text further on to the first running copy of ZeeTerm.

Info >ZT:0

Causes an "Info" to be redirected to ZTHandher, that sends the

text further on to the first running copy of ZeeTerm.

Info >ZT:1

Causes an "Info" to be redirected to ZTHandler, that sends the

text further on to the second running copy of ZeeTerm.

Info >ZT:0/c

Causes an "Info" to be redirected to ZTHandler, that sends the

text further on to the first running copy of ZeeTerm, and

converting any line feeds into carriage returns (<LF> becomes

<CR>).

Info >ZT:0/C

Same as above.

Info >ZT:0/l

Causes an "Info" to be redirected to ZTHandler, that sends the

text further on to the first running copy of ZeeTerm, and

converting any carriage returns into line feeds (<CR> becomes

<LF>).

Info >ZT:0/L

Same as above.

It is possible to run CLI "doors" like onlinegames using ZTHandler,

simply by redirecting both the input and the output through this device.

Example:

Run Game >ZT: <ZT:

It is also possible to start a new shell process this way, simply by

typing (from CLI): "NewShell ZT:". This is much more convenient than

the AUX: device from Commodore, because you are able to see on the

screen what the other end is doing, by switching to half duplex.

Under most conditions (as far as I have discovered) you should use the

"/L" option for the handler, causing all carriage returns to be converted

into line feeds, as the AmigaDOS shell/cli always terminates an input

condition with a line feed. This includes CLI "doors".

CAUTION!    Do not attempt to run "doors" while the Split-screen chatmode

is active! As ZeeTerm, when in Split-screen chatmode, always

convert a carriage return into carriage return + line feed,

amongst other things which will almost for sure cause

problems!

Do not attempt to redirect input or output from commands

executed from within ZeeTerm to the ZTHandler! This includes

starting a new process with "NewShell ZT:", unless you start

the commands with "Run".

Avoiding to do so will cause ZeeTerm to lock up, as ZeeTerm

will wait for the command to finish before doing anything

else, including to processing the message from the handler.

14 SUPPORT INFORMATION

Support for ZeeTerm is available through the ZeeTerm support BBS only.

14.1 ZeeTerm Support BBS

The support BBS is attached to the FidoNet BBS network, twenty-four

hour crashmail is supported via the Xenolink bulletin board system.

SAGA AmigaBBS       : +45 31163217

Baud rates supported: 24/96/14400 HST/V42bis/V32

FidoNet address     : 2:231/84

14.2 If you have problems

The need for a complete and detailed description cannot be

emphasized enough, statements like "The dialer doesn't work" or

"It won't upload/download" will be considered "junkmail", and

do neither of us any good.

Also, don't give up immediately. If something seems wrong ask the

sysop of the system you're connectet to first. Since his/her system

is more than likely to be less expensive to call than the support

BBS (and probably less busy too), it makes sense to see if he/she

can help you out first.

If you do post a problem report, please be sure to include

the name and number of the system you had a problem with, how you

created the problem (so that I can recreate it) and anything else

that is related to the problem. A short description of your system

may also be helpful. Hard disk users should include the type of

controller and how many partitions you're using.

Providing as much information as you can gather the first time

will usually result in a faster resolution of the problem and will

eliminate your having to call several times only to find me asking

for more information.

Good luck and happy modeming!

Per Hansen

ZeeTerm 0.92 Users Manual                               Electronic Edition