RELEASE NOTES

S3 OS/2 32 Bit Drivers

Release Date:	Oct. 26, 1994

Version:	2.41

1.	This is a release of the S3 OS/2 2.41 32 bit drivers supporting the 801/805/805i/928SD/864/764/732 products.

## 2.	Product Description

These drivers support OS/2 2.1, OS/2 2.11, and OS/2 WARP(3.0).

There are 4 sets of driver diskettes.  Each support certain chipset as follows:

- 	801/805, 805I with 8-bit/16-bit DAC, 928 with ATT DAC.

- 	928 chipset with Brooktree 9485 RAMDAC.

-	864 chipset with AT&T 498, AT&T 499, and SDAC RAMDAC and Trio764/732 chipset.

-	964 chipset with Brooktree 9485 RAMDAC, TI 3020/25 RAMDAC, and IBM 525/524/514

RAMDAC.

2.	The following resolutions are supported:

| Mode | Resolution | Memory Required | 805I(16) / 864 764/732 | 801/805/ 805I(8)/ 928ATT | 964 |
| --- | --- | --- | --- | --- | --- |
| 101 | 640x480x8 | 1M |  |  |  |
| 103 | 800x600x8 | 1M |  |  |  |
| 105 | 1024x768x8 | 1M |  |  |  |
| 107 | 1280x1024x8 | 2M |  |  |  |
| 207 | 1152x864x8 | 2M * |  |  |  |
| 120 | 1600x1200x8 | 4M * |  | N/S |  |
| 111 | 640x480x16 | 1M |  |  |  |
| 114 | 800x600x16 | 2M * |  |  |  |
| 116 | 1024x768x16 | 2M |  |  |  |
| 11a | 1280x1024x16 | 4M * | N/S | N/S |  |
| 212 | 640x480x24 | 1M | N/S | 112 | N/S |
| 112 | 640x480x32 | 2M |  | N/S |  |
| 115 | 800x600x32 | 4M * |  | N/S |  |
| 118 | 1024x768x32 | 4M | N/S | N/S |  |

Notes:	N/S 	-	Not Supported

1M only - 640x480x24 is supported for 1M memory configuration only, for 2M and above 32bpp

will be supported

(*) 	S3 BIOS supports 1152x864x8 with 1M, S3 OS/2 driver needs 2M of memory

S3 BIOS supports 1600x1200x8 with 2M, S3 OS/2 driver needs 4M of memory.

S3 BIOS supports 800x600x16 with 1M, S3 OS/2 driver need 2M of memory.

S3 BIOS supports 1280x1024x16 with 3M, S3 OS/2 driver needs 4M of memory.

S3 BIOS supports 800x600x32 with 2M, S3 OS/2 driver needs 4M of memory

The 32 bit per pixel driver may need a large swap space on the hard disk.

The recommended swap space is 15 MBytes.

Driver Limitations:

1.	Software Motion Video may require an aperture address setting on local bus systems.  If video playback is distorted or displays in the wrong location, add the following line to the CONFIG.SYS file:

SET VIDEO_APERTURE=XX

where "XX" is the actual aperture address.  Contact the system manufacturer for the address.

DOS Sessions and Applications

1.	With the program VPIC6 under FS DOS, GIFs cannot be viewed in the 1280x1024 resolution.

OS/2 Full Screen Sessions and Applications

1.	Context switching from Win-OS/2 full screen to OS/2 full screen may produce a distorted text in OS/2 full screen --Vision964 ONLY.