--------------------------------------------------------------------

IRDC - INFRA-RED Direct Control

--------------------------------------------------------------------

Version 2.11

February 1995

Copyright 1993-1995 - D.Huras - All Rights Reserved

------------------------------------------------IRDC.DOC v2.11------

CONTENTS

--------------------------------------------------------------------

1. DESCRIPTION

2. WARRANTY

3. FEATURE OVERVIEW

4. REQUIREMENTS

5. ONE FOR ALL REMOTES

6. SETUP INFORMATION

7. INSTALLATION INSTRUCTIONS

8. COMMAND LINE ARGUMENTS

9. COMMUNICATION PORTS

10. PROGRAM OVERVIEW

11. KEYBOARD USAGE

12. APPLICATIONS

13. PROBLEM RESOLUTION

14. REGISTRATION INFORMATION

15. UPGRADE INFORMATION

16. RELEASE HISTORY

1. DESCRIPTION

--------------------------------------------------------------------

IRDC is a program that sends commands to a ONE FOR ALL (*1) Remote

Control that instruct the remote control to emit INFRA RED (I/R)

signals.  Essentially, IRDC emulates a person pressing buttons on

the remote control keypad.  Any button sequence that can be

physically entered on the remote control keypad can be emulated with

the IRDC program including setting the Device type. The range for

the I/R signals can easily be extended through the use of the X-10

Powermid product.

IRDC will only operate with ONE FOR ALL* remote controls that have

an external port; the ONE FOR ALL 5*, ONE FOR ALL 6*, ONE FOR ALL 8*

and ONE FOR ALL 12* are the models with an external port that are

currently supported.  (See the ONE FOR ALL REMOTES section for

further information).  A special serial cable with a 3-pin male

connector at one end (that plugs into the ONE FOR ALL external port)

and an RS232 9-pin female connector on the other end (that plugs

into a PC serial port) is used to allow the software to communicate

with the remote control.  (This cable can be ordered from many Home

Automation Dealers --- see the CABLE.DOC file) The external port is

located under the battery cover and its primary purpose is for

upgrading the remote control (ie. loading new I/R signals into the

remote control).  The ONE FOR ALL remote controls cannot "learn" new

I/R signals; however, they can be "force fed" new I/R signals

through the external port.  This may be necessary if you acquire new

electronic equipment with I/R signals that are not defined within

your ONE FOR ALL remote control.

Unless your electronic equipment is located near your PC and you can

"aim" the ONE FOR ALL remote control appropriately, then the

benefits of IRDC may not be readily apparent.  To get the most out

of IRDC, you really need an I/R extender such as the X-10 Powermid

product.  This enables you to locate your PC and remote control

anywhere in your home or apartment and distribute I/R signals to

other locations.  See the 'APPLICATIONS' section of this document

for further information.

IRDC is a DOS program that uses a simple point & click user

interface.  (A Mouse though not required is highly recommended).

IRDC is designed to be robust software that is processor speed

independent.  This program is shareware and you are invited to

evaluate it for 30 days. If you continue to use the program after

the 30 day evaluation period you are asked to register it.  The

author retains all rights to the program.

----

(*1) ONE FOR ALL, ONE FOR ALL 5, ONE FOR ALL 6, ONE FOR ALL 8,

ONE FOR ALL 12 and The "Universal" Remote are trademarks of

Universal Electronics Inc.

2. WARRANTY

--------------------------------------------------------------------

IRDC is supplied AS IS.  The author disclaims all warranties,

expressed or implied, including, without limitation, the warranties

of merchantability and of fitness for any purpose.  The author

assumes no liability for damages, direct or consequential, which may

result from the use of IRDC.

3. FEATURE OVERVIEW

--------------------------------------------------------------------

- User-Friendly full screen point & click DOS character-based

Interface; your PC screen becomes a large remote keypad.

- All program functions can be controlled using a mouse; no keyboard

intervention is required.

- Supports Standard CommPorts 1 thru 4 and a User Defined CommPort

which can support any IRQ between 1 & 15 (inclusive) and any

Base I/O Address.

- Optional comprehensive on-line Help sub-system.

- All program parameters can be stored within an INI file thereby

obviating the need for any command line arguments.

- Extended Remote Codes can also be defined within the INI file

and presented in a List Box along with your description.  (Eg. TV

10  Surround ON).  You can simply double click on the list box

entry instead of trying to remember the extended code and then

clicking TV,A,1,0.  The entries in the list box are unsorted and

added in the same order as they are encountered within the INI

file.  This allows you to arrange the list in the order that you

prefer.

- The program WILL load without a ONE FOR ALL remote control

connected.  It will still attempt to use the serial CommPort; in

most cases (eg. if you have a modem attached to the serial port)

the results will be innocuous.  However, be aware that any device

connected to the CommPort configured for use with IRDC will

receive serial transmissions.

4. REQUIREMENTS

--------------------------------------------------------------------

- An IBM-Compatible PC with at least 1 serial port.

- A ONE FOR ALL 5, 6, 8 or 12 remote control.  See the 'ONE FOR ALL

REMOTES' section for further information on the supported models.

- A Special Serial Cable to connect the ONE FOR ALL remote to a PC

Serial Port.  (This cable can be ordered from many Home Automation

Dealers; see CABLE.DOC file).

- A Colour adapter/display is recommended although the program

should run with a monochrome display/adapter.  If you

experience problems with the default colors, then try the /BW

command line argument which will set everything to Black &

White.

The program requires approximately 360K available conventional memory

to run.  The memory requirements will increase as you add Extended

Remote Code definitions to the IRDC.INI file.  (eg. 100 definitions

will require an additional 6.5K of memory --- approximately).

5. ONE FOR ALL REMOTES

--------------------------------------------------------------------

IRDC supports 6 different ONE FOR ALL Remote Control Models.  The

Model Number is usually printed on the front of the remote or on the

battery cover. Be sure that you have correctly identified the model

of your remote before using the program.  You can identify your

model to IRDC by using the /Mn(n) argument as indicated in the

rightmost column below; you can also dynamically select the model by

using the Configuration Panel.

Model No.   Model Name    Description                       IRDC (/M)

--------  --------------  --------------------------------  ---------

URC-2005  ONE FOR ALL 5   Ergonomic Design (paddle shaped)   5

URC-4000  ONE FOR ALL 6   Conventional Design (rectangular)  6

URC-4050  ONE FOR ALL 6   Ergonomic Design (paddle shaped)   6A

URC-4005  ONE FOR ALL 6   Ergonomic Design (paddle shaped)   6B

URC-5000  ONE FOR ALL 12  Conventional Design (rectangular)  12

URC-6050  ONE FOR ALL 8   Ergonomic Design (paddle shaped)   8

6. SETUP INFORMATION

--------------------------------------------------------------------

Setup Instructions differ based on the model of the ONE FOR ALL

remote that you are using.  It is very important that you

correctly identify the model before using the program.  Refer to the

ONE FOR ALL REMOTES section to identify your remote and then follow

the appropriate Setup Instructions below.

SETUP INSTRUCTIONS FOR:

-----------------------

ONE FOR ALL 6  [URC4000]

ONE FOR ALL 12 [URC5000]

1) Plug the Special Serial Cable into the PC Serial Port.

2) Plug the Special Serial Cable into the ONE FOR ALL Remote Control.

3) Ensure that there are no loose connections.

4) Load the IRDC program.

5) VERY IMPORTANT: DO NOT TOUCH ANY BUTTONS on the remote control when

when using the software.

NOTES:

------

� The author assumes no liability for damages, direct or

consequential, which may result from the use of IRDC.

� If you physically press buttons on the keypad while the software

is communicating with the remote control strange things can happen

that could affect the operability of your remote control.  If the

memory contents of your remote control are altered then the only way

to restore it to normal operation is to have it reloaded by

Universal Electronics or a local Service Dealer.

--------------------------------------------------------------------

SETUP INSTRUCTIONS FOR:

-----------------------

ONE FOR ALL 5  [URC2005]

ONE FOR ALL 6A [URC4050]

ONE FOR ALL 6B [URC4005]

ONE FOR ALL 8  [URC6050]

1) Plug the Special Serial Cable into the PC Serial Port.

2) Plug the Special Serial Cable into the ONE FOR ALL Remote Control.

3) Ensure that there are no loose connections.

4) Load the IRDC program.

5) VERY IMPORTANT: You must physically press a button on the remote

control and then click on one of the on-screen buttons WITHIN 5

SECONDS to put the remote online. Once the remote is online it will

remain online (and will not respond to physical key presses) until

you explicitly take it offline by clicking on the 'Take Remote

Offline' button. When you click on the 'Take Remote Offline' button

the LED on the remote will flash twice in acknowledgement of the

request.

NOTE:

-----

� The author assumes no liability for damages, direct or

consequential, which may result from the use of IRDC.

7. INSTALLATION INSTRUCTIONS

--------------------------------------------------------------------

There are no special installation instructions; simply copy the

program to a sub-directory of your choice and run it.  The program

will look for 2 files in the directory it is run from.  However,

the program will run with or without the following 2 files:

1) IRDC.HLP

IRDC retrieves the on-line help text from this file. If the

file is not found then IRDC will disable the 'Help' command

button and the Help feature will be unavailable.  The program

will load slightly faster when the help sub-system is not

loaded; if you decide that you no longer require the online

help you can simply delete or rename the IRDC.HLP file.

2) IRDC.INI

IRDC will look for an IRDC.INI file in the current

directory.  All program parameters can be defined within

this file as well as Extended Remote Code definitions.  Any

errors found within the IRDC.INI file will be reported and

the program will be terminated.  IRDC will generate an

IRDC.INI file (or overwrite an existing IRDC.INI file) when

the 'Save' button on the Configuration panel is clicked.

Note that Command Line Args will ALWAYS OVERRIDE

conflicting parameters within the IRDC.INI file.  A sample

IRDC.INI file, generated by IRDC, is shown below:

;

;IRDC.INI File Created: 10-02-1993 07:37:33

;

; All Lines prefixed with ; are ignored

; Program Options

; ---------------

; CommPort=1,2,3 OR 4                DEFAULT=1

; Model=5,6,6A,6B,8 or 12            DEFAULT=6

; BW=YES or NO                       DEFAULT=NO (Colour)

; DisplaySetupInformation=YES or NO  DEFAULT=YES

;

Model=6

BW=NO

DisplaySetupInformation=YES

;

; User Defined CommPort

; ---------------------

; IRQ and Base_IO_Address apply to User Defined CommPort ONLY

;

; IRQ=1-15                 DEFAULT=4; IF Base_IO_Address is set

; Base_IO_Address=nnnn(n)  DEFAULT=1016; IF IRQ is set

;

IRQ=4

Base_IO_Address=1016

;

; Extended Remote Codes

; ---------------------

;

; The codes applicable to your I/R devices can be found in your

; your ONE FOR ALL manual.

;

CODE=TV,10,DISPLAY

CODE=TV,11,MTS

CODE=TV,12,PICTURE +

CODE=TV,13,PICTURE -

CODE=TV,14,SLEEP

CODE=TV,15,TV/VIDEO

CODE=TV,15,ANTENNA/AUX

CODE=TV,18,COLOUR UP

CODE=TV,19,COLOUR DOWN

CODE=TV,20,BRIGHT UP

CODE=TV,21,BRIGHT DOWN

CODE=TV,22,HUE UP

CODE=TV,23,HUE DOWN

CODE=TV,24,MATRIX/SURROUND

CODE=VCR,10,SLOW SRCH UP

CODE=VCR,11,SLOW SRCH DN

CODE=VCR,12,MENU

CODE=VCR,13,SELECT

CODE=VCR,14,ON SCREEN

CODE=VCR,15,CANCEL/SKIP

CODE=VCR,16,TIMER

;

; End of IRDC.INI (07:37:33)

8. COMMAND LINE ARGUMENTS

--------------------------------------------------------------------

DEFAULTS:  If the program is run without any command line arguments

it will default to COMMPORT 1,MODEL 6,COLOUR DISPLAY &

WARNING=YES.  Note that CommPort, Model & Display can

be toggled from within the program.

The following command line arguments can be entered in any order:

/Annnn(n) - Base I/O Address applicable to User Defined CommPort

(Must be specified as a decimal number)

If /I argument is present, but /A is not; then Base

I/O Address will default to 1016 (3F8 Hex).

/BW  - Use Black & White colours

/BW- - Use Colour (You would only use this argument to override

an INI BW=YES)

/Cn  - COMMPORT where n = 1,2,3 or 4

/In  - IRQ applicable to User Defined CommPort where n = 1 - 15

If /A argument is present, but /I is not; then IRQ

will default to 4.

/Mn  - MODEL where n = 5, 6, 6A, 6B, 8, or 12

/NW  - Suppress the automatic display of SETUP INFORMATION upon program

load. (This command line argument is not required if you have

elected to forego the online help by renaming or deleting

IRDC.HLP).

/NW- - Display Setup Information (You would only use this argument to

override an INI WARNING=NO)

/?   - Display Command Line Arguments

NOTE: - COMMAND LINE OPTIONS OVERRIDE IRDC.INI SETTINGS

- MULTIPLE OCCURRENCES OF THE SAME COMMAND LINE ARGUMENT

ARE ACCEPTED; THE LAST INSTANCE WILL BE USED BY IRDC.

9. COMMUNICATION PORTS

--------------------------------------------------------------------

IRDC supports "pseudo standard" CommPorts 1 thru 4 using the

following settings:

+----------+------+------------------+

|          |      | Base I/O Address |

| CommPort | IRQ  +---------+--------+

|          |      | Decimal |  HEX.  |

+----------+------+---------+--------+

|    1     |   4  |  1016   |  3F8   |

+----------+------+---------+--------+

|    2     |   3  |   760   |  2F8   |

+----------+------+---------+--------+

|    3     |   4  |  1000   |  3E8   |

+----------+------+---------+--------+

|    4     |   3  |   744   |  2E8   |

+----------+------+---------+--------+

If one of these CommPort settings matches your hardware

configuration simply use the /C command line argument with the

appropriate CommPort.  The CommPort= IRDC.INI directive can also be

used.

You can define your own CommPort by selecting User Defined CommPort

from the Configuration Panel screen and setting the IRQ interrupt

and Base I/O Address.  (You could also use the /I and /A Command

Line Args or IRQ= & Base_IO_Address= IRDC.INI file directives).

IRQ can be set to any value between 1 and 15 (inclusive).  Base I/O

Address can be set to any value greater than zero.  CAUTION: your

hardware may "lock up" if incorrect settings are used.  Once you

have enabled communications with the remote, then use the 'Save

Options' menu item to save the information to the IRDC.INI file.

NOTE: THE BASE I/O ADDRESS MUST BE SPECIFIED IN DECIMAL NOTATION.

IBM PS/2 CommPort Parameters are shown below:

+----------+------+------------------+

|   PS/2   |      | Base I/O Address |

| CommPort | IRQ  +---------+--------+

|          |      | Decimal |  HEX.  |

+----------+------+---------+--------+

|    1     |   4  |  1016   |  3F8   |

|    2     |   3  |   760   |  2F8   |

|    3     |   3  | 12832   | 3220   |

|    4     |   3  | 12840   | 3228   |

|    5     |   3  | 16928   | 4220   |

|    6     |   3  | 16936   | 4228   |

|    7     |   3  | 21024   | 5220   |

|    8     |   3  | 21032   | 5228   |

+----------+------+---------+--------+

A User Defined CommPort must be used for

PS/2 CommPorts 3-8.

10. PROGRAM OVERVIEW

--------------------------------------------------------------------

When the program loads, it looks for an IRDC.INI file in the current

directory and then proceeds to parse it if one is found.  If any

errors are detected within the INI file the program will terminate

with an appropriate message, otherwise it will parse the command

line arguments (which override any conflicting INI directives).

Once again, if any errors are found it will terminate with an

appropriate message.  Finally, it will look for a IRDC.HLP file and

activate the online Help if the file is found.  The IRDC full screen

interface will then be presented with the remote keypad displayed on

the right and the various program controls displayed on the left.

The upper left button of the remote keypad is the "active" control

at this time.  The program DOES NOT open the CommPort until you

actually press one of the remote keypad buttons; this allows you to

confirm the active CommPort before actually using it.

The Status Window on the upper left side of the screen will display

all remote keypad button activity as it is executed.  In addition

it will display a message each time a CommPort is opened.  (eg. the

first time you press a remote keypad button there will be a very

brief delay as the CommPort must be opened first).  There is also

a small LED indicator in the middle of the screen which will flash

briefly whenever a remote keypad button is clicked/pressed.

When the configuration button is clicked, the Configuration panel is

overlaid on the left side of the screen.  The current settings for

the Model, CommPort, Display and the Warning Flag are indicated with

option buttons.  If you change the Model or Display settings the

screen will immediately reflect the new settings.  To make your

settings permanent, click on Save; all current settings (including

any Extended Remote Code definitions) will be written to the

IRDC.INI file.  If an IRDC.INI already exists, the program will

display an Overwrite Confirmation before overwriting the existing

IRDC.INI file.  To continue with these new settings for the current

session only, click on OK; to restore the prior settings, click on

Cancel (or press ESCAPE).

As stated earlier, a mouse is highly recommended; you can simply

click on the remote keypad buttons, the control buttons and the

Extended Remote Code List Box scroll bars to perform program

functions.  To execute an Extended Remote Code contained within the

Extended Remote Code list box, simply Double Click on it.

11. KEYBOARD USAGE

--------------------------------------------------------------------

Use the TAB/Shift-TAB keys to move among the various remote keypad

buttons and the control buttons (eg. Configure, Help & Exit).  The

Up/Down arrows can be used to move up and down the Extended Remote

Code List Boxes.  Use the Space Bar key to select the current List

Box item, check/uncheck a check box, or invoke a command button.

(The Enter key can also be used for all controls except the Extended

Remote Code list box; the space bar must be used.) The ESCAPE key

will exit the current function or the program if no other windows

are visible.

The cursor keys can be used to navigate within the keypad area; you

can jump to adjacent buttons by pressing the appropriate key.  The

program will jump over 1 blank row/button if an ajacent button is

not present.

The following quick access keys are applicable when

the Configuration Panel IS NOT displayed:

Configure - Alt-c

Help - Alt-h

Exit - Alt-x

Remote Codes - Alt-d

The following quick access keys are applicable when the Configuration

Panel IS displayed:

Model - Alt-m

CommPort - Alt-p

Display - Alt-d

Setup Information - Alt-i

OK - Alt-o

Save - Alt-s

Cancel - Alt-a

12. APPLICATIONS

--------------------------------------------------------------------

After experimenting a little with IRDC you may find yourself

thinking: "its a neat 'toy' but what practical uses does it have?"

This section attempts to describe the practical applications for

this type of software.

Generating I/R Signals & IRDQ.EXE

---------------------------------

IRDC allows you to generate I/R signals simply by clicking the

buttons on the on-screen remote keypad.  It also allows you to build

a list box of those 'difficult to remember' Extended Remote Codes

which you can invoke with a double click.  It offers convenience

since you can leave it running in a window when working in a

multi-tasking or task-switching environment and simply switch to it

as required.  However, it does require 'human' intervention since

you have to physically click your mouse button in order to generate

I/R signals.  This is where IRDQ.EXE comes into play; IRDQ is a

command line driven version of IRDC that instructs ONE FOR ALL

remotes to emit I/R signals based on a button name passed on the

command line.  Alternatively, it will 'press' a series of buttons

that are contained within a file when the filename is passed to IRDQ

on the command line.  This means you can build "macros" by creating

files of the button names to be pressed.  In addition, it means the

entire library of ONE-FOR-ALL I/R codes is at your disposal because

you can dynamically set the device type at anytime.  For example,

you could build a button file containing the following button

names:

;

; IRDQ Button File - Turn on both TV's

; Note: lines prefixed with ";" are ignored

;

; Assign TV Code: 000 (Sony TV)

TV

A

B

C

0

0

0

; The Wait button name does not exist,but instructs IRDQ

; to wait for 1/2 second to allow the ONE FOR ALL remote

; to 'catch up'

Wait

; The SONY TV has an Extended Remote Code to turn the POWER ON

; use this code rather than the POWER button which is just

; a toggle

A

6

8

Wait

; Assign TV Code: 047 (GE TV)

TV

A

B

C

0

4

7

Wait

; The GE TV has an Extended Remote Code to turn the POWER ON

; use this code rather than the POWER button which is just

; a toggle

A

1

8

The end result being you can build a virtually unlimited library

of I/R signal sequences and generate them simply by invoking a

program from the DOS command prompt.

I/R Signal Distribution

-----------------------

The range of the I/R signals that the remote control generates based

on instructions from the software is limited to the same room as

where your PC is located.  This limitation is severe enough that the

program would not be very useful at all unless you happened to

locate all of your I/R electronic equipment in the same room as your

PC.  This is obviously unrealistic, but fortunately there is a

relatively inexpensive solution that will enable you to distribute

I/R signals to virtually anywhere in your home or apartment.  An I/R

extender can be used to "read" I/R signals from one location,

translate them into radio waves and beam them to one or more other

locations where they'll be translated back into I/R signals and sent

to the targeted electronic equipment.  The entire process takes

milliseconds and is virtually instantaneous.  The X-10 Powermid is

an I/R Extender; it has 2 components, a unit that receives the I/R

signals and transmits them via radio waves, and a unit that picks up

the radio signals and transmits the I/R signals.  Both units are

small 3" x 3" x 3" pyramid-shaped devices enclosed within a black

plastic casing.  You can install multiple Receiver/I.R emitters that

are sourced by the same Transmitter; this allows you to distribute

I/R signals to multiple locations.  Recoton also markets an IR

Extender.  To use an I/R extender with IRDC/IRDQ, you'd simply aim

the ONE FOR ALL remote at the I/R Transceiver.

Scheduling I/R Signals

----------------------

With IRDC/IRDQ, a ONE-FOR-ALL Remote & an I/R Extender you have the

capability to easily generate & distribute an I/R signal for just

about any electronic equipment that exists to virtually anywhere

within your home.  The capability to schedule I/R signal

transmissions at predetermined times would be a step closer towards

a completely automated I/R system.  Since IRDQ can be driven from

the DOS command prompt you could accomplish this through the use of

any DOS-based scheduler.  For example, a Scheduler is bundled with

Central Point's PC Tools.  You lodge the dates & times of the

programs you want to run and then load the resident portion of the

Scheduler into memory as a TSR from your AUTOEXEC.BAT.  The

Scheduler will kick in at the predetermined times and run the programs

you've defined.  If you happen to be working on your computer at a

scheduled program time, the scheduler will ask you if you want to

run the Scheduled program now or delay it; if you proceed to run it

the Scheduler will save the memory image of your current working

environment and restore it after it has run the scheduled program.

There are many other DOS schedulers around including several fine

shareware products.  Back'n Forth, a shareware Task-Switcher, has

many features including a built in scheduler.

Event-Driven I/R Signal Generation

----------------------------------

A truly automated I/R system should support event-driven I/R signal

generation.  The transmission of I/R signals should not be limited

to scheduled times, but rather, should be driven by events such as

the reception of X-10 signals, changes in temperature, motion

detection, contact closure, etc.  Most PC-based home automation

systems which react to received events support 'shelling to DOS' to

execute programs based on the events you specify; so IRDQ could be

used with these programs to integrate I/R signal generation.  The

HC2000 program is an example of a home automation system that is

based on 2-way X-10 communications.

The HC2000 Home Automation Program

----------------------------------

HC2000 is a software-based X-10 home automation system.  It requires

a dedicated PC but will run quite satisfactorily on a 4.77 MHz XT.

If you've upgraded your PC and have an older machine that is

collecting dust as a paperweight somewhere then HC2000 will allow

you to put it to good use as the 'brain' of your home automation

system.

HC2000 requires a TW523 device which is a two-way X-10 module that

sends and receives X-10 signals.  An Editor is supplied with HC2000

that allows you to build your home automation program.  You can

send X-10 signals at scheduled times, or when other X-10 commands

are received (allowing you to build remote 'macros'), or based on

the values of user variables that you define and manipulate.  In

addition to having the program respond to events by sending X-10

commands, you can also run DOS programs.  This means you can use

IRDQ.EXE to emit I/R signals based on the events that are trapped

by HC2000.  For example, you could define an X-10 command that would

invoke the button sequence to turn on the two TV's that was defined

earlier in this section; the following steps outline how you could

accomplish this:

1) Create the Button File: TURNTVS.ON

2) Start up the HC2000 editor and create a User Variable entitled

'DOS IRDQ.EXE TURNTVS.ON'.

3) Add an X-10 remote command using the House/Unit Code that you

want to define and add a 'LOG' statement using the User Variable

you defined in step 2: eg:

� A1 ON {Turn on both TVs} ::=

Log "DOS IRDQ.EXE TURNTVS.ON'

(This command would respond to House Code A, Unit 1.)

4) Exit the HC2000 editor and run HCDOS; HCDOS checks all of the

messages that HC2000 logs and when it encounters the IRDQ.EXE

message it will execute IRDQ.EXE and pass it the TURNTVS.ON

parameter.

IRDQ.EXE has been successfully tested with HC2000.

13. PROBLEM RESOLUTION

--------------------------------------------------------------------

The ONE FOR ALL remote control PC interface deploys a relatively

simple communications protocol and you should not experience any

problems provided that your special serial cable is in good

condition and there are no loose connections.  However, the timing

of the serial communications is very critical and there is not much

tolerance for even minor deviation.  Any resident software that

tampers with the system clock could throw the serial communications

out of sync.

If IRDC does not appear to be working at all, then read through the

following checklist to confirm your hardware/software setup.

1) Verify that your remote control is supported; See the ONE FOR ALL

REMOTES section for further information on the supported models.

2) Ensure that you are using the correct CommPort; click on the

Configuration button and verify that the active CommPort is

correct.

3) Ensure that the special serial cable connections are firm and

that the cable is not bent or stretched; The most likely cause is

the 3-pin serial connector not being seated properly in the

external port of the ONE FOR ALL remote.  In some cases, it may

appear to be properly seated but in fact it is not.  It may be

necessary to tape the connection to ensure that it is sound.

4) Ensure that memory resident software is not conflicting with

IRDC by booting your PC without any Device Drivers or TSR's.

(ie. use a stripped down CONFIG.SYS and AUTOEXEC.BAT).

If IRDC appears to be working but you run into a situation where the

LED on the remote control stays ON with an amber colour and will not

turn off, then communications between the remote control and the

software have been disrupted. The most likely cause for this is a

loose connection.  The remote control is in Serial Execution mode;

power OFF your PC, check cable connections, power ON your PC and

reload IRDC.  Click on a remote button and the LED should flicker

amber, then green, then turn off.  This problem can also be caused

by a CommPort IRQ conflict (ie. having a TSR loaded which uses

CommPort 4 while attempting to use IRDC with CommPort 2); in this case

it is necessary to perform a cold boot to reset serial devices and

then booting without loading the conflicting TSR.  If your remote

consistently becomes stuck in Serial Execution mode, then it is

probably the result of a timing problem; please contact the author

in this case, adjustments can be made to the software to work around

this.

If the ONE FOR REMOTE becomes "unresponsive" to IRDC, it could be

because commands were sent to it while it was executing a macro.

This only applies to a ONE FOR ALL 12 which will not act on commands

sent while it is executing a macro; you should wait for the macro

to complete before sending more commands.

As a last resort, you may want to try using another serial port to

see if this eliminates the problem. Other common problems &

solutions are listed below:

Interrupt Conflict

------------------

If you're using a COMMPORT that shares an interrupt with another

COMMPORT (eg. 1 & 3, 2 & 4), then you must ensure that no memory

resident software (eg. a FAX TSR) that uses the other COMMPORT is

loaded as it will not release the interrupt.

Serial Mouse Drivers

--------------------

If you use a Serial Mouse, this may cause a conflict if IRDC

attempts to communicate with a remote on the same CommPort.  IRDC

will not run properly if it attempts to communicate with a COMMPORT

that has been initialized with a Mouse Driver (ie.  you cannot use a

switchbox between the remote and a Mouse unless you reboot without

loading the Mouse Driver).

IRDC Encryption

----------------

The IRDC executable file is encrypted and there is a remote

possibility that the decryption process may fail with certain

processors; a blank screen immediately after attempting to run the

program typifies this problem.  Please contact the author if you

encounter this problem.

The open architecture of the PC compatible operating environment and

the near infinite combinations of hardware and software complicate

the compatibility of applications software.  While every effort has

been made to ensure that IRDC is compatible with as many hardware

and software combinations as possible, there may be operating

environments in which IRDC will not run properly.

Problems can be reported to:

David Huras

37 Ruddell Crescent

Georgetown, Ont. L7G 5N4

CANADA

FAX: 905-873-6429

CIS: 72767,472

Internet: david.huras@canrem.com

14. REGISTRATION INFORMATION

--------------------------------------------------------------------

The registration fee for IRDC is $25. (US Dollars for US shipping

addresses, CDN Dollars for Canadian shipping addresses).  IRDC is

currently supplied as fully functioning software without built-in

delays or "nag" screens prompting you to register. However, the

program will timeout after 10 minutes.  This limitation does not

apply to the Registered version.

A SIGNIFICANT amount of time and effort went into this program;

please register the program if you intend to use it beyond the 30

day evaluation period.  Registered Users will be shipped a printed

manual and a 3�" diskette (if you would prefer a 5�" diskette please

advise) containing the Registered Version of the IRDC.

Registered Users will also receive IRDQ.EXE, a command line driven

version of IRDC.  With IRDQ you can press buttons on ONE FOR ALL

remote keypads simply by entering commands at the DOS prompt, for

example you would type IRDQ POWER to press the POWER button.

Alternatively you can pass a file name containing a list of button

names to IRDQ, eg.  IRDQ /F:BUTTON.FIL where BUTTON.FIL contains

AUX

WAIT

POWER

TV

WAIT

POWER

VCR

WAIT

POWER

"<<"

This "macro" would turn on your Receiver, TV & VCR and rewind the

tape in the VCR.  IRDQ is a compact program which was designed to be

interfaced with software based home automation systems like HC2000.

You could also "grow your own" home automation system by using IRDQ

with a DOS-based program scheduler like the one provided with

Central Point's PC Tools.  All IRDQ program options can be imbedded

within the DOS environment thereby eliminating the need for any

command line arguments other than button names or a filename

containing button names.  (See the APPLICATIONS section for further

information about IRDQ).

To expedite your registration, you may wish to register using

CompuServe Shareware Registration; you will receive a Registration

Key via CompuServe E-Mail within 1 business day that will enable the

registered program features. (See CompuServe Shareware Registration

below).  If you elect to use this method of registration then your

your software will ship within 1 business day.

If you do not wish to register via CompuServe Shareware Registration

then complete the order form in the REGISTER.FRM file and send with

your payment to:

David Huras

37 Ruddell Crescent

Georgetown, Ont. L7G 5N4

CANADA

CompuServe Shareware Registration

=================================

You can register IRDC, IRDCw or BOTH using CompuServe Shareware

Registration.  The Registration Fee will be charged to your

CompuServe billing account and the latest version of the software

along with a printed User Guide will be shipped within one business

day.  In addition, you will receive a Registration Key via

CompuServe E-Mail (within one business day) that will immediately

allow you to run IRDC with the registered version features enabled

while you wait for your diskette to arrive.  GO SWREG while on

CompuServe to use Shareware Registration.

Program Title                      Program Name    Program ID#   Charge

===================                ============    ===========   =======

INFRA-RED Direct Control (DOS)        IRDC            1449       $25.00

INFRA-RED Direct Control (WINDOWS)    IRDCw           2521       $25.00

INFRA-RED Control Bundle (DOS & WIN)  IRCB            2522       $40.00

15. UPGRADE INFORMATION

--------------------------------------------------------------------

Quick Upgrade

=============

REGISTERED USERS of prior versions of IRDC can immediately run the

shareware version of IRDC v2.00 and all future shareware versions of

IRDC with the registered features enabled.   IRDC will look in the

current sub-directory for the IRDC.KEY file; if a valid file is

found then IRDC will enable all registered program features.   This

means you can upgrade by downloading the latest shareware version of

IRDC and ensuring that your IRDC.KEY file resides in the same

sub-directory.

Existing Registered Users

=========================

If you registered a version of IRDC prior to 2.00; then you are

entitled to an IRDC.KEY file that will enable you to immediately use

the shareware version of IRDC v2.00 with the registered features

enabled.  Your IRDC.KEY file can be forwarded via CompuServe E-Mail,

Internet E-Mail, or you can download it directly from the author.

Please contact the author via E-Mail or FAX to arrange for the

delivery of your IRDC.KEY file.

Diskette Upgrade

================

REGISTERED USERS of prior versions of IRDC can request a diskette

containing the current version of the software at ANY TIME by simply

sending a diskette containing their registered version of the

software and $2 for postage to the author.

REGISTERED USERS of IRDCw(WIN) can upgrade to IRDC(DOS) for

a special price of $15.00.  (See REGISTER.FRM for Details)

REGISTERED USERS of IRDC(DOS) can upgrade to IRDCw(WINDOWS) for

a special price of $15.00.  (See REGISTER.FRM for Details)

16. RELEASE HISTORY - IRDC Shareware/Registered Version

--------------------------------------------------------------------

Date      Version     Changes

------   ----------   ----------------------------------------------

FEB'95      2.11      -Upgraded encryption scheme to improve program

compatibility

JAN'95      2.10      -Added prompt to physically press a button

(to put remote ONLINE) upon first button click

after program load AND after 'Take Remote

Offline' is clicked for models URC2005,URC4005,

URC4050 & URC6050)

-Added prompt to 'Take Remote Offline' upon

program exit for models URC2005,URC4005,URC4050

& URC6050)

-Updated on-screen representation of URC4050

-Updated keycodes for models URC4005,URC4050 &

URC6050 (PROGRAM,RECORD,DISPLAY & MUTE keys)

NOV'94      2.00      -Added support for 4 additional ONE FOR ALL

remote control models (URC2005,URC4005,URC4050,

URC6050)

-Added Dynamic User Defined CommPort Support

-Added Quick Upgrade/Registration

-Changed initial warning prompt to contextual

jump into the Help Dialogue

APR'94      1.03      -Added Save IRDC.INI prompt when Exit attempted

without saving a changed Configuration

MAR'94      1.02      -Re-enabled ESCAPE key to exit Configuration

Panel

OCT'93      1.01      -Enabled ESCAPE key to exit Program

-Corrected typo in Help File Warning

(URC-5000-2 not supported should have

read URC-5000-1 not supported)

OCT'93      1.00      First Public Release

-Configuration Panel Re-organized

-SAVE Button Added

-Warning CheckBox Added

-'F1' Key to invoke Help added

-Cursor Key navigation added to keypad area

(cursor keys will navigate to adjacent

buttons - will skip 1 row or button if

no adjacent button).

-OFA-12 Button Definition Problems Corrected

(Mode Buttons, A,B,C, Power, CH+,CH-)

-OFA-12 Extended Remote Code support added

for F1 - F4.

-European Date Format Support Added

SEP'93      1.00�     Beta Test

------