������������������������������Ŀ

�MDRIVER.C Module Documentation�

��������������������������������

Date last modified: January 08, 1995

If you want to use any of the routines of this module, include "mdriver.h"

and link every soundcard driver you want to use with your program.

Available soundcard drivers:

Gus:

DRVGUS\GUSDEV.LIB

Soundblaster & compatibles:

DRVSB\SBDEV.LIB

if you want to use this, you'll also have to add VIRTCHAN\VIRTCHAN.LIB,

MDMA.C and MEMS.C to your project.

No sound:

DRVNOSND\NOSNDDEV.LIB

��������������������������������������������������������������������������������

General information:

As far as the other modules are concerned, the driver module is a set of

_universal_ routines for producing sound. The other modules don't have to

worry about how to drive a particular soundcard, they just 'ask' the driver

module to reserve so many voices, to mix at a certain rate and to play a

voice every once in a while. The driver module will adjust the requested

mixing mode and frequency if the actual soundcard doesn't support it.

When we take a closer look at the driver module we see that it's just a set

of 'glue' functions that redirect the driver function calls to the actual

selected soundcard driver:

����������������������������������������Ŀ

�DRIVER BLACKBOX�                        �

�����������������                        �

�                                        �

sb     �����������Ŀ                            �

hardware�Ŵsb driver ��ͻ <- selected driver      �

�������������  �                         �

�              �                         �

gus    �����������Ŀ  �                         �

hardware�Ŵgus driver��ĺ                         �

�������������  � ���������������������Ŀ �

�              �ʹDRIVER GLUE FUNCTIONS���<>� interface to

pas    �����������Ŀ  � ����������������������� �    other modules

hardware�Ŵpas driver��Ĵ                         �

�������������  �  - common driver        �

�              �    routines             �

other  �����������Ŀ  �                         �

hardware�Ŵetc..     ����  - common driver        �

�������������       variables            �

�                                        �

������������������������������������������

To prevent having to link _all_ lowlevel soundcard drivers to the player

(when you only want to use the gus-driver), the first thing the main module

has to do before using the driver module is to 'register' all drivers it

might need. This way only the registered drivers will get linked to the

executable, thus reducing the size of it. So registering a driver to the

driver module is a way of saying 'lookee here, another driver you can choose

from'.

��������������������������������������������������������������������������������

MDRIVER public variables:

DRIVER *md_driver       // pointer to the currently used driver

// this variable is set after a call to MD_Init()

UWORD  md_device	// device-number

UWORD  md_mixfreq	// current mixing frequency

UWORD  md_dmabufsize    // dma buffer size (for sb)

UWORD  md_mode		// output mode

UBYTE  md_numchn	// number of channels to use

UBYTE  md_bpm		// current bpm rate

UBYTE  md_mainvol	// current main volume

��������������������������������������������������������������������������������

void MD_InfoDriver(void)

========================

Input parms:

-

Returns:

-

Description:

This function produces a list of all registered drivers to stdout. Use it

_after_ you've registered all drivers using ML_RegisterDriver().

��������������������������������������������������������������������������������

void MD_RegisterDriver(DRIVER *drv)

===================================

Input parms:

drv     pointer to a DRIVER structure

Returns:

-

Description:

Before you can use any of the other device-routines you first have to

register the drivers you want to use. This way only the registered drivers

are linked to the resulting program, so if you only want to support a single

soundcard your program won't be so big.

Example:

[at the top of the program:]

extern DRIVER gusdriver,sbdriver;

[ at the start of the main program: ]

{

...

MD_RegisterDriver(&gusdriver);

MD_RegisterDriver(&sbdriver);

MD_InfoDriver();

...

...

}

��������������������������������������������������������������������������������

BOOL MD_Init(void)

==================

Input parms:

-

Returns:

True if soundcard init succeeded, FALSE otherwise.

Description:

Before calling this routine you have to initialize some variables:

md_mixfreq: 	What mixing frequency do you want to use (Hz)

md_dmabufsize: 	How big the soundcard dma-buffer should be (in bytes)

md_mode: 	Flags what output mode to use.. Use the bitwise OR (|)

to combine flags:

DMODE_16BITS : 16 bits output

DMODE_STEREO : stereo output

md_device:	The number of the driver you want to use. Use 0 to

use the first soundcard it detects.

MD_Init will call the initialisation routine of the specified driver. When

the soundcard doesn't support any of selected mixing modes or mixing

frequency it will adjust the md_mode & md_mixfreq to values that _are_

supported. For example, if you have a SB-pro and you try to init it with the

DMODE_16BITS flag set, it removes this flag after the MD_Init() call.

!!! It is ILLEGAL to change the variables md_mixfreq,md_dmabufsize, md_mode &

md_device AFTER you've called MD_Init()!!!

��������������������������������������������������������������������������������

**** The following routines may only be called if MD_Init succeeded ****

��������������������������������������������������������������������������������

void MD_Exit(void)

==================

Input parms:

-

Returns:

-

Description:

De-initialises the soundcard after you're done with it.

��������������������������������������������������������������������������������

BOOL MD_SampleLoad(FILE *fp,SAMPLEINFO *smp)

============================================

Input parms:

fp	filepointer to raw sample data

smp	pointer to a SAMPLEINFO structure, which describes the format

& size of the sample data.

Returns:

True when sampleload succeeded, FALSE otherwise.

Description:

Loads a raw sample from file to the soundcard. Apart from returning the BOOL,

it also sets the member 'handle' of the SAMPLEINFO structure to a value which

identifies the sample (you need the handle if you want to play the sample).

��������������������������������������������������������������������������������

void MD_SampleUnLoad(SAMPLEINFO *smp)

=====================================

Input parms:

smp	pointer to a SAMPLEINFO structure, which describes the format

& size of the sample data, and contains the handle of the

sample in the member 'handle'.

Returns:

-

Description:

Frees the sample which was previously loaded using MP_SampleLoad().

��������������������������������������������������������������������������������

void MD_PlayStart(void)

=======================

Input parms:

-

Returns:

-

Description:

Starts the mixing process of the soundcard (after calling this routine you

can use the MD_Voice* routines to make noise). Before calling this routine

you have to set the variable 'md_numchn' so it knows how many channels it has

to mix. This routine also installs a BPM interrupt which takes care of

updating the voice parameters every tick. The speed at which the interrupt

occurs can be controlled by adjusting the variable md_bpm, where a value of

125 (default) will make the interrupt go at 50Hz, and 250 will make the

interrupt go at 100Hz.

��������������������������������������������������������������������������������

void MD_PlayStop(void)

======================

Input parms:

-

Returns:

-

Description:

Stops soundcard output & stops the BPM interrupt.

��������������������������������������������������������������������������������

void MD_RegisterTickHandler(void (*tickhandler)(void))

======================================================

Input parms:

tickhandler	pointer to a routine that should be called every

bpm interrupt.

Returns:

-

Description:

Installs a custom BPM handler which you could use to update a song. Take a

look at MIKMOD.C to see how this could be done.

��������������������������������������������������������������������������������

void MD_VoiceSetVolume(UBYTE voice,UBYTE vol)

=============================================

Input parms:

voice	which voice to change the volume for.

vol	new volume setting for this voice (0-64)

Returns:

-

Description:

Sets the volume of a single voice. Note that it doesn't change the volume

immediately but at the next BPM tick (this is true for all MD_Voice*

routines).

��������������������������������������������������������������������������������

void MD_VoiceSetFrequency(UBYTE voice,ULONG frq)

================================================

Input parms:

voice	which voice to change the frequency.

frq	new playback frequency this voice.

Returns:

-

Description:

Sets the playback frequency of a single voice.

��������������������������������������������������������������������������������

void MD_VoiceSetPanning(UBYTE voice,UBYTE pan)

==============================================

Input parms:

voice	which voice to change the panning position.

pan	new panpos for this voice.

Returns:

-

Description:

Changes the panning position (=balance) for this voice. 0 is all left, 255 is

all right.

��������������������������������������������������������������������������������

void MD_VoicePlay(UBYTE voice,

WORD handle,

ULONG start,ULONG size,

ULONG reppos,ULONG repend,

UWORD flags)

============================================

Input parms:

voice	which voice to start playing a new sample on

handle	the handle that identifies the sample

start	the starting offset (in samples!)

size	the size of the sample (in samples!)

reppos	repeat start position offset.

repend	repeat end position offset.

flags	identifies the type of sample, and if it has to loop.

Returns:

-

Description:

Starts playing a new sample on the specified channel, using the requested

start,size and loop parameters (using the current volume, frequency and

panning parameters of a voice).

��������������������������������������������������������������������������������

void MD_RegisterTickHandler(void (*tickhandler)(void))

======================================================

Input parms:

tickhandler	pointer to a routine that should be called every

bpm interrupt.

Returns:

-

Description:

Installs a custom BPM handler which you could use to update a song. Take a

look at MIKMOD.C to see how this could be done.

��������������������������������������������������������������������������������