W X R E S E T

= = = = = = =

version 1.00                              compiled 08/01/95

by

Michael A. Conley

A program designed to reset the Heath Weather Station Model IDA-5001-3

stored high/low values and transmit any desired commands to the device.

SETTING UP:

===========

The first order of business is to place WXRESET.CFG and WXRESET.EXE in a

convenient directory on your system.

Next, you'll need to edit the WXRESET.CFG configuration file, in order to

set up the communication parameters to the device and to alter the table

of commands to suit your needs.  There is a complete list of available

commands beginning on page 3-15 of the Heath Weather Station manual.

Any commands may be sent, but WXRESET has no provision for displaying the

results.  You should examine WXDOOR, distributed with this program, if

it's desired to display the output data from the Weather Station.

CONFIGURATION FILE:

===================

There is a configuration file (WXRESET.CFG) distributed with this ZIP file,

and a copy is shown below for reference:

WXPORT=8

WXIRQ=0

WXCOM=8N1

WXFOSSIL=YES

WXBAUD=9600

WXHANDSHAKE=BOTH

COMMAND=ATAC    ;shut off any pending alarms

COMMAND=ATCBH   ;clear barometer high

COMMAND=ATCBL   ;clear barometer low

COMMAND=ATCHIH  ;clear humidity indoor high

COMMAND=ATCHIL  ;clear humidity indoor low

COMMAND=ATCHOH  ;clear humidity outdoor high

COMMAND=ATCHOL  ;clear humidity outdoor low

COMMAND=ATCR    ;clear rain

COMMAND=ATCTIH  ;clear temp indoor high

COMMAND=ATCTIL  ;clear temp indoor low

COMMAND=ATCTOH  ;clear temp outdoor high

COMMAND=ATCTOL  ;clear temp outdoor low

COMMAND=ATCWAH  ;clear wind average high

COMMAND=ATCWAL  ;clear wind average low

COMMAND=ATCWGH  ;clear wind gust high

COMMAND=ATCWGL  ;clear wind gust low

|Explanation of lines:

|

|   WXPORT=        This is the com port number of your weather station.

|                  If the specified port is 1 to 4, the program opens that

|                  port. If you specify the port as any other decimal value,

|                  the program will use that as the base address for the

|                  port. This allows you to work with non-standard I/O

|                  addresses. Note however that if you open a non-standard

|                  port, you must specify an WXIRQ value from 2 to 15 -

|                  There is no default IRQ when used in this manner.

|

|   WXIRQ=         Specify the IRQ of the port as 2 to 15.  If you're using

|                  a FOSSIL driver, this number is ignored and may be set

|                  to zero.

|

|   WXCOM=         These are the communication parameters. Specify 7 or 8

|                  databits, Odd, Even, or No parity, and 1 or 2 stopbits,

|                  in a form such as 8N1 or 7E1.

|

|   WXFOSSIL=      If using a FOSSIL driver, set this to YES, otherwise

|                  set it to NO.

|

|   WXBAUD=        The baud rate of your weather station, usually 9600,

|                  but must match the dip switch settings in the station.

|

|   WXHANDSHAKE=   SOFTWARE, HARDWARE, or BOTH are the options for the

|                  handshaking with the weather station.  Normally you

|                  should use BOTH, based on our test installation.

|

|   COMMAND=       You may have as many of these as desired.  Each command

|                  should begin with the letters AT, and any comments on

|                  the line should be separated by several spaces from

|                  the command.  The correct structure should look like

|                  the samples above in the distributed file.  You can

|                  disable a command by putting a semicolon in front of

|                  it, without removing it from the file.

<EOF>

RUNNING THE PROGRAM:

====================

Once the configuration file has been edited to suit your needs, the

program may be run from the DOS command line with no switches.  Just

be sure that the configuration file is present in the current directory.

------------------------------------------------------------------

PROBLEMS, SUGGESTIONS:

This program is public domain.  I don't seek or accept any donations or

registration fees for any of my Wildcat! utility programs, including

TODAYBBS, NEWFLOP, NETBULL, USTAT, and others ... however I invite you

to read the short textfile "CHARWARE.TXT" enclosed in this ZIPfile.  It

suggests that you make a contribution to YOUR favorite charity if you

find this program useful.

Current versions of all Cabin SysOp Utilities may be obtained from the

Charlatan's Cabin BBS at (213) 654-7337, or from Mustang Software's BBS,

(805) 873-2400, or via Mustan'g FTP server on the Internet. Their

address is "ftp://ftp.mustang.com/ftp/"

I welcome suggestions, and will try to help (no guarantees) if there are

problems.   Be sure to include specific information, including the

version number of this program, and the Wildcat! version number, when

you report problems.  A report such as "I downloaded it and it doesn't

work" doesn't give me much to go on!

You may contact me at The Charlatan's Cabin BBS  (213) 654-7337. Mention

your Wildcat! registration number in the comment and you'll be upgraded

to visiting sysop level without the usually required registration by

mail.

My Internet address is:    michael.conley@cabin.com

You may also leave a message via CompuServe EMail. My CompuServe account

number is 70250,220.  I also regularly check the Mustang Software head-

quarters BBS, especially the "3rd Party Utilities Help" conference.

Please, NO VOICE CALLS.  This isn't meant to be unfriendly, but since I

derive NO income from these programs, I just can't afford to be "on

call."  I WILL respond to your questions via my BBS, Internet,

CompuServe, or via the Mustang Software registered SysOp BBS.