This file contains the information for how to change the scanner

configuration settings.

1. INTRODUCTION

~~~~~~~~~~~~~~~

During the installation of Mustek TWAIN Data Source, the scanner settings

(i.e., IRQ, DMA and I/O address settings) will be automatically updated

in the WIN.INI file. However, there are instances when you may need to

change the IRQ, DMA or I/O address setting to avoid a hardware conflict.

2. HOW TO CHANGE THE SCANNER SETTING

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

To change the scanner settings, please execute the SCANTEST module which

can be found in the Mustek TWAIN group icon and follow the onscreen

instructions.

Note: To change the I/O address setting for some scanner interface card,

you should change it by changing the I/O address switch setting on

the scanner interface card and the Scantest module will detect the

new setting automatically. Please refer to the included manual for

how to change the I/O address.

3. CALIBRATE THE SCANNER

~~~~~~~~~~~~~~~~~~~~~~~~

For best scanning results, we recommend that you calibrate the scanner

the very first time you use it. The calibration procedure involves

scanning a white sheet of paper and adjusting the brightness control

until you get a Calibration OK message. After this you don't need to

calibrate again unless you notice a significant decrease in scan quality.

To calibrate the scanner, the steps are:

1.Set the Brightness Control of your scanner to the middle position.

2.In the Hand Scanner dialog box, click the Preference button.

The Preference dialog box appears.

3.Click the Calibration button and follow all onscreen instructions.

4. TWAIN COMPLIANT

~~~~~~~~~~~~~~~~~~

1. ABOUT OmniPage OCR (TWAIN-Compliant Version)

Please modify the SCANMGR.INI under the Windows directory as below:

1.1 Set

...

TWAINDS=Mustek Hand Scanner II

...

1.2 Add

...

[Mustek Hand Scanner II]

SHOW_UI=1    --------------> Show User Interface

INVERT=1     --------------> Invert image

AUTO_BRI=0   --------------> Text(LineArt) Scan Mode

DPI=1        --------------> Resolution = 300 dpi

DOCTYPE=0    --------------> Document Type = Black & White

ORIENT=4     --------------> Orientation = Landscape(L to R)

1.3 Switch scanner to TEXT scan mode, 300 DPI.

2. ABOUT GO-CR V1.3 & V2.1

Merge function is not supported for this application.