NISTIR 4513

Version 3.2

(1993 edition)

DISCOUNT--A Program for Discounting Computations

in Life-Cycle Cost Analyses

User's Guide and Reference Manual

Stephen R. Petersen

U.S. DEPARTMENT OF COMMERCE

National Institute of Standards and Technology

Office of Applied Economics

Computing and Applied Mathematics Laboratory

Gaithersburg, MD 20817

Sponsored by

The Federal Energy Management Program

U.S. Department of Energy

Washington, D.C. 20585

January 1991

U. S. DEPARTMENT OF COMMERCE

Robert A. Mosbacher, Secretary

NATIONAL INSTITUTE OF STANDARDS AND TECHNOLOGY

John W. Lyons, Director

ABSTRACT

The DISCOUNT program computes discount factors and related present values,

future values, and periodic payment values of cash flows occurring at known

points in time.  DISCOUNT computations are especially useful in life-cycle

cost analysis, financial analysis, and engineering-economics problems not

requiring comprehensive summation and reporting.  DISCOUNT performs all of the

functions of standard discounting tables, computing present values of future

amounts, future values of present amounts, present and future values of

periodic payments, and periodic payments corresponding to present and future

amounts.  In addition, DISCOUNT computes the present value of periodic

payments which increase at known rates over time, and the present value of

energy costs which increase at rates projected by the U.S. Department of

Energy for use in Federal life-cycle cost analyses.  DISCOUNT provides the

added flexibility of accepting non-integer discount rates, time periods, and

escalation rates in its computations.  DISCOUNT runs on most IBM PC and

compatible microcomputers with no special equipment requirements.

iii

PREFACE

The DISCOUNT computer program, version 3.2, has been prepared as an aid to

implementing life-cycle cost evaluations of potential energy conservation and

renewable energy projects in new and existing Federal buildings.  It can also

be used for evaluating similar projects in the private sector and at the state

and local government levels.

DISCOUNT is the fourth of a four-volume set of reports which together provide

a comprehensive set of methods, data, and computational tools for economic

evaluation of such projects.  Included in the four-volume set are the

following:

(1) Life-Cycle Costing Manual for the Federal Energy Management Program,

National Institute of Standards and Technology, Handbook 135 (revised

1991).

Handbook 135 is a guide to understanding life-cycle costing and related

methods of economic analysis as they are applied to Federal decisions.  It

describes the required procedures and assumptions, defines and explains how to

apply and interpret economic performance measures, gives examples of Federal

decision problems and their solutions, explains how to use the energy price

indices and discount factors which are updated annually in the supplement (2),

and provides worksheets and other computational aids and instructions for

calculating the required measures.

(2) Energy Prices and Discount Factors for Life-Cycle Cost Analysis, National

Institute of Standards and Technology, (Annual Supplement to NIST

Handbook 135 and NBS Special Publication 709), NISTIR 85-3273-5 (revised

10/90 and updated annually).

This report, which is updated annually, gives the energy price and discount

factor multipliers needed to estimate the present value of energy and other

future costs.  The data are based on energy price projections developed by the

Energy Information Administration of the U.S. Department of Energy.  Request

the latest edition.

(3) The NIST "Building Life-Cycle Cost" (BLCC) computer program (version 3.2),

National Institute of Standards and Technology, October 1991.

The Building Life-Cycle Cost (BLCC) computer program, version 3.2, has been

prepared as an aid to implementing life-cycle cost evaluations of potential

energy conservation and renewable energy projects in new and existing Federal

buildings.  It can also be used for evaluating similar projects in the private

sector and at the state and local government levels.  BLCC 3.2 supersedes and

incorporates both the Federal Building Life-Cycle Cost (FBLCC) and the

National Bureau of Standards Life-Cycle Cost (NBSLCC) programs.  For the

economic analysis of energy conservation investments in Federal buildings,

BLCC complies with NBS Handbook 135, Life-Cycle Cost Manual for the Federal

Energy Management Program.  For other investments in Federal buildings, BLCC

complies with OMB Circular A-94, "Discount Rate to Be Used in Evaluating Time-

iv

distributed Costs and Benefits," March 27, 1972.  BLCC complies with the

American Society for Testing and Materials (ASTM) standard practices for

building economics, including E917 (Practice for Measuring Life-Cycle Costs of

Buildings and Building Systems), E964 (Practice for Measuring Benefit-to-Cost

and Savings-to-Investment Ratios for Buildings and Building Systems), E1057

(Practice for Measuring Internal Rates of Return for Investments in Buildings

and Building Systems), and E1074 (Practice for Measuring Net Benefits for

Investments in Buildings and Building Systems).

(4) DISCOUNT--A Program for Discounting Computations in Life-cycle Cost

Analyses (version 3.2), NISTIR 4513, National Institute of Standards and

Technology, 1991.

The DISCOUNT program computes discount factors and related present values,

future values, and periodic payment values of cash flows occurring at known

points in time.  DISCOUNT computations are especially useful in life-cycle

cost analysis, financial analysis, and engineering-economics problems not

requiring comprehensive summation and reporting.  DISCOUNT performs all of the

functions of standard discounting tables, computing present values of future

amounts, future values of present amounts, present and future values of

periodic payments, and periodic payments corresponding to present and future

amounts.  In addition, DISCOUNT computes the present value of periodic

payments which increase at known rates over time, and the present value of

energy costs which increase at rates projected by the U.S. Department of

Energy for use in Federal life-cycle cost analyses.  DISCOUNT provides the

added flexibility of accepting non-integer discount rates, time periods, and

escalation rates in its computations.  DISCOUNT runs on most IBM PC and

compatible microcomputers with no special equipment requirements.

Both BLCC 3.2 and DISCOUNT 3.2 access DOE energy price escalation rates from

the same two files (currently ENCOST93.SEQ and ENCOST93.RAN).  These files are

updated and released at the same time that NISTIR 85-3273, Energy Prices and

Discount Factors for Life-Cycle Cost Analysis, is released each year.  These

disk files ensure that BLCC and DISCOUNT are compatible with computations

performed using the factors in NISTIR 85-3273.

For convenience, the BLCC 3.2 and DISCOUNT 3.2 programs are packaged and

distributed together on two 360K disks (one 3.5 inch disk) in MS-DOS format.

Text file versions of both user's guides are included on these disks.

For representatives of Federal government agencies and contractors to the

Federal government, these publications can be ordered from:

Advanced Sciences, Inc.

2000 North 15th Street

Suite 407

Arlington, VA 22201

(703) 243-4900

Please request the publications by name and number.

v

For others interested in obtaining the BLCC and DISCOUNT programs, please

contact one of the following organizations:

National Technical Information Service

5285 Port Royal Road

Springfield, VA 22161

(703) 487-4650

PC Software Interest Group

1030D East Duane Avenue

Sunnyvale, CA 94086

(408) 730-9291

MTS Software

5 Oak Forest Court

Saint Charles, MO 63303

(314) 441-1022

Energy Ware

P.O. Box 254

Littleton, NH 03561

(800) 522-4485

Workshops on the life-cycle costing method and energy analysis are conducted

at locations around the country each year.  The workshops include training and

software for both BLCC and an energy analysis computer program called "A

Simplified Energy Analysis Method" (ASEAM).  A schedule of workshops can be

obtained from the Office of Applied Economics, National Institute of Standards

and Technology, Bldg. 101, Room A415, Gaithersburg, MD 20899, (301) 975-6132.

An introduction to the workshop is provided in a video training film, "Least-

Cost Energy Decisions:  An Introduction to Life-Cycle Cost Analysis."  The

video film and companion workbook can be ordered from Video Transfer, Inc.,

5709-B Arundel Avenue, Rockville, MD 20852, (301) 881-0270.  Additional

training films in the series are in preparation.

Further information on the Federal Energy Management Program can be obtained

from the Federal Energy Management Program Staff, Office of the Assistant

Secretary for Conservation and Renewable Energy, U.S. Department of Energy.

Please direct communication to:  FEMP, CE 10.1, U.S. Department of Energy,

1000 Independence Avenue, SW, Washington, D.C. 20585.

vi

ACKNOWLEDGMENTS

The author wishes to thank Mr. Richard W. Brancato and Mr. K. Dean DeVine of

the Federal Energy Management Program of the U.S. Department of Energy (DOE)

for their continued support and direction of this work.  Appreciation is also

extended to Harold Marshall, Rosalie Ruegg and Siglinde Fuller of the Office

of Applied Economics and Gregory Tassey of the NIST Program Office for their

careful review of this User's Guide.

vii

CONTENTS

Page

ABSTRACT . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .   iii

PREFACE. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .    iv

ACKNOWLEDGEMENTS . . . . . . . . . . . . . . . . . . . . . . . . . . .   vii

1.  INTRODUCTION . . . . . . . . . . . . . . . . . . . . . . . . . . .     1

2.  INSTALLATION AND FILE REQUIREMENTS . . . . . . . . . . . . . . . .     3

3.  RUNNING DISCOUNT . . . . . . . . . . . . . . . . . . . . . . . . .     3

4.  COMPUTATIONAL METHODS. . . . . . . . . . . . . . . . . . . . . . .     8

List of Figures

Figure 1.  Discounting applications menu. . . . . . . . . . . . . . .      2

Figure 2.  Selecting Terms to be Defined. . . . . . . . . . . . . . .      2

Figure 3.  Present Value when Future Value is Known . . . . . . . . .      4

Figure 4.  Uniform Periodic Payment when Present Value is Known . . .      4

Figure 5.  Present Value of a Non-Uniform Periodic Amount Increasing

at a Constant Escalation Rate. . . . . . . . . . . . . . .      5

Figure 6.  Present Value of a Non-Uniform Periodic Amount Increasing

at a Changing Escalation Rate . . . . . . . . . . . . . . .     6

Figure 7.  Present Value of Annually Recurring Energy Expenditures

Using DOE's Projected Energy Price Escalation Rates . . . .     7

Figure 8.  DOE Energy Price Escalation Rates Used in Example . . . . .     8

vii

1.  Introduction

The DISCOUNT program provides a convenient method of computing many of the

values and factors needed for life-cycle cost analyses, engineering-economic

analyses, and financial analyses.  DISCOUNT performs the functions of standard

discounting tables, computing present values of future amounts, future values

of present amounts, present and future values of periodic payments, and

periodic payments corresponding to present and future amounts.  In addition,

DISCOUNT computes the present value of periodic payments which increase at

known rates over time, and the present value of energy costs which increase at

rates projected by the U.S. Department of Energy (DOE) for use in life-cycle

cost analyses of buildings and building systems owned or operated by the

Federal government.  The DOE projections of energy price increases are in data

files included with the DISCOUNT program and are automatically retrieved from

the disk as needed.  These data files are updated annually by NIST, following

release of the new projections by DOE.

DISCOUNT provides the added flexibility of using non-integer discount rates,

time periods, and escalation rates in its computations.  All computations are

made using double-precision mathematics.  Dollar values up to

$999,999,999,999.99 can be accurately calculated.  Definitions of terms used

in the DISCOUNT program can be displayed on the screen as needed.  These

features make the DISCOUNT considerably more useful and accurate than even the

most voluminous standard tables of discount factors.

An end-of-year discounting convention is used in DISCOUNT.  That is, all cash

flows are assumed to be incurred at the end of each time period and are

discounted from that point in time.  Interest compounding through multiple

time-periods occurs once per time period.  Thus, if each time period

represents one year, compounding occurs once per year.  If each time period

represents one month, compounding occurs 12 times per year.

Figure 1 shows the discounting applications menu, which lists the discounting

operations performed directly by DISCOUNT.  The desired operation is selected

by highlighting the application and pressing <Enter> or by entering its

corresponding application number (1-9).  Additional discounting functions are

possible by using a combination of operations.  All relevant input variables

and computational results are maintained in memory from application to

application.  Thus, after finding the present value of annually recurring

costs which escalate at some known rate (applications 7, 8, and 9), the

corresponding future value can be found by pressing <Esc> to return to the

main menu.  Then select application 2 (future value of a present value amount)

and press the <Enter> key for each of the values displayed: the present value

amount, number of time periods, and discount rate.  The future value of these

annually recurring costs will then be displayed.

Figure 2 shows a list of terms used in the DISCOUNT program that can be

defined on the screen.  Press 'D' to display the list; use the up and down

arrows to highlight the term to be defined and press <Enter>.  Most of the

definitions used are from ASTM E 833-89, "Standard Terminology of Building

Economics."

1

����������������������������������������������������������������ͻ

�   �������������������� SINGLE AMOUNTS ���������������������    �

�   1. PRESENT VALUE when future value is known.                 �

�   2. FUTURE VALUE when present value is known.                 �

�                                                                �

�   �������������� UNIFORM SERIES OF PAYMENTS ���������������    �

�   3. PRESENT VALUE when periodic payment amount is known.      �

�   4. FUTURE VALUE when periodic payment amount is known.       �

�   5. PERIODIC PAYMENT AMOUNT when present amount is known.     �

�   6. PERIODIC PAYMENT AMOUNT when future amount is known.      �

�                                                                �

�   ������������ NON-UNIFORM SERIES OF PAYMENTS �������������    �

�   7. PRESENT VALUE when periodic payment amount increases      �

�      at constant rate.                                         �

�   8. PRESENT VALUE when periodic payment amount increases      �

�      at changing rate.                                         �

�   9. PRESENT VALUE using DOE energy price escalation rates.    �

�                                                                �

����������������������������������������������������������������ͼ

Figure 1.  Discounting applications menu.

�����������������������������ͻ

� Base Year                   �

� Discount Rate               �

� Escalation Rate             �

� Future Value                �

� Non-Uniform Series          �

� Periodic Payment Amount     �

� Present Value               �

� Study Period                �

� Time Period                 �

� Uniform Series              �

�����������������������������ͼ

Figure 2.  Selecting Terms to be Defined.

2

DISCOUNT is intended to be used for assistance in discounting individual cost

components in an economic analysis.  If more comprehensive analysis and

reporting are required, including summation of cash flows on a time-equivalent

basis and computation of economic decision factors for alternative investments

(e.g., net savings, savings-to investment ratio, or adjusted internal rate of

return), a life-cycle cost program such as the NIST Building Life-Cycle

Computer Program (BLCC version 3.2) may be more appropriate*.

2.  Installation and File Requirements

DISCOUNT can be run directly from the original disk or installed on a hard

disk.  Once loaded, DISCOUNT runs equally fast from a hard drive or from the

original disk, except when accessing the DOE energy price projections from the

disk (application 9 only).  If it is to be run from the original disk, it is

suggested that this disk be copied and the copy be used.  If it is to be

installed on a hard disk, use the Change Directory command (CD) to move to the

(sub)directory where it is to be installed.  Then place the original disk in

the A drive and type 'COPY A:*.*' to copy the files to the hard disk:

DISCOUNT.EXE, ENCOST93.RAN, ENCOST93.SEQ, and DISCOUNT.DOC.  DISCOUNT.EXE is

the executable program file.  The two ENCOST93 files contain the DOE energy

price escalation rates used with discounting application 9 only.  (These are

the same files used with the NIST BLCC program (version 3.2).  The

DISCOUNT.DOC file is the name of the data file containing this user's guide

and reference manual.

DISCOUNT does not require graphics capability, math coprocessor, or other

special hardware to run.

3.  Running Discount

To run DISCOUNT from the current directory, type DISCOUNT at the DOS ready

prompt.  After reading the initial screen, press any key to bring up the main

discounting applications menu, as shown in figure 1.  Use the cursor arrows to

highlight the application desired, and press <Enter>, or simply press the

application number (1-9).  To see the definition of any term shown in figure

2, press "D", and then highlight the term to be defined and press <Enter>.

Press "Q" to quit the program and return to DOS.

For discounting applications 1 through 7, the data entry form will appear on

the screen in a format similar to that shown in figure 3.  Enter the parameter

values requested on the screen, pressing <Enter> or the up or down cursor key

after each entry.  Once all values are entered, the answer will appear below

the box.  A change in any of the parameters will immediately change the answer

shown.  IMPORTANT:  The <Enter> or the up or down cursor keys MUST be pressed

to effect this change.

___________

* Petersen, S. R., The NIST Building Life-Cycle Cost (BLCC) Program

(version 3.2), NISTIR 4481, National Institute of Standards and Technology,

Gaithersburg, MD, January 1991.

3

������������������������������������������������������������������ͻ

�  Future amount (dollars)                        100              �

�  Number of time periods (usually years):        10               �

�  Discount rate (percent per time period):       10    %          �

������������������������������������������������������������������ͼ

Present Value =   $38.55

Single Present Value Factor = 0.3855

Figure 3.  Present value when future value is known.

When entering dollar values, do not use the dollar sign ($) or commas.  When

entering discount or escalation rates, enter them in percentage terms, not in

decimal form.  For example, 10 percent is entered as 10, not 0.10.  Do not

include the percent (%) sign.

If fractional time periods (e.g., 10.5) are entered for applications with

uniform or non-uniform series of payments (applications 3-9), a corresponding

fractional payment is assumed in the final period.  For example, if the

present value of annual maintenance costs is to be calculated over 10.5 years,

and $100 is the annual cost, then the final half-period maintenance cost is

assumed to be $50 (0.5 � $100).  If the periodic payment increases at a

specified escalation rate per time period, then the payment in the final

fractional time period is assumed to be P � f � (1 + e)f, where P is the

payment amount at the end of the last full (integer) time period, f is the

fractional time period, and e is the escalation rate for the fractional

period.

If time periods other than one year are used, the discount (interest) rate

must reflect the rate for the period actually used and not the annual rate.

For example, if the monthly payment on a $100,000 loan over 30 years at 12%

annual interest (1% monthly) is desired, press 5, "Uniform periodic payment

when present value is known."  Enter 100000 for the present amount, 360 (30

years � 12 months per year) for the number of time periods and 1 for the

monthly interest (discount) rate.  This example is shown in figure 4.  (Note:

because of compounding effects, the sum of 12 monthly payments is not the same

as the annual payment calculated using 30 years and 12 percent interest.)

������������������������������������������������������������������ͻ

�  Present amount (dollars):                      100000           �

�  Number of time periods (usually years):        360              �

�  Discount rate (percent per time period):       1     %          �

������������������������������������������������������������������ͼ

Periodic Payment =   $1,028.61

Uniform Capital Recovery Factor = 0.0103

Figure 4.  Uniform periodic payment when present value is known.

4

For the calculation of the present value of a non-uniform series of payments,

starting with an initial amount changing from period to period at a constant

rate (application number 7), the escalation rate (i.e, the periodic rate of

change in the periodic amount) must be entered.  See figure 5 for an example

of this application.  In this example, an annual payment which has a value of

$100 initially, but increases at 5% per year over 30 years and is discounted

at 10%, has a present value of $1,579.86.  Note that the first payment (at the

end of year 1) is actually $105.00 (not $100.00), since the escalation rate

is applied uniformly in every year.

������������������������������������������������������������������ͻ

�  Initial periodic amount (base year dollars):   100.00           �

�  Number of time periods (usually years):        30               �

�  Discount rate (percent per time period):       10    %          �

�  Escalation rate (percent per time period):     5     %          �

������������������������������������������������������������������ͼ

Present Value =   $1,579.86

Uniform Present Value Factor =  15.799

Figure 5.  Present value of a non-uniform periodic amount

increasing at a constant escalation rate.

For the calculation of the present value of a non-uniform series of payments,

starting with a known initial amount and changing from period to period at a

changing rate (application number 8), up to ten rates of change and the

duration in consecutive time periods can be entered. (The number of rates

entered cannot exceed the number of periods in the analysis.)  An example of

this application is shown in figure 6.  After entering the initial payment

amount, the total number of periods in the analysis, and the discount rate,

the number of escalation rates to be used is entered (1-10).  Then a separate

box will appear on the screen, in which the escalation rates and corresponding

duration (number of consecutive time periods for each rate) are to be entered.

The total number of periods for these escalation rates must add up to the

total number of time periods specified for the analysis.  (If a non-integer

number of time periods is used, then the total number of periods for the

escalation rates must equal the total number of time periods in the analysis,

rounded up to the next integer.)  Press <Esc> when the escalation rates and

number of periods each have been entered and the answer will be displayed.

If the number of time periods in the analysis is changed, the number of

periods must also be changed for at least one of the escalation rates to make

the total number of periods correspond.  To move into the escalation rate box,

move the cursor to the "number of escalation rates to be used" and press

<Enter>.

5

������������������������������������������������������������������ͻ

�  Initial periodic amount (base year dollars):   100              �

�  Number of time periods (usually years):        30               �

�  Discount rate (percent per time period):       10    %          �

�  Number of Escalation Rates to be used (1-10):  3                �

������������������������������������������������������������������ͼ

Escalation  Number of

Rate (%)   Periods              PRESENT VALUE =   $1,446.35

�������������������ͻ             Modified Uniform Present

1  �   5            10 �               Value Factor = 14.464

2  �   3            10 �

3  �   2            10 �

�������������������ͼ

Figure 6.   Present value of a non-uniform periodic amount

increasing at a changing escalation rate.

Figure 6 shows the present value of a non-uniform series of annual payments

over 30 years, valued at $100 initially (i.e., at the beginning of the first

time period), and increasing at 5% per year for the first 10 years, 3% for the

next 10 years, and 2% for the final 10 years.  This kind of discounting

operation is frequently used in energy cost analyses, when the periodic rate

of energy price increase is expected to vary over time instead of remaining

constant.

To calculate the present value of a series of annual energy costs, starting

with a known initial amount, and increasing annually at rates projected by the

U.S. Department of Energy, use application number 9.  An example of this

discounting application is shown in figure 7.

Because the DOE forecasts vary with region, rate type, energy type, base year,

each of these must be specified.  There are four DOE regions: (1) North East,

(2) North Central, (3) South, and (4) West.  The states in each region are

displayed on the screen when the cursor is located on the region field.  There

are three energy rate types: (1) residential, (2) commercial, and

(3) industrial.  And there are six energy types: (1) electricity,

(2) distillate fuel oil, (3) residual fuel oil, (4) natural gas, (5) LPG, and

(6) coal.  Not all combinations of rate types and energy types have DOE

projections.  There are no DOE projections for residential coal or residual

fuel oil, or for commercial and industrial LPG.  An error message will be

displayed if one of these combinations is entered.

The escalation rates for energy prices projected by DOE vary from year to

year, and thus the base year must be specified.  The base year must be in the

range 1990 to 2000.  The initial energy cost must be specified in base-year

dollars, and all future costs will be discounted to the beginning of this base

year.

6

The DOE projections of energy price escalation rates do not include general

inflation; that is, they are in real terms.  If the analysis is to include

general inflation, then the user must specify the underlying average rate of

inflation over the period of analysis.  If the analysis does not include

inflation, then set the general inflation rate to 0.  If inflation is

included, the discount rate should reflect this same inflation rate.  If

inflation is excluded, general inflation should be excluded from the discount

rate.  Note that the nominal discount rate (i.e., including inflation) is not

simply the sum of the real discount rate (excluding inflation) and the

underlying general inflation rate, but is calculated as*:

D = d + I + dI

where D = the nominal discount rate,

d = the real discount rate,

I = the underlying rate of general inflation.

������������������������������������������������������������������ͻ

�  Annual energy expenditure (base year dollars): 100              �

�  DOE region (1-4):                              1                �

�  Rate type (1-3):                               1    Residential �

�  Energy type (1-6):                             1    Electricity �

�  Base year (1990-2000):                         1991             �

�  Number of years (1-50):                        30               �

�  Discount rate (0-30):                          4.7              �

�  General inflation rate (percent/year):         0                �

�  Display corresponding DOE rates (Y/N):         N                �

������������������������������������������������������������������ͼ

Present Value =   $1,629.37

Uniform Present Value Factor =  16.294

Figure 7.  Present value of annually recurring energy expenditures

using DOE's projected energy price escalation rates.

The DOE rates used in this last analysis can be displayed on the screen by

moving the cursor to the "Display corresponding DOE rates" field and pressing

"Y".  Figure 8 shows the rates corresponding to the example in figure 7.  If

the analysis is performed with a non-zero general inflation rate, the rates

displayed will include this rate.  Note that, as with the discount rate, the

compounding effect of real price increases with the rate of general inflation

must be considered, so that

E = e + I + eI

___________

* The additional cross product (dI) results from the compounding effect

of general inflation and the real discount rate, i.e., D = (1 + d)(1 + I) - 1.

7

where E = nominal escalation rate,

e = real escalation rate, and

I = general inflation rate.

�������������������������������������������������������������������������Ŀ

� DOE Region = 1;  Energy Type = Electricity;   Rate Type = Residential   �

�               Rates do not include general inflation.                   �

�                                                                         �

�            1991  -1.39%     2001   0.54%     2011   0.04%               �

�            1992   0.10%     2002   0.83%     2012   0.04%               �

�            1993   0.71%     2003   0.60%     2013   0.05%               �

�            1994  -0.72%     2004   0.52%     2014   0.05%               �

�            1995   0.17%     2005  -0.23%     2015   0.03%               �

�            1996   0.60%     2006   0.17%     2016   0.04%               �

�            1997   0.94%     2007   0.59%     2017   0.05%               �

�            1998   0.48%     2008   0.02%     2018   0.04%               �

�            1999   0.40%     2009   0.33%     2019   0.04%               �

�            2000   0.83%     2010   0.05%     2020   0.04%               �

�                                                                         �

���������������������������������������������������������������������������

Figure 8.  DOE energy price escalation rates used in example.

The DOE projections of energy price increases do not extend beyond the year

2029.  For any year beyond 2029, the rate of change used in the analysis is

set to the rate of general inflation, or zero if no inflation is specified.

4.  Computational Methods

The equations used to calculate the various discounting factors and the

corresponding dollar amounts in DISCOUNT are documented here.  Equations 1 and

2 can be used with an integer or non-integer number of time periods.  For

equations 3 through 9, which include periodic payment amounts, the use of a

non-integer number of time periods requires special treatment.  In these

latter equations, the final payment is assumed to be a fraction of the

periodic payment, corresponding to the length of the fractional period.  The

use of standard discounting equations having periodic payment amounts and a

non-integer number of payment periods will lead to erroneous results.

The following notation is used in the following equations:

P = present (initial) amount

F = future (final) amount

A = annual (periodic) amount

e = escalation rate (per period)

d = discount rate (per period)

n = total number of periods

t = period index (1-n)

8

Note that d and e can be in either real or nominal terms, but when used

together they must be consistent.  Do not mix real and nominal rates in the

same analysis.

Discounting applications:

(1) Present value when future value is known:

P = F � Single Present-Value factor (SPV)

n

SPV = 1 / (1 + d)                                                (1)

(2) Future value when present value is known:

F = P � Single Compound Amount factor (SCA)

n

SCA = (1 + d)                                                    (2)

(3) Present value when uniform periodic payment amount is known:

F = A � Uniform Present Value factor (UPV)

UPV = n                  for d = 0, and                          (3a)

n

(1 + d)  - 1

UPV = ������������       for d not = 0 and integer values of n.  (3b)

n

d(1 + d)

For d not = 0 and non-integer values of n, use equation 3b to compute UPV for

the integer value of n; then adjust UPV as follows:

fraction

UPV = UPV + ��������                                             (3c)

n

(1 + d)

where fraction = the difference between n and the integer value of n.

9

(4) Future value when uniform periodic payment amount is known:

F = A � Uniform Compound Amount factor (UCA)

UCA = n                 for d = 0, and                           (4a)

n

(1 + d)  - 1

UCA =  ������������     for d not = 0 and integer values of n.   (4b)

d

For d not = 0 and non-integer values of n, use equation 4b to compute UCA for the

integer value of n; then adjust the UCA factor as follows:

fraction

UCA = UCA � (1 + d)         + fraction                           (4c)

where fraction = the difference between n and the integer value of n.

(5) Uniform periodic payment amount when initial (present) amount is known:

A =  P � Uniform Capital Recovery factor (UCR)

UCR = 1 / n                 for d = 0, and                         (5a)

n

d(1 + d)

UCR = ������������          for d not = 0 and integer values of n. (5b)

n

(1 + d)  - 1

For d not = 0 and non-integer values of n, first compute UPV using equations 3b

and 3c.  Then

UCR = 1 / UPV                                                      (5c)

(6) Uniform periodic payment amount when final (future) amount is known:

A = F � Uniform Sinking Fund factor (USF)

USF = 1 / n               for d = 0, and                          (6a)

d

USF = ������������       for d not = 0 and integer values of n.   (6b)

n

(1 + d)  - 1

10

For d not = 0 and non-integer values of n, first compute UCA using equations 4b

and 4c.  Then

USF = 1 / UCA.                                                    (6c)

(7) Present value when periodic payment amount increases from period to period

at a constant rate:

For integer values of n:

P = A � Modified Uniform Present Value factor (UPV*)

0

where:

A = initial periodic amount (base year dollars)

0

UPV* = n                               when e = d, and             (7a)

n

(1 + e)          (1 + e)

UPV* =  �������  � [1 -  ������� ]     when e not = d.             (7b)

n

(d - e)          (1 + d)

For d not = 0 and non-integer values of n, use equation 7b to compute UPV* for the

integer value of n; then adjust the UPV* factor as follows:

n

(1 + e)

UPV* = UPV* +  fraction � �������� .                               (7c)

n

(1 + d)

(8 & 9) Present value when periodic payment amount increases from period to

period at a changing rate.  The same computational methods are used when the

escalation rates are entered by the user and when the escalation rates are

retrieved from the DOE data files on the disk.  For the former, the periodic

rate may be the same over several time periods.  For the latter, the periodic

(annual) rate changes every year.

For integer values of n:

P = A  � Modified Uniform Present Value factor (UPV*)

0

where:

A = periodic payment amount as of beginning of first time period,

0

11

Index

n         t

UPV* =  sum  ��������,                                            (8a)

t = 1        t

(1 + d)

t

Index  =  product (1 + e ), and                                    (8b)

t     j = 1        j

e  = the escalation rate in period j.

j

For non-integer values of n, use equations 8a and 8b to compute UPW* for the

integer value of n; then adjust the UPV* factor as follows:

fraction

Index  � (1+e   )

k       k+1

UPV* = UPV* + fraction � �����������������������,                 (8c)

n

(1 + d)

where  k = integer value of n,

e   = the escalation rate in the final (fractional) period,

k+1

fraction = the difference between n and k,

k

Index  = product (1 + e ), and

k    j = 1        j

e  = the escalation rate in period j.

j

12