>> FREEWARE <<

GIF RESolution utility (C)1993 R.Janorkar

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

v1.00 release date 03/04/93

What is GIFRES?

~~~~~~~~~~~~~~~

GIFRES is a simple utility that checks the header of GIF image

files and returns a DOS ERRORLEVEL indicating:

ERRORLEVEL 0 = GIF image is 640x480 or better

ERRORLEVEL 1 = GIF image is under 640x480

ERRORLEVEL 2 = Not a GIF file!

ERRORLEVEL 255 = File not found, or read error.

This little utility can be run in upload scan batch files on

Bulletin Board Systems, to reject substandard GIF uploads.

GIFRES is Freeware, no payment to the author is required for

use. The assembler source code has also been included for those

who wish to make modifications to the code. The author retains

all rights to the source code, though you are free to modify the

source code for your own use.

This program and source code is provided with NO warrantees,

expressed or implied. Use it at your own risk. The author of

this program will NOT be liable for any damages, direct or

indirect, arising from the ability or disability to use this

program. The use of this program on your computer system

indicates your acceptance of these disclaimers.

Technical junk

~~~~~~~~~~~~~~

The GIF file structure is as follows:

7 6 5 4   3 2 1 0 (individual bits)

- - - -   - - - -

Byte 0 | 0 1 0 0   0 1 1 1 = 47h = "G"

Byte 1 | 0 1 0 0   1 0 0 1 = 49h = "I"

Byte 2 | 0 1 0 0   0 1 1 0 = 46h = "F"

Byte 3 |                     <ignored>

Byte 4 |                     <ignored>

Byte 5 |                     <ignored>

Byte 6 | L L L L   H H H H = Width LSB MSB

Byte 7 | L L L L   H H H H = Height LSB MSB

GIFRES first check the first three bytes of the file for the

signature "GIF", if this is not found it returns ERRORLEVEL 2

(not a GIF file). It then checks the image width, comparing it

against 640. If the image is less than 640 pixels, it returns

ERRORLEVEL 1 (less than 640x480). If the width is acceptable, it

checks the image height against 480 pixels. If the height is

less it returns ERRORLEVEL 1 (less than 640x480). Otherwise it

returns ERRORLEVEL 0 indicating the image is 640x480 or higher.

GIFRES was compiled with MicroSoft Macro Assembler (MASM) v5.1

and linked with MicroSoft Linker v5.13.

Comments, criticism and feedback welcomed. You can contact the

author via snail mail at: Ron Janorkar

651 Richmond Road,

Staten Island, NY 10304-2409

or at the support BBS (718) 876-0337 @9600 v.32