The SuperVGA Test Library

-------------------------

The SuperVGA Test Library is a set of routines for automatically detecting

a number of different SuperVGA chipsets, and performing bank switching

and extended page flipping on supported SuperVGA's. It will also detect

the presence of HiColor and TrueColor video DAC's and provides support

for video modes that use these DAC's.

This library supports many of the extended features of today's SuperVGA

adapters, such as being able to separately set the read and write banks

and support for extended page flipping. Not all of these features are

available on each SuperVGA, but providing support for them if they are

available is made extremely easy.

NOTE: All source code is formatted with 4 space tabs, including the

assembly language source.

Bug fixes, changes etc

----------------------

If you have a SuperVGA card that it not supported by this library, I

would appreciate it if you mail me some code to to add support for your

video card. If your video card supports extended page flipping or

separate read/write banks, but support for this is not included in the

library, I would apprecite it if you mail me some code to add support for

this to the library also.

Also, if the library misbehaves on your system, I would appreciate

feedback on the problem so we can attempt to rectify it (see the list

of cards that have been successfully tested, before mailing me a yes

answer :-).

I would appreciate it if people could fill in the following form and mail it

back to me (see the read.me for for more details).

----------------------------------------------------------------------------

Name:  ____________________________________________________

Email: ____________________________________________________

Library Version: ___________

SuperVGA board: ___________________________________________

Chipset Type:   ___________________________________________

Demo Results:

16 color demo:  Ran Correctly [ ], Incorrectly [ ], Unsupported [ ]

If incorrectly, which parts did not work correctly:

Bank Switching [ ], Separate read/write banks [ ], Page Flipping [ ]

Explain: __________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

256 color demo: Ran Correctly [ ], Incorrectly [ ], Unsupported [ ]

If incorrectly, which parts did not work correctly:

Bank Switching [ ], Separate read/write banks [ ], Page Flipping [ ]

Explain: __________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

32k color demo: Ran Correctly [ ], Incorrectly [ ], Unsupported [ ]

If incorrectly, which parts did not work correctly:

Bank Switching [ ], Separate read/write banks [ ], Page Flipping [ ]

Explain: __________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

16m color demo: Ran Correctly [ ], Incorrectly [ ], Unsupported [ ]

If incorrectly, which parts did not work correctly:

Bank Switching [ ], Separate read/write banks [ ], Page Flipping [ ]

Explain: __________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

__________________________________________________

----------------------------------------------------------------------------

Test programs

-------------

The library has four test programs, for 16 color, 256 color, 32k color

and 16 million color SuperVGA video modes. The tests are now more

informative, providing you with a menu of video modes to test, and

providing feedback on what is supposed to happen. Simply run the programs

and follow the instructions.

Note that some video adapters support the 64k (16 bit) video modes.

These are essentially the same as the 32k color video modes, but have a

diffent layout for the bits in each pixel (usually 1 extra bit of green),

and thus are a great cause of pain as bitmaps are not compatible between

the two modes. This kit does not support the 64k color video modes (most

video cards that support 64k color also support 32k colors).

If any of the programs do not work correctly, you will probably see garbage

on the screen or stange results (at least not what the program says should

happen).

SuperVGA Support:

-----------------

The following SuperVGA chipsets are supported:

- VESA Compatible SuperVGA's

- ATI Technologies 18800, 28800

- Ahead A & B

- Chips & Technologies 82c451/452/453

- Everex

- Genoa Systems GVGA

- OAK Technologies OTI-037C, OTI-067, OTI-077

- Paradise PVGA1A, WD90C00/10/11/30/31

- NCR 77C20/21/22E

- Trident 88/8900

- Video7 V7VGA versions 1-5

- Tseng Labs ET3000, ET4000

- AcuMos AVGA2, AVGA3

- S3 86c911/924/801/806/928

- Advance Logic AL2101 SuperVGA

- MXIC 86010 SuperVGA

- Primus 2000 SuperVGA

- RealTek 3106 SuperVGA

Extended page flipping is currently available on the following

adapters:

- VESA Compatible SuperVGA's

- ATI Technologies 18800, 28800

- Ahead B

- Chips & Technologies 82c452/453

- Paradise PVGA1A, WD90C00, WD90C10, WD90C11

- NCR 77C20/21/22E SuperVGA

- Trident 88/8900

- Video7 V7VGA versions 1-5

- Tseng Labs ET3000, ET4000

- S3 86c911/924/801/805/928

- Primus 2000 SuperVGA

The following adapters support separate read and write banks for faster

performance in certain areas:

- VESA Compatible SuperVGA's

- ATI Technologies 18800-1, 28800

- Ahead B

- Genoa Systems GVGA

- Paradise WD90C10, WD90C11

- Video7 V7VGA version 5 or later

- Tseng Labs ET3000, ET4000

- MXIC 86010 SuperVGA

- Primus 2000 SuperVGA

Areas needing clearing up

-------------------------

- The RealTek RT3106 SuperVGA has bank switching registers in 3D6h and

3D7h. The register in 3D7h works by default for reading and writing,

but there must be some way of putting this card in a mode that can use

separate read/write banks. Also, I would like to figure out how to do

extended page flipping on this card.

- The biggest problem seems to be with the incompatible mode numbering

schemes for video cards based on the ET4000 chipset with True Color

video DAC's. The HiColor modes seem to be all compatible, but the True

Color 640x480 24 bit mode is an add on and different cards have

different numbers and methods to invoke this mode.

Anyone have a solution to this problem?

- The new Genoa 7900 seems to be a problem, since it uses a different

mode number than other ET4000 based cards for 640x480 24 bit graphics

mode. If I can figure out a way of detecting that the board is a Genoa

7900 then we can solve the problem. Anyone know how to do this?

Problems

--------

All S3 cards are VESA compatible, but some like the STB WIND/X have stuffed

BIOSes. This driver has fixed to work with these cards in 256 color modes,

but not in 1024x768x16 VESA mode 104h, since the incorrect scanline value

is also returned.

However, the S3 also supports 800x600x16, 1024x768x16 and 1280x1024x16

accelerated video modes (20xh mode numbers). These video modes are in a

packed 16 color mode, where two pixels are packed into a single byte. These

modes are not supported directly by this test kit, but could be supported by

a special S3 video driver (using the accelerator). Thus the only S3 video

modes supported directly by these test programs are the VESA 10xh video

modes (which are not accelerated).

Also I have noticed that setting up the 20xh modes on S3's stuffs up the

video palette, but using the VESA 10xh numbers doesn't.

Another problem that I found was that the 86c911 chip seems to have trouble

doing page flipping in the 1024x768x16 video mode (VESA mode number 102h, not

S3 mode number 202h). This problem is non-existant on the 86c924.

SuperVGA chipsets successfully tested

-------------------------------------

- VESA

- ET4000 with/without HiColor, 1024k

- Diamond Speedstar 24 (ET4000 with SS24 TrueColor DAC)

- S3 86c911, 1024k

- S3 86c924 with HiColor, 1024k

- Trident 8900C, 512k

- Oak OTI-037C, 512k

- Oak OTI-067C, 256k

- RealTek RT3106, 1024k

- Primus P2000

- Paradise WD90c30

The following is a list of SuperVGA chipset manufacturers and BIOS mode

numbers for the standard 16 color and 256 color SuperVGA video modes.

=============================================================================

VESA

----

Width       Height      Colors

-----       ------      ------

0x102         800          600          16

0x104        1024          768          16

0x106        1280         1024          16

0x100         640          400         256

0x101         640          480         256

0x103         800          600         256

0x105        1024          768         256

0x107        1280         1024         256

0x10D         320          200      32,768

0x110         640          480      32,768

0x113         800          600      32,768

0x116        1024          768      32,768

0x119        1280         1024      32,768

0x10E         320          200      65,536

0x111         640          480      65,536

0x114         800          600      65,536

0x117        1024          768      65,536

0x11A        1280         1024      65,536

0x10F         320          200  16,777,216

0x112         640          480  16,777,216

0x115         800          600  16,777,216

0x118        1024          768  16,777,216

0x11B        1280         1024  16,777,216

AX=4F02 BX=MODE

=============================================================================

ACUMOS

------

Width       Height      Colors      Chip

-----       ------      ------      ----

0x58          800          600          16      AVGA1

0x5D         1024          768          16      AVGA1

0x5E          640          400         256      AVGA1

0x5F          640          480         256      AVGA1

0x5C          800          600         256      AVGA1

AX=MODE

=============================================================================

AHEAD

-----

Width       Height      Colors      Chip

-----       ------      ------      ----

0x71 or 0x6A  800          600          16      A, B

0x74         1024          768          16      A, B

0x60          640          400         256      A, B

0x61          640          480         256      A, B

0x62          800          600         256      A, B

0x63         1024          768         256      B

AX=MODE

=============================================================================

AL2101

------

Width       Height      Colors      Chip

-----       ------      ------      ----

0x2B          800          600          16      AL2101

0x30         1024          768          16      AL2101

0x36         1280         1024          16      AL2101

0x29          640          400         256      AL2101

0x2A          640          480         256      AL2101

0x2C          800          600         256      AL2101

0x31         1024          768         256      AL2101

0x37         1280         1024         256      AL2101

0x40          320          200      65,536      AL2101

0x42          640          400      65,536      AL2101

0x43          640          480      65,536      AL2101

0x44          800          600      65,536      AL2101

AX=MODE

=============================================================================

ATI

---

Width       Height      Colors      Chip

-----       ------      ------      ----

0x54          800          600          16      18800 up

0x55         1024          768          16      18800-1 up

0x61          640          400         256      18800 up

0x62          640          480         256      18800 up

0x63          800          600         256      18800 up

0x64         1024          768         256      28800 up

0x72          640          480      32,768      28800-4 (15 bit DAC)

0x73          800          600      32,768      28800-4 (15 bit DAC)

AX=MODE

=============================================================================

CHIPS & TECH

------------

Width       Height      Colors      Chip

-----       ------      ------      ----

0x70 or 0x6A  800          600          16      451 up

0x72         1024          768          16      452 up

0x78          640          400         256      451 up

0x79          640          480         256      452 up

0x7B          800          600         256      453

0x7E         1024          768         256      453

AX=MODE

=============================================================================

EVEREX

------

Width       Height      Colors      Chip

-----       ------      ------      ----

0x02          800          600          16      all

0x20         1024          768          16      Ev629, Ev678

0x21         1280         1024          16      Ev629, Ev678

0x13          640          350         256      all

0x14          640          400         256      all

0x30          640          480         256      Ev629, Ev678

0x31          800          600         256      Ev629, Ev678

0x32         1024          768         256      Ev629, Ev678

0x62h         320          200      32,768      Viewpoint TC

0x71h         640          480      32,768      Viewpoint TC

0x77h         640          480  16,777,216      Viewpoint TC

AX=0x70 BL=MODE

=============================================================================

GENOA

-----

Width       Height      Colors      Chip

-----       ------      ------      ----

0x79 or 0x6A  800          600          16      GVGA

0x5F         1024          768          16      GVGA

0x5B          640          350         256      GVGA

0x7E          640          400         256      GVGA

0x5C          640          480         256      GVGA

0x5E or 0x6C  800          600         256      GVGA

AX=MODE

Other Genoa boards are based on the Tseng ET3000 and ET4000 chipsets and

use standard Tseng video mode numbers.

=============================================================================

MXIC

----

Width       Height      Colors      Chip

-----       ------      ------      ----

0x55          800          600          16      MXIC

0x56         1024          768          16      MXIC

0x57          640          350         256      MXIC

0x58          640          400         256      MXIC

0x59          640          480         256      MXIC

0x5A          800          600         256      MXIC

0x5B         1024          768         256      MXIC

AX=MODE

=============================================================================

NCR

---

Width       Height      Colors      Chip

-----       ------      ------      ----

0x58          800          600          16      77C20,21,22E

0x5D         1024          768          16      77C20,21,22E

0x67         1280         1024          16      77C22E

0x5E          640          400         256      77C20,21,22E

0x5F          640          480         256      77C20,21,22E

0x5C          800          600         256      77C20,21,22E

0x62         1024          768         256      77C22E

0x6A         1280         1024         256      77C22E

0x70          640          480      32,768      77C22E (15 bit DAC)

0x71          800          600      32,768      77C22E (15 bit DAC)

0x78          640          480      65,536      77C22E (15/16 bit DAC)

0x79          800          600      65,546      77C22E (15/16 bit DAC)

AX=MODE

=============================================================================

OAK TECH

--------

Width       Height      Colors      Chip

-----       ------      ------      ----

0x52          800          600          16      OTI-037C up

0x56         1024          768          16      OTI-067 up

0x58         1280         1024          16      OTI-067 up

0x53          640          480         256      OTI-067 up

0x54          800          600         256      OTI-067 up

0x59         1024          768         256      OTI-077

0x5B          640          400      32,768      OTI-077 (15 bit DAC)

0x5C          640          480      32,768      OTI-077 (15 bit DAC)

AX=MODE

=============================================================================

PARADISE

--------

Width       Height      Colors      Chip

-----       ------      ------      ----

0x58          800          600          16      pVGA1A,WDC90Cxx

0x5D         1024          768          16      WDC90Cxx

0x64         1280         1024          16      WDC90C31

0x5E          640          400         256      pVGA1A,WDC90Cxx

0x5F          640          480         256      pVGA1A,WDC90Cxx

0x5C          800          600         256      WDC90C1x,WDC90C3x

0x60         1024          768         256      WDC90C3x

0x62          640          480      32,768      WDC90C3x (15 bit DAC)

0x63          800          600      32,768      WDC90C3x (15 bit DAC)

AX=MODE

=============================================================================

PRIMUS

------

Width       Height      Colors      Chip

-----       ------      ------      ----

0x2A          800          600          16      P2000

0x30         1024          768          16      P2000

0x36         1280         1024          16      P2000

0x2C          640          400         256      P2000

0x2D          640          480         256      P2000

0x2B          800          600         256      P2000

0x31         1024          768         256      P2000

0x37         1280         1024         256      P2000

0x42          640          350      32,768      P2000 (15 bit DAC)

0x43          640          400      32,768      P2000 (15 bit DAC)

0x44          640          480      32,768      P2000 (15 bit DAC)

0x45          800          600      32,768      P2000 (15 bit DAC)

AX=MODE

=============================================================================

REALTEK

-------

Width       Height      Colors      Chip

-----       ------      ------      ----

0x1F          800          600          16      RT 3106

0x21         1024          768          16      RT 3106

0x2A         1280         1024          16      RT 3106

0x25          640          400         256      RT 3106

0x26          640          480         256      RT 3106

0x27          800          600         256      RT 3106

0x28         1024          768         256      RT 3106

0x37         1280         1024         256      RT 3106

AX=MODE

=============================================================================

S3

--

Width       Height      Colors      Chip

-----       ------      ------      ----

0x102         800          600          16      86c911 up

0x104        1024          768          16      86c911 up

0x108        1280         1024          16      86c911 up

0x101         640          480         256      86c911 up

0x103         800          600         256      86c911 up

0x105        1024          768         256      86c911 up

Acclerated modes:

-----------------

0x202         800          600          16      86c911 up

0x204        1024          768          16      86c911 up

0x208        1280         1024          16      86c911 up

0x201         640          480         256      86c911 up

0x203         800          600         256      86c911 up

0x205        1024          768         256      86c911 up

0x301         640          480      32,768      86c911 up

AX=0x4F02 BX=MODE

=============================================================================

TRIDENT

-------

Width       Height      Colors      Chip

-----       ------      ------      ----

0x5B          800          600          16      8800,8900

0x5F         1024          768          16      8800,8900

0x5C          640          400         256      8800,8900

0x5D          640          480         256      8800,8900

0x5E          800          600         256      8900

0x62         1024          768         256      8900

AX=MODE

=============================================================================

TSENG

-----

Width       Height      Colors      Chip

-----       ------      ------      ----

0x29          800          600          16      ET3000,ET4000

0x37         1024          768          16      ET3000,ET4000

0x3D         1280         1024          16      ET4000

0x2D          640          350         256      ET3000,ET4000

0x2F          640          400         256      ET4000

0x2E          640          480         256      ET3000,ET4000

0x30          800          600         256      ET3000,ET4000

0x38         1024          768         256      ET4000

AX=MODE

ET4000 with 15/16 bit DAC's

---------------------------

Width       Height      Colors      Chip

-----       ------      ------      ----

0x2D          640          350      32/64k      ET4000 (15/16 bit DAC)

0x2F          640          400      32/64k      ET4000 (15/16 bit DAC)

0x2E          640          480      32/64k      ET4000 (15/16 bit DAC)

0x30          800          600      32/64k      ET4000 (15/16 bit DAC)

AX=0x10F0 BL=MODE

AX=0x10F2 BL = 01h  Set 15 bit HiColor mode

02h  Set 16 bit HiColor mode

ET4000 with SS24 24 bit DAC (SpeedStar 24)

------------------------------------------

Width       Height      Colors      Chip

-----       ------      ------      ----

0x2E          640          480  16,777,216      ET4000 (24 bit DAC)

AX=0x10E0 BL=MODE

ET4000 with SS24 24 bit DAC (Genoa 7900)

----------------------------------------

Width       Height      Colors      Chip

-----       ------      ------      ----

0x3E          640          480  16,777,216      ET4000 (24 bit DAC)

AX=0x10F0 BL=MODE

=============================================================================

VIDEO7

------

Width       Height      Colors      Chip

-----       ------      ------      ----

0x62          800          600          16      V7VGA

0x65         1024          768          16      V7VGA

0x66          640          400         256      V7VGA

0x67          640          480         256      V7VGA

0x69          800          600         256      V7VGA

0x6A         1024          768         256      V7VGA

AX=0x6F05 BL=MODE

=============================================================================

References:

-----------

Richard Wilton: Programmer's Guide to PC & PS/2 Video Systems.

Microsoft Press 1987. ISBN 1-55615-103-9

Richard F. Ferraro: Programmer's guide to the EGA and VGA cards 2nd ed.

Addison-Wesley 1990. ISBN 0-201-57025-4

George Sutty and Steve Blair: Advanced Programmers Guide to the EGA/VGA.

Brady Books 1988. ISBN 0-13-729039-X

Jake Richter & Bud Smith: Graphics Programming for the 8514/A.

M&T Books 1990. ISBN 1-55851-074-5

Jake Richter: Power Programming... The IBM XGA.

MIS Press 1992. ISBN 1-55828-127-4

S3 Software Development Kit, version 2.0

John Bridge's VGAKIT52.

(Simtel: vga/vgakit52.zip)

Finn Thoegersen's VGADOC2.ZIP

(Simtel: vga/vgadoc2.zip)

+------------------------------------------+-------------------------------+

| Kendall Bennett                          | Internet:                     |

| RMIT Advanced Computer Graphics Centre   | kjb@citri.edu.au              |

| CITRI Building, 723 Swanston Street      | rcskb@minyos.xx.rmit.oz.au    |

| Carlton Victoria 3053 AUSTRALIA.         |                               |

+------------------------------------------+-------------------------------+