XGA Programmer's toolkit - version 1.1

(Version 1.1 includes several 800x600 modes that were not in

the original release.  The 800x600 modes require a multisynch

adapter.)

This package contains a set of simple routines providing direct-video

access to IBM's new XGA adapter.  These routines have been tested on

both single-monitor and dual-monitor (XGA & VGA) setups.  If you have

multiple XGA adapter/monitors (and you can have up to eight of them!),

you'll have to modify these routines yourself - I ran out of money after

buying one XGA adapter <grin>.

The routines and detailed explanations of their operation are in

XGAKIT.ASM, an assembly module written to interface with a driving

"C" program using the medium model.  There is also a simple XGADEMO.C

program provided as proof that these routines work (XGADEMO was also an

invaluable tool for me in *getting* them to work, which is why the

commented-out function calls in it exist).

Basically, the routines provide functions for throwing an XGA adapter

into any of the modes listed below.  For the graphics modes, there are

also routines for setting the XGA palette (which is *different* from the

VGA palette), writing/reading individual pixels, and writing/reading

entire (or portions of) individual scan lines.  The routines use the

64K aperture of the XGA as opposed to its 1MB or 4MB aperture, so they're

not as fast as they could be - but they work, and they're relatively

simple.

The XGA video modes supported by these routines are:

normal VGA text mode

132-col VGA text mode

1024x768x256 graphics mode

1024x768x16 graphics mode

640x480x256 graphics mode

640x480x65536 graphics mode

800x600x16 graphics mode       |  The 800x600 modes require

800x600x256 graphics mode      |  a multisynch monitor (they've

*    800x600x65536 graphics mode    |  been tested on a NEC 2A)

* - but the 800x600x65536 mode doesn't work on the 2A.  The pixels are

doubled up hirozontally so that only the left-hand side of the

screen is visible.  No, I don't know why.  Help!

---------------------

The limitations (of both the XGA adapter and this code):

- Although the XGA adapter works just fine as a VGA adapter when it is in

a VGA mode (which it can do on single-mode setups), it has a few uglies

when it is in one of its extended modes.

For one, you can't get an XGA adapter back into a VGA mode just by

calling a BIOS INT 10H routine, because the (current) PS/2 BIOS doesn't

know about the XGA's special registers and so can't reset them - you have

to invoke 'xga_mode(0)' first to get the adapter(s) back into a "normal"

VGA mode.  On dual-adapter setups, 'xga_mode(0)' leaves the XGA image

intact but switches the focus back to the VGA monitor.

For another, when it is in an extended mode, the XGA uses its own

palette registers, which are *different* from those of the VGA.  They are

accessed differently (use 'xga_setpalette()'), they are *not* initialized

to the VGA palette (you have to set them explicitly), and they use the

*top* six bits of a byte instead of the *bottom* six bits (IE, RGB value

10,20,30 in VGA mode is the same as color 40,80,120 in extended mode).

'xga_setpalette()' uses XGA-style values - 0 thru 255.

Also, the (current) Windows drivers that are distributed with the XGA

prevent the XGA routines from working properly, probably because the

XGA register locations are not virtualized.  The monitor ID is returned

as an invalid '0000', the memory-detect routines do not work, and

everything else is equally messed up.  For the moment, if 'xga_detect()'

receives that impossible '0000' monitor ID, it just returns a "no XGA

found" and inhibits the other routines from doing anything.