BGI-TEST and CHR-TEST were written by Sam Denton, St. Louis, MO,

CompuServ ID 76314,1512.

The programs and this documentation are Copyright 1988 by Sam Denton,

all rights reserved.  Private use is permitted, otherwise call me.

(My rates are reasonable, and/or you can hire me if you like my code.)

----------------------------------------------------------------------------

NOTE/WARNING:

If, like me, you make the .BGI and .CHR files ReadOnly, you should insert

"System.FileMode := 0;" before calling InitGraph. I spent an entire night

tracing the code before I realized what I had done to myself.

----------------------------------------------------------------------------

BGI-TEST.PAS

To use with DOS DEBUG, try "DEBUG C:\TP4\TURBO.EXE C:\MYPGMS\BGI-TEST.PAS".

At the DEBUG prompt, enter the "G" command.  BGI-TEST will stop of its own

volition at the start of the .BGI driver if you remove the "$" from the

"{$define trace}" at the start of the program.  Use the "RIP" command to

manually advance the program counter past the "INT 3" instruction.

All .BGI files start with a 2 byte signature = 'pk' (checked for by code in

the Graph unit), followed by two backspace characters.  For comparison, .CHR

files start with 'PK', and are otherwise similar.  BTW, I presume that 'pk'

is Philipe Kahn, and the backspaces are so you can TYPE the files and be

able to read the comments without seeing the 'pk'.  Then comes free-form

ASCII comments ending in Ctrl-Z, then:

dw  offset within file to the start of code

dw  driver id, a value from 0 to 5,

0=CGA, 1=EGA/VGA, 2=reserved(?), 3=Hercules, 4=AT&T, 5=PC3270

dw  size of just the code, NOT the entire file

db  a non-zero value

db  zero

db  either 0 or 1

db  dup(0) until start of code

GRAPH normally does a FAR CALL to the start-of-code address (normalized so

that the offset is 0) with a value in SI determining the type of call.

The value is always even, so the driver may use it to index a branch-table.

All numbers from here on are given in HEXADECIMAL, and all offsets are

taken from CGA.BGI and are relative to start-of-code.

org 0111

dw  ?   ; cp.x

dw  ?   : cp.y

BitBlt code is at 0EAC, it is invoked at 0F60, it is really weird stuff.

In the following table, for each legal value of SI is shown the offset into

the driver's code of the routine.  Also, if there seems to be a direct

correspondence between the routine and a GRAPH entry point, the name of the

entry point and how the arguments are passed is shown.  Some GRAPH calls

turn into a pair of calls, first a MoveTo to set the X and Y parameters,

then a call to pass the rest of the parameters.  In this case, the first

two parameters are shown as CP.X and CP.Y.  During InitGraph, the driver

gets a CALL FAR instruction stuffed into offset 0010.  The CALL FAR points

back into GRAPH and will execute many complicated "primitives" by doing

recursive calls back into the driver.  This means that if you write your

own graphic device driver, you don't have to write 'hard' routines.

si  addr  name (if any), args :- results

--  ----  ------------------------------

00  01D7  AL=1 :- CX=nbr of modes?

AL=2,CL=mode :- [ES:BX]=string(name of mode)

AL=?,CL=mode :- [ES:BX]=descriptor table, other stuff happens

02  0231  :- set graphics hardware to mode

04  0248  ClrScr?

06  0015 ?No Operation

08  0297  MoveTo(AX,BX)

0A  029F  LineTo(AX,BX)

0C  02B3  Line(AX,BX,CX,DX)

0E  0010* Do Polygon [ES:BX]=point list, CX=nbr pts, ?=draw or fill

10  0010* Bar3D(cp.x,cp.y,AX,BX,CX,DX)

12  07FE* Bar(AX,BX,CX,DX)

14  0010* Ellipse(cp.x,cp.y,AX,BX,CX,DX)

(this entry point is also used by Arc and Circle)

16  0010* PieSlice(cp.x,cp.y,AX,BX,CX)

18  0015 ?No Operation

1A  00BD  SetBkColor(BX)

1C  00DE ?SetColor(BX)

1E  0286  SetColor, AL=AH=color

20  00E3*?SetFillStyle/Pattern, AL=pattern nbr, or if FF then [ES:BX]=pattern

22  03D2  SetLineStyle(AX,BX,CX)

24  0398* SetTextStyle AL=direction, BX=8*scale, CX=8*scale

When AH has bit 80 on, DX is also significant

26  035C* OutText @ CP, [ES:BX]=string, CX=length

28  00FD* TextSize [ES:BX]=string, CX=length :- BX=width, CX=height in pixels

2A  0015 ?No Operation

2C  009F  FloodFill(AX,BX,CX)

2E  0FDC  GetPixel(AX,BX) :- DX

30  0FF5  PutPixel(AX,BX,DL)

32  006C  really strange, see below

34  0084  GetImage CX=x1,DX=y1,[ES:BX]=image(dx,dy,dup(?))

36  0099  PutImage AL=BitBlt,CX=x,DX=y,[ES:BX]=image(dx,dy,data...)

38  0050  SetViewport(AX,BX,CX,DX)

SI=32 returns [ES:BX] pointing to a table of offsets into the code that GRAPH

uses as additional entry points.  The code GRAPH uses is kludgy, but the net

effect is to simulate CALL [ES:[ES:BX+n]].  There are only 6 entries and

most of them point to RETF instructions, but [ES:BX+8] will return the number

of bits per pixel in the current graphics mode.  However, in EGAVGA.BGI only,

[ES:BX+0A] does some sort of segment arithmetic involving display memory.  In

any event, SI=32 is used by GetImageSize and SetActivePage

When InitGraph is called, GRAPH calls the device driver seventeen times with

SI= 00, 02, 24, 38, 08, 1E, 1E, 1E, 1E, 20, 24, 1E, 1E, 20, 22, 24, and 08,

and only Frank Borland knows why.  SetGraphMode issues the same sequence of

calls.  Also, when LineTo or LineRel is called, GRAPH calls SI=08 (MOVE! to a

new place), SI=0A (draw a line to the OLD! point), SI=08 (move back to new

point), even though a single call to SI=0A (draw a line to the new point)

would have the same net effect.  OutText and OutTextXY both call SI=08 TWICE

in succession with IDENTICAL arguments before writing out the string.  I

guess they want to be absolutely certain the text lands in the right place.

(As nearly as I can tell, both GRAPH and the .BGI drivers maintain their own

copies of the current position, but the copy in the driver is used to pass

positioning data when Bar3D, Arc, PieSlice, etc. are called.  Only the

current position kept by GRAPH matches the documentation.)

(P.S.  If, like me, you make the .BGI files ReadOnly, you should insert

"System.FileMode := 0;" before calling InitGraph. I spent an entire night

tracing the code before I realized what I had done to myself.)

----------------------------------------------------------------------------

CHR-TEST.PAS

All .CHR files start with a 2 byte signature = 'PK' (checked for by code in

the Graph unit), followed by two backspaces (which are not checked anywhere).

For comparison, .BGI files start with 'pk', and are otherwise similar.  Then

comes free-form ASCII comments ending in Ctrl-Z, then:

dw  offset within file to the start of descriptors

db  driver id, four characters

GOTH=gothic, LITT=small, SANS=sans serif, TRIP=triplex

dw  size of just the code, NOT the entire file

db  one

db  zero

db  one

db  dup(0) until start of descriptors

Within the descriptors, first is a ten byte area laid out as:

db  unknown, apparently alway $2B

dw  number of characters defined

db  unknown, apparently alway zero

db  the first character drawable with this character set

(usually blank ($20))

dw  offset to table of plot commands

db  unknown, apparently alway zero

db  Y value of top of character cell (shortint)

db  unknown, apparently alway zero

db  Y value of bottom of character cell (shortint, usually negative)

db  5 bytes, unknown, apparently alway zero

Next is an array of words pointing to the plot commands:

dw  0,4,...    ; offset to blank, exclaimation point, ...

Thirdly is an array of bytes containing the widths of each character:

db  6,3,...    ; width of blank, exclaimation point, ...

Finally come the plot commands. Each consists of 16 bits broken into several

fields.  The high order bit ($80) from both bytes are used to decide what to

do.  The cases are (expressed in binary):

1xxxxxxx 0yyyyyyy -- move to (x,y)

1xxxxxxx 1yyyyyyy -- line to (x,y)

0xxxxxxx 0yyyyyyy -- done, (x,y) is ignored

0xxxxxxx 1yyyyyyy -- no operation, (x,y) is ignored

The remaining seven bits of each byte are two's complement numbers.  Each

character is drawn within a box with the leftmost portion and the base of

the image lying on the y and x axii, respectively. Lower-case descenders

and some punctuation will drop below the x axis.

The first data area is of fixed size at the start of the 'segment', so

addressing it is no problem, and the second data is also easy to address,

although it is of variable size.  The third and fourth areas are more

problematical.  To calculate the offset of the third area, use the formula

2*(number of characters defined) + 16, and then index to the needed byte.

To find the plot commands in the fourth area for a given character, c, use

2*(ord(c) - (first char defined)) + 16 to get the offset of a word whose

value is then added to the offset of the table of plot commands.  See the

program for examples of Pascal code.  It's actually somewhat easier to do in

assembler (or C), however.