�����������������������������������������������������������Ŀ

� ***       Upper Bounds on Nonconformance Rates       ***  �

�             for Hypergeometric Distributions              �

�                                                           �

�                HYPRGEOM.EXE...Version 9103                �

�����������������������������������������������������������Ĵ

�             A Quality Assurance Training Tool:            �

�     Statistics Committee of the QA Section of the PMA     �

�                                                           �

�       PMA = Pharmaceutical Manufacturing Association      �

�                                                           �

�         Bob Obenchain, CompuServe User [72007,467]        �

�                                                           �

�������������������������������������������������������������

Given the size and the number of nonconformances found in a random

sample taken "without replacement" from a finite population of 1 million

or fewer units, HyprGeom.EXE calculates an upper confidence limit (with

specified probability content) for the percent of nonconforming units in

that population.  The sampling theory is based on the "hypergeometric"

distribution; the probability of getting exactly x nonconformances in a

sample of size n when the total population of size N contains exactly X

nonconforming units is then...

( X choose x ) times ( N-X choose n-x )

Pr( x, n, X, N ) = ---------------------------------------

( N choose n )

where ( A choose B ) denotes the binomial coefficient defined to be A!

(read "A factorial") divided by the product of B! times (A-B)!, A and B

are integers such that 0 <= B <= A, zero factorial equals 1, A! denotes

A * (A-1) * (A-2) * ... * 1 when A >= 1, and the range of x is all

integers values such that max[ 0, n+X-N ] <= x <=  min[ n, X ].

Given the values of N, n, and x (in the above notation), HyprGeom finds

the smallest value of X that is sufficiently large to make the sum of

hypergeometric probabilities from nonconformances = 0 to the observed x

value small...i.e. no more than a specified "alpha" level, like 0.05.

One then has confidence of AT LEAST 100*(1-alpha) PERCENT that the

unknown, true X value is no larger than the computed bound.

HYPRGEOM Prompts and Parameter Limits...            MIN           MAX

========================================

What is the total Population Size? [N] :              2     1,000,000

What is the Sample Size ? [n] :                       1           N-1

How many Nonconformances in Sample ? [x] :            0             n

Probability Content of Upper Bound ? [1-alpha] :   0.50          0.99

Example Batch Input File...

===========================

FIVE500.INP....Batch Input File for n=500, x=5, 95% Confidence,

and N= 750, 1000, 1250, 1500, 2000, 5000 & 10000.

NOTE: Batch input files must be formatted EXACTLY as shown below...

-------------------------------------------------------------------

filename    \__ the first two lines contain DOS filenames (at most

filename    /   characters) for the Primary and Detailed output files.

200         <-- first N       = population size

100         <-- first n       = sample size

1           <-- first x       = observed nonconformances

0.95        <-- first 1-alpha = probability content

<-- blank line

300         <-- second N       \   Repeat these four types of

100         <-- second n        \  rows for each subsequent set

1           <-- second x        /  of parameter values, and

0.95        <-- second 1-alpha /   separate sets with blank rows.

n           <-- LAST LINE: the letter N ==> no more data

Invoking HyprGeom...

====================

DOS Prompt> hyprgeom

Once HyprGeom has been invoked from the DOS prompt (as shown above), the

user is prompted to choose between K = keyboard and B = batch data file

entry.  Pick K you use your keyboard console to respond to HyprGeom's

prompts for input.  Each prompt displays a [default] value that you may

accept simply by pressing the ENTER or RETURN key.  Otherwise, type in

the value (or character string) you wish and then press ENTER.

The HYPRGEOM Primary Output File: filename.OUT

==============================================

The Primary Output file from HyprGeom saves only the "most important"

subset of the information that is displayed on your computer screen as

HyprGeom executes...

Parameter Settings specified by the user:

Population Size: N =  750

Sample Size: n =  500

Observed Nonconformances: x =    5

Probability Content = 0.95

Summary of HyprGeom calculations:

Observed Percent Nonconforming: 100 * 5 / 500 =   1.00%

At Confidence  95.0%, the Upper Limit on Percent Nonconforming

is... 100 * 13 / 750 =   1.73%

Secondary DETAILED Output from HYPRGEOM: filename.DTL

=====================================================

The secondary output file from HyprGeom saves computational details not

recorded in the primary output file.  For example, secondary output

includes...

Cumulative hypergeometric probability sums:

For Total Defectives =     13, Cum.  Prob. = 0.033.

NOTE: The cumulative sum above implies that the exact probability

content of 13 as an upper bound on X is 0.967; this implies slightly

higher confidence (96.7%) than what was actually sought (95%)... due to

the discreteness of parameters of hypergeometric distributions.

Execution   The calculations performed by HyprGeom.EXE are slightly

Time:       more time consuming than those of Binomial.EXE.  For

=========   example, concluding that 0.0204 is the 95.1% upper bound on

the nonconformance rate when x=5 defects are observed in a

sample of size n=500 from a population of size N=5000 takes

about 3 seconds on a 20 Mhz 80386 machine with a numeric

math coprocessor (80387) or about 6 seconds when there is no

"math chip."  On an IBM PC/XT class machine (4 Mhz 8088),

these calculations require about 7 seconds with an 8087 math

coprocessor or about 25 seconds without the "math chip."

Source Code...

==============

C-language source code for HyprGeom is provided in the self-extracting

archive file, AudiCODE.EXE, that also contains the source code for

Binomial.EXE.  The source code for the gammln() function is not in the

archive because it is a copyrighted work by Press et.al.(1988).  Source

code for these functions is also available on floppy disks [Copyright

(c) 1987, Numerical Recipes Software], also distributed by Cambridge

University Press.

Validation of Numerical Results...

==================================

Lieberman and Owen(1961) published individual and cumulative

hypergeometric probabilities (with 6 decimal places) computed from

logarithms of factorials (with 15 decimal places) ranging from

log(2!)= 0.30102 99956 63981 to log(2000!)= 5735.52065 05490 73045.

When N=2000 and n=1000, Lieberman and Owen(1961) reported cumulative

probabilities of observing at most x=481 nonconformances of 0.053691

when X=999 and 0.048982 when X=1000.  These numerical values are

identical to those given by the PROBHYPR() function of SAS.

HyprGeom.EXE was able to verify that X=1000 is indeed the 95.1%

minimum-confidence upper bound in this case; this search consumed

slightly more than 5 minutes on a 20 Mhz 80386 machine.  HyprGeom prints

cumulative probabilities to "only" 4 decimal places; for example, 0.0537

and 0.0490 for the X=999 and X=1000 cases considered here.

REFERENCES

Johnson, N.L. and Kotz, S. (1969).  Distributions in Statistics:

Discrete Distributions.  (Chapter 6: Hypergeometric

Distribution.)  New York: John Wiley and Sons.

Lieberman, G.J. and Owen, D.B. (1961).  Tables of the Hypergeometric

Probability Distribution.  Stanford: Stanford University Press.

Press, W.H., Flannery, B.P., Teukolsky, S.A. and Vetterling, W.T.

(1988).  Numerical Recipes in C: The Art of Scientific

Computing.  [Chapter 6: Special Functions.]  Cambridge:

Cambridge University Press.

SAS/PC (Release 6.04) Function: PROBHYPR( N, X, n, x ).  SAS Institute

Inc.; SAS Circle; Cary, North Carolina 27512-8000.