������������������������������������������������������Ŀ

�             Sampling Strategy Simulator              �

�             SamStrat.EXE...Version 9304              �

������������������������������������������������������Ĵ

�          A Quality Assurance Training Tool:          �

�  Statistics Committee of the QA Section of the PMA   �

������������������������������������������������������Ĵ

�      Bob Obenchain, CompuServe User [72007,467]      �

�     5261 Woodfield Drive North, Carmel, IN 46033     �

��������������������������������������������������������

SamStrat.EXE is a simulation module that treats the generalization of

the "(K1,K2) Game" of Deming(1983) and Burke, Davis and Kaminsky(1993)

in which the true nonconformance rate can vary between stated minimum

and maximum values.

Filenames:     Files related to the Sampling Strategy Simulator are:

----------

SamStrat.EXE ...Main executable module

SamStrat.DOC ...This ASCII documentation file

SamStrat.C   ...C-language source code for SamStrat

CONTENTS:

(i) Introduction to Deming's (K1,K2) Game

(ii) Invocation of SamStrat.EXE and Use of the Menus and Prompts.

(iii) Numerical Examples.

(iv) Literature References and Software Update History

INTRODUCTION

============

The primary parameters used by the Sampling Strategy Simulator are:

����������������������������������������Ŀ

�     K1 = Per Unit Cost of Inspection   �

�     K2 = Per Unit Cost of Rework       �

�   Pmin = Minimum Nonconformance Rate   �

�   Pmax = Maximum Nonconformance Rate   �

������������������������������������������

When Pmin = Pmax, the true nonconformance rate is STABLE, and the

process is said to be "in a state of statistical control."  Under

these circumstances, Mood(1943) showed that there is no correlation

between the nonconformance rates observed in random samples with the

nonconformance rate in the remainder of that same lot.  And

Deming(1983) argued that minimum expected cost is always achieved in

this case by either 100% inspection or Zero Inspection.

When Pmin = Pmax < K1/K2, Zero Inspection is optimal; when Pmin =

Pmax > K1/K2, 100% Inspection is optimal.  When Pmin = Pmax =

K1/K2, all three methods have the same expected cost.  In other

words, Acceptance Sampling methods are of no advantage in expected

cost when the nonconformance rate is stable.

Sampling Strategy Simulator [ver.9304]               Page 2 of 12

When Pmin < Pmax, SamStrat allows the true nonconformance rate to vary

following a SINUSOIDAL PATTERN for consecutive lots, were the number

of lots with nonconformance rates strictly > Pmin and < Pmax is given

by the value of the "cycle" parameter.  In cases like this [especially

when K1/K2 is approximately the average nonconformance rate,

(Pmin+Pmax)/2], Sampling Inspection can be more cost effective on

average than either 100% Inspection of Zero Inspection.

To achieve maximum speed, accuracy, and MS-DOS portability, SamStrat

was compiled using Microsoft C/C++ version 7.00.  For uniform

pseudo-random number generation, SamStrat includes implementations of

both the 32-bit and 16-bit L'Ecuyer(1988) algorithms.  A "bucket" of

100 32-bit numbers is randomly sampled using the 16-bit values.

The parameters, limits, and defaults used in SamStrat are:

Variable                            Minimum Default Maximum

Name      Brief Explanation          Value   Value   Value

-------   ------------------------  ------- ------- -------

Pmin      Min Fraction Nonconform     0.01    0.10    Pmax

Pmax      Max Fraction Nonconform     Pmin    0.40    0.50

cycle     Lots between Pmax & Pmin       0      24      49

K1        Unit Cost of Inspection     0.01       2   50.00

K2        Unit Cost of Rework         0.01       8  500.00

N=lotsiz  Number items in a Lot         10      20    5000

n=samsiz  Number of items in Sample      1       5     N-1

c=accept  Sampling acceptance number     0       1       n

idum      Random Number SEED Value       0       0*  32767**

replics   Maximum Total No. of Reps.    50     500     500

* Note: idum = 0 causes SamStrat.EXE to use your personal computer

clock to generate each start-up seed value; different runs (and

resets) of SamStrat.EXE usually start out (and progress) differently

in this case.  A positive value (1<=idum<=32767) will cause each

run (or reset) of SamStrat.EXE to start at the exact same point of a

pseudo-random sequence; "final" cumulative results will then

depend only upon how long the user waits before pressing the ESC

key to terminate SamStrat.EXE.

USAGE of SamStrat.EXE

=====================

To start up the Sampling Strategy Simulator, ENTER the command

SamStrat at your MS-DOS (>) prompt:

MyPrompt> samstrat

The following screen will then appear...

Sampling Strategy Simulator [ver.9304]               Page 3 of 12

������������������������� SamStrat Simulator �����������������������Ŀ

�                                                                    �

�                    SAMSTRAT.EXE...Version 9304                     �

�        ****   Determine Optimal Sampling Strategies   ****         �

�                                                                    �

�                 A Quality Assurance Training Tool:                 �

�         Statistics Committee of the QA Section of the PMA          �

�                                                                    �

�            Bob Obenchain, CompuServe User [72007,467]              �

�  ��������������������������������������������������������������ͻ  �

�  �                                                              �  �

�  �   Press a key to SET Simulation Scenario Parameters...       �  �

�  �                                                              �  �

�  ��������������������������������������������������������������ͼ  �

����������������������������������������������������������������������

������������������������� SamStrat Simulator �����������������������Ŀ

�   At colon Prompts : ...simply press ENTER to get the [default].   �

�                                                                    �

�   Specify filename to save Output [SAMSTRAT.OUT] :   <---take default

�   The SamStrat Output file is to be: SAMSTRAT.out                  �

�                                                                    �

�   What will be the MINIMUM Fraction Nonconforming for              �

�   any LOT ? (min=0.01;max=0.50) [0.10] :             <---take default

�   Minimum Fraction Nonconforming = 0.10.                           �

�                                                                    �

�   What will be the MAXIMUM Fraction Nonconforming for              �

�   any LOT ? (min=0.01;max=0.50) [0.40] :             <---take default

�   Maximum Fraction Nonconforming = 0.40.                           �

�                                                                    �

�   Fraction Nonconforming will cycle UP and DOWN between            �

�   Pmax = 0.40 and Pmin = 0.10 following a regular,                 �

�   sinusoidal pattern.  How many LOTS will be produced between      �

�   successive Pmin and Pmax rates ? (min=0;max=49) [24] : <---take  �

�   Lots per Sinusoidal Cycle = 25.                           default�

����������������������������������������������������������������������

Sampling Strategy Simulator [ver.9304]               Page 4 of 12

������������������������� SamStrat Simulator �����������������������Ŀ

�   What will be the cost, K1, of inspecting one                     �

�   unit ? (min=0.01;max=50) [ 2.00] :                 <---take default

�   Unit Cost of Inspection, K1 =  2.00.                             �

�                                                                    �

�   What will be the cost, K2, of reworking one                      �

�   nonconforming unit ? (min=0.01;max=500) [  8.00] : <---take default

�   Unit Cost of Rework, K2 =   8.00.                                �

�                                                                    �

�   What will be the lot size, N ?                                   �

�   (min=10;max=5000) [  20] :                         <---take default

�   Number of items in a lot, N =     20.                            �

�                                                                    �

�   What will be the sample size, n ?                                �

�   (min=1;max=N-1) [   5] :                           <---take default

�   Number of items in a sample, n =    5.                           �

����������������������������������������������������������������������

������������������������� SamStrat Simulator �����������������������Ŀ

�                                                                    �

�   What will be the accept number, c ?                              �

�   (min=0;max=n-1) [   1] :                           <---take default

�   Sampling acceptance number, c =    1.                            �

�                                                                    �

�   Input Simulation Start-up Seed ( 0 means use your Clock ): <---take

�   Equivalent Start-Up Seed = -28262.                        0 default

�                                                                    �

�           Press a Key to Continue...                               �

����������������������������������������������������������������������

Let us first view the SamStrat HELP screen...

������������������������� SamStrat Simulator �����������������������Ŀ

�                ������������������������������������ͻ              �

�                �             Main Menu              �              �

�                �   H = display HELP screen          �              �

�                �   D = Define Sampling Scenario     �              �

�                �   S = Simulate Strategy Costs      �              �

�                �   X = eXit SamStrat                �              �

�                �   choice --> H                     �              �

�                ������������������������������������ͼ              �

����������������������������������������������������������������������

Sampling Strategy Simulator [ver.9304]               Page 5 of 12

������������������������� SamStrat Simulator �����������������������Ŀ

��������������������������� SamStrat HELP �������������������������ͻ�

��                                                                  ��

��   Welcome to the SamStrat Simulation...                          ��

��                                                                  ��

��   The object of the simulation is to determine which of          ��

��   three strategies (NO inspection, 100% inspection,              ��

��   or SAMPLING inspection) yields minimum average cost            ��

��   for a given set of quality scenario parameters.                ��

��                                                                  ��

��   You must first specify maximum and minimum values for the      ��

��   for fraction nonconforming, P.  When Pmax = Pmin, fraction     ��

��   nonconforming is then STABLE.  In this situation, Deming has   ��

��   shown that either NO inspection or 100% inspection is best.    ��

��   When Pmax > Pmin, SamStrat will allow P to vary up and down    ��

��   following a sinusoidal pattern for consecutive lots.           ��

��                                                                  ��

��   Among the other parameter settings you must specify are: the   ��

��   cost, K1, of inspecting a single unit and the cost, K2, of     ��

��   reworking a nonconforming unit that had already been assembled ��

��   or shipped.                                                    ��

�������������������������������������������������������������������ͼ�

����������������������������������������������������������������������

Now, let us run the simulation for all of the above default values!!!

������������������������� SamStrat Simulator �����������������������Ŀ

�                ������������������������������������ͻ              �

�                �             Main Menu              �              �

�                �   H = display HELP screen          �              �

�                �   D = Define Sampling Scenario     �              �

�                �   S = Simulate Strategy Costs      �              �

�                �   X = eXit SamStrat                �              �

�                �   choice --> S                     �              �

�                ������������������������������������ͼ              �

����������������������������������������������������������������������

Sampling Strategy Simulator [ver.9304]               Page 6 of 12

������������������������� SamStrat Simulator �����������������������Ŀ

����������������������� Sampling Simulation ���������������������ͻ  �

��                                                                �  �

��                                                                �  �

��   Welcome to the SAMSTRAT Simulation...                        �  �

��                                                                �  �

��   Please respond to the following prompts for Information.     �  �

��                                                                �  �

��   How many Monte-Carlo Replications do you wish                �  �

��   to evaluate ? (nin=50,max=500) [500] :      <---take default �  �

��   Now performing 500 replications...              here too     �  �

��                                                                �  �

��   Press a Key to Stop Simulating.                              �  �

��                                                                �  �

��   Replication Number = 63 <---this counter shows how many      �  �

��                               replications have been performed �  �

�����������������������������������������������������������������ͼ  �

����������������������������������������������������������������������

When either the 500 replications are completed or else the user

presses a key, summary statistics will be listed for all three

primary strategies...

������������������������� SamStrat Simulator �����������������������Ŀ

����������������������� Sampling Simulation ���������������������ͻ  �

�� ������������������������ SamStrat Results ����������������������ͻ�

�� �                                                                ��

�� �                                                                ��

�� �   For the 100% Sorting Strategy...                             ��

�� �           Maximum Observed Cost =    94.00                     ��

�� �           Average Observed Cost =    54.30                     ��

�� �           Minimum Observed Cost =    40.00                     ��

�� �           Average Expected Cost =    54.43                     ��

�� �   For the Sampling Strategy...                                 ��

�� �           Maximum Observed Cost =   162.00                     ��

�� �           Average Observed Cost =    52.32                     ��

�� �           Minimum Observed Cost =    10.00      Sampling       ��

�� �           Average Expected Cost =    51.76  <---is best on     ��

�� �   For the Zero Inspection Strategy...           average here.  ��

�� �           Maximum Observed Cost =   216.00                     ��

�� �           Average Observed Cost =    57.22                     ��

�� �           Minimum Observed Cost =     0.00                     ��

�� �           Average Expected Cost =    57.73                     ��

�� �                                                                ��

�� �           Press a Key to Display Histograms...                 ��

��ͺ                                                                ��

�  ����������������������������������������������������������������ͼ�

����������������������������������������������������������������������

Next, SamStrat draws 13-cell histograms of the costs incurred under all

three primary strategies and also lists the cost standard deviations...

Sampling Strategy Simulator [ver.9304]               Page 7 of 12

������������������������� SamStrat Simulator �����������������������Ŀ

����������������������� Sampling Simulation ���������������������ͻ  �

�� ������������������������ SamStrat Results ����������������������ͻ�

�� �                                                                ��

�� �    LowLimit Freq                                               ��

�� �    -------- --- ---------------------------------------------  ��

�� �       94.00   1                                                ��

�� �       89.50   1                                                ��

�� �       85.00   0                                                ��

�� �       80.50   2 *                                              ��

�� �       76.00  13 ******                                         ��

�� �       71.50  12 ******                                         ��

�� �       67.00  34 *****************                              ��

�� �       62.50  42 *********************                          ��

�� �       58.00  72 *******************************                ��

�� �       53.50  66 ****************************                   ��

�� �       49.00  72 *******************************                ��

�� �       44.50  87 ***************************************        ��

�� �       40.00  98 *********************************************  ��

�� �    -------- --- ---------------------------------------------  ��

�� �    LowLimit Freq       Press a Key...                          ��

�� �    Observed Standard Deviation =      9.87                     ��

��ͺ    Costs of 100% Sort...                                       ��

�  ����������������������������������������������������������������ͼ�

����������������������������������������������������������������������

������������������������� SamStrat Simulator �����������������������Ŀ

����������������������� Sampling Simulation ���������������������ͻ  �

�� ������������������������ SamStrat Results ����������������������ͻ�

�� �                                                                ��

�� �    LowLimit Freq                                               ��

�� �    -------- --- ---------------------------------------------  ��

�� �      216.00   1                                                ��

�� �      198.00   1                                                ��

�� �      180.00   0                                                ��

�� �      162.00   2 *                                              ��

�� �      144.00  13 ******                                         ��

�� �      126.00  12 ******                                         ��

�� �      108.00  34 *****************                              ��

�� �       90.00  42 *********************                          ��

�� �       72.00  72 *******************************                ��

�� �       54.00  66 ****************************                   ��

�� �       36.00  72 *******************************                ��

�� �       18.00  87 ***************************************        ��

�� �        0.00  98 *********************************************  ��

�� �    -------- --- ---------------------------------------------  ��

�� �    LowLimit Freq       Press a Key...                          ��

�� �    Observed Standard Deviation =     39.50                     ��

��ͺ    Costs of Zero Inspection...                                 ��

�  ����������������������������������������������������������������ͼ�

����������������������������������������������������������������������

Sampling Strategy Simulator [ver.9304]               Page 8 of 12

������������������������� SamStrat Simulator �����������������������Ŀ

����������������������� Sampling Simulation ���������������������ͻ  �

�� ������������������������ SamStrat Results ����������������������ͻ�

�� �                                                                ��

�� �    LowLimit Freq                                               ��

�� �    -------- --- ---------------------------------------------  ��

�� �      162.00   1                                                ��

�� �      149.33   1                                                ��

�� �      136.67   2                                                ��

�� �      124.00   3 *                                              ��

�� �      111.33   6 **                                             ��

�� �       98.67  13 *****                                          ��

�� �       86.00  14 *****                                          ��

�� �       73.33  38 ***************                                ��

�� �       60.67  97 **********************************             ��

�� �       48.00 123 *********************************************  ��

�� �       35.33  61 ************************                       ��

�� �       22.67  72 **************************                     ��

�� �       10.00  69 **************************                     ��

�� �    -------- --- ---------------------------------------------  ��

�� �    LowLimit Freq       Press a Key...                          ��

�� �    Observed Standard Deviation =     25.53                     ��

��ͺ    Costs of Acceptance Sampling...                             ��

�  ����������������������������������������������������������������ͼ�

����������������������������������������������������������������������

Then SamStrat draws three high-resolution graphical displays.

The first two look like control charts for nonconformance rates, in the

sample or in the remainder of the lot.  But these charts also show (in

green) how, if any, the true nonconformance rate varies from lot to lot.

The lower limit in these charts is zero, while the upper limit is at

least Pmax.

The third plot shows the nonconformance rates for the sample and the

remainder of the lot plotted against each other.  Here, many points

usually will plot on top of each other, so successive values are

connected with lines to give some indication of their multiplicity.

Sampling Strategy Simulator [ver.9304]               Page 9 of 12

������������������������� SamStrat Simulator �����������������������Ŀ

����������������������� Sampling Simulation ���������������������ͻ  �

�� ������������������������ SamStrat Results ����������������������ͻ�

�� �                                                                ��

�� �   The VGA screens will show charts of nonconformance           ��

�� �   rates in the sample or in the remainder of the lot,          ��

�� �   along with the true nonconformance rate.  And the            ��

�� �   last plot shows the sample rate on the horizontal            ��

�� �   axis versus the remainder rate on the vertical axis.         ��

�� �                                                                ��

�� �   SCREEN DUMPS to slave parallel PRINTERS on LPT1:             ��

�� �     Press the L key or F10 for HP Laser/Desk Jet.              ��

�� �     Press the G key or F09 for IBM graphics dot matrix.        ��

�� �     Press the E key or F08 for Epson FX, JX or LQ.             ��

�� �                                                                ��

��ͺ   Press a Key to initiate Graphics...                          ��

�  ����������������������������������������������������������������ͼ�

����������������������������������������������������������������������

Finally, SamStrat calculates the sample correlation between the

nonconformance rates for the sample and the remainder of the lot.

This correlation is usually quite small when Pmin=Pmax, of course.

But here it was quite large!!!

������������������������� SamStrat Simulator �����������������������Ŀ

����������������������� Sampling Simulation ���������������������ͻ  �

�� ������������������������ SamStrat Results ����������������������ͻ�

�� �                                                                ��

�� �    Average Sample Nonconformance Rate = 0.2428                 ��

�� �    Average Remainder Nonconformance Rate = 0.2495              ��

�� �    Sample/Remainder Nonconformance Correlation = 0.3637        ��

�� �                                                                ��

��ͺ           Press a Key to Continue...                           ��

�  ����������������������������������������������������������������ͼ�

����������������������������������������������������������������������

The user of SamStrat may now made additional simulation runs for the

given set of ( K1, K2, Pmin, Pmax, etc. ) parameters using the "S"

option on the Main Menu.  Or he/she may change parameters with the "D"

option.  Here, we simply illustrate exiting from SamStrat by pressing

the "X" key...

Sampling Strategy Simulator [ver.9304]               Page 10 of 12

������������������������� SamStrat Simulator �����������������������Ŀ

�                ������������������������������������ͻ              �

�                �             Main Menu              �              �

�                �   H = display HELP screen          �              �

�                �   D = Define Sampling Scenario     �              �

�                �   S = Simulate Strategy Costs      �              �

�                �   X = eXit SamStrat                �              �

�                �   choice --> X                     �              �

�                ������������������������������������ͼ              �

����������������������������������������������������������������������

REMINDER(S) :

SAMSTRAT created an output file named: SAMSTRAT.out

Use the DOS invocation... TYPE SAMSTRAT.out | MORE

to review detailed information about this

run of the SAMSTRAT simulator.

NUMERICAL EXAMPLES

==================

Burke, Davis and Kaminsky(1993) illustrate two scenarios with Pmin =

Pmax = 0.25 in their Table 1 and Figures 1 through 4.  In both of

these cases, N = 20, n = 5, and c = 0.

**********************************************************************

When K1 = $1 and K2 = $10, inspection is inexpensive relative to

rework in the sense that K1/K2 = 0.10 is less than P = 0.25, and

100% inspection has lowest average cost.

100% Sorting Strategy...     Avg Observed Cost =    26.63

Expected Cost =    26.67  <<BEST

Sampling Strategy...         Avg Observed Cost =    33.86

Expected Cost =    32.01

Zero Inspection Strategy...  Avg Observed Cost =    66.28

Expected Cost =    66.67

Sample/Remainder Nonconformance Correlation = 0.0044

**********************************************************************

When K1 = $5 and K2 = $10, inspection is expensive relative to

rework in the sense that K1/K2 = 0.50 is greater than P = 0.25, and

zero inspection has lowest average cost.

100% Sorting Strategy...     Avg Observed Cost =   133.66

Expected Cost =   133.33

Sampling Strategy...         Avg Observed Cost =   122.41

Expected Cost =   124.43

Zero Inspection Strategy...  Avg Observed Cost =    67.32

Expected Cost =    66.67  <<BEST

Sampling Strategy Simulator [ver.9304]               Page 11 of 12

Sample/Remainder Nonconformance Correlation = 0.0155

**********************************************************************

With the SamStrat default values of Pmin = 0.10 and Pmax = 0.40, the

average value of P will again be Pavg = 0.25.  As a result,

interesting things tend to happen when K1/K2 is also approximately

0.25.

After all, zero inspection will dominate 100% inspection when P is

below its 0.25 average, and 100% inspection will dominate zero

inspection when P is above its 0.25 average!

In fact, sampling inspection does well on average in these cases

precisely because...

few nonconformances are found in the

a smallish sample (and inspection stops) when

P is relatively small and

zero inspection is favorable,

and yet

more nonconformances are found in the

a smallish sample (and inspection continues) when

P is relatively large and

100% inspection is favorable.

For example, when K1 = $2, K2 = $8 and c = 1, we have...

100% Sorting Strategy...    Avg Observed Cost =    53.91

Avg Expected Cost =    54.44

Sampling Strategy...        Avg Observed Cost =    51.50

Avg Expected Cost =    51.83  <<BEST

Zero Inspection Strategy... Avg Observed Cost =    55.65

Avg Expected Cost =    57.78

**********************************************************************

REFERENCES

==========

Burke, R. J., Davis, D. D., and Kaminsky, F. C. (1993).  "The ( K1,

K2 ) Game."  Quality Progress (February) 49-53.

Deming, W. E. (1983).  Out of Crisis.  (Chapter 15: Plan for Minimum

Average Total Cost of Incoming Materials and Final Product.)

Center for Advanced Engineering Study, MIT Press, 407-464.

L'Ecuyer, P. (1988).  "Efficient and Portable Combined Random Number

Generators."  Communications of the ACM 31, 742-749,774.

Sampling Strategy Simulator [ver.9304]               Page 12 of 12

Mood, A. M. (1943).  "On the Dependence of Sampling Inspection Plans

Upon Population Distributions."  Annals of Mathematical

Statistics 14, 415-425.

Papadakis, E. P. (1985).  "The Deming Inspection Criterion for

Choosing Zero or 100 Percent Inspection."  Journal of Quality

Technology 17, 121-127.

Press, W. H., Flannery, B. P., Teukolsky, S.A., and Vetterling, W. T.

(1988). "Numerical Recipes in C: The Art of Scientific

Computing."  [especially Chapter 7:  Random Number Generation.]

Cambridge University Press. { Source Code Disk:  Copyright 1987

by Numerical Recipes Software, P.O.Box 243, Cambridge, MA 02238.}

Schilling, E. G. (1982).  Acceptance Sampling in Quality Control.  New

York: Marcel Dekker.

Vardeman, S. B. (1986).  "The Legitimate Role of Inspection in Modern

SQC."  The American Statistician 40, 325-328.

SamStrat Update History:

===================

9304 ...Original version.

+---------------------------End of SamStrat.DOC-----------------------------+