========================================

TIGA-340 VIDEO RESOURCE FILE  (TIGA.VRS)

========================================

Before you can use WordPerfect 5.1 or DrawPerfect 1.0 with your TIGA-340

graphics card, you will need to copy the TIGA.VRS file into the same

directory where all of the other *.VRS video resource files are located.

In addition, you must create an Absolute Load Module for your particular

TIGA-340 graphics card by following these steps:

1)  Type CD <path> at the DOS prompt where <path> represents

the complete DOS directory path where the TIGA-340 driver

files that came with your graphics card are located.

2)  Type SET TIGA=-M<path> -L<path><Enter> at the DOS prompt

to make sure that the TIGA environment variable is properly

set.  (Please note that <path> represents the same directory

described in step 1 -- the TIGA driver files directory.)

3)  Type TIGACD<Enter> at the DOS prompt to make sure that

the TIGA Communications Driver is running.

4)  Type TIGALNK -LX<Enter> at the DOS prompt to Load and

eXecute the TIGA graphics manager on your TIGA-340 graphics

card.

5)  Type TIGALNK -CA EXTPRIMS<Enter> at the DOS prompt to

Create an Absolute load module called EXTPRIMS.ALM.

*** NOTE:  Steps 2 and 3 above can be placed in your AUTOEXEC.BAT file

so that they will automatically be executed each time your computer starts

up.  Steps 1, 4 and 5 needs to be executed only once.