VIDBLITZ reads and writes data to your video card, through the CPU window

located at address A0000. The data transfer is timed as "frames". A "frame"

is the time it takes for the raster beam of your monitor to scan from top

to bottom. In normal MCGA mode, the refresh rate of your monitor is set to

70 Hz. This means that 70 "frames" pass per second. This relationship,

together with the CPU clock speed, is used to calculate the various

statistics that appear on the screen when the program is run.

Note that VIDBLITZ is very accurate. You will achieve nearly identical

results on machines with the same video card, provided that the bus

type/speed is equal. CPU speed is only a minor factor in determining the

speed of your video card.

A typical run may look like this:

VIDBLITZ v1.20 - A MAGE Utility - (C) Fabian Gonzalez, 1994

1024 x 4096 doublewords = 16 megabytes were read from video memory

Timing results: 140 frames, 2.00 seconds

Megabytes per second    :     8.00

Kilobytes per frame     :   117.03

Frames per 16 kilobytes :     0.14

CPU clocks per read     :    31.79

4096 x 16386 doublewords = 64 megabytes were written to video memory

Timing results: 176 frames, 2.51 seconds

Megabytes per second    :    25.45

Kilobytes per frame     :   372.36

Frames per 64 kilobytes :     0.17

CPU clocks per write    :     9.99

The number of frames used in transferring the data is shown, together with its

equivalent in seconds. The throughput is shown in megabytes per second (the

amount of data that can be transferred to/from video memory in a second), as

kilobytes per frame (the amount of data that can be transferred in a single

frame) and as frames per 64 KB. Frames per 64 KB shows the number of frames

that pass in the amount of time it takes to transfer 64 KB to/from video

memory. The last entry shows the CPU clock cyles used to transfer one

doubleword (4 bytes) to/from video memory.

Reads from video memory generally are a lot slower than writes. This is the

reason for the reduced data transfer amount in reads contra writes.

VIDBLITZ accepts the following options:

-h         Displays the help screen

-c[speed]  Sets the CPU speed. This setting is used to calculate the number

of CPU clock cycles required per write/read. This setting defaults

to 66.67 MHz. Note that some motherboards are clocked at 66, others

at 66.67 MHz. The same applies to 33/33.33 Mhz and 99/100 Mhz

motherboards.

-w[frames] Shows full timing statistics for writes, based on the number of

frames used. This option is useful for getting the full statistics

from video cards listed in VIDCARDS.TXT.

-r[frames] Shows full timing statistics or reads, based on the number of

frames used. This option is useful for getting the full statistics

from video cards listed in VIDCARDS.TXT.

-m         This option lets you test the main memory of your computer instead

of the video memory.