BRINGING UP FASTFLOP ON A NEW SYSTEM

FastFlop achieves its speed by reading the sectors on a track

beginning where the head is likely to be, rather than always starting

with sector 1, thus avoiding the wait for sector 1 to come around under

the head.  Thus, it might read sectors 5-9 followed immediately

by a read of sectors 1-4 of the SAME track.  TOS however

waits for 30 ms before reading each track, so when one attempts to

read sectors 1-4, sector 1 has already passed under the head before

TOS is ready to read it, and the disk make another revolution (200 ms)

before the read begins.  The purpose of this delay

is to allow the head to settle down after moving to a new track, but

unfortunately, TOS inserts the delay even after a seek to the SAME track.

FastFlop overcomes this problem by modifying the floppy disk

driver.  In order to do this, it needs to know about the locations

of particular instructions in TOS, which may vary from country to country

and from version to version of TOS.  FastFlop has a table of

the necessary locations for several versions of TOS.

However, if you want to run FastFlop on a version of TOS that FastFlop

does not know about, you must determine the proper 'magic locations' for

your version of TOS, add them to the FastFlop source code,

and recompile FastFlop.  This document explains how to do that.

You will need a machine-level debugger.

The magic locations needed by FastFlop are stored in the following

structure:

typedef struct {

LONG version;   /* The composite version # for this record */

LONG base_adr;  /* Base address for this version of TOS */

unsigned length;    /* Length of floppy code */

int floprd;     /* Location of floprd() */

int flopwr;     /* Location of flopwr() */

int flopver;    /* Location of flopver() */

int patch_place;/* Place to patch in jmp... */

int ctrack;            /* Place to find CTRACK.w */

int flop_cmds_offbase; /* Base address for computing flop_cmds() */

} offset_rec;

They are explained individually below.

1.  The "composite" version number holds the TOS version number in the

upper 16 bits and the TOS date (in GEMDOS format) in the lower 16.

This is used to identify the particular version of TOS you are running.

(We do not know if two different versions of TOS ever have both the

same TOS version number and the same TOS date.)

The version number and date are stored in the OS header which is

at a fixed offset from the location pointed to by the

documented system variable SYSBASE.

The following code obtains the version number for you (using my

special e_osbind.h include file) and builds the composite version

number needed by FastFlop.

LONG usp;

LONG version;

usp = Super(0);

version = SYSBASE->version;

version <<= 16;

version |= SYSBASE->tos_date;

Super(usp);

2.  base_adr is the starting address of the floppy disk driver

and is the only absolute address needed by FastFlop.

All other magic values, from floprd to

flop_cmds_offbase, are offsets from base_adr.

The floppy code starts at flopini(), and base_adr should be set

to its starting address.  Since flopini() has no XBIOS or BIOS calling

sequence, finding it is a little involved.

It is the routine that comes right

before floprd() (generally $48 bytes before),

which does have an XBIOS calling sequence.

floprd().

To find the address of floprd() in

memory, in the jump table for the XBIOS trap

handler.  The jump table immediately follows

the XBIOS trap handler, whose address is generally stored at

location $B8 (XBIOS trap vector).

Some programs, such as TurboST, install

themselves in this trap, obscuring the location of the trap handler in

ROM.  To most easily find the ROM trap handler, de-install all

programs which change the contents of $B8.

Now, to find the jump table, look directly past the trap handler for a

long string of addresses, or inspect the code of the trap handler

carefully to find where the jump table starts.  In ROM versions, all

addresses stored in the jump table start with $00FC, making it easy to

simply search for a series of longwords of the form $00FCxxxx,

directly after

the trap handler.  Determine the start of the jump table and then

add 8 longwords (32 bytes) to this to determine the pointer to the

address of floprd().  Look at the code and determine that it matches

the code fragment below, with the arrowed line being the start address of

floprd().

bne

bsr

beq

bra

FLOPRD:

bsr

moveq   #~~~,d0

bsr     ~~~

bsr     ~~~

bsr     go2track

bne

Subtract $48 from the address of FLOPRD(), and you have the start of

the floppy handler.  Store this as the value of base_adr.  Just to make

sure, check its code against the following:

FLOPINI:

lea     ~~~, A1

tst.w   0C(A7)

beq

lea     ~~~, A1

move.w  ~~~, 2(A1)

moveq

clr.w

bsr

bsr

move.w

3.  'length' should be set to the number of bytes in the floppy disk driver.

To determine the length, look about $780 bytes past

FLOPINI for the following code:

clr.l     (a0)

move.w    0x10(a7), ~~~~

clr.w     0x10(a7)

rts

lea       0xECC(a5), a0

move.b    d0, -(a7)

move.w    0xA08(a5), d0

move.b    (a7)+, 0x0(a0,d0)

rts

lea       ~~~~, a0

tst.w     0x4(a7)

beq       ~~~~

lea       ~~~~, a0

move.w    0x2(a0), d0

move.w    0x6(a7), d1

cmp.w     #-1, d1

beq       ~~~~

move.w    d1, 0x2(a0)

ext.l     d0

rts             ; This is the last statement in the floppy code.

Keep in mind that link or unlnk instructions are hallmarks of compiled

code.  The floppy handler was written in assembly code, so it should

(probably) contain no link or unlk instructions.  In my version of

the ROMs, there's a link instruction right after the floppy handler,

making it rather easy to find the end.

If you're not sure where the code ends, it doesn't hurt FastFlop's

functionality to assume that the code is a bit longer than it is.

Guessing too small, on the other hand, could lead to spectacular

crashes at random times.

All remaining addresses are offsets from the

start of the floppy handler.  You can use the offsets found in

the FastFlop source code for a close

version of TOS as a starting point in your search.  To compute each

offset, first find the absolute address of the desired location, and

then subtract base_adr from it.

4.  floprd is the offset for the floprd() routine which you found above.

It should be about $48.

5.  flopwr is the offset for flopwr(), the floppy disk write routine.

It is an XBIOS routine and can be found as above by looking in the

XBIOS trap handler's jump table at the entry 9 longwords past the beginning.

The code around it should look as follows:

bne

move.w ~~~(A5), D1

move.w D1, ~~~(A5)

rts

FLOPWR:

bsr

moveq #F6, D0

6.  flopver is the offset for flopver(), the floppy disk verify routine.

It is an XBIOS routine and can be found by looking in the XBIOS trap

handler's jump table at the entry 19 longwords past the beginning.

The code around it should look as follows:

rts

move.b D0, (A2)+

dbf D1, ~~~

rts

FLOPVER:

bsr

moveq #F5, D0

bsr

bsr

bsr

7.  patchplace, ctrack, and flop_cmds_offbase.  These are all addresses (stored

as offsets from base_adr) into

the middle of the driver routines 'go2track' and 'restore'.  To find

the location of this code, look back at the beginning of the code

for floprd(), and

notice that the 5th instruction is a 'bsr go2track'.

patchplace and offbase are the addresses shown below.  ctrack is

xctrack - 2.

PATCHPLACE, CTRACK, OFFBASE:

move.w  #10, D6

bsr

bne

move.w

go2track:

move.w

move.w

move.w

XCTRACK:                 (CTRACK = XCTRACK - 2)

bsr

moveq   #14, D6

PATCHPLACE:

bsr

bne

move.w

and.b

rts

restore:

clr.w D6

bsr

FLOP_CMDS_OFFBASE:

bne

btst #02, D7

eor #04, CCR

That's it!  Put these magic values into a new entry of the 'loc' array,

recompile, and enjoy.

Please, when you make a new version of FastFlop, send the table of

"magic values" and the date and version of your TOS to me (Robert

Fischer).  I will incorperate those changes into a new official

release of FastFlop.

Don't hesitate to call or write if you have trouble.  My address is:

80 Killdeer Road

Hamden, CT   06517   (USA)

(203) 288-9599

or E-mail c/o Michael Fischer:  <fischer-michael@cs.yale.edu>