INFORMATIQUE & ARTS

1925, Bergemont, Qubec

(Qubec) G1J 3T5

CANADA

Tel. (418) 529-7616

S O U N D M A S T E R   K 4

version 1.2 1990

by  M.Sc. L. Stoyanov

CompuServe 76546,724

GEnie Stoyanov,Latchesar I.

SOUNDMASTER K4 is a single and multiedi-

tor,soundlibrary-manager - drums and ef-

fects-processor and  randomgenerator for

the new  Kawai K4 16 Bit Synthesizer. It

runs on all ATARI ST/Mega/Stacy/STe with

monochrome or color monitor.

It is programmed in Assembler and becau-

se of that is  working much more quickly

and it is smaller.It is musician-friend-

ly

and absolutely indispensable for profies

and beginners, who  wants to exploit all

the capabilities of the K4 synthesizer.

Now the features of the programm:

- Overview -  the sound-parameters /over

140!/  are  all  at  the same time on the

screen. If you know how much time it tak-

es to switch  between different pages you

will appreciate this. For graphic edition

you need to click in the special boxes.

/ DCA,Curves ets./

- Page-princip - full K4-complex from the

K4/r or disk  opens a new page. The pages

0 - 2 are reserved:

Page 0 - edit-area.

Page 1 -  the  original complex from the

K4/r -  automatically  received after the

start of the programm. If there isn't any

K4/r it is empty.

Page 2 - Store -  the store for the good

patches.

You have the possibility to work with

max 60 complexes(pages) or 120 blocks !!!

/on 1 MB/ at the same time.

- Virtually keyboard - to test the sou-

nds.It is possible to play tones and  ac-

cords   /min,maj,4/ from  c -2 till C 6 ,

set the velocity- and octave,switch bet -

ween the original- and edit-sound, change

the MIDI- channel.

_ Drums-processor - easy acces and edi-

ting  all the notes of the K4-drums-sec -

tion of  the K4 with  hear-control ; copy

from a note to another note.

- Effects-/outputs-processor -  analyse

and synthese the intern effects / outputs

of the K4/r with hear-control ; copy.

- copy function - with extensive filter

, variable  possibilities of restriction,

copy from every patch in the memory.

- Random -  powerful  random  generator

with flexible filter,with many new featu-

res, developed first by the  Informatique

& Arts:

- Art  - artistic creation based on the

long  experiance  in  the  domain  of the

sound design;

- Base - random-transformation of a ba-

sic-sound;

6 Steps/random-speeds/.

A powerful  instrument for research and

creation !!!

- Block-editor - compose , classify and

save  of single- or multi-blocks from all

available patches.

- Envelope-processor - graphic real ti-

me - editor of  the envelopes of  all the

four sources at the same time.

- Graphic  representation  and  editing

of all  the velocity / KS-curves and vib-

rato/DC filter and shapes.

- automatically data-transfer -   every

modification is simultaneously transfered

in the K4/r and the result is ready to be

heard.

- direct connection  between floppy disk

and K4 -  in the both directions  without

any influence of the edit-area.

- save on disk on 3 ways -  in patches,

blocks or complexes/all/.

- Central control from  the programm of

all the K4-functions -

no need to touch the synthesizer; control

of the MIDI-connections.

- 18 differente  messages -   MIDI  /no

card/, MIDI /write error/, LOAD  /invali-

de file/ etc.

- invincible speed -  by the new routi-

nes accelerators developed by  Informati-

que & Arts.

- no 1 MB condition - the programm runs

on min 170 KB !!!

- Undo-function - everywhere.

-easy and musician-friendly utilisation

- unique quality - price report.

M A N U A L

A. The SINGLE-editor

1. Mode /K4,K4r/ - you click in the box

to switch.

2. Kind of patch /S-SINGLE, M-MULTI/

3. Position/I-Intern,E-card/ -  you can

switch.

4. Patch-number in Kawai-standart.

5. MIDI-number.

6. Patch-name.

If you click 4. or 5.  you scroll through

the patches in the edit-area -  left but-

ton up right - down.

Click in 6. starts  the patchname-editor.

Click in all other parameterboxes changes

the value:

left  button -1

right button +1

both buttons: a) first  left -10

b) first right +10.

pic.B The MULTI-editor

The F1-key or click in the test-box opens

the  virtually  keyboard  for testing the

sounds./pic. K/

The F2-key or click in the random-box pro

duces a random-patch.

The F3-key  without shift or click in the

list-box with the left button opens a ca-

talogue of all the patches/pic. E/

The F3-key with shift or right button sta

rts the block-editor/pic. F/

The F4-key with  shift or  click with the

left button in the corresponding box sc-

rolls up, without shift or right button -

down trough the edit-area.

The  F5 - key  or  the  corresponding box

switch  between  the single-  and  multi-

editor.

The  F6-key or  click in  the  /CARD/-box

with the left button changes the position

/I-Inern,E-Extern/ .

The  F6-key  with  shift or  click in the

/CARD/-box with  the right button changes

the mode (K4, K4r) of the editor.

The F7-key or  click in  the /CURVES/-box

starts  the graphic curve-editor./pic. C/

Vibrato- / DCF-shape ,poly-mode and velo-

city-/KS- curves are represented and edi-

table as graphics.

The F8-key  or click in  the  /DRUMS/-box

starts the drums-processor./pic. G/

The F9-key  or  click   in the /EFFECTS -

OUTPUTS/-box starts the effects-/outputs-

processor./pic. H/

The F10-key  or  click in the /STORE/-box

switch between  the aktually patch in the

edit-area and  the corresponding patch in

the store-area/page 2/.

The Undo-key or box removes all the orig-

inal parameters.

The click in the /DCA/-box on pic.A opens

the graphic envelope - processor./pic. D/

All  the standart windows-operations are

enable.

The MENU

The menu has 8 Titles:

A. Desk -

A1. Soundmaster -     opens the info-box.

B. File - All Load/save routines, opens

the file-box

B1./3./5./7./9. - load one/block/drums/

effects(outputs)/all.

B2./4./6./8./10./- save one/block/drums

/effects(outputs)/all.

B11. - delete file- /to win a place for

saving/.

B12. - format disk.

B13. - quit - leave the editor.

C. Direct -  automatically receives the

K4-datas and  directly saves on disk wit-

hout  any  influence on  the edit - area.

/pic. J/

C1./2./3./4./5. -   one/block/drums/ef-

fects(outputs)/all.

D. Options -

D1. - test sound -  opens the virtually

keyboard./pic. K/

D2. - save modus -   saves the mode /K4

, K4/r and gives the possibility to begin

with  the programm-start  in the mode you

want.

E. MIDI - all the MIDI routines.

E1. MIDI THRU - opens the MIDI-box.

/pic. P/

E2./3./4./5./6. - receive one/block/dr-

ums/effects(outputs)/all - opens the re -

ceive-box/pic. J/, automatically receives

the K4-datas in  the edit-area, intern or

extern /card/ - via MIDI.

E7./8./9./10./11./12. -  send one/block

/drums/effects(outputs)/all -   sends K4-

datas to the synth, intern or to the card

(if you have one).

F. Edit

F1./2./3./4./5. one/block/drums/effects

(outputs)/all- edit the corresponding ob-

jekts.

G. Copy - opens the copy-box.  /pic. M/

G1./2./3. -  copies  source/one/block ,

set before the sources and groups impli-

cated in the copy-process !

H. Random - opens the random-box.

H1. - do -   sets the random-parameter

and generates a random-patch.

C. The CURVES-editor

Starts from the F7-key or the  /CURVES/

-box in the single-editor.The  vibrato- /

DCF-shapes, polymode,KS-and velocity cur-

ves are represented as graphic. The aktu-

ally shapes and polymode are reverse. You

can select  the new  shapes and mode. The

curves-numbers are to change with left or

right button.

D. The envelope-processor.

Starts from  the /DCA/ -Box in the single

-editor or from the menu.All the four en-

velope are at the same time on the screen

. You click in the  corresponding box the

mouse  changes  to cross and the box gets

movable in the definite limits.The corres

ponding values  changes in real time. You

don't need to keep down the mouse-button.

The values  are  direct changeable in the

value-boxes with left or right button.

E. LIST

Starts from the F3-key without shift or

a click in  the /LIST/-Box  with the left

button. The list-window  closes  when the

mouse reaches the top of the screen.

On the first line the page-number and the

page-name  (for numbers less than 3). Use

the window-arrow  and  vertical-slider to

scroll the pages up and down.

The follow operationa are possible:

copy - click on the source , it will be

reverse, the mouse  change to cross. Move

the mouse over the destination (you don't

need to keep  the  button  down), click a

second time without shift.The destination

is in  the edit-area (page 0), after copy

you are every time in the page 0.

place-changing - like the copy, but the

second click is with shift.

If you click with the right  button the

patch  in a page  with a  number  greater

than 0,  you copy it on the same place in

the edit-area(page 0). If you do the same

in the page 0, you open the single-/multi

editor for this patch.

The click in the boxes - SINGLE , MULTI

,DRUMS , EFFECTS(OUTPUTS) copies the cor-

responding group in edit-area(page 0).

The click whit the right button copies all

in the edit-area(page 0).If you are alre-

ady in page 0, nothing happens.

F. The block-editor.

Starts  from the F3-key with shift or the

/LIST/-box with the right button.You leave

the block-editor,if the mouse reaches the

top of the screen.

On the first line you see the page-number.

On the left side of the screen you see the

patch-list of the SINGLE-/MULTI-block, on

the right- the block you want to make up.

Only  the right /EDIT/-block is editable.

The left is only the source.

The click in the SINGLE-/ MULTI-Box  with

the right button  switch  between MULTI-/

and SINGLE-block. The click with the left

button and  shift copies the source-block

to the edit-block.

The click in the /EDIT/-Box with the right

button and shift clears the edit-block.

The click in the /PAGE/-Box with the left

/right button scrolls through the pages

you have - up/down.

If you  select  a patch from  the source-

block, you can copy it to the edit-block.

If you select a patch from the edit-block

, there are 3 possibilities:

A.copy - if you click  a destination in

the edit-block without shift.

B. place-changing (classify) - the same

with shift.

C.clear -if you click a destination out

of the edit-block.

The /SAVE/-Box is  to save the edit-block.

Only  a full edit-block  can be saved, if

the block is not full,appears the corres-

ponding warning-box.

G.The drums-processor.

Starts from the F8-key,from the /DRUMS/-

box or the menu - edit.

You leave from the /EXIT/-box or return-

key.

The values are to  change by  the left or

right mouse-button.

The click  in the /DONE/-box  copies  the

selected source- to the selected destina-

tion-note.

The click in the /NOTE/-Box scrolls  from

C 1 till C 6 - by the left or right  but-

ton up or down.

If you  select one of  the both sources

of a drum-note,it will be reverse,and you

see  down in  the corresponding boxes the

wave-names.

If you click in one note-box you can hear

this note.

/DUMMY/ is a data-area without any influ-

ence  of  the sound  and never appears on

the K4-window, you can use it as a perso-

nal mark.

H. The effects-processor.

Starts from the F9-key,from the /EFFECTS/

-Box or the menu - edit.

You leave it  from  the /EXIT/-Box or the

return-key.

In the K4r-mode all effects-boxes are di-

sabled and only the panorama is editable.

In the K4-mode there are 16 effects-types

,6 - double.Every effects-type has 3 dif-

ferent global parameter and     /SEND 1/-

/SEND 2/- parameters for every channel.

The /DONE/- and /DUMMY/-boxes  have  the

same functions, like  the  corresponding

boxes in the drums-processor.

The receive-box.

Appears from  the menu -  Direct  and the

MIDI receive-group. Select  the objekt to

receive.Receiving from a card is possible

. If you don't have a card, but try to re

ceive from, appears the corresponding war

ning box. The receive-box doesn't open,if

you don't connect any K4/r synth .

K. Virtually keyboard.

Starts from the F1-key,from the  /TEST/-

Box or  the menu -optionen. You can play

from the ATARI-keyboard or mouse.You can

switch between the original-and the edit

-patch -very useful to comparate.You can

play tones or accords.

L. QUIT - leaving the editor.

Starts from the menu - File.

You have 4 possibilities:

A. befor leaving ,send the original da-

tas back in the K4/r.

B. befor leaving ,save the original da-

tas on a disk.

C. leaving without any saving.

D. return back in the editor./CANCEL/

M. COPY-Box.

Starts from the menu - Copy.Only the re-

verse - represented  sources  and groups

will be copied.

It is possible to copy from all the pages

in the memory.

N. Random-box.

Starts from the menu - Random.Use it to

set the parameters of the random-genera-

tor. Only reverse-represented sourcen and

groups will be randomized.

O. Formatbox.

Set one- or two-sided and by click on the

/OK/-button or  return-key, the formating

begins.

P. MIDI-Box

Switch  on - off the MIDI-bridge. Set the

send and  auto off  MIDI-channel. Set the

input- and the output- MIDI-filters.