The official MIDI 1.0 specification is available from:

IMA

the International MIDI Association

11857 Hartsook St.

North Holllywoood, CA  91607

(818) 505-8964

The complete MIDI spec as developed by the Japanese manufacturers

and adopted by the "World" at the Summer '85 NAMM show

is available to IMA members ($40/yr) foor $30 non-members $35.

The sketchy hardware and byte definitions are free with membership.

The following is an expanded MIDI definition (sort of in-between

IMA MIDI 1.0 and the new $35 booklet) entered into the public

domain on USENET net.musi.synth by an altruistic musically

inclined engineer:

Bob McQueer

22 Bcy, 3151

All rites reversed.  Reprint what you like.

The USENET MIDI Primer

Bob McQueer

PURPOSE

It seems as though many people in the USENET community have an interest

in the Musical Instrument Digital Interface (MIDI), but for one reason

or another have only obtained word of mouth or fragmentary descriptions

of the specification.  Basic questions such as "what's the baud rate?",

"is it EIA?" and the like seem to keep surfacing in about half a dozen

newsgroups.  This article is an attempt to provide the basic data to

the readers of the net.

REFERENCE

The major written reference for this article is version 1.0 of the MIDI

specification, published by the International MIDI Association, copyright

1983.  There exists an expanded document.  This document, which I have not

seen, is simply an expansion of the 1.0 spec. to contain more explanatory

material, and fill in some areas of hazy explanation.  There are no

radical departures from 1.0 in it.  I have also heard of a "2.0" spec.,

but the IMA claims no such animal exists.  In any event, backwards

compatibility with the information I am presenting here should be

maintained.

CONVENTIONS

I will give constants in C syntax, ie. 0x for hexadecimal.  If I

refer to bits by number, I number them starting with 0 for the low

order (1's place) bit.  The following notation:

>>

text

<<

will be used to delimit commentary which is not part of the "bare-

bones" specification.  A sentence or paragraph marked with a question

mark in column 1 is a point I would kind of like to hear something

about myself.

OK, let's give it a shot.

PHYSICAL CONNECTOR SPECIFICATION

The standard connectors used for MIDI are 5 pin DIN.  Separate sockets

are used for input and output, clearly marked on a given device.  The

spec. gives 50 feet as the maximum cable length.  Cables are to be

shielded twisted pair, with the shield connecting pin 2 at both ends.

The pair is pins 4 and 5, pins 1 and 3 being unconnected:

2

5       4

3           1

A device may also be equipped with a "MIDI-THRU" socket which is used

to pass the input of one device directly to output.

>>

I think this arrangement shows some of the original conception

of MIDI more as a way of allowing keyboardists to control

multiple boxes than an instrument to computer interface.  The

"daisy-chain" arrangement probably has advantages for a performing

musician who wants to play "stacked" synthesizers for a desired

sound, and has to be able to set things up on the road.

<<

ELECTRICAL SPECIFICATION

Asynchronous serial interface.  The baud rate is 31.25 Kbaud (+/- 1%).

There are 8 data bits, with 1 start bit and 1 stop bit, for 320 microseconds

per serial byte.

MIDI is current loop, 5 mA.  Logic 0 is current ON.  The specification

states that input is to be opto-isolated, and points out that Sharp

PC-900 and HP 6N138 optoisolators are satisfactory devices.  Rise and

fall time for the optoisolator should be less than 2 microseconds.

The specification shows a little circuit diagram for the connections

to a UART.  I am not going to reproduce it here.  There's not much

to it - I think the important thing it shows is +5 volt connection

to pi 4 of the MIDI out with pin 5 going to the UART, through 220

ohm load resisors. It also shows that you're supposed to connect

to the "in" side of the UART through an optoisolator, and to the

MIDI-THRU on the UART side of the isolator.

>>

I'm not much of a hardware person, and don't really know what

I'm talking about in paragraphs like the three above.  I DO

recognize that this is a "non-standard" specification, which

won't work over serial ports intended for anything else.  People

who do know about such things seem to either have giggling

or gagging fits when they see it, depending on their dispos-

itions, saying things like "I haven't seen current loop since

the days of the old teletypes".  I also know the fast 31.25

Kbaud pushes the edge for clocking commonly available UART's.

<<

DATA FORMAT

For standard MIDI messages, there is a clear concept that one device

is a "transmitter" or "master", and the other a "receiver" or "slave".

Messages take the form of opcode bytes, followed by data bytes.

Opcode bytes are commonly called "status" bytes, so we shall use

this term.

>>

very similar to handling a terminal via escape sequences.  There

aren't ACK's or other handshaking mechanisms in the protocol.

<<

Status bytes are marked by bit 7 being 1.  All data bytes must

contain a 0 in bit 7, and thus lie in the range 0 - 127.

MIDI has a logical channel concept.  There are 16 logical channels,

encoded into bits 0 - 3 of the status bytes of messages for

which a channel number is significant.  Since bit 7 is taken over

for marking the status byte, this leaves 3 opcode bits for message

types with a logical channel.  7 of the possible 8 opcodes are

used in this fashion,  reserving the status bytes containing all

1's in the high nibble for "system" messages which don't have a

channel number.  The low order nibble in these remaining messages

is  eelly ffuther  pcodee

>>

II you are  nterested in rrceiving MIDI input, look over the

SYSTEM messages even if you wish to ignore them.  Especially the

"system exclusive" and "real time" messages.  The real time

messages may be legally inserted in the middle of other data,

and you should be aware of them, even though many devices won't

use them.

<<

VOICE MESSAGES

I will cover the message with channel numbers first.  The opcode determines

the number of data bytes for a single message (see "running status byte",

below).  The specification divides these into "voice" and "mode" messages.

The "mode" messages are for control of the logical channels, and the control

opcodes are piggybacked onto the data bytes for the "parameter" message.  I

will go into this after describing the "voice messages".  These messages are:

status byte   meaning        data bytes

0x80-0x8f     note off       2 - 1 byte pitch, followed by 1 byte velocity

0x90-0x9f     note on        2 - 1 byte pitch, followed by 1 byte velocity

0xa0-0xaf     key pressure   2 - 1 byte pitch, 1 byte pressure (after-touch)

0xb0-0xbf     parameter      2 - 1 byte parameter number, 1 byte setting

0xc0-0xcf     program        1 byte program selected

0xd0-0xdf     chan. pressure 1 byte channel pressure (after-touch)

0xe0-0xef     pitch wheel    2 bytes giving a 14 bit value, least

significant 7 bits first

Many explanations are necessary here:

For all of these messages, a convention called the "running status

byte" may be used.  If the transmitter wishes to send another message

of the same type on the same channel, thus the same status byte, the

status byte need not be resent.

Also, a "note on" message with a velocity of zero is to be synonymous

with a "note off".  Combined with the previous feature, this is intended

to allow long strings of notes to be sent without repeating status bytes.

>>

From what I've seen, the "zero velocity note on" feature is very

heavily used.  My six-trak sends these, even though it sends

status bytes on every note anyway.  Roland stuff uses it.

<<

The pitch bytes of notes are simply number of half-steps, with

middle C = 60.

>>

On keyboard synthesizers, this usually simply means which

physical key corresponds, since the patch selection will

change the actual pitch range of the keyboard.  Most keyboards

have one C key which is unmistakably in the middle of the

keyboard.  This is probably note 60.

<<

The velocity bytes for velocity sensing keyboards are supposed

to represent a logarithmic scale.  "advisable" in the words

of the spec.  Non-velocity sensing devices are supposed to

send velocity 64.

The pitch wheel value is an absolute setting, 0 - 0x3FFF.  The

1.0 spec. says that the increment is determined by the receiver.

0x2000 is to correspond to a centered pitch wheel (unmodified

notes)

>>

I believe standard scale steps are one of the things discussed

in expansions.  The six-trak pitch wheel is up/down about a third.

I believe several makers have used this value, but I may be wrong.

The "pressure" messages are for keyboards which sense the amount

of pressure placed on an already depressed key, as opposed to

velocity, which is how fast it is depressed or released.

?       I'm not really certain of how "channel" pressure works.  Yamaha

is one maker that uses these messages, I know.

<<

Now, about those parameter messages.

Instruments are so fundamentally different in the various controls

they have that no attempt was made to define a standard set, like

say 9 for "Filter Resonance".  Instead, it was simply assumed that

these messages allow you to set "controller" dials, whose purposes

are left to the given device, except as noted below.  The first data

bytes correspond to these "controllers" as follows:

data byte

0 - 31       continuous controllers 0 - 31, most significant byte

32 - 63      continuous controllers 0 - 31, least significant byte

64 - 95      on / off switches

96 - 121     unspecified, reserved for future.

122 - 127    the "channel mode" messages I alluded to above.  See

below.

The second data byte contains the seven bit setting for the controller.

The switches have data byte 0 = OFF, 127 = ON with 1 - 126 undefined.

If a controller only needs seven bits of resolution, it is supposed to

use the most significant byte.  If both are needed, the order is

specified as most significant followed by least significant.  With a

14 bit controller, it is to be legal to send only the least significant

byte if the most significant doesn't need to be changed.

>>

This may of, course, wind up stretched a bit by a given manufacturer.

The Six-Trak, for instance, uses only single byte values (LEFT

justified within the 7 bits at that), and recognizes >32 parameters

<<

Controller number 1 IS standardized to be the modulation wheel.

?       Are there any other standardizations which are being followed by most

manufacturers?

MODE MESSAGES

These are messages with status bytes 0xb0 through 0xbf, and leading data

bytes 122 - 127.  In reality, these data bytes function as further

opcode data for a group of messages which control the combination of

voices and channels to be accepted by a receiver.

An important point is that there is an implicit "basic" channel over which

a given device is to receive these messages.  The receiver is to ignore

mode messages over any other channels, no matter what mode it might be in.

The basic channel for a given device may be fixed or set in some manner

outside the scope of the MIDI standard.

The meaning of the values 122 through 127 is as follows:

data byte                   second data byte

122       local control     0 = local control off, 127 = on

123       all notes off     0

124       omni mode off     0

125       omni mode on      0

126       monophonic mode   number of monophonic channels, or 0

for a number equal to receivers voices

127       polyphonic mode   0

124 - 127 also turn all notes off.

Local control refers to whether or not notes played on an instruments

keyboard play on the instrument or not.  With local control off, the

host is still supposed to be able to read input data if desired, as

well as sending notes to the instrument.  Very much like "local echo"

on a terminal, or "half duplex" vs. "full duplex".

The mode setting messages control what channels / how many voices the

receiver recognizes.  The "basic channel" must be kept in mind. "Omni"

refers to the ability to receive voice messages on all channels.  "Mono"

and "Poly" refer to whether multiple voices are allowed.  The rub is

that the omni on/off state and the mono/poly state interact with each

other.  We will go over each of the four possible settings, called "modes"

and given numbers in the specification:

mode 1 - Omni on / Poly - voice messages received on all channels and

assigned polyphonically.  Basically, any notes it gets, it

plays, up to the number of voices it's capable of.

mode 2 - Omni on / Mono - monophonic instrument which will receive

notes to play in one voice on all channels.

mode 3 - Omni off / Poly - polyphonic instrument which will receive

voice messages on only the basic channel.

mode 4 - Omni off / Mono - A useful mode, but "mono" is a misnomer.

To operate in this mode a receiver is supposed to receive

one voice per channel.  The number channels recognized will be

given by the second data byte, or the maximum number of possible

voices if this byte is zero.  The set of channels thus defined

is a sequential set, starting with the basic channel.

The spec. states that a receiver may ignore any mode that it cannot

honor, or switch to an alternate - "usually" mode 1.  Receivers are

supposed to default to mode 1 on power up.  It is also stated that

power up conditions are supposed to place a receiver in a state where

it will only respond to note on / note off messages, requiring a

setting of some sort to enable the other message types.

>>

I think this shows the desire to "daisy-chain" devices for

performance from a single master again.  We can set a series

of instruments to different basic channels, tie 'em together,

and let them pass through the stuff they're not supposed to

play to someone down the line.

This suffers greatly from lack of acknowledgement concerning

modes and usable channels by a receiver.  You basically have

to know your device, what it can do, and what channels it can

do it on.

I think most makers have used the "system exclusive" message

(see below) to handle channels in a more sophisticated manner,

as well as changing "basic channel" and enabling receipt of

different message types under host control rather than by

adjustment on the device alone.

The "parameters" may also be usurped by a manufacturer for

mode control, since their purposes are undefined.

Another HUGE problem with the "daisy-chain" mental set of MIDI

is that most devices ALWAYS shovel whatever they play to their

MIDI outs, whether they got it from the keyboard or MIDI in.

This means that you have to cope with the instrument echoing

input back at you if you're trying to do an interactive session

with the synthesizer.  There is DRASTIC need for some MIDI flag

which specifically means that only locally generated data is to

go to MIDI out.  From device to device there are ways of coping

with this, none of them good.

<<

SYSTEM MESSAGES

The status bytes 0x80 - 0x8f do not have channel numbers in the

lower nibble.  These bytes are used as follows:

byte    purpose              data bytes

0xf0    system exclusive     variable length

0xf1    undefined

0xf2    song position        2 - 14 bit value, least significant byte

first

0xf3    song select          1 - song number

0xf4    undefined

0xf5    undefined

0xf6    tune request         0

0xf7    EOX (terminator)     0

The status bytes 0xf8 - 0xff are the so-called "real-time" messages.

I will discuss these after the accumulated notes concerning the

first bunch.

Song position / song select are for control of sequencers.  The

song position is in beats, which are to be interpreted as every

6 MIDI clock pulses.  These messages determine what is to be played

upon receipt of a "start" real-time message (see below).

The "tune request" is a command to analog synthesizers to tune their

oscillators.

The system exclusive message is intended for manufacturers to use

to insert any specific messages they want to which apply to their

own product.  The following data bytes are all to be "data" bytes,

that is they are all to be in the range 0 - 127.  The system exclusive

is to be terminated by the 0xf7 terminator byte.  The first data byte

is also supposed to be a "manufacturer's id", assigned by a MIDI

standards committee.  THE TERMINATOR BYTE IS OPTIONAL - a system

exclusive may also be "terminated" by the status byte of the next

message.

>>

Yamaha, in particular, caused problems by not sending terminator

bytes.  As I understand it, the DX-7 sends a system exclusive

at something like 80 msec. intervals when it has nothing better

to do, just so you know it's still there, I guess.  The messages

aren't explicitly terminated, so if you want to handle the

protocol (esp. in hardware), you should be aware that a DX-7

will leave you in "waiting for EOX" state a lot, and be sending

data even when it isn't doing anything.  This is all word of

mouth, since I've never personally played with a DX-7.

<<

some MIDI ID's:

Sequential Circuits   1      Bon Tempi     0x20     Kawai     0x40

Big Briar             2      S.I.E.L.      0x21     Roland    0x41

Octave / Plateau      3                             Korg      0x42

Moog                  4      SyntheAxe     0x23     Yamaha    0x43

Passport Designs      5

Lexicon               6

PAIA                  0x11

Simmons               0x12

Gentle Electric       0x13

Fairlight             0x14

>>

Note the USA / Europe / Japan grouping of codes.  Also note

that Sequential Circuits snarfed id number 1 - Sequential

Circuits was one of the earliest participators in MIDI, some

people claim its originator.

Two large makers missing from the original lineup were Casio

and Oberheim.  I know Oberheim is on the bandwagon now, and

Casio also, I believe.  Oberheim had their own protocol previous

to MIDI, and when MIDI first came out they were reluctant to

?       go along with it.  I wonder what we'd be looking at if Oberheim

had pushed their ideas and made them the standard.  From what I

understand they thought THEIRS was better, and kind of sulked

for a while until the market forced them to go MIDI.

?       Nobody seems to care much about these ID numbers.  I can only

imagine them becoming useful if additions to the standard message

set are placed into system exclusives, with the ID byte to let

you know what added protocol is being used.  Are any groups of

manufacturers considering consolidating their efforts in a

standard extension set via system exclusives?

<<

REAL TIME MESSAGES.

This is the final group of status bytes, 0xf8 - 0xff.  These bytes

are reserved for messages which are called "real-time" messages

because they are allowed to be sent ANYPLACE.  This includes in

between data bytes of other messages.  A receiver is supposed to

be able to receive and process (or ignore) these messages and

resume collection of the remaining data bytes for the message

which was in progress.  Realtime messages do not affect the

"running status byte" which might be in effect.

?       Do any devices REALLY insert these things in the middle of

other messages?

All of these messages have no data bytes following (or they could

get interrupted themselves, obviously).  The messages:

0xf8   timing clock

0xf9   undefined

0xfa   start

0xfb   continue

0xfc   stop

0xfd   undefined

0xfe   active sensing

0xff   system reset

The timing clock message is to be sent at the rate of 24 clocks

per quarter note, and is used to sync. devices, especially drum

machines.

Start / continue / stop are for control of sequencers and drum

machines.  The continue message causes a device to pick up at the

next clock mark.

>>

These things are also designed for performance, allowing control

of sequencers and drum machines from a "master" unit which

sends the messages down the line when its buttons are pushed.

I can't tell you much about the trials and tribulations of drum

machines.  Other folks can, I am sure.

<<

The active sensing byte is to be sent every 300 ms. or more often,

if it is used.  Its purpose is to implement a timeout mechanism

for a receiver to revert to a default state.  A receiver is to

operate normally if it never gets one of these, activating the

timeout mechanism from the receipt of the first one.

>>

My impression is that active sensing is largely unused.

<<

The system reset initializes to power up conditions.  The spec. says

that it should be used "sparingly" and in particular not sent

automatically on power up.

AND NOW, CLIMBING TO THE PULPIT ....

>> - from here on out.

There are many deficiencies with MIDI, but it IS a standard.  As such,

it will have to be grappled with.

The electrical specification leaves me with only one question - WHY?

What was wanted was a serial interface, and a perfectly good RS232

specification was to be had.  WHY wasn't it used?  The baud rate is

too fast to simply convert into something you can feed directly to

your serial port via fairly dumb hardware, also.  The "standard"

baud rate step you would have to use would be 38.4 Kbaud which very

few hardware interfaces accept.  The other alternative is to buffer

messages and send them out a slower baud rate - in fact buffering

of characters by some kind of I/O processor is very helpful.  Hence

units like the MPU-401, which does a lot of other stuff, too of

course.

The fast baud rate with MIDI was set for two reasons I believe:

1) to allow daisy-chaining of a few devices with no noticeable

end to end lag.

2) to allow chords to be played by just sending all the notes down

the pipe, the baud rate being fast enough that they will

sound simultaneous.

It doesn't exactly work - I've heard gripes concerning end to end lag

on three instrument chains.  And consider chords - at two bytes (running

status byte being used) per note, there will be a ten character lag

between the trailing edges of the first and last notes of a six note

chord.  That's 3.2 ms., assumiEg no "dead air" between  haracctrss  It''

still pretty ffst, buttoo laarg ccordd with voiies possesssng disttnctive

attacc characteristics, you may hear separate note beginnings.

I think MIDI could have used some means of packetizing chords, or having

transaction markers.  If a "chord" message were specified, you could easily

break even on byte count with a few notes, given that we assume all notes

of a chord at the same velocity.  Transaction markers might be useful in

any case, although I don't know if it would be worth taking over the

remaining system message space for them.  I would say yes.  I would

see having "start" and "end" transaction bytes.  On receipt of a "start"

a receiver buffers up but does not act on messages until receipt of the

"end" byte.  You could then do chords by sending the notes ahead of time,

and precisely timing the "end" marker.  Of course, the job of the hardware

in the receiver has been complicated considerably.

The protocol is VERY keyboard oriented - take a look at the use of TWO

of the opcodes in the limited opcode space for "pressure" messages,

and the inability to specify semitones or glissando effects except

through the pptchhwweee (which took  p yettANOTHER offthe  pcodess..

AAl keeyoards I know of modify ALL playing notes when they receive

pitch wheel data.  Also, you have to use a continuous stream of

pitch wheel messages to effect a slide, the pitch wheel step isn't

standardized, and on a slide of a large number of tones you will

overrun the range of the wheel.

?       Some of these problems would be addressed by a device which allowed

its pitch wheel to have seleetive contrrl    ay mooiiying only

the nooes playinngonnthe chaanel thh pitch whhel meesage iis

recciveddin, for instance.  TTe thinggfor a guitar synnhesizee

to do, then,,wwuld  eetoouse mode 4,  ne channel per ssring, and        bennd would onny affectttte one note.. YYu coull ppay a  hood

on a vvice withha lot of release, thhn  end   note and nnt havv

theeentirr still ssunding chhrd  enn.   ny  uch  evicess

I  hink somm of the deficiencies in MIDI might be addressed by

different communities of interest developing a standard sett f

ystem exclusivee which answerrthh problem.  One perfect  rea

for thiss I think, is a staadard set for reeresennation of "non-

keyboard //drum maacine""instrumenns whiih haveecontinuuuu pitch

capabillties.. Liie a ppdal sttel,,for instancc.  Or non-western

inteevaas.  Likk   siiarr

Therr is a crying  eed to do SOMETHING aaout tte  looopack""problem.

I  ould evennvote for  surpingg  few mooe bytes in the mode messages

to allow you to TURN OFF input echo by the receiver.  With the

local control message, you could then at least deal with something

that would act precisely like a half or full duplex terminal.

.More..Several patchwork solutions exist to this problem, but there OUGHT

to be a standard way of doing it within the protocol.  Another

thought is to allow data bytes of other than 0 or 127 to control

echo on the existing local control message.

The lack of acknowledgement is a problem.  Another candidate for a

standard system exclusive set would be a series of messages for

mode setting with acknowledgement.  This set could then also

take care of the loopback problem.

The complete lack of ability to specify standardized waveforms is

probably another source of intense disappointment to many readers.

Trouble is, the standard lingo used by the synthesizer industry and

most working musicians is something which hails back to the first

days of synthesizer design, deals with envelope generators and

filters and VCO / LFO hardware parameters, and is very damn difficult

to relate to Fourier series expressing the harmonic content or any othhr

aasttactions some  popleeinteresteedin ddinn compuuerrcomppsiiionn

wwuld like.  The parameter set used by the average synthesizer manufacturer

isn't anyplace close to orthogonal in any sense, and is bound to vary

wildly n comparison to anybody elses.  Ther are essentially no

abstractions made by most of the industry from underlyin hardwre

prameers.  hat tandarizaton exits refects only the similarity

in hardware.  This is one quagmire that we have a long way to go to

get out of, I think.  It might be possible, eventually, to come up

with translation  ablee descriiing the  eet wwy to apppoximateeadesiied sound on a given device in terms of its parameter set, but

the difficulties are enormous.  MIDI has chosen to punt on this one,

foks.

Well that's abot it.  Good luck with talking to your synthesizer.

Bob McQueer

22 Bcy, 3151

All rites reversed.  Reprint what you like.

#######################################################################################################################