A few quick notes on Junovoic.bas.

The program was written for a Roland Alpha-Juno 1.  It should

work on a Juno-2 and an MKS-50 although it has not been tested.

The program is not intended to be a full blown editor/librarian.

I originally wrote it to only display all of the parms for a patch

on the screen simultaneously.  I later added the file save/retrieve,

patch print, and directory options for convenience.

Obviously the program has some limitations.  It views a group

of Juno voices as 8 banks of 8 cells (numbers on the synth) just like

the Juno.  I had trouble overflowing the midi buffer when i tried a

bulk dump so the program receives voices one at a time.  The program

does not transmit any data to the Juno and uses the front panel of the

Juno as a transmission control in order to receive the voice parms.

The following tips should help.

1) Be sure to set MIDI EXCL=ON on the JUNO.

2) When selecting option 1 from the menu

a)   The Juno transmits the patch data as soon as you

press a select button on its front panel.  The program,

therefore pauses before every bank, including the first

one, to allow you to set the bank.  For instance,

after selecting option 1 the program prompts you to hit

enter.  Be sure to set to bank 1 before hitting enter.

After the prompt is gone, select cells (numbers) 1 - 8

in the ascending sequence.  After selecting 8, the

program again prompts you.  Select bank 2, hit enter, and

select the cells as described above.  When you have

finished with bank 8 the program will return you to the

main menu.

b)   Option 2 allows you to receive only 1 voice instead of the

entire group.  With this option, the program needs to know

where to place the voice in the 8 by 8 group.  It will

prompt you for bank and cell, then wait for you to select

the voice from the Juno's front panel.  Again, be sure to

position the Juno before responding to the prompts.

3) The program prints and displays the envelope parms numerically.

The Juno has graphic representations for these parms.  I used

the midi data value, 0 thru 3, in the same order as the options

appear on the Juno.  For example, the second DCO ENV parm has

four options for you to choose.  The first is option 0, etc.

If you have any comments about this program please let me know.

There are many modifications to be made to make this shell more

useful.  I request, for posterities sake, that if you modify

Junovoic.bas, you upload it as a new version, document your changes,

and leave my name as well as yours in the code so that we all may

share in the feedback.

Good luck,

Loren Claypool