SONGDUMP.TTP songdump.c

Dump a music studio song, playing the notes as they occur.

The source is provided so a true player can be made, or a better dumper or

converter for different sequencer formats.  Read the source for some details

which will not be documented here.

Song dump:

key:xx,				key signature

tempo, quarter note = n,	tempo marking (M.M.)

time:x/x,			time signature

loud=xx				loudness, pp 15-127 ff

n*[	begin n times repeat

]	end repeat

|	bar

{instrument:note(marks) duration(marks)}

note is not pitch, if no incidentals #,(nat),b adjust for key.

(	begin tie

)	end tie

-	rest

>	accent

.	dotted note

(3)	triplet

lines correspond to vertical columns in the normal display

Things that don't work well (yet), mainly in the player

The pause does not compensate for processing time, so 32nd notes will

go slower.

Ties aren't supported, it sounds as discrete notes.

accents are not supported

all notes are quieted at the end of the bar's duration, so if it has

both a half and a quarter note, it will cut the half note off when

the quarter note ends.

I am using my own pause routine compatible with Micro RTX from

Beckemeyer Development Tools

---------------------------------------------------------------------------

Song file structure

512 byte header structure:

10 byte header CD,'Mstudio',CD,02

15*10 byte atari instrument names (null terminated)

15*8 byte atari instrument settings

15*10 byte casio instrument names (null terminated)

15 bytes casio settings

15 bytes casio settings

5 pointers (file displacement) to lyric lines

32 byte null terminated title string

after the 512 byte header (hexadecimal unless noted):

track 1-4 bitmap (4 * 1 word)

instrument active, [15:14:13...2:1:track enable]

00:80:01:key

1= C G D A E B F# C# F Bb Eb Ab Db Gb Cb =15

83:time

1= 2/2 3/2 2/4 3/4 4/4 5/4 6/8 =7

81:tempo

quarternote = tempo, 57..200

84:01:volume

0F pianissimo .. fortissimo 7F

music, 00 delimits beginning of column data, 0xff marks end of music info

82

bar

85:number

repeat number of times

86

end repeat

note format

0 : tie start : tie end : rest(not note) : [instrument:4]

[0=inkey 1=nat 2=sharp 3=flat :2] : accent : [duration:5]

pitch in C major midi coding (18(24d)=C2)

duration: 12d = quarter note, (+2 for dotted(*3/2) , -2 for triplet (*2/3) )

9d = 1/8th , 6d = 1/16th  15d = half, 18d = whole, etc.

pitch is always the natural note, later adjusted for key, or explicit sharp,

natural, or flat.

d means decimal.  all other numbers in hex.