GRIDFIND.EXE

by

LTC Ray Andrews, CAP

HQ, Great Lakes Region

GRIDFIND is intended to be an aid in ELT-type missions.  It will  take  a

set  of  latitude-longitude coordinates and determine the sectional  and  grid

number within that sectional where the target is located.  It then displays  a

graphic  representation  of  the grid and marks the  location  of  the  target

coordinates within the grid.

GRIDFIND  can  make use of 2 optional data files to  enhance  the  information

displayed.   These   2 files are named VOR.DAT and AIRPORT.DAT.   The  VOR.DAT

file  contains a list of all VOR's in the area of interest.  If this  file  is

present,  the program will calculate and display the distance and  bearing  of

the  target coordinates from the 3 closest VOR's.  The AIRPORT.DAT file  is  a

list  of  all airports in the area.  If this file is  present,  GRIDFIND  will

display  a  list  of  all airports within a specified  radius  of  the  target

coordinates.

STARTING GRIDFIND

The program is entered by typing GRIDFIND followed by up to 3 optional command

line parameters.  These parameters answer the 3 questions normally asked  when

the  program is started.  You can answer any or all three of the questions  by

use of the command-line parameters.  The parameters must be separated from the

word  GRIDFIND  and from each other by one or more spaces.   The  command-line

parameters may be in either upper or lower case.

When GRIDFIND is started, it takes the following actions:

1.)  Displays a sign-on message.

2.)  Looks for and loads the VOR.DAT and AIRPORT.DAT files, if found.

3.)  Displays some further information about the program.

4.)  Asks  for  the distance unit of measurement,  the  maximum  airport

distance, and whether a printed copy is desired or not.

Distance  Unit  of Measurement -- The program is capable  of  calculating  and

displaying  distances  in statute miles, nautical miles, and  kilometers.   If

either  the VOR.DAT file or the AIRPORT.DAT file is present, the program  will

ask  which  unit  of  measurement you wish to use.   Answer  the  question  by

pressing the 'S' key for statute miles, the 'N' key for nautical miles, or the

'K' key for kilometers.  The unit of measurement may be selected by placing an

's', 'n', or 'k' on the command line.

Maximum  Airport Distance -- If the AIRPORT.DAT file is present,  the  program

will  ask  you to specify a maximum distance.  If an airport  is  within  this

distance  from  the  target coordinates, it will be included in  the  list  of

airports.  The distance must be a whole number between 1 and 99.  The distance

may be specified by means of a command-line parameter.

Printed  Copy -- If desired, the program will generate a printed copy  of  all

information  entered  or displayed.  Press the 'Y' key if you wish  a  printed

copy; press the 'N' key if you do not.  This question may also be answered  by

means of a command-line parameter.  Place a 'p+' on the command line if wish a

printed copy; place a 'p-' on the command line if no printed copy is desired.

ENTERING COORDINATES

First, you will enter the target latitude.  The program will first ask you for

the degrees, then the minutes, and then the seconds.  If you wish to enter the

latitude  in degrees and a decimal fraction of a degree, type a  period  after

the  degrees and follow it with the fractional part.  If you type a period  as

part  of  the degrees, the program will not ask for the minutes  and  seconds.

Similarly, if you wish to enter degrees, minutes, and a fraction of a  minute,

type  a period after the minutes and follow with the fractional part; and  the

program will not ask you for the seconds.

The target longitude is entered in the same way.

If you enter a value of 0 degrees for either the latitude or the  longitude,

or if you press the 'ESC' key, the program will exit and return you to DOS.

VOR.DAT FILE

The VOR.DAT file is an ASCII file with the data for one VOR on each line.

The file may contain a maximum of 128 VOR's.  If more VOR's are placed in  the

file, the program will only use the first 128; all others will be ignored.

Each  line  consists  of  six (6) fields separated by  commas.   The  line  is

terminated by a carriage return, or a carriage return and a line feed.

Example -- IND,Indianapolis,39,48.9,86,22.1

The 1st field is the 3-letter identifier for the VOR.

The 2nd field is the name of the VOR.

The  3rd & 4th fields are the latitude of the VOR in degrees (3rd  field)  and

minutes (4th field).

The  5th & 6th fields are the longitude of the VOR in degrees (3rd field)  and

minutes (4th field).

AIRPORT.DAT FILE

The  AIRPORT.DAT file is an ASCII file with the data for one airport  on  each

line.

The file may contain a maximum of 2048 airports.  If more airports are  placed

in  the  file, the program will only use the first 2048; all  others  will  be

ignored.

Each  line  consists  of eight (8) fields separated by commas.   The  line  is

terminated by a carriage return, or a carriage return and a line feed.

Examples -- Ace Airpark,Anderson,40,02,45.0,85,40,00.0

(H) Dammon,Battleground,40,30,23.0,86,51,46.0

(S) Lake Monroe,Bloomington,39,03,31.0,86,26,46.0

The 1st field is the name of the airport.

The 2nd field is the name of the city where the airport is located.

The  3rd,  4th, & 5th fields are the latitude of the airport in  degrees  (3rd

field), minutes (4th field), and seconds (5th field).

The  6th, 7th, & 8th fields are the longitude of the airport in  degrees  (6th

field), minutes (7th field), and seconds (8th field).