TRACK 2 READER UNIT

Assembly instructions:

Contents:

Magnetic strip reader unit

PCB prototype board

7805 +5 volt regulator

7 pin right angle header connector

9 volt battery snap

100 uF capacitor

DB-25 Connector with cable

Program disk

1) Notice that the 7 pin connector interfaces with the female connector

on the card reader unit. Notice that the card reader female connector

only has 5 holes. Break off two of the pins on the 7 pin connector.

2) Position the 5 pin connector so that it is a single row of holes away

from the edge of the pc-board. Be sure that the pins curved at a 90

degree angle are the pins which go into the board. Solder all

5 pins into position.

3) Examine the wires at the end of the DB-25 cable. Feed the wires through

the holes on the pc-board in the row behind the 5 pin connector

with each wire corresonding to the proper pin.

Solder them to the 5 pin connectors.

4) Feed the 9V battery snap wires through the holes indicated in the

diagram. Solder into position. Also position the +5 volt regulator and

the capacitor as shown. Be sure that the black wire and the minus wire

on the capacitor are postioned in the correct direction.

Track 2 Pinouts

Card Reader                       DB-25 connector (Parallel Port)

-----------------------------------------------------------------------------

1       Black   Ground          	<---->     23   Green      Ground

2       Green   Strobe          	<---->     10   Black      Acknowledge

3       Yellow  Card Present    	<---->     12   Red        Paper Empty

4       Blue    Data            	<---->     13   Yellow     De/Select

5       Red     +5 Volts

/--->     11              Busy

\--->     24              Ground

Summary:

5 pin Connector        DB-25

pin 1                  Green

pin 2                  Black

pin 3                  Red

pin 4                  Yellow

pin 5

Note: The Busy to ground is shorted within the hood of the DB-25 connector.

There is no need to short it out as it has already been done!

5) Bend the capacitor minus wire, voltage regulator center wire, and

black battery snap wires together and solder.

6) Bend the voltage input regulator wire to meet the red battery snap wire.

Solder together.

7) Bend the voltage regulator output wire and capacitor wire to meet

"pin 5" on the 5 pin connector. Solder together.

8) Feed the jumper wire through the holes designated on the diagram.

Bend one end to meet with the black battery snap wire and solder.

Then bend the other end to meet with pin 1 on the connector and solder.

8) Plug the magnetic strip connector on to the 5 pin connector. Make sure

that you do not plug it on UPSIDE DOWN! The correct direction is when

the four ridges on the connector are facing up.

9) Plug the DB-25 connector to the printer port of your PC.

10) Boot the CARDOMATIC disk. Run cardomatic by typing "CARDOMAT" at the

A:> prompt.

11) Press "R" and then swipe a magnetic card through the reader.

12) If the program does not respond, hit "p" then "TAB" down do the lpt: port

settings and change the port number to the one corresponding to your

computer setup. Read the CARDOMATIC help file for more information.

For track2 readers, the pattern for decoding is likely to be

ANSI5B.