New Season software

--------------------

Season 95 is a derivate from the season9.exe, which initially appeared

at utelscin.el.utwente.nl. Ever since, the combination between season9

and the new Pace-Receivers showed up critical, eventually locking up the

decoder after about 5 minutes.

Finally, while logging the decoder-/card data traffic with a

piratecard, I found a solution, which I implemented into this version.

This version does not cause brief "blinks" in 5 minute-intervals, as

some other Pace-fixes do.

If you start season95 without any parameters, season95 should behave

the way you used to be. If you run season95 with a new Pace-Decoder, you

have to specify another timing (wc) to avoid decoder lockups. Values

around 150000 have been reliable in conjunction with a 386DX-33 Notebook

and a Pace MSS1000. When you specified this timing, you will notice two

things:

- The computer screen will alternativly display 'RESET' and 'Decrypting

xxx (xx xx xx)'. If you read a stable 'Decrypting...'-message, increase

the value in wc.

- The decoder onscreen messages won't disappear. This can easily be fixed

by using season's m-option to supress any onscreen displays.

Starting season95 with a Pace-Decoder should therefore look like this:

'season95 1 wc150000 m'   (the 1 is for the com port)

There may be problems, when the interface's power is supplied by the

decoder; if this version doesn't work, when there's a value for wc set,

use an external 5 volt power supply to the interface.

Post any suggestions please to alt.satellite.tv.europe

07-Aug-95

Max