[ netinfo/x25.doc ]

ACKNOWLEDGMENTS

This specification was prepared by BBN Communications Corporation

under contract to the Defense Data Network Program Management

Office of the Defense Communications Agency.

The specification has been reviewed by the Defense Communications

Engineering Center for accuracy and completeness.  The draft of

this specification has been disseminated to industry by the

National Bureau of Standards for review and comments which have

been incorporated in the final specification.  This specification

has been approved for use on the Defense Data Network by the DoD

Protocol Standards Steering Group.

Comments on this specification should be directed to the Defense

Communications Agency, ATTN: Defense Data Network Program Managment

Office, Code B610, Washington, D.C.   20305

Table of Contents

1  INTRODUCTION.......................................... 1

1.1  Background.......................................... 1

1.1.1  X.25 and FIPS 100/Federal Standard 1041........... 1

1.1.2   X.25-to-X.25 and X.25-to-1822

Interoperability................................ 2

1.2  Compliance.......................................... 4

1.2.1   Compliance With CCITT X.25 and FIPS

100/Fed. Std. 1041.............................. 4

1.2.2  DTE Compliance With This Specification............ 4

2  INTERFACE SPECIFICATION............................... 6

2.1  Call Establishment Conventions...................... 6

2.1.1  Addressing........................................ 6

2.1.1.1  Address Formats and Fields...................... 6

2.1.1.1.1  Reserved...................................... 7

2.1.1.1.2  Flag.......................................... 7

2.1.1.1.3  DDN host Identifier........................... 7

2.1.1.1.4  Sub-Address................................... 7

2.1.1.2  Supplying Missing Address Information........... 7

2.1.2  DDN-Specific Facilities........................... 8

2.1.2.1  Type of Service Selection....................... 8

2.1.2.2  Call Precedence................................. 9

2.1.3  Protocol Identification.......................... 10

2.1.4  Logical Channel Assignment....................... 10

2.2  Packet Level Procedures............................ 11

2.3  Link Level Procedures.............................. 12

2.3.1  Link Level Parameters and Options................ 12

2.3.2  Timer T1 and Parameter T2........................ 12

2.3.3  Maximum I Frame Size............................. 13

2.4  Physical Level Specifications...................... 14

3  BIBLIOGRAPHY......................................... 16

APPENDICES

APPENDIX A:  DDN X.25 Implementation Details............ A-1

A-1  Introduction...................................... A-1

A-2  Operational Features of DDN X.25 DCE Releases..... A-1

A-2.1  Initial Feature Support......................... A-1

A-2.2  Exception-Handling Procedures................... A-2

A-2.2.1  Non-Octet-Aligned Data........................ A-2

A-2.2.2  RESTART REQUEST Packet........................ A-2

A-2.2.3  RESET REQUEST Packet.......................... A-2

A-2.2.9  CLEAR REQUEST Packet.......................... A-3

A-2.3  Virtual Circuit Resource Availability........... A-3

A-3   Detailed Features and Facilities

Specifications.................................. A-3

A-3.1  Additional Diagnostic Codes..................... A-3

A-3.2  X.25 IP Interoperability Considerations......... A-6

A-3.3  The DDN Logical Addressing Facility............. A-7

A-3.3.1  Logical Addresses............................. A-7

A-3.3.2  Enabling and Disabling Logical Addresses...... A-7

A-4  Limitations of DDN Basic X.25 Service............. A-8

A-5  Derivation of DDN X.25 Addresses.................. A-9

APPENDIX B: DDN Synchronous Level 1 Specification...... B-1

B-1  Introduction...................................... B-1

B-2  Supported Interfaces.............................. B-1

APPENDIX C:  Federal Information Processing Standard

Publication 100...................................... C-1

TABLES

DDN X.25 Address Fields................................... 7

"Derivation of Maximum I Frame Size".................... 14

DDN X.25 Physical Signaling Rates and Interfaces......... 15

Additional Packet Level Diagnostic Codes................ A-4

IP Precedence to X.25 Precedence Mapping................ A-6

EIA and CCITT Interchange Circuits...................... B-3

Signal Selection by CCITT Interchange Circuit

Number................................................ B-4

Typical Level 1 Connection Schemes...................... B-5

Interface Type by Service Speed......................... B-7

RS-232-C Interface...................................... B-8

MIL-188-114 Interface (and equivalents)................. B-9

V.35 Interface......................................... B-10

FIGURES

Typical Level 1 Connection Schemes...................... B-4

INTRODUCTION

This report specifies the attachment of an X.25 host to the

Defense Data Network (DDN).  In particular, this report describes

specific options and features of CCITT Recommendation X.25 (1980)

and Federal Information Processing Standard (FIPS) 100/Federal

Standard (Fed.  Std.) 1041 (July 1983) required of a host X.25

implementation to enable that host to communicate with a DDN X.25

Interface Message Processor ("IMP", the DDN packet switching

node).  This report, in conjunction with FIPS 100/Fed. Std.

1041, should enable DDN host site managers and others planning to

attach a host by means of X.25, rather than the 1822 interface,

to determine, first, whether or not the X.25 implementation of

the host in question is adequate for operation with DDN, and,

second, what options, parameter settings, etc. must or may be

selected for operation with DDN.

This report assumes that the reader is familiar with CCITT

Recommendation X.25 and FIPS 100/Fed. Std. 1041.  A copy of FIPS

100/Fed. Std. 1041 is attached as Appendix C of this report.

In this document, the term "Administration" refers to the

Defense Communications Agency (DCA Code B610, Washington, D. C.

20305).

1.1   Background

1.1.1 X.25 and FIPS 100/Federal Standard 1041

The CCITT Recommendation X.25 describes the interface

between host computers (data terminal equipment, or DTEs) and

data   circuit-terminating   equipment   (DCEs,   which    effect

communication with remote hosts over computer networks) for hosts

operating in the packet mode on public data networks.  The X.25

interface standard is defined as three independent architectural

levels, following the Open Systems Interconnection (OSI)

Reference Model.  The three levels are:

Level 1:  The PHYSICAL level of the connection.  The

physical,    electrical,    functional,   and

procedural characteristics to activate,

_____________

* As used in this report, "1822 interface" refers to the

interface specified in Bolt Beranek and Newman Inc. (BBN) Report

No. 1822, "Specification for the Interconnection of a Host and an

IMP," revision of December 1981.

-1-

maintain, and deactivate the physical link

between the DTE and the DCE.

Level 2:  The LINK level of the connection.  The link

access procedure for data interchange across

the link between the DTE and the DCE.

Level 3:  The PACKET level of the connection.  The

packet format and control procedures for the

exchange of packets containing control

information and user data between the DTE and

the DCE, and between the DTE and a remote

DTE.

CCITT Recommendation X.25 contains many options and

implementation choices.  FIPS 100/Fed. Std. 1041, which specifies

the general use of X.25 for the Federal Government, defines some

of the choices left open in X.25.  This document describes the

X.25 interface to a particular network, DDN.  Thus in several

areas where X.25 allows a choice, a single choice appropriate for

DDN is specified; in areas which X.25 leaves unspecified,

addressing in particular, conventions are specified that are

consistent with the overall architecture of DDN and the

interoperability goals described below.  The effect of this

approach is to make DDN service available to hosts in a way that

requires no changes to a host DTE implementation that is

compliant with FIPS 100/Fed. Std. 1041 and CCITT Recommendation

X.25.      By   implementing   extensions  described   in   this

specification, a host will be able to take advantage of

additional DDN features required in military networks, such as

precedence and logical addressing.

The reader is referred to CCITT Recommendation X.25 and to

FIPS 100/Fed. Std.  1041 for detailed information not provided in

the body of this document.

1.1.2 X.25-to-X.25 and X.25-to-1822 Interoperability

A   key   goal   of   the   DDN   X.25   implementation   is

interoperability among all DDN subscribers.  That is, effective

communication should be possible, not only between subscribers

attached   to   the  DDN  using  identical  vendor-supplied  X.25

implementations, but between subscribers using different X.25

implementations, and between a subscriber using an X.25 interface

to the DDN and a subscriber using an 1822 interface to the DDN.

Achieving this goal of interoperability requires that all DDN

-2-

X.25 subscribers conform to this interface specification and

implement the DoD standard higher level protocols.  True

interoperability among DDN hosts requires, in particular,

implementation of the DoD standard protocols TCP (Transmission

Control Protocol) and IP (Internet Protocol), as well as the

higher-level protocols which implement DDN standard services,

" when such services are provided by the host:  the Telnet Protocol

for character-oriented terminal support, the File Transfer

Protocol (FTP) for file movement between hosts, and the Simple

Mail Transfer Protocol (SMTP) for communication between

electronic mail service hosts.

The DDN X.25 DCE offers two types of service to X.25 DTEs:

1.   DDN Standard X.25 Service, which, when used in

conjunction with DoD standard protocols, provides

interoperable communication between an X.25 DTE

and other DDN hosts that also implement the DoD

standard protocols, whether they are connected to

DDN via the 1822 interface or via the X.25

interface;

and

2.   DDN  Basic   X.25   Service,    which    provides

communication only between an X.25 DTE and other

DDN X.25 DTEs implementing compatible higher-level

protocols.

Section 2.1.2.1 of this report describes the conventions to

be used by a DTE to specify the type of service desired for each

X.25 virtual call.  All DDN X.25 DTEs will be required to develop and

initiate a plan to use the DoD standard protocol architecture and DDN

standard X.25 service.

Use of DDN basic X.25 service imposes some restrictions on the

nature of the network communications service that a host can obtain.

These restrictions are discussed in Appendix A, Section A-4.

-3-

1.2 Compliance

1.2.1 Compliance With CCITT X.25 and FIPS 100/Fed. Std. 1041

The DDN X.25 Interface Specification is compliant with CCITT

Recommendation X.25 and FIPS 100/Fed. Std.  1041.  The DDN X.25 DCE

supports all facilities specified as E (essential) by FIPS 100/Fed.

Std.  1041, and no facilities specified as A (additional).  The

additional facilities not supported are:

(i) datagrams and associated facilities,

and

(ii) bilateral closed user groups.

In that FIPS 100/Fed. Std. 1041 describes features for a DCE,

DDN X.25 DTEs may support any or all facilities specified as either E

or A by FIPS 100/Fed Std. 1041.  However, DDN X.25 DTEs must not use

the facilities identified above that are not supported by the DDN X.25

DCE.

1.2.2 DTE Compliance With This Specification

This document specifies several areas in which the DDN X.25 DCE

is capable of operating in several modes.  For example, Section 2.4

lists a number of signaling rates supported by the DCE.  In such

cases, a DDN X.25 DTE must implement at least one of the options

listed (or the set of options required of a DTE by FIPS 100/Fed.  Std.

1041) but need not implement all of the options listed (unless

required by FIPS 100/Fed.  Std.  1041).  Determining the adequacy of

the options supported by a DTE vendor for meeting a DDN subscriber's

requirements is the responsibility of the subscriber.

In addition to the CCITT X.25 and FIPS 100/Fed.  Std.  1041

requirements described in Section 1.2.1 above, DDN X.25 DTEs may wish

to take advantage of additional DDN-specific features that are

compatible extensions to the public standards.  Implementation of a

DDN-specific feature by a host is required only if the host wishes to

take advantage of the service or information provided by the feature.

For example, a host that wishes to establish calls only at the default

precedence level assigned to it need not implement the precedence

facility described in Section 2.1.2.2.  However, a host that wishes to

have flexibility in the precedence of the calls it establishes must

implement this facility.

-4-

Any deficiencies with respect to this specification in a

vendor-supplied X.25 DTE implementation contemplated for use with the

DDN X.25 DCE should be rectified so as to attain compliance with this

specification.  Proper operation with DDN of an X.25 DTE that is not

compliant with this specification cannot be guaranteed and should not

be attempted.  To this end, a test program is available through the

Administration.

-5-

2 INTERFACE SPECIFICATION

2.1 Call Establishment Conventions

This section specifies DDN X.25 call establishment conventions.

2.1.1 Addressing

DDN addresses are assigned to subscriber DTEs by the

Administration.  Two basic forms of address are provided: physical

addresses, which correspond to the node number and DCE port number of

the node to which the DTE is connected, and logical addresses, which

are mapped transparently by DCE software into a corresponding physical

network address.  Each DTE is assigned one physical address, and may

be assigned one or more logical addresses.  All DDN addresses are

either twelve or fourteen BCD (binary-coded decimal) digits in length.

A calling DTE need not determine whether a given address is a physical

or logical address, in order to establish a call to that address.

.2.1.1.1 Address Formats and Fields

DDN addresses have the following format:

ZZZZ F DDDDDDD (SS)

The various fields of the address are presented in Table 2.1 and are

explained below.

Length

Field Meaning (BCD digits)

ZZZZ Reserved (must be zero) 4

F Flag 1

DDDDDDD DDN Host Identifier 7

(SS) Sub-address (optional) 0 or 2

TOTAL 12 or 14

Table 2.1 DDN X.25 Address Fields

-6-

2.1.1.1.1 Reserved

The Reserved field corresponds to the DNIC field generally used

in public data networks.  Pending assignment of a DDN DNIC, this field

must be zero.

2.1.1.1.2 Flag

The Flag field is used to differentiate physical and logical

addressing.  The value zero indicates physical addressing, while the

value one indicates logical addressing.  A value of nine is used in

the setup of calls to enable and disable logical addresses; see

Appendix A, Section A-3.3.1.

2.1.1.1.3 DDN Host Identifier

The DDN Host Identifier is a seven-digit address, either logical

or physical, assigned to a subscriber DTE by the DDN Administration.

2.1.1.1.4 Sub-Address

The Sub-Address may be used by a DTE for any.purpose.  It is

carried across the network without modification.  Its presence is

optional.

2.1.1.2 Supplying Missing Address Information

The DDN X.25 DCE incorporates a mechanism to supply "missing"

address information in CALL REQUEST and CALL ACCEPTED packets received

from an attached DTE.  This mechanism is useful in DTE software

testing and physical address determination.

If a DTE sends a CALL REQUEST packet with no calling address

field, the local DCE will insert the physical calling DDN Host

Identifier with no subaddress field.  If a DTE sends a CALL REQUEST or

CALL ACCEPTED packet with either or both calling or called addresses

that contain F = zero and DDDDDDD = zero, the local DCE will replace

the DDN Host Identifier field (DDDDDDD) with the physical address of

the DTE.

-7-

DTE implementors are cautioned that use of this mechanism in

accepting calls to a DTE's logical address (See Appendix A, Section

A-3.3) can result in confusion on the part of the calling DTE and is

not advised.

2.1.2 DDN-Specific Facilities

Two DDN-specific features are requested by means of "private" or

non-CCITT facilities in CALL REQUEST and CALL ACCEPTED packets.  If

either or both of these facilities are requested in a CALL REQUEST or

CALL ACCEPTED packet, they must follow all CCITT X.25 facilities and

must be preceded by a single facility marker, two octets of zero.

2.1.2.1 Type of Service Selection

The DDN X.25 provides two types of service, DDN basic X.25

service and DDN standard X.25 service.  DDN standard X.25 service

provides only local DTE to local DCE support of the X.25 connection.

Data is carried via the network to its destination (using protocols

internal to the network), where it is delivered using the access

protocol of the destination host (i.e., either 1822 or DDN standard

X.25 service).  This access method is oriented towards DDN X.25 hosts

using the DoD standard TCP/IP higher level protocols.  No X.25

procedures change when using DDN standard X.25 service; however, the

significance of the procedures changes (see Appendix A, Section

A-3.2).  There is no end-to-end X.25-level acknowledgment or guarantee

of delivery of data packets with DDN standard X.25 service;

reliability of DDN standard X.25 service is provided instead by the

use of a reliable transport protocol.

DDN basic X.25 service provides end-to-end call management with

significance as described in CCITT Recommendation X.25 and FIPS

100/Fed. Std.  1041.  This access method is oriented towards hosts

that have existing higher level protocol implementations that require

reliable packet delivery at the network level.

Selection of DDN standard or DDN basic X.25 service must be made

on a call-by-call basis by the DDN X.25 DTE at the time of call setup.

To specify DDN standard X.25 service, a DTE must include in the CALL

REQUEST packet a facility two octets long, coded as follows:

00000100 00000001

-8-

If this facility is not specified, DDN basic X.25 service will be

provided.

2.1.2.2 Call Precedence

The precedence of a call is negotiated by an X.25 DTE by means

of a facility two octets long, coded as:

00001000 000000XX

where XX is the precedence, from 0 (lowest precedence) to 3 (highest

precedence).  If this facility is not used, the call will be

established at the subscriber's default precedence.

A DTE is not permitted to establish a call at a precedence level

higher than that authorized for that DTE by the Administration.  An

attempt to do so will result in the DDN X.25 DCE returning to the DTE

a CLEAR INDICATION packet with clearing cause 00001001, "Out of

order," with diagnostic code 194, "Requested precedence too high".

Calls of a lower precedence may be cleared by a DCE if DCE or

other network resources are required, or if access to the local or

remote DTE is required (for a call of higher precedence).  In this

event, a CLEAR INDICATION packet will be sent with the clearing cause

00000101, "Network congestion," and with a diagnostic code specifying

the reason for the preemption.  The diagnostic codes employed for this

purpose are 192, "Cleared due to higher precedence call at local DCE,"

and 193, "Cleared due to higher precedence call at remote DCE".

Similarly, an attempt to establish a call may be unsuccessful if

network resources are engaged in calls of higher priority than that

requested.  In this case, a CLEAR INDICATION packet will be sent with

the clearing cause 00001001, "Out of order," and with either

diagnostic code 192 or 193, as appropriate.

The diagnostic codes described in the preceding paragraphs are

DDN-specific diagnostic codes; additional information about these

codes may be found in Appendix A, Section A-3.1.

-9-

2.1.3 Protocol Identification

X.25 DTEs employing the DoD standard TCP/IP protocol

architecture must indicate this by means of the call user data field

of the CALL REQUEST packet.  The first octet of this field must be set

to 11001100 to identify the DoD standard protocol architecture.

Indication of the use of the DoD standard protocol architecture

is independent of the selection of DDN standard or DDN basic X.25

service by means of the facility specified in Section 2.1.2.1 above.

Therefore, a host employing the DoD standard protocol architecture and

using DDN standard X.25 service must include both the DDN standard

X.25 service facility and the call user data DoD standard protocol

identification in its CALL REQUEST packet.

A DTE using a protocol architecture other than the standard DoD

protocol architecture is free to use any call user data protocol

identification recognized by the DTEs with which it wishes to

communicate.  Identification of protocol architectures other than the

DoD standard architecture is not standardized or enforced by the

Administration.  Subscribers are cautioned, therefore, that conflicts

among various vendor-assigned protocol identifications may arise.

2.1.4 Logical Channel Assignment

The assignment of logical channels by the DDN X.25 DCE follows

the requirements and guidelines of FIPS 100/Fed. Std.  1041 and Annex

A of CCITT X.25.  Within the guidelines of CCITT X.25 Annex A, the

range of logical channel numbers assigned to permanent virtual

circuits, incoming, two-way, and outgoing virtual calls for DDN DCEs

is configured for each DTE attached to a DCE by the Administration.

DDN X.25 DTEs must follow the logical channel selection

requirements of FIPS 100/Fed. Std. 1041.

The number of logical channels available to a DTE is dependent

upon the configuration of the DCE to which the DTE is attached, and

upon the dynamic requirements placed upon other DCEs that share the

same DDN packet switching node.

-10-

2.2 Packet Level Procedures

DDN X.25 packet level procedures are as specified by FIPS

100/Fed.  Std.  1041 and CCITT X.25.  The following additional

information is provided:

1. The maximum window size that may be negotiated is

seven.

2. Modulo 128 packet level sequence numbering is not

supported.

3. Maximum packet sizes of 16, 32, 64, 128, 256, 512,

and 1024 octets may be negotiated.

4. The DDN X.25 DCE uses additional packet level

diagnostic codes, specified in Appendix A, Table

A-1.  DDN X.25 DTEs may, but are not required to,

make use of the information conveyed by these

codes.

5. The Qualifier bit (Q-bit) is passed transparently

by the DDN X.25 DCE in DDN basic X.25 service.

DTEs using DDN basic X.25 service may use the Q-

bit in any way that is consistent with FIPS

100/Fed. Std. 1041.

6. The DDN X.25 DCE implements the diagnostic packet.

It is sent under conditions specified in Annex D

of CCITT X.25.  The DTE is not required to act on

the information provided in diagnostic packets.

7. DTEs using DDN standard X.25 service must restrict

the maximum number of data bits in a complete

packet sequence to be no more than 8056.  This

ensures that the data from a packet sequence

transmitted by an X.25 host will fit within the

maximum 1822 message length limit upon delivery to

an 1822 host.  This restriction is necessary as

existing 1822 host implementations are not re-

quired to accept messages longer than 8063 bits. *

________________ * DTEs using DDN standard X.25 service will generally

be transmitting Internet Protocol datagrams, the length of which, by

convention, does not approach this limit.  Therefore, unless a

protocol other than the Internet Protocol is used with DDN standard

X.25 service, this is a technical restriction that will have no

practical impact upon the design of DTE software.  See Appendix A,

Section A-3.2.

-11-

DDN X.25 DTEs connecting to DDN through an X.25

Internet Private Line Interface (IPLI) must reduce

the maximum complete packet sequence length by an

additional 256 bits to allow for IPLI overhead.

2.3 Link Level Procedures

DDN X.25 link level procedures are as specified by FIPS 100/Fed.

Std.  1041 and CCITT X.25.  This section presents additional

information.

2.3.1 Link Level Parameters and Options

1.  The default value of K, the maximum number of

sequentially numbered I frames that the DCE will

have outstanding (unacknowledged) at any given

time, is seven.  A DDN X.25 DCE may be configured

on a per-DTE basis to provide optional values of K

from one to six.

2.  The default value of N2, the maximum number of

transmissions and retransmissions of a frame

following the expiration of the T1 timer, is

twenty.  This value can be changed to any value

from one to 200 as a DCE configuration parameter

on a per-DTE basis.

3.  The optional 32-bit FCS is not supported.

2.3.2 Timer T1 and Parameter T2

The period of the timer T1 used by the DDN X.25 DCE reflects

assumptions about the processing speed of the DTE.  The DCE assumes

that parameter T2, the response latency of the DTE to a frame from the

DCE, is no greater than 1/2 second.  Likewise, the DCE guarantees that

its parameter T2, the latency in responding to frames from the DTE, is

1/2 second for signaling rates of 19.2 Kb/s or slower, and 1/4 second

for faster links.

A lower bound for timer T1 may be computed to be 4X + T2, based

on the assumptions that:

* the link propagation time is negligible,

-12-

* the worst-case frame transmission time is X,

* timer T1 is started when a frame is scheduled for

output,

* each frame is scheduled just as transmission of

the previous frame starts,

* frames are not aborted, and

* each frame and its predecessor are of maximum

length Nl = 8248 bits (see Section 2.3.3 below).

As an example, for a signaling rate of 9.6 Kb/s, this yields X =

.86 sec.  If T2 is .5 sec., the total time for the DTE to respond in

the worst case should be 3.9 seconds.  In fact, the DCE uses a T1

timer value of 4 seconds for a link speed of 9.6 Kb/s.

In no case does the DCE use a value for T1 smaller than 3

seconds.  This means that, for faster links, the DTE's T2 parameter

may be lengthened because the X term in the above formula is smaller.

For links of 19.2 Kb/s or faster, DTEs are expected to satisfy latency

requirements that allow the DCE to use the formula 4X + T2 (DTE) < 3

seconds = T1 (DCE).

The DTE may choose any value for T1 that is compatible with the

DCE's T2 parameter values.  The value of T1 used by the DTE may always

be set longer than the formula indicates, with the result that

recovery from certain types of link errors will be slower.  However,

the DCE's parameter T2 cannot be reduced, so the formula should be

viewed as yielding a lower bound on the DTE's T1 timer.

2.3.3 Maximum I Frame Size

The maximum number Nl of bits in an I Frame is 8248,

accommodating a data packet with up to 1024 data octets.  The

derivation of this number is shown in Table 2.2.

DTEs using DDN standard X.25 service must observe the

restriction on the number of data bits in a complete packet sequence

given in Section 2.2 above.

-13-

X.25 No. of

Field Name Level Bits

Address 2 8

Control 2 8

General Format Identifier 3 4

Logical Channel Number 3 12

Packet Type 3 8

User Data 3 8192 (max)

Frame Check Sequence 2 16

TOTAL 8248 (max)

Table 2.2 Derivation of Maximum I Frame Size

2.4 Physical Level Specifications

The DDN X.25 physical level specification is in conformance with

FIPS 100/Fed.  Std.  1041 and CCITT X.25.  This section presents

additional information.

A DDN X.25 DTE may either be collocated with its DCE or may be

connected to it via an access line.  In all cases the DTE presents a

physical DTE interface; the DDN will supply the matching DCE

interface.  DDN X.25 service offers four physical level interfaces:

RS-232-C (CCITT V.28), RS-449, both balanced and unbalanced (CCITT

V.ll and V.10, respectively; also MIL-188- 114 balanced and

unbalanced), and CCITT V.35.  Appendix B of this document describes in

detail the choices of physical interface available to the DDN

subscriber and the specifications for each type of interface.  Table

2.3, below, summarizes the physical interfaces available at each data

rate supported by the DDN X.25 DCE, and indicates which interfaces are

recommended at each signaling rate.

A DDN X.25 DTE may implement any or all of the signaling rates

shown.  At each signaling rate implemented, the DTE must offer at

least one of the physical interface options listed as "R"

(recommended) or "A" (available) for that rate in Table 2.3.

Implementors are encouraged to offer the widest variety of signaling

rates and physical interfaces practical to maximize the ease of use of

their equipment in DDN.

-14-

Physical Signaling Rate in Kb/s Interface 1.2 2.4 4.8 9.6 14.4 48 50

56 64 100

RS-232-C R R R R R - - - - -

RS-449 unbal.  A A A A - - - - - - (and equiv.)

RS-449 balanced A A A A A A A A A R (and equiv.)

CCITT V.35 - - - - - R A R R A

Legend

R = Recommended

A = Available

- = Not available

(Taken from Appendix B, Table B-4

Table 2.3 DDN X.25 Physical Signaling Rates and Interfaces

-15-

3 BIBLIOGRAPHY

1.  "Specification for the Interconnection of a Host and an IMP".

Report No.  1822, Bolt Beranek and Newman Inc" Cambridge,

MA, revision of December 1981.

2.  CCITT Recommendation X.25, "Interface Between Data Terminal

Equipment (DTE) and Data Circuit Terminating Equipment (DCE)

for Terminals Operating in the Packet Mode on Public Data

Networks," International Telegraph and Telephone

Consultative Committee Yellow food, Vol.  VIII.2, Geneva,

1981.

3.  "Defense Data Network Subscriber Interface Guide," Defense

Communications Agency, Washington, DC, July 1983.

4.  "Internet Protocol Transition Workbook," SRI International,

Menlo Park, CA, March 1982.

5.  "Internet Protocol Implementation Guide," SRI International,

Menlo Park, CA, August 1982.

-16-

APPENDIX A: DDN X.25 Implementation Details

A-1 Introduction

This Appendix serves three purposes.  First, it provides

information concerning the planned evolution of DDN X.25 capabilities.

Second, it provides information on the use of certain DDN X.25

features and facilities at a greater level of detail than is

appropriate for inclusion in the body of the DDN X.25 Interface

Specification.  Specifications for the use of DDN X.25 features and

facilities given in this Appendix are mandatory on the part of DDN

X.25 DTEs that wish to make use of these features and facilities.

Finally, this Appendix presents a discussion of the limitations on the

use of DDN services that will be encountered by hosts using only DDN

basic X.25 service.

A-2 Operational Features of DDN X.25 DCE Releases

The capabilities of the DDN X.25 DCE will evolve over time from

an initial set of capabilities to the full capabilities of this DDN

X.25 Interface Specification.  This section describes

release-dependent features of the DDN X.25 DCE.  Implementors should

note that not all optional facilities of the specification will

initially be available for use by DTEs.

Releases of new DCE capabilities will be compatible with DTE

hardware and software implementations that meet the full DDN X.25

Interface Specification.

A-2.1 Initial Feature Support

The initial release of the DDN X.25 DCE will support flow control

parameter negotiation and fast select.  In addition, the DDN X.25 DCE

may be configured by the DDN Administration to provide non-standard

default window and packet sizes as described in CCITT X.25 Sections

7.1.2 and 7.2.1.  The call precedence and type of service selection

facilities will be accepted, but not acted upon, by the network.  Only

DDN basic X.25 service will be supported.  Planned future DCE releases

will support all facilities specified in FIPS 100/Federal Standard

1041 with the exception of those "additional" facilities that are

listed in Section 1.2.1 of this document.

A-1

A detailed schedule of DDN X.25 DCE releases and the capabilities of

each release will be supplied in a separate document.

A-2.2 Exception-Handling Procedures

Certain of the exception- or error-handling procedures of the

initial release of the DDN X.25 DCE differ in detail from the

procedures specified in FIPS 100/Federal Standard 1041.  These

differences are described below.  A later release of the DDN X.25 DCE

will bring these procedures into conformance.  In the interim, the

variances in these procedures will not preclude satisfactory operation

between the DCE and a DTE, provided the DTE operates in accordance

with FIPS 100/Federal Standard 1041.

A-2.2.1 Non-Octet-Aligned Data

Data packets received by the DDN X.25 DCE that are not aligned on

an octet boundary are discarded at the link level.  They are not

passed to the DCE packet level, and no packet level diagnostic code is

returned to the DTE.

A-2.2.2 RESTART REQUEST Packet

The DDN X.25 DCE will not discard, but will instead act upon, a

RESTART REQUEST packet that

(i) is too long (unless it exceeds the maximum frame

size for the link level),

or

(ii) contains a non-zero cause field.

A-2.2.3 RESET REQUEST Packet

The DDN X.25 DCE will not discard, but will instead act upon, a

RESET REQUEST packet that contains a non-zero reset cause field.

A-2

A-2.2.4 CLEAR REQUEST Packet

The DDN X.25 DCE will not discard, but will instead act upon, a

CLEAR REQUEST packet that contains a non-zero clearing cause field.

A-2.3 Virtual Circuit Resource Availability

In its current implementation, the DDN X.25 packet switching node

is capable of supporting a minimum of one hundred simultaneous virtual

circuits.  As was discussed in Section 2.1.4, resources of the node

are shared dynamically among the DCEs attached to the node.

Therefore, no explicit guarantees are made of the number of

simultaneous virtual circuits that can be made by a single DTE.

Depending upon the configuration of the node, the number of

simultaneous circuits supported by the node can be significantly

greater than one hundred.

A-3 Detailed Features and Facilities Specifications

This section provides detailed specifications and descriptions of

use for certain DDN X.25 features and facilities.

A-3.1 Additional Diagnostic Codes

The DDN X.25 DCE is capable of providing additional information

to DTEs in RESTART, RESET, CLEAR INDICATION, and DIAGNOSTIC packets by

means of diagnostic codes that are extensions to the set of diagnostic

codes given in Annex E of CCITT Recommendation X.25.  These codes are

taken from the set of codes "reserved for network specific diagnostic

information," and are thus not in conflict with code assignments made

in Annex E.  The values of these codes, and their meanings, are given

in Table A-1 below.

A-3

Code Value Meaning

128 IMP is unavailable.  The packet-forwarding

mechanisms of the network are unavailable to the

DCE.  Sent in RESET, CLEAR and RESTART packets.

130 Link level came up.  Sent in RESTART and RESET

packets.

131 Link level went down at remote DTE.  Sent in CLEAR

and RESET packets.

132 Remote DTE restarted.  Sent in CLEAR and RESET

packets.

133 Local resources not available for call

establishment.  The local DCE has too few

resources to establish another call.  Sent in

CLEAR and DIAGNOSTIC packets.

134 Remote resources not available for call

establishment.  The remote DCE has too few

resources to establish another call.  Sent in

CLEAR packets.

136 Remote host dead.  The link to the remote DTE is

down.  Sent in CLEAR and RESET packets.

137 Remote IMP dead.  The IMP to which the remote DTE

is attached is down.  Sent in CLEAR and RESET

packets.

138 Logical subnetwork access barred.  The remote DTE

cannot be reached because of a communities-of-

interest prohibition.  Sent in CLEAR and RESET

packets.

139 Connection lost.  An internal error has occurred

at either the remote or the local DCE which has

made their virtual circuit data structures

inconsistent.  Sent in CLEAR and RESET packets.

140 Response lost.  A response from the remote DCE

failed to arrive within a reasonable time.  Sent

in CLEAR and RESET packets.

A-4

141 Calling logical address not enabled or not

authorized.  Sent in CLEAR packets.

142 Calling logical name incorrect for this DTE.  Sent

in CLEAR packets.

143 Called logical name not authorized.  Sent in CLEAR

packets.

144 Called logical name not enabled.  Sent in CLEAR

packets.

145 Called logical name has no enabled DTEs.  Sent in

CLEAR packets.

146 Use of logical addresses invalid in this network.

Sent in CLEAR packets.

147 Declared logical name now in effect.  Sent in

CLEAR packets.

148 Declared logical name was already in effect.  Sent

in CLEAR packets.

149 Declared logical name is now disabled.  Sent in

CLEAR packets.

150 Declared logical name was already disabled.  Sent

in CLEAR packets.

151 Incoming calls barred.  Sent in CLEAR packets.

152 Outgoing calls barred.  Sent in CLEAR packets.

192 Cleared due to higher precedence call at local

DCE.  Sent in CLEAR packets.

193 Cleared due to higher precedence call at remote

DCE.  Sent in CLEAR packets.

194 Requested precedence too high.  The DTE is not

authorized to establish a call at the requested

precedence level.  Sent in CLEAR packets.

Table A-1. Additional Packet Level Diagnostic Codes

A-5

A-3.2 X.25 IP Interoperability Considerations

When DDN standard X.25 service is requested at call

establishment (as described in Section 2.1.2.1), the call is in effect

established between the DTE and a local X.25 entity.  This entity

subsequently extracts the IP datagrams from the X.25 data packets for

transmission through the DDN Internet.  This approach requires that

certain conventions be followed:

1. IP datagrams are to be sent as X.25 complete

packet sequences.  That is, datagrams begin on

packet boundaries and the M ("more data") bit is

used for datagrams that are larger than one

packet.  Only one IP datagram is to be sent per

X.25 complete packet sequence.

2. By convention, the maximum IP datagram size is 576

octets.  This packet size can most efficiently be

accommodated by negotiating an X.25 maximum packet

size of 1024; alternatively, a DTE may use an X.25

complete packet sequence to transmit an IP

datagram.

3. Because the X.25 connection is in effect

terminated locally, the D and Q bits have no

significance and should be set to zero.

4. The precedence bits of the IP type-of-service

field are to be mapped into X.25 precedence bits

(see Section 2.1.2.2) as specified in Table A-2.

IP Precedence X.25 Precedence

000 00

001 01

010 10

011 - 111 11

Table A-2. IP Precedence to X.25 Precedence Mapping

A-6

A-3.3 The DDN Logical Addressing Facility

The DDN logical addressing facility allows references to hosts by

either their physical network address or by one or more

location-independent logical addresses, and allows hosts to exercise

partial control over the logical address(es) by which they can be

referenced.  Implementation of DDN logical addressing by a host is

optional.

The DDN Administration will assign seven-digit logical addresses,

and will maintain a logical addressing data base.  The host is then

responsible for notifying the network ("enabling") of the "names"

(logical addresses), if any, by which it wishes to be known.  It

cannot receive calls addressed to a name or originate calls under that

name unless it has enabled that name.  It also cannot enable a name

that is not authorized for that physical address.  Names can also be

enabled automatically by the network, under the control of the

Administration.

A-3.3.1 Logical Addresses

Logical addressing is invoked when a called address is supplied

to the IMP with the flag digit F = one.  The logical address consists

of seven BCD digits.  This name is mapped by the logical addressing

facility into a DDN physical network address.  The logical name need

not be unique for the physical address, nor is the physical address

necessarily unique for the name.

A-3.3.2 Enabling and Disabling Logical Addresses

To enable and disable logical addresses, the DDN X.25 host must

send declarative CALL REQUEST packets to the DCE using a called

address with the format:

ZZZZ F DDDDDDD (SS)

where the address fields are as described in Section 2.1.1.  The Flag

F must be set to nine, the DDN Host Identifier field specifies the

logical address under consideration, and the subaddress field, which

must be present, specifies the type of transaction.  Declarative calls

are cleared immediately by the local DCE.

A-7

If SS is zero, the logical name is enabled in normal mode,; that is,

that physical port will accept incoming calls to that name, and allow

outgoing calls from that name.  If SS is one, the logical name is

disabled.  If SS is two, the logical address is enabled in reverse

translation mode; in this mode, the called address field of incoming

call packets will be translated into a physical address (i.e., an

address containing a flag F = 0), if it was given by the calling DTE

(X.25 host), as a logical address (i.e., containing a flag F = 1).

Whenever a DTE comes up, or restarts, the logical names for that

DTE are returned to their default state, which may be either enabled

or disabled, as configured by the DDN Administration.

A-4 Limitations of DDN Basic X.25 Service

The Defense Data Network is an Internetwork environment.  That

is, DDN as a whole is made up of a number of constituent packet

switching networks that are interconnected via gateways.

Communication across gateways requires the use of the Internet

Protocol, which, for a host accessing DDN using X.25, requires that

the host implement the DoD standard protocol architecture and employ

DDN standard X.25 service.  In addition, a classified host is attached

to a DDN constituent network of lower classification by means of an

Internet Private Line Interface (IPLI).  IPLIs, which themselves

contain gateways, also require the use of the Internet Protocol;

moreover, they do not, as currently designed, offer an X.25 host

interface.  These attributes of the DDN Internet have two implications

for users of DDN basic X.25 service:

1.  DDN hosts that do not implement IP and higher-

level DDN protocols, and which use only DDN basic

X.25 service, cannot communicate across gateways.

Their network communication is therefore

restricted to a single DDN constituent network.

2.  X.25 hosts cannot be provided classified service

on a constituent network of lower classification.

Should X.25 host access be developed for the IPLI

in the future, classified network access will be

made available to hosts using DDN standard X.25

service only.

A-8

A-5 Derivation of DDN X.25 Addresses

All DDN hosts are assigned addresses by the Administration.

The address of a DDN host may be obtained from the Network

Information Center (NIC), represented as an ASCII text string in

what is called "host table format".  This section describes the

process by which DDN X.25 addresses in the format described in

Section 2.1.1 may be derived from addresses in NIC host table

format.

A NIC host table address consists of the ASCII text string

representations of four decimal numbers separated by periods,

corresponding to the four octets of a thirty-two bit Internet

address.  The four decimal numbers are referred to in this

section as "n", "h", "l", and "i."  Thus, a host table address

may be represented as "n.h.l.i" Each of these four numbers will

have either one, two, or three decimal digits and will never have

a value greater than 255.  For example, in the host table address

"10.2.0.124", n=10, h=2, l=0, and i=124.  To convert a host table

address to a DDN X.25 address:

1.  If h < 64, the host table address corresponds to

the DDN X.25 physical address

ZZZZ F IIIHHZZ (SS)

where:

ZZZZ = 0000

as required in Section 2.1.1.1.1;

F = 0 because the address is a physical

address;

III is a three decimal digit

representation of "i", right-adjusted

and padded with leading zeros if

required;

HH is a two decimal digit representation

of "h", right-adjusted and padded

with leading zeros if required;,

ZZ = 00

and

(SS) is optional, as described in Section

2.1.1.1.4.

A-9

In the example given above, the host table address

10.2.0.124 corresponds to the DDN X.25 physical

address 000001240200.

2.  If h > 64 or h = 64, the host table address

corresponds to the DDN X.25 logical address

ZZZZ F RRRRRZZ (SS)

where:

ZZZZ = 0000

as required in Section 2.1.1.1.1;

F = 1 because the address is a logical

address;

RRRRR is a five decimal digit

representation of the result "r" of

the calculation

r = h * 256 + i

(note that the decimal representation

of "r" will always require five

digits);

ZZ = 00

and

(SS) is optional, as described in Section

2.1.1.1.4.

Thus, the host table address 10.83.0.207

corresponds to the DDN X.25 logical address

000012145500.

In both cases, the "n" and "l" fields of the host table address

are not used.

A-10

APPENDIX B: DDN Synchronous Level 1 Specification

B-1 Introduction

A host may connect to the Defense Data Network at the link level

using the asynchronous bit serial protocol described in BBN Report No.

1822 as either a local host (LH) or a distant host (DH).  A host may

also connect to the DDN by means of a synchronous bit serial protocol

at the link level, using either the method described in BBN Report No.

1822, HDH, or the DDN X.25 interface.  Neither LH nor DH is

recommended for new implementations.

This section describes the functional, electrical, and mechanical

connection (the level 1 connection) that is required when either an

HDH or an X.25 host is connected to the DDN.  Hosts connecting to the

DDN via HDH or X.25 require a synchronous modem connection or the

equivalent, which will be supplied as part of the DDN service.  The

host will present the DTE interface while the DDN-provided equipment

will present the DCE interface.

A long-term goal of the DDN is for all level 1 connections to be

accomplished with the MIL-188-114 balanced interface.  Its general

equivalents are EIA RS-449/422, CCITT V.ll, and Fed. Std.  1031/1020.

The DDN cannot implement this at present due to the limited

availability of commercial vendor hardware.  In order to facilitate

future DDN compatibility, all new system acquisitions should specify

MIL-188-114 balanced as a required interface, in addition to an

alternate interface.  The selection of an alternate interface should

not preclude utilization of the MIL- 188-114 balanced interface when

it becomes supportable.

B-2 Supported Interfaces

DDN presently supports four synchronous level 1 interfaces.  They

are:

1.  EIA RS-232-C, CCITT V.28 & V.24;

2.  MIL-188-114 balanced, EIA RS-449&422, CCITT V.ll,

Fed. Std. 1031/1020;

3.  MIL-188-114 unbalanced, EIA RS-449&423, CCITT

V.10, Fed. Std. 1031/1030; and

B-1

4.  CCITT V.35.

Table B-1 is a dictionary of terms that relates the CCITT signal

ID to the EIA signal ID and to the more common abbreviations.  Table

B-2 identifies signals as either required, optional, or not used.

Figure B-1 and Table B-3 identify typical DTE connections to the

DDN.  The required subscriber services will dictate which scheme is

selected for a particular DTE.

Table B-4 relates required speed of service to interface type.

Together, these tables and figures serve as a guide to level 1

interface selection.  From these, most systems will be able to

identify the most appropriate interface.  However, this information is

not all-inclusive.  Other interface arrangements may be possible;

contact your DDN representative for assistance as required.

Demarcation Point

(mating connectors)

DTE DCE

|------------] [------(1) Modem RS-232-C

|

| |---------] [------(2) Modem V.35 |---|--|----| | |----]

[------(3) LDM RS-232-C, MIL-188-119 | | | |----] [------(4) Null

Modem Cable | HOST | | |----] [------(5) SME Cable plus clock source |

| | |----] [------(6) DCS MIL-188-114 |--|--|--|--|

| | |-------] [------(7) DES RS-232-C, RS-449, V.35

| |

| |----------] [------(8) KG MIL-188-114 balanced

|

|-------------] [------(9) IPLI MIL-188-114 balanced

Figure B-1. Typical Level 1 Connection Schemes

B-2

EIA CCITT ABBRM NAME ID ID NAME --- ----- ------

--------------------------------- AA 101 FG Frame (Chassis/Protective)

Ground AB 102 SG Signal/Supply Common SC 102a -- RS-449 DTE Common RC

102b -- RS-949 DCE Common BA 103 TD Transmit Data BB 104 RD Receive

Data CA 105 RTS Request to Send CB 106 CTS Clear to Send CC 107 DSR

Data Set Ready CD 108.2 DTR Data Terminal Ready CF 109 DCD Data

Carrier Detect CG 110 SQ Signal Quality CH 111 -- Signal Rate Selector

to DCE CI 112 -- Signal Rate Selector to DTE DA 113 ETC External

Transmit Clock DB 114 TC Transmit Clock DD 115 RC Receive Clock -- 116

-- Select Standby -- 117 -- Standby Indicator SBA 118 STD Secondary

Transmit Data SBB 119 SRD Secondary Receive Data SCA 120 SRS Secondary

Request to Send SCB 121 SCS Secondary Clear to Send SCF 122 SCD

Secondary Carrier Detect SCG 123 SSQ Secondary Signal Quality -- 124

-- Select Frequency Group CE 125 RI Ringing Indicator -- 126 -- Select

Transmit Frequency -- 127 -- Select Receive Frequency -- 128 --

External Receive Clock -- 129 RR Request to Receive -- 130 --

Secondary Transmit Tone -- 131 -- Receive Character Timing -- 132 --

Return to Non-Data Mode -- 133 RTR Ready to Receive .  -- 134 --

Received Data Present -- 136 -- New Signal -- 140 RL Remote loopback

-- 141 LL Local loopback -- 142 TM Test Status Monitor -- 191 --

Transmit Voice Answer

192 -- Receive Voice Answer

Table B-1. EIA and CCITT Interchange Circuits

Required: 101, 102, 103, 104, 105, 106, 107, 108.2,

109, 113, 114, and 115

Optional: 110, 125, 140, 141, and 142

(These may be required IAW future DDN

developments; it is strongly recommended

that these at least be available for

implementation upon requirement)

Not used: 111, 112, 116, 117, 118, 119, 120, 121, 122,

123, 124, 126, 127, 128, 129, 130, 131, 132,

133, 134, 136, 191, and 192

Table B-2. Signal Selection by CCITT Interchange Circuit Number

B-3

Scheme (From

Fig. B-1) Explanation

(1) Modem RS-232 at spe eds of 1200, 2400, 4800, 9600 or

14400 b/s over long haul leased voice grade

telephone facilities

(2) Modem CCITT V.35 at speeds of 48, 50, 56, 64 Kb/s over

leased group (37KHz) grade facilities or in CONUS

the Digital Data Service facilities.

(3) Limited Distance Modem

LDM generally available at 9600 b/s and below in

an RS-232 version.  Other types are available for

all speeds.

(4) Null modem A Null Modem is a length of cable with the signal

leads crossed so as to present a DCE interface.

To be used in local connection schemes where

either the DTE or the DCE has a clocking source

capability.  All four supported level 1

interfaces are available.  If DTE clock and DCE

clock are both available, DTE clock will be

preferred.

(5) Synchronous Modem Eliminator

SME is a length of cable with a hardware device

interjected.  The device allows convenient

crossing of signals so as to present a DCE

interface.  The device also provides clocking

when neither the DTE nor the DCE has such

capability.  All four supported level 1

interfaces are available.

(6) DCS Microwave

DCS is generally a military microwave system

which provides the MIL-188-114 balanced or

unbalanced interfaces.  It implies a speed of 50

Kbps and is usually found O-CONUS.  Selection of

this scheme requires selection of (4) or (5).

(7) Data Encryption Standard

DES is a commercial encryption device used by the

DoD as a privacy device.  DES is available with

either RS-232, V.35, or RS-449/422.

B-4

(8) KG KG devices are U. S. Government encryption

devices under strict NSA control.  The

requirement for security and KG devices requires

the selection of the MIL-188-114 balanced

interface.

(9) Internet Private Line Interface

IPLI devices are security level community of

interest isolation devices.  The requirement for

IPLI service requires the selection of the MIL-

188-114 balanced interface.

Notes and Considerations

1.  Interface (2), Modem, 48Kb/s is generally only

available O-CONUS.

2.  MIL-188-114 balanced is deemed equivalent to RS-449

with RS-422, the difference being that MIL-188-114 is

more tolerant of noise on signal common and more

tolerant of common mode noise.

3.  MIL-188-114 unbalanced is deemed equivalent to RS-449

with RS-423.  In most cases where MIL-188-114 balanced

is specified, MIL-188-114 unbalanced is also available,

but it is not recommended.

4.  There are system enhancements under long term

development for use in the DDN which may request

additional control leads beyond those listed as

required.  The implementation of these enhancements

will not limit operational capabilities but may impact

the ability of the Network Monitoring Center to assist

with host and host access line diagnosis.  These

enhancements may request signals from the optional

category.

Table B-3. Typical Level 1 Connection Schemes

B-5

Signaling Rate in Kb/s Physical Interface 1.2 2.4 4.8 9.6 14.4 48 50

56 64 100

RS-232-C R R R R R* - - - - -

MIL-188-114 A A A A - - - - - - unbal. (& equiv.)

MIL-188-114 A A A A A* A A A A R** bal. (& equiv.)

CCITT V.35 - - - - - R A R R A

Legend

R = Recommended

A = Available

- = Not available

* = Only available using modems

** - Only available using a local cable

connection

Table B-4. Interface Type by Service Speed

Signal Name Abbrev Pin No.  EIA ID Signal Source ----------- ------

------- ------ ------------- Frame Ground FG 1 AA DTE/DCE Transmitted

Data TD 2 BA DTE Received Data RD 3 BB DCE Request to Send RTS 4 CA

DTE Clear to Send CTS 5 CB DCE Data Set Ready DSR 6 CC DCE Signal

Ground SG 7 AB DTE/DCE Data Carrier Detect DCD 8 CF DCE Transmit Clock

TC 15 DB DCE Receive Clock RC 17 DD DCE Data Terminal Ready DTR 20 CD

DTE Ext. Transmit Clock ETC 24 DA DTE Wired Spare -- 18 -- --- Wired

Spare -- 22 -- --- Wired Spare -- 25 -- ---

Required pins: 1, 2, 3, 4, 5, 6, 7, 8, 15, 17, 20, 24

Optional pins: 9, 10, 18, 22, 25

Notes

1.  The DTE will present a CANNON DB-25P male connector

with pinouts as above or equivalent hardware with

identical pinouts.

2.  The DCE will present a CANNON DB-2SS female

connector or equivalent.

Table B-5. RS-232-C Interface

B-6

B-8 Signal Name Abbrev Pin Nos EIA ID

Signal Source ----------- ------ ------- ------ ------------- Send D

ta SD 4,22 BA DTE Send Timing ST 5,23 DB DCE Receive Data RD 6,24 BB

DCE Request to Send RTS 7,25 CA DTE Receive Timing RT 8,26 DD DCE

Clear to Send CTS 9,27 CB DCE Local 100pback LL 10 -- DTE Data Mode DM

11,29 CC DCE Terminal Ready TR 12,30 CD DTE Receiver Ready RR 13,31 CF

DCE Remote 100pback RL 14 -- DTE Terminal Timing TT 17,35 DA DTE Test

Mode TM 18 -- DCE Signal Ground SG 19 AB DTE/DCE Receive Common RC 20

RC DCE Send Common SC 37 SC DTE Wired Spare -- 1 -- --- Wired Spare --

3,21 -- ---

Required pins: 4,22; 5,23; 6,24; 7,25; 8,26; 9,27,;

11,29; 12,30; 13,31; 17,35; 19; 20; 37

Optional pins: 10; 14; 18; 1; 3,21

Notes:

1.  The DTE will present a CANNON DC-37P male connector

with pinouts as above or equivalent hardware with

identical pinout.

2.  The DCE will present a CANNON DC-37S female

connector or equivalent.

Table B-6. MIL-188-114 Interface (and equivalents)

B-7

Signal Name Abbrev Pin Nos.  EIA ID Signal Source ----------- ------

-------- ------ -------------

Frame Ground FG A AA DTE/DCE Signal Ground SG B AB DTE/DCE Transmit

Data TD P/S BA DTE Receive Data RD R/T BB DCE Request to Send RTS C CA

DTE Clear to Send CTS D CB DCE Data Set Ready DSR E CC DCE Data

Carrier Detect DCD F CF DCE Local 100pback LL K -- DTE Ext. Transmit

Clock ETC U/W DA DTE Transmit Clock TC Y/aa DB DCE Receive Clock RC

V/X DD DCE

Required Pins: A; B; P/S; R/T; C; D; E; F; U/W; Y/aa;

V/X

Optional Pins: K

Notes:

1.  The DTE will present a Winchester MRA(C)-34D-JTCH-H8

male connector with pinout as above or equivalent

hardware with the identical pinout.

2.  The DCE will present a mating female connector.

Table B-7. V.35 Interface

B-8

APPENDIX C

FEDERAL INFORMATION

PROCESSING STANDARDS PUBLICATION 100

FEDERAL STANDARD 1041

1983 JULY 6

ANNOUNCING THE JOINT STANDARD FOR

INTERFACE BETWEEN DATA TERMINAL EQUIPMENT (DTE)

AND DATA CIRCUIT-TERMINAL EQUIPMENT (DCE)

FOR OPERATION WITH PACKET-SWITCHED DATA

COMMUNICATIONS NETWORKS

Federal Information Processing Standards Publication are developed and

issued by the National Bureau of Standards pursuant to section 111(f)(2)

of the Federal Property and Administrative Services Act of 1949, as

amended, Public Law 89-306 (79 Stat.1127), Executive order 11717 (38 FR

12315 dated May 11, 1973), and Part 6 of Title 15 Code of Federal

Regulations (CFR).

Federal Standards in the "telecommunication" series are developed by the

Office of the Manager, National Communication System. These Federal

Standards are issued by the General Services Administration pursuant to

the Federal Property and Administrative Services Act of 1949, as amended.

Name of Standard: Interface Between Data Terminal Equipment (DTE) and Data

Circuit-Terminating Equipment (DCE) for Operation with Packet-Switched

Data Communications Networks.

Category of Standard: Hardware, Data Transmission.

Explanation: Federal automated data processing equipment, services, and

telecommunication equipment using public packet-switched data

communications networks (PSDCN) based on the family of CCITT

Recommendations derived from X.l and X.2 shall employ the interface and

protocols specified in this joint standard. In addition, designers of

these internally operated and maintained Federal networks employing

packet-switched technology should consider the use of this interface as

appropriate. The joint standard provides:

- A family of physical layer interfaces, from which a particular

interface may be selected; and

- A single data link layer control procedure; and

- Packet level procedures for virtual calls and permanent virtual

circuits, and an optional datagram operation.

The mandatory interface attributes of this joint standard are summarized

as follows:

PHYSICAL LEVEL

Transmission rates:      2.4, 4.8, 9.6 Kbits/s

Interface:              one or more of the following: RS-232-C, X.2l, RS-449

LINK LEVEL:

Procedure:               LAPB

Parameter K:             7

Smallest N l:            l64 Octets

PACKET LEVEL:

Services:               Virtual call and permanent virtual circuit

Packet types:           All basic plus Diagnostic packets. Packet Reject

shall  not be used.

User data field         Octet-aligned

length:

Packet sequence         Modulo 8

numbering:

D bit procedure:        Supported  by all DCEs;  DTE need not  employ the

D  bit when sending to

the DCE, but no DTE shall reject incoming packet

with the D bit set to l or  0 as having this bit

in error unless it is known by receiver that the

sender  has no D bit capability.

X.25 diagnostic         Use standard codes whenever they apply; non-std

codes:                codes may be used for events not listed in X.25

within a period of 24 months after the effective

date of this standard.

Fast Select:            DCEs shall implement fast select; DTE need not

employ fast select when sending to DCE, but all

DTEs with higher level functionality which

allows response to fast select must be able to

accept incoming fast  select packet.

Interrupt packet:       Receipt of a DTE interrupt packet before a

previous DTE interrupt  packet has been confirmed

is an error condition.

Duplicated facility     The last appearing facility code should be

codes:               treated by the DTE as if it  were the only

appearance of that code.

Non-zero cause field    Discarded

of restart request

packet:

Restart request too     Discarded

long in state r1:

This joint standard is intended to enhance interoperability by specifying

certain subsets and other constraints on Federal use of CCITT

Recommendation X.25.

The Government's intent in employing this joint standard is to reduce the

cost of acquiring and using Federal automated data processing equipment,

services, and telecommunication equipment with PSDCN.  The joint standard

is also intended to reduce the cost of acquiring and using

Government-owned or leased PSDCN. These goals will be achieved by:

- increasing the available alternative sources of supply;

- Increasing the reutilization of Government resources; and,

- Assuring the required interoperability.

Approving Authority: Secretary of Commerce (Federal Information Processing

Standards).  Administrator, General Services Administration (Federal

Standards).

Maintenance Agency: The National Bureau of Standards and the Office of the

Manager, National Communications System will jointly maintain this

standard coordinating as necessary with the General Services

Administration (GSA).

Cross Index: The following are related standards upon which this FIPS PUB

is based. The inclusion of a particular standard on this list does not

necessarily mean that the standard is applicable in all cases to which

this FIPS PUB applies.

(a) International Standard 2110-1980: Data Communication-25 pin DTE/DCE

Interface Connector and Pin Assignments.

(b) International Telegraph and Telephone Consultative Committee

(CCITT) recommendations V.24 (1980): List of Definitions for Interchange

Circuits Between Data Terminal Equipment and Data Circuit Terminating

Equipment.

(c) CCITT Recommendation V.28 (1980) Electrical Characteristics for

Unbalanced Double-Current Interchange Circuits.

(d) Electronics Industries Association (EIA) RS-232-C (1969 August):

Interface Between Data Terminal Equipment and Data Communication Equipment

Employing Serial Binary Data Interchange.

(e) International Standard 4902-1980: Data Communication-37-Pin and

9-Pin DTE/DCE Interface Connectors and Pin Assignments.

(f) CCITT recommendation V.11(X.27) (1980): electrical Characteristics

for Balanced Double-Current Interchange Circuits for General Use with

Integrated Circuit Equipment in the Field of Data Communications.

(g) EIA RS-422-A (1978 June): Electrical Characteristics of Balanced

Voltage Digital Interface Circuits.

(h) Federal Standard 1020A (1980 January): Telecommunications:

Electrical Characteristics of Balanced Voltage Digital Interface Circuits.

(i) CCITT Recommendation V.10 (X26) (1980): Electrical Characteristics

for Unbalanced Double-Current Interchange Circuits for General Use with

Integrated Circuit Equipment in the Field of Data Communications.

(k) Federal Standard 1030A (1980 January): Telecommunications:

Electrical characteristics of Unbalanced Voltage Digital Interface

Circuits.

(l) CCITT Recommendation X.21bis (1980): Use on Public Data Networks of

Data Terminal Equipment which are Designed for Interfacing to Synchronous

V-series Modems.

(m) CCITT Recommendation V.54 (1980): Loop Test Devices for Modems.

(n) EIA RS-449 (1977 November): general Purpose 37-Position Interface

Between Data Terminal Equipment and Data Circuit-Terminating Equipment.

(o) Federal Standard 1031 (1980 June): Telecommunications General

Purpose 37-position and 9-position Interface Between Data Terminal

Equipment and Data Circuit Terminating Equipment (implementing

instructions in the form of a Federal Property Management Regulation have

not yet been issued. the General Services Administration is considering

canceling FED-STD 1031. Furthermore, a Federal Information Processing

Standard for ADP applications corresponding to Federal Standard 1031 has

not been adopted by the National Bureau of Standards.)

(p) International Standard 4903-1980: Data Communication-15-pin DTE/DCE

Interface Connector and Pin Assignments.

(q) EIA Industrial Electronics Bulletin No. 12 (1977 November):

Application Notes on Interconnection Between Interface Circuits Using

RS-449 and RS-232-C.

(r) Draft International Standard 2593 (1980): Data Communication-34-pin

DTE/DCE Interface Connector and Pin Assignments.

(s) CCITT Recommendation V.35 (1980): Data Transmission at 48 Kilobits

per second Using 60-108 kHz Group Band Circuits.

(t) CCITT Recommendation X.21 (1980): general Purpose Interface Between

Data Terminal Equipment and Data Circuit-Terminating Equipment for

Synchronous Operation on Public Data Networks.

(u) CCITT recommendation V.5 (1980): Standardization of Data-Signalling

Rates for Synchronous Data Transmission in the General Switched Telephone

networks.

(v) CCITT Recommendation V.6 (1980): Standardization of Data-Signalling

Rates for Synchronous Data Transmission on Leased Telephone-Type Circuits.

(w) American National Standard X3.1-1976: Synchronous Signalling Rates

for Data Transmission.

(x) Federal Information Processing Standard Publication 22-1 (1977

September): Synchronous Signaling Rates Between Data Terminal and Data

Communication Equipment. (FIPS PUB 22-1 is identified also as FED-STD

1013.)

(y) Federal Standard 1013 (1977 August): Telecommunications:

Synchronous Signaling Rates Between Data Terminal Equipment and Data

Circuit-Terminating Equipment utilizing 4 kHz Circuits (FED-STD 1013) is

identified also as FIPS PUB 22-1.)

(z) American National Standard X3.36-1975: Synchronous High-Speed Data

Signaling Rates Between Data Terminal Equipment and Data Communication

Equipment.

(aa) Federal Information Processing Standards Publication 37 (1975

June): Synchronous High Speed Data Signaling Rates Between Data Terminal

Equipment and Data Communication Equipment. (FIPS PUB 37 is identified

also as FED-STD 1001.)

(ab) Federal Standard 1001 (1975 June): Telecommunications: Synchronous

High-Speed Data Signaling Rates Between Data Terminal Equipment and Data

Communications Equipment. (FED-STD 1001 is identified also as FIPS PUB

37.)

(ac) EIA RS-269-B (1976 January): Synchronous Signaling Rates for Data

transmission.

(ad) International Standard 3309-1979: Data Communication-High Level

Data Link control Procedures-Frame Structure.

(ae) International Standard 4335-1979: Data Communication-High Level

Data Link control Procedures-Elements of Procedures.

(af) Addendum 1 to International Standard 4335-1979: Data

Communication-High Level Data Link control Procedures-Elements of

Procedures.

(ag) Addendum 2 to International Standard 4335-1979: Data

Communication-High Level Data Link Control Procedures-Elements of

procedures.

(ah) International Standard 6256-1980: Data Communication-High -Level

Data Link Control Procedures-Balanced Class of Procedures.

(ai) American National Standard X3.66-1979: Advanced Data Communication

Control procedures (ADCCP).

(aj) Federal Information Processing Standards Publication 71 (1980 May)

as revised by the Federal Register notice 47 FR 23798, dated June 1, 1982

and corrected by the notice 47 FR 25397 dated June 11, 1982: Advanced Data

Communication Control Procedures (ADCCP). (FIPS PUB 71 is technically

consistent with FED-STD 1003A.)

(ak) Federal Information Processing Standards Publication 78 (1980

September): Guideline for Implementing Advanced Data Communication Control

Procedures (ADCCP).

(al) Federal Standard 1003A (1981 August): Telecommunications:

Synchronous bit-Oriented Data Link Control Procedures (FED-STD 1003A is

technically consistent with FIPS PUB 71.)

(am) CCITT Recommendation X.25 (1980): Interface Between Data Terminal

Equipment (DTE) and Data Circuit-Terminating Equipment (DCE) for Terminals

Operating in the Packet Mode on Public Data Networks.

(an) Draft Proposed International Standard 7498: Data Processing-Open

Systems Interconnection-Basic Reference Model.

(ao) CCITT Recommendation X.1 (1980): International User Classes of

Service in Public Data Networks.

(ap) CCITT Recommendation X.2 (1980): International User Facilities in

Public Data Networks.

(aq) CCITT Recommendation X.96 (1980): Call Progress Signals in Public

Data Networks.

Applicability: The technical specifications of this joint standard shall

be employed in the acquisition, design, and development of all federal

automated data processing equipment, services, and telecommunication

equipment and PSDCN whenever an interface based on CCITT Recommendation

X.25 (1980), Interface Between Data Terminal Equipment (DTE) and Data

Circuit-Terminating Equiment (DCE) for Terminals Operating in the Packet

Mode on Public Networks>1, is required.  Referred to below as CCITT

Recommendation X.25, Recommendation X.25, or X.25.

Implementation: The provisions of this joint standard are effective July

6, 1983. Any applicable equipment or service ordered on or after the

effective date, or procurement action for which solicitation documents

have not been issued by that date, must conform to the provisions of this

standard unless a waiver has been granted in accordance with the

procedures described below.

This joint standard shall be reviewed by the Institute for Computer

Sciences and Technology, National Bureau of Standards and the Office of

the Manager, National Communications System, within five years after its

effective date.  This review shall take into account technological trends

and other factors to determine if the joint standard should be affirmed,

revised, or withdrawn.

Specifications: This joint standard adopts a subset, identified below, of

the International Telegraph and Telephone Consultative Committee's

Recommendation X.25.

(a) At the physical level, the provisions of Section 1 of CCITT

Recommendation X.25 shall be used.  As a minimum, networks shall support

dedicated circuit access; other types of access (e.g., through the general

switched telephone network) may also be offered.

CClTT Recommendation X.1 standardizes data signalling rates of

2.4, 4.8, 9.6, and 48 kbits/s for packet mode interfaces.  At a minimum,

networks shall support the synchronous data signalling rates of 2.4, 4.8,

and 9.6 kbits/s full duplex; other speeds (e.g., 19.2 kbits/s) may also be

offered. The 48 kbits/s rate need not be supported in those locations

where it is not available; 56 kbits/s is recommended in its place (see

American National Standard X3.36-1975 and related documents referenced

above).  The term "user class of service" used in X.25 refers to the data

signalling rate of DTE/DCE interface.

In accordance with CCITT Recommendation X.25, networks shall

provide one or more of the following interface options:

i.  CCITT Recommendation X.21;

ii.  EIA RS-232-C, which is essentially equivalent to one of

the options in CCITT Recommendation X.21bis;

iii.  CCITT Recommendation X.21bis option that is equivalent to

RS-449 using only the EIA RS-423A unbalanced electrical characteristics.

Interworking between EIA RS-232-C on one side of the interface

and RS-449 on the other side is permitted in accordance with EIA

Industrial Electronics Bulletin Number 12. Where interworking with

RS-232-C equipment is not required, the provisions described below

employing RS-449 with the RS-422A electrical characteristics may

optionally be employed at signalling rates below 48 kbit/s.

Networks which support 48 or 56 kbits/s data signalling rates

shall provide one or more of the following interface options:

i.  CCITT Recommendation X.21;

ii.  CCITT Recommendation X.21bis option that specifies CCITT

Recommendation V.35; or

iii.  CCITT Recommendation X.21bis option that specifies CCITT

Recommendation V.36 which is equivalent to EIA RS-449.

NOTE: Current study in national and international standards groups may

result in the development of additional physical interfaces.  Each such

physical interface will be evaluated for inclusion in this joint standard.

If there are significant savings, one physical interface may be selected

as the future mandatory physical interface.

NOTE: DTE purchasers and designers should determine which physical

interface(s) is provided by the associated DCE(s).

(b) Only the LAPB link level procedures shall be used.

NOTE: These procedures are a subset of those described in FIPS PUB 71

and Federal Standard 1003A and correspond to FIPS PUB 78 recommended class

B.  This subset is identified as follows:

i.  Link configuration: two combined stations on a

point-to-point link.

ii.  Class of procedures: balanced asynchronous (BA) with

options two and eight. The RSET command shall not be used. (RSET is found

in option 11 of the Fips PUB 71.  RSET is part of the basic repertoire in

Federal Standard 1003A; option 11 of federal Standard 1003A deletes the

RSET command.  Note that RSET is not part of CCITT Recommendation X.25.)

iii.  Two-way simultaneous operation shall be employed.

iv.  The smallest N1, (the maximum number of bits in an

information frame excluding flags and zero bit insertion for

transparency), which shall be supported shall be 164 octets (the maximum

length of) fast select caIl setup packet). If a DTE neither transmits, nor

receives for processing by higher level functionality fast select packets,

an N1 as small as 135 octets may be supported by the DTE.

v.  The address of the combined station provided by the network

shall be 10000000; the address of the other combined station shall be

11000000; where the left-hand bit is the least significant bit (bit number

1) and shall be transmitted first.  This convention is consistent with the

provisions of FIPS 71 and Federal Standard 1003A.

vi.  The FCS shall be a 16-bit sequence as indicated in Section

2.2.7. DTE/DCE may also employ the 32-bit FCS as indicated in FIPS PUB 71

(revised) and FED-STD 1003A. DTE/DCE equipment using the 32-bit FCS shall

be able to also operate with the 16-bit FCS. The smallest N1 shall be 166

octets when the 32-bit FCS is used. If a DTE neither transmits, nor

receives for processing by higher level functionality fast select packets,

an Nl as small as 137 octets may be supported by the DTE when the 32-bit

FCS is used.

NOTE: FIPS PUB 78 provides a detailed discussion of the relative

merits of the 16-bit and 32-bit FCS.

vii.  The frame reject information field shall be padded with 4

zero bits in bit positions 21 through 24 of the information field to

provide a length of three octets.

viii. It is required that all implementations be capable of

operating with K=7; optionally, values of 1 to 6 are permissible with

modulo 8 operation and values 1 to 127 are permissible with modulo 128

operation.

NOTE: DTE purchasers and designers should determine that values of k

other than 7 are supported by the associated DCE(s).

(c) The user data field of packets shall be an integral number of

octets.  If a packet is received which shows a user data field not equal

to an integral number of octets, the receiving DTE/DCE shall follow the

packet level procedures for processing a packet type which is too long.  A

new diagnostic code "non-octet aligned packet," consistent with the Data

Communications-X.25 Packet Layer Specification for Terminal Equiment, ISO

DP 8208, November 8, 1982, is recommended as #82.

(d) The reject packet shall not be used.

(e) All DCE restart confirmation, DCE reset confirmation, and DCE

clear confirmation packets shall be interpreted by the DTE as having local

significance only.

(f) The D-bit shall be implemented by all networks. DTE's need not

employ the D-bit procedures when transmitting to the network, but no DTE

shall reject incoming packets with the D-bit set to 1 or 0 as having this

bit in error unless the receiving DTE knows the remote DTE has not

implemented the D-bit procedure; in this case, the receipt of a D-bit set

to 1 may be treated by the receiving DTE as an error condition.

(g) The selection of logical channel number for new virtual calls

shall follow the procedures suggested in Section 4.1.2 Note 2, Annex A

Note 5, and Annex A Note 6, of the CCITT Recommendation X.25.

(h) It is required that all implementations be capable of operating

with packet sequence numbering modulo 8; optionally, implementations of

packet sequence numbering modulo 128 are also permitted.

NOTE: DTE purchasers and designers should determine if the associated

DCE(s) support packet sequence numbering modulo 128.

(i) All DTE's and DCE's shall follow the flow control principles

outlined in the first two sentences of the first paragraph of Section 4.4.

1.3 of CCITT Recommendation X.25.

(j) The alternative procedure for passing packets containing a P(S)

that is out of sequence but within the window as described in the third

paragraph of Section 4.4.1.3 of CCITT Recommendation X.25 shall not be

used.

(k) The second sentence of Section 4.4.1.4 Note 2 shall not apply.

This sentence permits networks to defer updating the window for data

packets with D =0, and sent within the window but before a data packet

with D= 1, until the network receives a corresponding P(R) for the packet

with D= 1.

(1) The resetting cause field of a reset request packet shall be set

to zero. If a reset request is received with a non-zero resetting cause

field, the packet shall be discarded. The network shall then initiate the

resetting procedure with the resetting cause field indicating local/remote

procedure error.

(m) The clearing cause field of a clear request packet shall be set to

zero. If a clear request packet is received with a non-zero clearing cause

field, the packet shall be discarded.  The network shall then initiate the

clearing procedure with the clearing cause field indicating local/remote

procedure error.

(n) The restarting cause field of a restart request packet shall be

set to zero. If a restart request packet is received with a non-zero

restart cause field, the restart request packet shall be discarded without

further action.  Optionally, the DCE may generate a diagnostic packet with

a recommended diagnostic code #81 (improper cause code from DTE), which is

consistent with the <1Data Communication-X.25 Packet Layer>1

<1Specification for Data Terminal Equiment,>1 ISO DP 8208, November 8,

1982.

(o) A diagnostic code shall be provided in all clear request, reset

request, and restart request packets in accordance with the codes listed

in Annex E of CCITT Recommendation X.25 whenever they apply; non-assigned

codings in X.25 may be used for events not listed in X.25 within the

period of 24 months after the effective date of this standard. Prior to

the end of this 24 month period, this standard will be reviewed by NBS to

determine whether the standard should be revised to incorporate a

different table.  After this revision, codes not specifically listed shall

not be used.

(p) A generic diagnostic code shall not be used when a more specific

diagnostic code is known to be applicable.

(q) The network diagnostic codes shall be used in accordance with the

codes listed in Annex E of CCITT Recommendation X.25 whenever they apply;

non-assigned codings in X.25 may be used for events not listed in X.25

within the period of 24 months after the effective date of this standard.

Prior to the end of this 24 month period, this standard will be reviewed

by NBS to determine whether the standard should be revised to incorporate

a different table. After this revision, network diagnostic codes not

specifically listed shall not be used.

(r) The network shall consider the receipt of a DTE interrupt packet

before a previous DTE interrupt packet has been confirmed as an error, and

shall execute the error procedure described in Annex C, Table C-4/X.25 and

the corresponding note 2.

(s) The timeouts and time limits specified in Annex D shall be

observed by all DTE and DCE equipment. T21 shall not be less than the

value given in table D-2/X.25.  The preferred actions listed in table

D-2/X.25 shall be followed.

(t) When the link level procedures enter the logically disconnected

state, the associated packet level procedures shall clear all virtual

calls and reset all permanent virtual circuits and datagram logical

channels.  When the link level procedures reenter the information transfer

state, the associated packet level procedures shall execute the restart

procedure.  The terms "logically disconnected state" and "information

transfer state" are used as defined in American National Standard

X3.66-1979 (referenced above).  Link level procedures enter the logically

disconnected state when a DISC command is sent and a UA response is

received, for example. The link level procedure shall also be considered

to be in the logically disconnected state after N2 (re)transmissions of

SABM or DISC, where N2 is as defined in CClTT Recommendation X.25. The

logically disconnected state is not assumed after N2 (re)transmissions of

other types of frames.

(u) lf a restart request packet is received in state rl which exceeds

the maximum permitted length, the DCE shall discard the restart request

packet without further action. Optionally, the DCE may generate a

diagnostic packet with diagnostic code #39 (packet too long).

(v) In the event that a facility code appears more than once in a

facility field, the receiving DTE detecting this condition should treat

the last appearance of the particular code as if it were the only

appearance of that code.

(w) All networks shall supply diagnostic packets when their use is

suggested in CClTT Recommendation X.25. No DTE shall rejcct diagnostic

packets as errors.

(x) ln Section 6.1.1, the second paragraph, the last phrase, "and is

set to 0 in all other packets", shall be interpreted that the Qualifier

bit is set to 0 in all other packets except data packets.  For the case of

data packets, the Qualifier bit is set to 0 or 1 as indicated in Section

4.3.6 of CClTT Recommendation X.25.

(y) The list of user facilities for packet-switched data networks,

extracted from CCITT Recommendation X.2, is given below.  These facilities

are described in Section 7 of CCITT Recommendation X.25. The following

further constraints apply:

i.  Networks shall provide the facilities designated as

essential "E" below.

ii.  Networks shall also implement the Fast Select and Fast

Select Acceptance facilities to facilitate more efficient operation in

conveying higher layer protocol information or user data during call

establishment.  DTE's need not employ fast select packets when

transmitting to the network, but all DTE's associated with the higher

level functionality which allows response to a fast select packet must be

able to accept incoming fast select packets.

iii.  The packet retransmission facility shall not be used.

iv.  All DTE's which employ any of the facilities labelled as

additional "A" below (except Fast Select and Fast Select Acceptance) shall

also be capable of operating without employing any A facilities (except

Fast Select and Fast Select Acceptance).

v.  The throughput class value of 48,000 bits/s may be

interpreted as 56,000 bits/s in those locations where 56,000 bits/s access

is used.

Facilities of packet-switched data networks:

User Facility                             VC       PVC     DG*

Optional user facilities assigned

for an agreed contractual period:

Extended packet sequence numbering

(modulo)                                        A        A       A*

Non-standard default window sizes               A        A       A*

Non-standard default packet sizes

16, 32, 64, 256, 512, 1024                    A        A       -

Default throughput class assignment             A        A       A*

Flow control parameter negotiation              E        -       -

Throughput class negotation                     E        -       -

Packet retransmission                           A***     A***    A***

Incoming calls barred                           E        -       E*

Outgoing calls barred                           E        -       E*

One-way logical channel outgoing                E        -       A*

One-way logical channel incoming                A        -       A*

Closed user group                               E        -       E*

Closed user group with outgoing

access                                          A        -       A*

Closed user group with incoming

access                                          A        -       A*

Incoming calls barred within a

closed user group                               A        -       A*

Outgoing calls barred within a

closed user group                               A        -       A*

Bilateral closed user group                     A        -       A*

Bilateral closed user group with

outgoing access                                 A        -       A*

Reverse charging acceptance                     A        -       A*

Fast select acceptance                          A**      -       -

Datagram queue length selection*                -        -       A*

Datagram service signal logical

channel*                                        -        -       A*

Datagram non-delivery indication*               -        -       E*

Datagram delivery confirmation*                 -        -       E*

D-bit modification                              A        A       -

Optional user facilities requested

by the DTE on a per call basis

Closed user group selection                     E        -       E*

Bilateral closed user group selection           A        -       A*

Reverse charging                                A        -       A*

RPOA selection                                  A        -       A*

Flow control parameter negotiation              E        -       -

Fast select                                     A**      -       -

Throughput class negotiation                    E        -       -

Abbreviated address calling                     FS       -       A*

Datagram non-delivery indication                -        -       E*

Datagram delivery confirmation                  -        -       E*

NOTE: Detailed  explanations  of  these  facilities  are  provided  in  CCITT

Recommendation X.25.

LEGEND:

E   =  An essential user facility to be offered by all networks.

A   =  An  additional  user  facility which  may  be  offered  by  certain

networks.

FS  =  Further study is required. This standard will be modified when this

study is complete.

-   = Not applicable.

DG  = Applicable when the datagram service is being used.*

VC  = Applicable when the virtual call service is being used.

PVC = Applicable when the permanent virtual circuit service is being used.

* - The  datagram  service and  its  related facilities  may  be  used

only when:

- there is  to be  a one-way transfer  of information  which does  not

require recovery at the network layer; and,

- a response to  this transfer of information  is not required at  the

network layer.

NOTES: 1. At the present time,  the transfer of datagram packets  across

international  borders  through  public  packet-switching  networks  is   not

permitted 2. DCE's are  not required to provide  datagram service. DTE's  are

not required to generate or accept datagrams and datagram-related packets.

** - Fast select shall  be provided by all  DCE's. All DTE's associated  with

the higher level functionality which allows response to a fast select  packet

must be  capable of  accepting incoming  fast select  packets, but  need  not

generate fast select packets.

***  The packet retransmission facilities shall not be used.

(z) The list  of the  applicable call progress  signals, extracted  from

CCITT Recommendation X.96, is given below. These signal definitions apply  to

the  cause  codes  specified  in  CCITT  Recommendation  X.25.   The  related

circumstances giving rise  to each call  progress signal is  also defined  in

table 1 below. The significance of  categories indicates broadly the type  of

action expected of the DTE receiving the signal:

Category           Significance

A                  Requested  action  confirmed  by  network.

B                  Call   cleared  because  the procedure is complete.

C1 and C2          Call cleared. The calling DTE should  call again soon: the

next attempt may be successful. However, after a number of

unsuccessful call attempts with the same  response, the

cause could be assumed to be in Category D1 or D2. The

interval between  successive attempts and the number of

maximum attempts will depend on a number of circumstances

including:

- nature of the call progress signal

- users' traffic pattern

- tariffs

- possible regulations by the network provider.

OR

Reset. The DTE may continue to transmit data

recognizing that data loss may have occurred.

D1 and D2          Call cleared. The calling DTE should take other action to

clarify when the call  attempt might be successful.

OR

Reset (for permanent virtual circuit only).

The DTE should cease data transmission and take other action

as appropriate.

C1 and D1          Due to subscriber condition.

C2 and D2          Due to network condition.

The sequence of call  progress signals in table  1 implies, for Categories  C

and D, the order of  call set-up processing by  the network. ln general,  the

DTE can assume, on receiving a call progress signal, that no condition higher

up in  the table  is present.  Network  congestion is  an exception  to  this

general rule. The actual coding of call progress signals does not necessarily

reflect this sequence.

Users and DTE  manufacturers are  warned to  make due  allowance to  possible

later extensions to this table by providing appropriate fallback routines for

unexpected signals.

-------------------------------------------------------------------------------

-------------------------------------------------------------------------------

Call Progress               Definition                             Category

Signal

-------------------------------------------------------------------------------

Delivery               The datagram has been                      A

confirmation           accepted by the destination DTE.

Local procedure        A procedure error caused by the DTE        C1

error                  is detected by the DCE at the local

DTE/DCE interface.

Network                A condition exists in the network          C2

congestion             such as:

1) temporary network congestion

2) temporary fault condition within

the network, including procedure error

within a network or an international link.

Invalid                 A facility requested by the calling        D1 or D2

facility                DTE is detected as invalid by the DCE

request                 at the local DTE/DCE interface.

Possible reasons include:

-  request for a facility which has not

been subscribed to by the DTE;

-  request for a facility which is not

available in the local network:

-  request for a facility which has not

been recognized as valid by the local DCE.

RPOA out                The RPOA nominated by the calling DTE is   D2

of order                unable to forward the call.

Not                     The called DTE address is                  D1

obtainable              out of the numbering plan or not

assigned to any DTE.

Access barred           The calling DTE is not permitted           D1

the connection to the called DTE.

Possible reasons include:

-  unauthorized access between the calling

DTE and thc called DTE.

-  incompatible closed user group.

Reverse charging        The called DTE has not subscribed          D1

acceptance not          to the reverse charging acceptance

subscribed              facility.

Fast select             The called DTE has not subscribed          D1

acceptance not          to the fast select acceptance

subscribed              facility.

Incompatible            The remote DTE/DCE interface or the        D1

destination             or the transit network does not support

a function or facility requested (eg.the

datagram service).

Out of Order		  The remote number is out of order.         D1 or D2

Possible reasons include:

- DTE is Uncontrolled Not Ready:

- DCE Power off:

- Network fault in the local loop:

- X.25 Level 1 not functioning:

- X.25 Level 2 not in operation.

Number busy		  The called DTE is detected by the DCE	     C1

as engaged on other call(s), and

therefore as not being able to accept

the incoming call. (In the case of the

datagram service..the queue at the

destination DCE is full.)

Remote		  A procedure error caused by the             D1

procedure		  remote DTE is detected by the DCE

error			  at the remote DTE/DCE interface.

Network		  Network is ready to resume normal	     C1

operational		  operation after a temporary failure

or congestion.

Remote DTE		  Remote DTE/DCE interface is ready	     C1 or D1

operational             to resume normal operation after a

temporary failure or out of order

condition (e.g., restart at the remote

DTE/DCE interface. Loss of data may

have occurred.

DTE originated	  The remote DTE has intiated		     B or D1

a clear, reset, or restart procedure.

Waivers: Waiver of this standard is required when an interface based on CCITT

Recommendation X.25  (1980) is  to be  employed  and has  either one  of  the

following conditions: 1) The interface has options that are not permitted  by

this standard; 2) The  interface does not implement  all options mandated  by

this standard.

Heads of agencies  desiring a  waiver from  the requirements  stated in  this

standard, so as to acquire applicable equipment or service not conforming  to

this standard,  shall  submit a  request  for waiver  to  the  Administrator,

General Services Administration,  for review and  approval. Approval will  be

granted if, in the judgment of the Administrator after consultation with  the

Assistant Secretary of Commerce for Productivity, Technology and  Innovation,

based on  all available  information including  that provided  in the  waiver

requests, a major adverse economic or operational impact would occur  through

conformance with this standard.

A request for waiver shall include a justification for the waiver,  including

a description and discussion  of the adverse  economic or operational  impact

that would  result  from conforming  to  this  standard as  compared  to  the

alternative for which  the waiver  is requested.  ICST and  NCS will  provide

technical assistance, as required, to GSA.

Where to  Obtain Copies:  Copies of  this  publication are  for sale  by  the

National  Technical  Information  Service,  U.S.   Department  of   Commerce,

Springfield,  VA  22161.   When   ordering,  refer  to  Federal   Information

Processing Standards Publication  100 (FIPS-PUB-  l00)/Federal Standard  1041

(FED-STD 1041),  and  title.  When  microfiche  is desired,  this  should  be

specified. Payment may be made by check, money order, purchase order,  credit

card, or deposit account.

The CCITT X.25 specifications upon which  this publication is based may  also

be obtained from NTIS.  Specify PB82-187766; the cost is $50; telephone (703)

487-4650.

Downloaded From P-80 International Information Systems 304-744-2253