************ CAR TRACKER *************

Uploaded By: The Phoney Phoner

And The Skeleton Krew

**************************************

By: Ford Prefect

"my name is bond, james bond."

My next gadget is what is called a

bumpbeeper.  It attaches to the under

side of a car with magnets and then

sends out a signal that can be heard on

a radio.

Parts list: () = diagram #

(1) 2n635a transistor

(2) 4.3K 1/2 watt resistor

(3) 1 meg potentiometer

(4) 10k 1/2 watt resistor

(5) 50pf capacitor

(6) 365pf variable capacitor

(7) .005Uf capacitor

(8) .01Uf capacitor

(9) ferrite loopstick (from a crys-

(10) coil [see text]    tal radio)

(11) single-pole single-throw switch

(12) 9 volt battery

(opt.) Battery clip & case

(13) antenna

this device is a constant tone signal-

ling source that can be tuned to any

clear spot below 1000 khz.  If magnets

are attached to the case, it can quick-

ly be installed under the persons car.

(9) Is a standard ferrite loopstick tha

t can be purchased at radio shack.

(10) Is simply 12 turns of plastic cov-

ered hookup wire wound over (9).

13

(*)-------\(----+-----______--- >>a<<

\              \     ^^^^^^

\        5     \       \   9

---             -\(--   --\(--->>b<<

-           6      \   7

v                  \

--->>a<<

>>a<<------+------+------o/ o-----

\      \       11     \

/     ---             \

\      -              \

/      v              \

4  \              --------

\              \

\              --[][]->>c<<

--->>b<<          12

"@�                                   j��  >>c<<----/\/\/\/\----+-----+-------

2       \     \      \

\     \      \

>>d<<---)\-------     \      \

v    >>e<<

8      -----/\/\/\--

\       3

>>f<<-----------+

/---\

1 (  \  )

(   / /-)---->>d<<

(   \  >  )

( ----- )

(  \  )

\---/

\

\

>>b<<

>>e<<-------

\

(

)

(

10    )

\

------>>f<<

>>d<<---

\

---

-

v  (ground)

to tune the transmitter:

1)pick an empty spot on the am car

radio below 1000 khz.

2)Switch on the transmitter with the

spst switch.

3)Tune the 365 variable capacitor

slowly until a shrill note can be

heard from the car radio.

4)The pitch of the note is adjustable

by turning the 1 meg potentiometer

resistantly,

Ford Prefect

Uploaded to: The  Party  Line

(415) 447-0963

///////////////////////////////////////////////////////

//               The PIRATES' HOLLOW                 //

//                  415-236-2371                     //

//         over 12 Megs of Elite Text Files          //

//                  ROR-ALUCARD                      //

//             Sysop: Doctor Murdock                 //

// C0-Sysops: That One, Sir Death, Sid Gnarly & Finn //

//                                                   //

//    "The Gates of Hell are open night and day;     //

//     Smooth is the Descent, and Easy is the way.." //

///////////////////////////////////////////////////////