HIGH FREQUENCY ANTENNAS

JD DELANCY, K1ZAT/3

1.  Long Wire Antennas:

a.	The fundamental wire-type antenna is the horizontal half-

wave center-fed antenna.    The nominal impedance of this type of

antenna  is about 73 ohms,  which will give a 1.5:1 standing wave

ratio (SWR) for a 50 ohm transmitter/receiver.	 Various matching

networks (delta and gamma types for example) can be used for  1:1

ratio,	but the power loss with a 1.5:1 is minimal, and the value

of messing around with matching networks is  questionable.   Like

all  balanced  antennas,  the dipole should be fed through a  1:1

balun  (balanced to unbalanced transformer).   The  problem  with

half-wave  dipoles for 75 meter frequencies (3.9-4.0 Mhz) is that

the  antenna  is approximately 120  feet  long.      The  antenna

should	be  erected horizontally as high as  possible.	 In  most

cases,	the  antenna will not be at least a 1/4 wavelength  above

ground (at 75 meters that equates to 55 to 60 feet).   This means

the  directional effect of the dipole is virtually  non-existant,

so  any geographical orientation can be used for omni-directional

use.   Dipoles are sometimes referred to as "long wire" antennas,

an  erroneous  term  since  a  true long-wire  is  at  least  one

wavelength  long.   Variations	of the half-wave  dipole  in  use

includes the commonly called "coaxial" dipole, which uses coaxial

cable  for  part of the radiating element.   The  coaxial  dipole

seems to have a better bandwidth, probably due to the diameter of

the  coax used as a radiating element versus the number 14 or  12

wire  diameter used with a regular dipole.   The half-wave dipole

provides a good match only at the frequency for which it is "cut"

and  it  has a high angle of radiation (60-80 degrees)	which  is

correct for the 100-500 mile operating range.

b.	  The  trap  dipole  is  a  variation  of  the	half-wave

horizontal dipole which provides multi-band operation.	Wave-trap

networks  are inserted along the length of the dipole and act  to

disconnect part of the antenna on higher bands to permit matching

at  different frequencies.   The characteristic impedance of  the

trap dipole is also about 73 ohms,  the same as the basic dipole.

The  length of the antenna is still the same on the lowest  band,

so  a  large amount of space is still required.   The  traps  are

frequency-dependent  networks,	and  thus  provide  a  relatively

narrow bandwidth of matched operation.	 These antennas are  also

non-directional at the heights normally used.

c.	The  inverted "V" dipole is a variation of the half-wave

dipole	that  can be used in restricted  space.   The  center-fed

portion  of the antenna is supported at a height of  40-50  feet,

and  the  radiating  elements  are ruin diagonally  down  to  the

ground.  The  angle  between  the elements should  be  about  100

degrees  for a good match to 50 ohms.	The element  lengths  are

approximately  the  same  as a	regular  dipole.   The	angle  of

radiation  is  about  50  to 70 degrees  which	is  suitable  for

operation in the 100-500 mile range.

d.   The  rotatable dipole is a center-loaded dipole about 40

feet long with aluminum conduit elements.   When mounted about 50

feet high,  where the directional effect becomes apparent, it can

be rotated and permits desired signals to be peaked, or undesired

signals to be suppressed.  Initial testing of such an antenna has

indicated  no real advantage over a dipole or inverted	"V"  that

would make it worth the time and effort.

e.	True  long-wire  antennas are at least	one  wave-length

long.  Such antennas are directional off the ends of the antenna.

If  one end of the long-wire is terminated in its  characteristic

impedance   (500-600  ohms),   the  antenna  can  be  made   uni-

directional,  obviously, a good sized backyard is needed for a 75

meter long-wire antenna (it's at least 220 feet long!!).

2.  Parasitic Arrays:

a.	The  "beam"  antenna  commonly used is	a  three-element

parasitic array - reflector,  driven element,  and director.  The

antenna  is directional and will give 3-5 DB gain.   The elements

are half-wave in length,  which means they are practical for  10,

15,  and  20 meters -- get a little unwieldy for 40 meters -- and

just  about  impractical  for  80  meters!!	The  characteristic

impedance is about 73 ohms,  with most antennas having a gamma or

delta matching network for operation at 50 ohms.

b.   The YAGI antenna is the "pure" form of the beam antenna.

YAGIs usually  have 5 to 10 elements.    The driven element in  a

YAGI  is  a folded dipole,  making the	characteristic	impedance

about  300 ohms.   A 6:1 balun is normally used with YAGIs for	a

match  to 50 ohms.   A ten element YAGI will have a gain of about

10 DB,	with a narrow bandwidth.  Again, the physical size of the

antenna  with half-wave elements is the limiting factor  in  low-

frequency usage.

c.   The  Quad  or delta-loop antenna has become  popular  in

recent	years.	 the biggest advantage of Quad antennas is  their

ability  to  provide gain with broad bandwidths.   The gain of	a

Quad  is similar to that of a beam or YAGI (with an equal  amount

of  elements).	 The  quad  is	a  directional	antenna  with  an

impedance of about 200 ohms (very approximate) and thus  requires

a matching network for 50 ohm operation.   Again,  the limitation

in  use  for HF work is the physical size,  since the Quad  is	a

square-shaped antenna with each side 1/4 wave length long, while

the delta-loop is triangular with each side 1/3 wave length long.

Needless to say, there are not many 80 meter Quads around.

3.  Vertical antennas:

a.	The  most  common vertical for fixed-station use is  the

trap vertical,	such as made by Hustler and Cushcraft.	 As  with

the  trap dipole,  this antenna uses frequency dependent networks

to  isolate  portions  of the antenna for  proper  resonating  on

different bands.   Although the antenna can be mounted at  ground

level, greatly improved performance will result from installation

on top of a 20-30 foot high mast.   The efficiency of the antenna

is  dependent  upon installation of adequate radials.	At  least

four,  cut to 1/4 wave length, should be installed for each band.

These radials can also be used as the top guy wires for the mast.

These antennas can be adjusted for a very low SWR at 50 ohms, but

have a narrow bandwidth.   The radiation angle is 30-40  degrees,

making	them perform well for long-distance operation (1500  mile

range)	while  still usable for short-distance (500 mile  range).

These are omni-directional antennas.

b.	The  single  band  vertical for 75  meter  operation  is

usually a loaded type, either with a coil at the bottom for base-

loading  or a coil in the center of the mast for center  loading.

While top-loading would be preferred,  the size and weight of the

coil  required would make for an unwieldy  mechanical  structure.

Center-loading is preferred to base-loading since a large portion

of  the  radiation  is	performed by the  coil	itself,  and  the

elevated  coil	will provide better performance  and  efficiency.

The  antenna  can be resonated for a  50  ohm  impedance.   Being

frequency-dependent,  due  to the loading coil,  the bandwidth is

relatively narrow.	Again,	the efficiency of the antenna is

highly dependent upon the ground radial system.   There should be

a minimum of four (1/4 wave) radials,  with the more, the better.

The  antenna  can be elevated on a mast for  better  performance.

The radiation angle is similar to the trap vertical.

c.	Towers utilized to elevate VHF antennas or beams can  be

loaded	and  resonated	as vertical antennas.	The base  of  the

antenna  can be grounded or insulated.	 Adequate ground  radials

are required for proper operation.   The radio amateur's handbook

shows  several methods of feeding and matching towers for use  as

vertical antennas,  even for 160 meters.  A common use for towers

is as a counterpoise for "half sloper" antennas.   This is a form

of inverted-antenna with the tower acting as one of the radiating

elements.   The  antenna is fed at the top of the tower with  the

coax  shield  connected to the tower,  and the	center	conductor

connected to the sloping element.   A "slope" of approximately 45

degrees will permit matching to 50 ohms.

d.	Due to mechanical problems involved,  the only practical

antenna  for  mobile  HF operation is a  vertical  whip  antenna.

These can be base-loaded,  center-loaded,  or trap verticals with

the body of the car acting as the counterpoise.   The  efficiency

of  such  an  antenna,	though,  is only about 5  to  10  percent

(working DX from a car usually means driving to the top of Pike's

Peak!!).   For	local (100 mile) operation,  the mobile  vertical

whip  does  a good job.   Two important considerations	apply  to

safety	in mobile antenna operation -- one,  the antenna must  be

mechanically  suitable	to  withstand the  whipping  and  shaking

involved  in  traveling  -- two,   the	antenna  height  must  be

considered in clearing overhead obstructions,  PARTICULARLY power

lines.

4.  Examples of simple design antennas:

\ctr\G5RV MULTIBANDER ANTENNA

I  I	      51 FEET

I  I

I  I

I  I

I  I   --------  300 OHM LINE

I  I	     L = 36 FT FOR

I  I	     HORIZONTAL DIPOLES

I  I

I  I	     L = 29 FEET FOR

I  I	     INVERTED VEE DIPOLES

I  I

I  I

I  I

--------------

|	     |	----------1:1 50 OHM

|	     |		      BALUN

--------------

I

I

I -------------------50 OHM COAX

I			  (RG8) ANY

I			  LENGTH

XMTR/RCVR

The  impedance	at the lower end of the 300 ohm line is 50 to  60

ohms.	A  1:1 balun is recommended for RFI or	TVI  suppression.

The antenna is slightly "short" on 80 meters and the 300 ohm line

section  acts as a sort of matching stub on that band and acts as

an  impedance transformer on the other bands.	Trim the 300  ohm

line for best SWR on 15 or 20 meters (most critical bands).   SWR

should	be 2:1 or better across any band.   SWR of 1.3:1  can  be

obtained  for the most resonant frequency in each band.   Antenna

is  good  for about 400 watts.	 For higher  power,  use  regular

amateur  transmitting open-wire line (300 ohm).   Shielded  twin-

lead  can  be used,  but length will be different  due	to  cable

propagation factor.  You will have to cut and try.

\ctr\DJ4BQ DOUBLE-DIPOLE

(55 feet)			  (55 feet)

------------------------------O    O------------------------------

-----------------------O    O-----------------------

(40.5 feet)	      I    I	  (40.5 feet)

I    I

I    I   --------200 OHM OPEN-WIRE

I    I	       LINE, ANY LENGTH

I    I

I    I

------------

I	      I   -------- 4:1 BALUN

I	      I 	   (50:200 OHMS)

------------

I

I

I    ------------- 50 OHM COAX

I		    (RG8) ANY LENGTH

I

I

XMTR/RCVR

This  antenna operates from 80 to 10 meters.   The longer  dipole

operates  on  80,  20,	and  15 meter while  the  shorter  dipole

operates on 40 and 10 meters.	SWR of 1.2:1 to 1.5:1 are typical

on each band.	This is a 200 ohm antenna and a 4:1 balun must be

used  to match it to 50 ohm coax.   Dipole wires should be spaced

at least six inches apart to eliminate interaction  of	dipoles.

Heavy-duty  twin  lead can be utilized for the	dipoles  (forming

both dipoles at the same time) but the length of the dipoles will

be  different from the standard dimensions,  and you will have to

"cut and try" for the lowest SWR.   Amateur transmitting 200  ohm

open-wire line with plastic spacers should be used,  with about a

six  inch  separation  from tower legs	or  mast.   Inverted  Vee

operation  may	require shortening of the dipole lengths  due  to

extra capacity to ground.

\ff