CINCINNATI MICROWAVE ENGINEERING REPORT

ON

TTTTTTT  RRRRRR    AAAAA   FFFFFFF  FFFFFFFF  IIIIIII   CCCCC

T     R     R  A     A  F        F            I     C

T     RRRRRR   AAAAAAA  FFFFF    FFFFFF       I     C

T     R    R   A     A  F        F            I     C

T     R     R  A     A  F        F         IIIIIII   CCCCC

RRRRRR    AAAAA   DDDDDD    AAAAA    RRRRRR

R     R  A     A  D     D  A     A   R     R

RRRRRR   AAAAAAA  D     D  AAAAAAA   RRRRRR

R    R   A     A  D     D  A     A   R    R

R     R  A     A  DDDDDD   A     A   R     R

INTRODUCTION

====================================================================

This report explains in simple language the principles and

limitations of traffic radar. It also outlines documented errors that

can cause radar to give incorrect readings. ( It is estimated that as

many as 30% of radar tickets are in error.) Every motorist should

know these facts.

PART I - HOW RADAR WORKS

====================================================================

There are many kinds of radar. Military radar, weather radar,

and air-traffic control radar fall into one category. Traffic radar

is quite different and falls into its own category.

The first category of radar is highly sophisticated

equipment. The popular concept of a radar antenna - a dish mounted on

a tower, rotating to sweep the horizon - applies to this category.

The rotating dish produces the familiar radar sweep you see on TV

weather reports. These sophisticated radars sweep with a modulated

beam to measure speed of objects, distance to those objects, and

general shap of those objects as an aid to identification by ground

crew. Such radars cost hundreds of thousands of dollars.

Traffic radar, by contrast, is very simple. It must be simple

because it can be no larger than will fit on the dashboard of a

mid-size cruiser. And it must be simple becauseit can cost no more

than the low bid of a municipality's procurement process. Sometimes

this is $400 or less.

The necessary simplicity means that traffic radar has

important limitations.

Traffic radar does not sweep. Instead it uses a stationary

beam, much like a searchlight, and it shines down the road,

either forward or backward, but not both ways at once. This means the

area under surveillanceby traffic radar is quite limited.

Traffic radar does not use a modulated beam. It uses a

constant beam. This means that, if there are multiple moving objects

within range, traffic radar is unable to distinguish between them.

Traffic radar does not have a radar screen. It has only a

single digital readout. So, at any given time, the maximum amount of

information traffic radar can provide is one number. This is a very

significant limitation, and we'll talk more about it later.

TRAFFIC RADAR: THE BASIC IDEA

Traffic radar's constant beam acts very much like a

searchlight, both in shape and in its method of operation. That's

because microwaves behave much like light waves. They travel in

straight lines and they are easily reflected. Metallic objects like

cars, trucks, guard rails and overpasses make the most effective

reflectors, sending glints of microwaves around in unpredictable

directions, just like glints of light.

Unlike light, however, you can't see the radar beam because

microwaves are invisible. But they are very easily received by a

radio tuned to microwave frequency. And, in fact, such a radio

connected to a compact antenna forms the basis of all radars.

Traffic radar works by shining its microwave searchlight down

the road. When you come in range, the beam bounces off your car, and

the radar antenna looks for the reflections.

HOW TRAFFIC RADAR MEASURES YOUR SPEED

Traffic radar calculates speed from the reflections it

receives. It uses a phenomenon of physics known as the DOPPLER

PRINCIPLE. We've all heard how the Doppler principle works with sound

waves. The classic example is heard along railroad tracks. As the

train approaches, you hear the sound at a fixed pitch. The instant

the train passes and begins to move away, you hear a lower pitch. The

train itself is making the same sound both coming and going, but to a

stationary listener, the speed of the train adds to the pitch of its

sound as it approaches, and subtracts as it departs. This change from

true pitch is called the Doppler shift, and the magnitude of the

change depends only upon the speed of the train.

Traffic radar applies this Doppler principle to microwaves.

It compares the shifted frequency of the reflection to the original

frequency of the beam it sent out, and from the difference it

calculates speed, which it then displays on its digital readout.

That's all there is to traffic radar.

HOW FAR AWAY CAN RADAR CLOCK YOU?

Going back to the searchlight analogy, we all know there's a

limit to the effectiveness of any spotlight. The more powerful it is,

the farther it reaches. The same applies to radar. Since power costs

money, traffic radar is far less powerful than military radar.

Traffic radar's low power means that it has limited range.

It's a fact of microwave life that the strenght of the beam

diminishes with distance. The farther it has to travel, the less

energy it'll have when it gets there. For example, the radar operator

may spray your car with microwaves while you're still a mile away.

But the reflection signal has to travel that same mile back to the

radar before it's of any value. If it's so weak when it gets back

that the radar's electronics can't read it, then no speed computation

is possible. You're out of range.

Radar range depends upon two things: power of the radar and

the reflectivity of the target. The amount of power is determined by

the radar's designer: to the motorist, it's not a variable.

Reflectivity of the target, however, varies with each vehicle and is

therefore of great importance to each motorist.

For highway vehicles, radar reflectivity is mostly a matter

of size and shape. The smaller the vehicle, the smaller its

reflection, and therefore the shorter the range. Some cars are out of

range on some radars until they drive within 500 feet of the antenna.

On the other hand, flat surfaces perpendicular to the beam make

excellent reflectors. The same radar that may be blind to a small car

500 feet away can see a semi a mile and a half away. From this you

should conclude that the principle of radar is quite easily

understood, but the details of its operating behavior are hard to

predict with accuracy.

PART II - TYPES OF TRAFFIC RADAR

====================================================================

All traffic radar operates according to the technical

principles outlined in Part I. But just as carpenters use different

tools for different jobs, so are there different types of traffic

radar with different capabilities. Here are the categories of

equipment:

1. STATIONARY RADAR - Virtually all radar can be used in a

stationary mode. This simply means that the radar setup operates from

a fixed position. Stationary radar can be either mounted to a car, or

it can be hand held ( the so-called "radar gun" ).

2. MOVING RADAR - This is more complex system designed to

allow the operator to check speed of patrol traffic from a moving

patrol car. The principles involved are the same as for stationary

radar; a microwave beam looks forward, monitoring the strongest

reflection, which in this case is oncoming terrain. Simultaneously

the beam monitors the second strongest reflection, which it assumes

to be traffic. An internal calculator then subtracts the terrain

speed ( same as patrol car speed ) from the closing speed of the

patrol car and the target vehical. The result - if there are no

errors - should be the road speed of the target vehical. Like

stationary radar, moving radar displays its conclusion as a single

number on a digital readout.

3. INSTANT-ON ( OR PULSE ) RADAR - This is a method of

operation rather than a type of radar equipment. Most traffic radar

can be operated in the instant-on mode. It can be used either

stationary or moving.

The sole purpose of instant-on radar is to defeat radar

detectors. The instant-on radar trap is set up just like anordinary

radar trap. The only difference is that the radar doesn't transmit

until the operator pushes a button. So there is no radar signal for a

radar detector to find.

Then when the target is within speed-measuring range, the

operator triggers the beam. Hence the term "instant-on". Instant-on

radar locks on to target speed within a fraction of a second after

it's triggered.

4. PHOTO RADAR - This is a type of stationary radar designed

to operate unmanned. It takes a photo of any car it thinks is

speeding. The owner is identified from the license number and a

ticket is mailed to the owner. This type of radar is common in Europe

but is still an experiment in the United States. In early 1988, it

was operated in two locations, pending court challenges. The

locations were Paradise Valley, Arizona, and Pasadena, California.

Photo radar is expected to face legal difficulty in the

United States because America laws hold the driver responsible for

moving violations, not the owner. And it's almost impossible to

prove, at a later date, who was driving.

PART III - WHY RADAR GETS WRONG READINGS

====================================================================

The greatest weakness of traffic radar is the way in which it

gives its information. It has only one readout and it displays only

one number. You can point the radar antenna down a two-lane road with

traffic going in both directions. Vehicals can range in size from

economy cars to semi trucks. And all the radar will show is one

number.

Where does it get its one number? And how can the operator

know which of the coming and going vehicles is responsible?

The truth is, offen the operator cannot know for sure. He has

to guess. This is the most serious of all traffic radar's limitation.

Because it's made to a low-bid price, it must necessarily be a

relatively simple device. Without the modulated beam of the military

radar, it cannot distinguish between targets within range. Without

the radar screen, it cannot identify for the operator which target

it's reading.

At least the low power of traffic radar is an asset here in

that it limits surveillance to line of sight.

GUESSING THE OFFENDER

To make up for the lack of a modulated beam and the absence

of a radar screen, traffic radar makers use the cheapest imaginable

substitute. They simply program the electronics to disregard all but

the strongest reflection in the case of stationary radar, all but the

two strongest reflections in the case of moving radar. This is the

method of all traffic radar. The number displayed on the readout

comes from simplification. It's up to the operator to decide which -

if any - of the moving vehicles within range is producing the unseen

reflection. If there is only one vehicle in range, probably that

vehicle is responsible for the number, although it could be caused

by an electrical interference or blowing trash or some other less

obvious distraction to the microwaves.

If there is more than one vehicle in range, it's up to the

operator to decide which one is producing the reflection. Is it the

closest one to the antenna, or is it the largest one in the pack? In

truth, it could be either, depending upon a host of subtleties.

A skilled operator intend on justice wouldn't write a ticket

unless he was absolutely sure. A less skilled operator might write

the ticket thinking he had the right answer, and be wrong. A careless

operator intent on filling his quota might see the number and single

out a likely operator - the red sports car - and be done with it.

When you deal with humans, you take your chances.

Because traffic radar is built to a low-cost target, it is

not the infallible electronic instrument that it purports to be. And

because operators have a tough time keeping track of invisible beams,

traffic radar invites human error in vehicle identification.

NO GOVERNMENT STANDARDS

While military radar is produced to exact government

standards to assure accuracy, traffic radar is not subject to any

government standards whatsoever.

In the late 1970's there was wide-spread puplicity about

radar errors including an 86-mph tree and a 28-mph house. The

National Highway Traffic Safety Administration ( NHTSA ) assigned to

the National Bureau of Standards the task of testing all brands of

traffic radar in use at the time, for the purpose of discovering the

source of these errors and proposing federal standardsto eliminate

them. In January 1981, these proposed standards were published in the

Federal Register. The newly-installed Reagan administration book took

no action on the proposal.

After three years of government inaction on the problem, the

International Assocition of Chiefs of Police ( IACP ) provided

non-government standards by which all traffic radar units could be

tested to assure accuracy: Volume I of the standards was published in

April, 1984, Volume II in June, 1984.

In June 1986, the traffic radar manufacturers announced the

formation of their own trade association, saying that they would not

submit traffic radar units for IACP testing. Instead, the radar

makers would use their own standards.

The industry standards have not been published thus far. So,

in effect, there are no performance requirements for traffic radar,

and the claims of 86-mph trees and 28-mph houses have never been

refuted.

PART IV - HOW RADAR DETECTORS WORK

====================================================================

A radar detector is, in essence, a radio tuned to receive

traffic radar frequency.

The technology involved is straightforward. Remember that

traffic radar transmits a microwave beam out through an antenna,

then listens for the reflection of the transmission. A radar detector

amounts to the listening section of a radar unit.

The key question is, Can a radar detector find radar before

radar finds you? The answer is yes. Consider the worst-case scenario

for pure range. You're driving down a straight and level highway,

heading right for the radar trap. The beam is reaching out, feeling

for you. But remember that microwvaes lose energy as they cover

distance. And to measure your speed, the beam has to reach out to

you, be reflected back, and then arrive at the radar unit with enough

strenght to activate the calculating circuits.

Let's say you come in range when you're a half mile away. The

microwave beam traveled a half mile out to you and a half mile back

to the radar. That's a mile of total travel. If your detector has the

same sensitivity as the radar's listening section, you can find the

radar a half mile before it finds you.

It is not at all difficult to build a detector as sensitive

as the listening section of the traffic radar. In fact, the

electronic components used in traffic radar are standard industry

components available to anyone.

Of course, radar is not always used out on the open road

where a detector can get a good look at it. Radar operators favor the

ambush; they hide over a hill or around a curve, and when you pop

into view, you're already in range. So how does a detector work in

this case?

The classic analogy for the radar beam likens it to a

flashlight on a foggy night. The guy holding the flashlight can't see

you until the beam scores a direct hit. But you can see the beam

tracing its pattern through the fog long before it falls on you. You

can see it because the light reflects off moisture, dust and other

objects in its path, sending off small glints of light easily seen

from a distance.

Microwave beams behave like light beams. They operate on a

strict line-of-sight basis. You can't be "seen" over a hill or around

a curve or behind a semi. Radar has to wait for you to drive into

view. But a detector can pick up the scatter from the microwave beam

well before the beam itself hits you directly.

Again, a receiver as sensitive as the traffic radar's will be

sufficient. But unlike the radar, a detector doesn't need to

calculate speed. It just needs to know that radar waves are present.

So a detector can act on a weaker signal than that necessary for the

radar itself.

This is not to say that detectors are infallible. Radar

operators have one trick that's hard to defend against. This is radar

used in the "instant-on" mode. Here a hidden radar is kept in standby

condition as a target approaches. Its internal ciruits are warmed up

and ready, but it is not emitting a beam. So, naturally a detector

can't find it. Then when the target vehicle is well within range, the

operator switches the unit on and it quickly calculates speed.

A good radar detector protects against instan-on radar by

warning you when the operator zaps one of the cars up ahead. It has

to be sensitive enough to respond to weak signals. Moreover, it must

be reliable enough so you don't interpet these brief, weak signals as

random false alarms and disregard them. Of course, if there is no car

ahead, you're on your own.

A good radar detector, like any sophisticated electronic

equipment, requires careful engineering. You will have the best

chance of satisfaction if you buy from a reliable source. Pick a firm

that will refund your money if you're not satisfied. There is no

better guarantee.

COMMON OPERATOR ERRORS

====================================================================

The proposed Federal standards for traffic radar were never

implemented. However, in an attempt to reduce radar errors, several

local governments used the Federal research to develope better

training programs.

The Texas Department of Puplic Safety produced a

comprehensive manual based on the Federal tests. It cautions

operators, "...the radar does not generate `false' readings. Anytime

a reading appears, the radar has sensed a signal. The radar operator

must be familiar with situations that can produce `error' readings."

If the operator does not detect the error, a ticket will be

wrongfully issued.

Here are the radar "errors" detailed by the Texas manual:

1. ANTENNA POSITIONING ERROR - The radar beam travels in a

straight line, neither bending around curves nor following the

contour of hilly terrain. If the antenna is not properly positioned,

it may seem to clock an approaching car when, in fact, it's clocking

another car in the background.

2. LOOK-PAST ERROR - Even if the operator aims his antenna

properly, radar is still subject to "look-past" error. this is caused

by the radar looking past a small reflection in the foreground to

read a larger reflection behind. This error is all the more insidious

because poorly-trained operators assume it can't happen.

Texas instructors warn, "It is a widely-held misconception

that the reflected target signal received by the radar antenna will

always be that of the closest vehicle to the antenna. There are many

times, due to traffic conditions, that the closest vehicle is not

returning the strongest signal."

Evidence of the potential size of this error appeared in CAR

AND DRIVER ( October, 1979 ). The author measured the effective range

of a Kustom Signals KR11 traffic radar against various vehicles. The

typical small sedan did not show up on the radar until it was less

than 1200 feet away from the antenna, but the same radar unit locked

on to a Ford 9000 semi at 7600 feet. This shows how common vehicles

reflect microwaves differently.

The Texas instructors confirm this problem with radar, saying

" It is not unfair to say that the reading you register could be a

larger, better target three-quarters of a mile down the road."

3. VEHICLE INTERFERENCE ERROR - Because moving radar tries to

do a more complicated job than stationary radar, it is subject to all

the errors of stationary radar, plus several additional errors that

apply to it alone.

"Vehicle Interference" error occurs when moving radar is used

in traffic. For example, traffic ahead can confuse the radar's

estimate of patrol speed. Moving radar calculates target speed by

subtracting patrol speed  from closing speed of the target. Therfore

anything that produces a low evaluation of patrol speed will

automatically result in a high reading of target speed.

Texas tells its radar operators that this... " situation

becomes more critical if the difference in patrol speed and

interference-vehicle speed is five to ten mph. A target vehicle

moving 61 mph may be recorded at 66-71. These borderline speeds are

more difficult to detect with the eye."

4. COSINE ERROR - Cosine error produces a result similar to

Interference error except no moving traffic need be present. A

stationary object adjacent to the road, such as a building, or road

machinary, or even a sign, makes a more efficent reflector than

horizontal pavement. Therefore the radar uses that reflection as a

basis of patrol speed.

If this reflector were positioned straight ahead on a

collision path, the patrol speed estimate would be close enough. But

the further the object is located off a direct line to the target,

the lower will be the estimate of the patrol speed. This is a simple

trigonometry problem relating to the cosine of the angle between the

target and the ground reflector, hence the name Cosine error. Since

Cosine error always makes the patrol speed seem smaller than it

actually is, it always acts to raise the reading of the target speed.

5. DOUBLE-BOUNCE ERROR - Microwavesare easily reflected.

That's what makes radar possible. But the operator must be aware of

the difference between an ordinary reflection and a bad bounce. Big

objects such as trucks are very efficent reflectors, and it's

possible for the radar beam to bounce off several moving trucks at

once, always producing erroneous readings.

6. BEAM-REFLECTION ERROR - Because microwaves are so readily

reflected, Texas instructors recommend caution, even in mounting the

antenna within the patrol car. They say it's possible that a

reflective path can be set up through the rearview mirror that will

produce radar readings on vehicles behind the patrol car when the

radar is aimed forward. And those vehicles behind can be either

coming or going, since radar does not distingguish direction.

7. ROAD-SIGN ERROR - The ready reflectability of microwaves

means that road signs are also be a source of errors.

8. RADIO-INTERFERENCE ERROR - According to the Texas course,

" UHF radio now in use can force radar to read various numbers when

you transmit, or just key the mike. Citizens band radio transmissions

from within the patrol vehicle can cause ghosting ( false readings )."

It recommends that no radio transmissions be made while clocking

target vehicles.

9. FAN-INTERFERENCE ERROR - When the antenna is mounted

inside the patrol car, the Texas course says, " Radar will have a

tendency to read the pulse of the fan motor ( air conditioner,

heater, or defroster )." The instructors go on to say, however, that

the fan reading will disappear when a target comes into range, and

that the fan will not distort the speed reading of the target car.

However, in the case of moving radar, they say " Sometimes a steady

fan will override patrol car speed reflected from the roadway." This

error is particularly egregious because the fan speed will be

substituted for patrol speed in the moving radar's calculation of

target speed. Since this calculation consists of subtracting patrol

speed from closing speed, if the fan reading is less than patrol

speed, then the speed displayed for the target will be incorrectly

high. The Texas course offers no safe guard for this error.

In conclusion, the Texas Department of Puplic Safety notes

" Radar cannot identify ( the ) speeding vehicle: ( the ) officer

must do that."

For further information on the Passport or Escort Radar

Detectors, Traffic Radar or an illustrated, color copy of this

report, contact:

CINCINNATI MICROWAVE, INC.

One Microwave Plaza

Cincinnati, Ohio 45249-9502

1-800-543-1608##############################################################################