Converting a Tone Dialer into a Red Box

---------------------------------------

By Noah Clayton

A very simple modification to Radio Shack pocket tone dialer part

#43-141 ($24.95) can make it into a red box.  The modification

consists of changing the crystal frequency used to generate the

microprocessor's timing.  To make this modification you will need

a Phillips screwdriver, a flat bladed screwdriver, a soldering

iron, a pair of long nose pliers, a pair of wire cutters and a

6.5536 MHz (megahertz) crystal.

Orient the dialer with the keypad down and the speaker at the

top.  Remove the battery compartment cover (and any batteries) to

expose two screws.  Remove these two screws and the two on the

top of the dialer near the speaker.  There are four plastic clips

that are now holding the two halves of the dialer together.  Push

on the two bottom clips near the battery compartment and pull up

to separate the bottom part.  Now slide a flat screwdriver into

the seam on the left starting from the bottom and moving towards

the top.  (You may have to do this on the right side as well.)

When the two halves separate, slide the speaker half underneath

the other half while being careful not to break the wires

connecting the two.  Locate the cylindrical metallic can (it's

about half an inch long and an eighth of an inch in diameter) and

pull it away from the circuit board to break the glue that holds

it in place.  Unsolder this can, which is a 3.579545 MHz crystal,

from the circuit board.

The hard part of this modification is getting the new crystal to

fit properly.  Bend the three disk capacitors over so that there

will be room for the new crystal.  You may also need to remove a

screw.  Since the 6.5536 MHz crystal you have is probably much

bigger than the crystal you are replacing, you will need to bend

the leads on the new crystal so that they will match up with the

pads on the circuit board.  Place the new crystal on the circuit

board, making sure that it will fit once you replace the bottom

half.  Solder the new crystal in place.  As an added touch you

might peel the QC sticker off of the PC board and place it on top

of the crystal.  Now carefully snap the two halves back together

while checking to make sure that none of the wires are getting

pinched or are in the way of the screw holes. Put the case screws

back in and insert three AAA batteries into the battery

compartment.

Your dialer is now ready to test.  Switch the unit on.  The LED

on the dial pad should be lit.  Set the lower slide switch to

STORE mode.  Press the MEMORY button on the dial pad.  Press the

* key five times.  Press the MEMORY key again and then press the

P1 key.  A beep tone should be heard when any key is pressed and

a long beep should sound after the P1 key has been pressed to

indicate that the programming sequence was performed correctly.

Switch the unit into DIAL mode.  Press the P1 key, and five tone

pulses that sound remarkably like coin tones should come out of

the speaker.  I usually program P1 to be four quarters (insert

one or two PAUSE's between each set of five tones), P2 to be two

quarters, and P3 as one quarter.

Of course, you can no longer use the unit to generate touch

tones.

History and Theory

------------------

A friend of mine and I were sitting around his house one day

trying to come up with a way to build a reasonable red box.  I

had built one with analog sine wave generators in the past, but

it was difficult to adjust the frequency of the outputs and keep

them accurate over time and with changes in temperature.  The

electronic project box I had assembled it in was bulky, hard to

conceal, and definitely suspicious-looking.

My friend was playing with his calculator while I was wishing we

had the money and time to design a microprocessor-controlled

device with its own custom PC board.  After a while, he announced

that he had an idea.  He had been looking at a data sheet for a

DTMF (Dual Tone MultiFrequency aka touch tone) generator chip. He

calculated the ratio of the coin tone frequencies of 1700 Hz and

2200 Hz to be .7727.  He then went through all of the tone pairs

used for DTMF, calculating each of their ratios. He discovered

that the ratio of the tone pair used for * was very close to the

ratio for the coin tone frequencies.  This ratio, 941/1209 =

0.7783, differed from the coin tone ratio by less than one

percent.

What this meant was that since the tones generated by such a chip

are digitally synthesized from a divider chain off of a reference

crystal, if one changed the reference crystal to the "right"

frequency, the coin tones would be generated instead of the DTMF

*.  Most DTMF chips use a TV color-burst crystal with a frequency

of 3.579545 MHz.  To determine the crystal frequency that would

generate the coin tones, one would compute:

3,579,545 / 941  * 1700 = 6,466,766 MHz

3,579,545 / 1209 * 2200 = 6,513,647 MHz

(6,466,766 + 6,513,647) / 2 = 6,490,206 MHz

Unfortunately, this is not a standard crystal value and getting

custom crystals made is a real pain for the hobbyist.  The

closest I could find was 6.5536 MHz.  I tried a crystal of this

value and it worked.

(The actual frequencies produced by a DTMF generator chip depend

on the particular manufacturer's design.  The color-burst

crystal's frequency is divided down to DTMF tones by an integer

divider chain.  Because the color-burst crystal's frequency is

not an integer multiple of the DTMF tones there will be a small

difference in the frequencies produced from the standard.)

When we first tried this, we were using one of Radio Shack's

earliest tone dialers.  It consisted of a DTMF generator chip

only, and as such could not produce a sequence of tones

automatically.  Tones were generated as long and as fast as one

could press the buttons.  We were able to simulate nickels using

this device but doing so was fairly slow and tedious.  Because

our manual timing was so far off of the mark, our attempts at

producing dime or quarter signals were a miserable failure.  A

live operator would be instantly connected to the line whenever

we tried it.

The Shack's next model had a microprocessor and a tone generator

in it, each with separate crystals controlling their respective

timing.  It was just a matter of changing the micro's crystal to

get the right on-off timing for a quarter's timing for a

quarter's tone sequence as well as the tone generator's crystal

to get the proper coin frequencies.

Later Radio Shack came out with the model used in this project.

I promptly bought one because it was lower cost and more compact

than their older model.  I put some batteries in it and tried it

out.  It generated DTMF sequences with very long on and off

times, but other than that, seemed like a nice unit.  Upon

disassembling it though, I became unhappy.  There was only one

crystal.  It controlled the timing for a microprocessor that was

specifically designed to synthesize DTMF.  There was no way to

independently adjust the output frequency of the tones from their

on-off timing.  I was just about to say, "Oh well, yet another

tone dialer for my collection" when it hit me.  Why not try the

higher frequency crystal?  The timing might come out close enough

to simulate either a quarter or a dime.  I made the mod and

tested it out.  It worked!

Thank you Radio Shack, for giving us a convenient to use, easily

concealable and non-suspicious-looking red box.

Reference

---------

The crystal is available from Fry's Electronics in Freemont, CA

for $0.89 plus the charge for UPS Red or Blue. Their number is

415-770-3763. I would suggest buying five, some for future use

and some just in case you cut the leads too short when trying

this project.

Coin Frequencies

----------------

1700 Hz and 2200 Hz +/- 1.5%.

Timing

------

5 cents:  One tone burst for 66 ms (milliseconds) +/- 6 ms.

10 cents: Two tone bursts each 66 ms, with a 66 ms silent period

between tones.

25 cents: Five tone bursts each 33 ms +/- 3 ms with a 33 ms

silent period between tones.

[Originally published in 2600 magazine, Vol. 7 #3, Autumn 1990]