CPE INTERFACE REQUIREMENTS FOR

SINGLE-STAGE INBAND ANI DELIVERY

018-0035-01.1F-ER

MCI

NETWORK ARCHITECTURE AND REQUIREMENTS ENGINEERING

NETWORK STANDARDS AND ARCHITECTURE ENGINEERING

August 8, 1989

Revision 1.1

*** FINAL ***

Copyright  MCI COMMUNICATIONS CORPORATION 1989

PROPRIETARY INFORMATION OF

MCI TELECOMMUNICATIONS

Network Standards and Architecture Engineering

018-0035-01.1F-ER

REVISION HISTORY

REV       DATE      AUTHOR         REASON FOR CHANGE

1.0       6/6/89    A. Patel       Draft

A. Merchant

1.1       8/8/89    A. Patel       Final

A. Merchant

PROPRIETARY INFORMATION OF

MCI TELECOMMUNICATIONS

TABLE OF CONTENTS

SECTION                                                  PAGE

1.0  Introduction                                        1-1

2.0  Overview                                            2-1

2.1  ANI Applications                                  2-2

3.0  Terminology                                         3-1

4.0  Signaling Sequence                                  4-1

4.1  ANI Delivery Using MF Signaling                   4-1

4.2  ANI Delivery Using DTMF Signaling                 4-3

5.0  Timing, Tolerances and Restrictions                 5-1

5.1  Hook Flash                                        5-1

5.2  Hits                                              5-1

5.3  Wink Start Operation                              5-1

5.3.1  Wink Start Guard                              5-2

5.4  Answer                                            5-2

5.5  Incomplete Calls                                  5-2

5.6  Glare Resolution                                  5-2

5.7  Register Signaling                                5-3

5.7.1  MF Characteristics                            5-4

5.7.2  DTMF Characteristics                          5-6

5.8  Audible Signals                                   5-8

5.9  Disconnect                                        5-8

5.9.1  Busy Guard Time                               5-8

LIST OF FIGURES

FIGURE         TITLE                                     PAGE

4-1            ANI Delivery Using MF Signaling           4-2

4-2            ANI Delivery Using DTMF Signaling         4-4

LIST OF TABLES

TABLE          TITLE                                     PAGE

5-1            MF Characteristics                        5-4

5-2            DTMF Characteristics                      5-6

1.0  INTRODUCTION

This document addresses the technical requirements for PBX/ACD

vendors who wish to interface with MCI switches to receive ANI.

Delivery of this ANI between the MCI terminating switch and the

Customer Premise Equipment (CPE) will be inband, and will be

supported on dedicated/hard-wired terminations only.

2.0  OVERVIEW

Automatic Number Identification (ANI) relates to the identification

of the telephone number of the calling party at the premise of the

called party.  This document specifies the technical requirements

necessary for the interface between the MCI terminating switch and

the CPE.

Initially, MCI will deliver real time ANI for 800 and 900 service

calls to the CPE of the called party over dedicated trunks.  The

terminating switch will outpulse this ANI to the CPE using a

single-stage signaling sequence.  See Section 4.0 for details.

It should also be noted that:

(1)  If a call originates in a foreign country, no ANI or

originating country code will be delivered to the PBX/ACD.

(2)  If a 800/900 call overflows to an LEC network for call

completion, no ANI will be delivered to the terminating LEC and

PBX/ACD.

(3)  If the MCI switch does not receive the full 10-digit ANI from

the LEC, i.e. if the call originates from a non-conforming end

office, then the Originating NPA + 555 - 5555  will be outpulsed to

the CPE.

If a specific customer subscribes for the real time ANI feature,

ANI will be delivered to the actual termination of that call.  In

order to receive the ANI, customers need to ensure that all

possible dedicated trunks and terminating PBX/ACDs, as specified in

the routing control, are equipped with ANI receiving and processing

capabilities.

2.1  ANI Applications

Some of the current and potential applications are:

(1)  Telemarketing:  This generic term includes locating dealer,

customer service and order taking.

(2)  Financial services:  This is a particularly large application

including customer service, automated balance/credit inquiry,

financial information retrieval and funds transfer.

(3)  Home Shopping:  ANI could be used as an identifier to recover

the called party profile and appropriately route the call.  Payment

records could be checked.

However, a few important limitations must be noted at this point.

(1)  As pointed out earlier, ANI cannot be delivered for calls

originating from non-conforming end offices.

(2)  A customer could originate a call from a telephone with a

number different from that customer's home or office telephone

number.  Hence no check of the customer's record can be performed

in this case.

3.0  TERMINOLOGY

The following abbreviations, acronyms and terms have been used

throughout this document.

( )            Digits in parenthesis may or may not have been

dialled or outpulsed.

3/10           3 or 10 digits.

ACD            Automatic Call Distributor.

ANI            Automatic Number Identification.

CPE            Customer Premise Equipment.

DTMF           Dual Tone Multi Frequency.

KP             Start Digit (Key Pulse).

LEC            Local Exchange Carrier.

MF             Multi Frequency.

NPA            Numbering Plan Area

ST             Stop digit.

TLP            Transmission Level Point.

4.0  SIGNALING SEQUENCE

This section describes the signaling sequence for the interface

between the terminating MCI switch and the CPE.  Real time ANI will

be delivered to the 800/900 Service customer during the call setup

process.

4.1  ANI Delivery Using MF Signaling

If the MCI switch determines that ANI is to be delivered for a

particular call, the switch will perform single-stage signaling as

specified below.  Refer to Fig. 4-1 on page 4-2.

A.   The MCI terminating switch will seize a trunk to the CPE.

B.   The CPE shall respond to the trunk seizure with a wink start

signal when ready to receive pulsing.

C.   On receipt of the wink start signal from the CPE, the MCI

switch will outpulse the 10 digit ANI (KP + 10D ANI + ST).

D.   If ANI is not received, the switch will outpulse

KP + NPA + 555 - 5555 + ST.

E.   When the telephone/terminal answers, an answer off hook signal

shall be sent from the CPE to the MCI terminating switch.

F.   The MCI switch will cut through the voice path when it

receives an answer off hook from the CPE.

G.   If the CPE transmits an on-hook towards MCI for at least 350

ms, the call will be disconnected.  Similarly, the MCI switch

may disconnect the call by transmitting an on-hook for at

least 350 ms.

*Note: If the MCI switch does not receive the ANI digits from the

LEC, the switch will outpulse KP + Originating NPA + 555 -

5555 + ST to the CPE using MF Signaling.

Figure 4-1

ANI DELIVERY USING MF SIGNALING

4.2  ANI Delivery Using DTMF Signaling

If the MCI switch determines that ANI is to be delivered for a

particular call, the switch will perform single-stage signaling as

specified below.  Refer to Fig. 4-2 on page 4-X.

A.   The MCI terminating switch will seize a trunk to the CPE.

B.   The CPE shall respond to the trunk seizure with a wink start

signal when ready to receive pulsing.

C.   On receipt of the wink start signal from the CPE, the MCI

switch will outpulse the 10 digit ANI.

D.   If ANI is not received, the switch will outpulse NPA + 555 -

5555.

E.   When the telephone/terminal answers, an answer off hook signal

shall be sent from the CPE to the MCI terminating switch.

F.   The MCI switch will cut through the voice path when it

receives an answer off hook from the CPE.

G.   If the CPE transmits an on-hook towards MCI for at least 350

ms, the call shall be disconnected.  Similarly, the MCI switch

may disconnect the call by transmitting an on-hook for at

least 350 ms.

*Note: If the MCI switch does not receive the ANI digits from the

LEC, the switch will outpulse the Originating NPA + 555 -

5555 to the CPE using DTMF Signaling.

Figure 4-2

ANI DELIVERY USING DTMF SIGNALING

5.0  TIMING, TOLERANCES AND RESTRICTIONS

This sections describes the timing, tolerances and restrictions

that are applicable to the MCI switch/CPE interface.

5.1  Hook Flash

Hook flashes shall not be passed to the MCI switch.  All hook flash

timing shall be handled internally by the CPE.

5.2  Hits

A short change in the received supervisory state is called a hit.

All hits shall be ignored.  A hit may be on hook or off hook and is

dependant on conditions as specified below:

A.   The CPE shall interpret an off hook of less than 150 ms as a

hit and not a seizure signal.

B.   The MCI switch will interpret an off hook of less than 70 ms

as a hit and not a wink start signal.

C.   An on hook duration of less than 200 ms shall be interpreted

as a hit.

5.3  Wink Start Operation

The off hook wink (wink start) shall be sent as soon as the CPE

detects a seizure signal.  The CPE shall return the wink start

signal within 3.5 seconds of the trunk seizure.  However, it shall

not be provided earlier than 100 ms after receiving the seizure

signal.  The off-hook period shall be a minimum of 140 ms and

maximum of 290 ms.  The MCI switch will interpret an off hook

signal in the range of 100 ms to 350 ms as a wink signal.  If the

MCI switch receives an off hook signal lasting over 350 ms on a two

way trunk, a glare condition will be interpreted.  Glare resolution

is discussed in Section 5.6.

5.3.1  Wink Start Guard

The CPE must be prepared to receive Multi Frequency (MF) or Dual

Tone Multi Frequency (DTMF) digits 35 ms after the end of the wink-

start signal.  The MCI switch will wait for 50 ms after the end of

the wink-start signal before initiating the MF or DTMF pulsing.

Characteristics of these signals are defined in Section 5.7.

5.4  Answer

The CPE shall provide a steady off-hook state indicating called

party answer.

5.5  Incomplete Calls

If a time out occurs while waiting for a response from the CPE, the

MCI switch shall treat the call as follows.  If the CPE responds to

seizure on a trunk with an off-hook longer than 350 ms instead of a

wink-start signal, a false answer condition is indicated.  The MCI

switch shall try to set up another call using a different circuit

within the same trunk group.  If an idle circuit is not available,

the switch will try to set up the call on a different trunk group,

failing which all other routes shall be tried.  If all of the above

fail, the MCI switch shall disconnect the call and return a reorder

towards the calling party.

5.6  Glare Resolution

Glare is a condition resulting from seizure attempts for the same

two way access line at about the same time by both the MCI switch

and the CPE.  If the returned off hook lasts longer than 350 ms

instead of a wink start signal, a glare condition will be

interpreted.

The MCI switch will always be the controlling end when a glare is

detected.  The CPE shall maintain the outgoing off hook state while

preparing for reception of address signals from the MCI switch.

When ready to receive digits, the outgoing off hook signal shall be

changed to on hook.  The outgoing call initiated by the CPE shall

be re-tried on another trunk.

5.7  Register Signaling

The MCI switch supports the following two types of address

signalling to deliver ANI.

A.   Multi Frequency (MF); and

B.   Dual Tone Multi Frequency (DTMF).

5.7.1  MF Characteristics

One of the two types of address signaling that the MCI switch will

support is the Multi Frequency (MF) type.  Conformity with the

criteria indicated in this subsection will ensure compatibility of

the signals between the MCI switch and the CPE.

The MF pulsing system consists of transmitting and receiving

various combinations of two, and only two, out of five frequencies

in the voice band.  Each combination of two frequencies represents

a pulse and each pulse represents a digit.  This system provides

sufficient unique combinations to encode 10 digits.  A sixth

frequency is provided for additional control signals.  Table 5-1

identifies the signaling codes.

Digits      Frequencies

(Hz)

1

2        700  +  900

3        700  +  1100

4        900  +  1100

5        700  +  1300

6        900  +  1300

7       1100  +  1300

8        700  +  1500

9        900  +  1500

0       1100  +  1500

KP      1300  +  1500

ST      1100  +  1700

Not Used   1500  +  1700

Not Used    700  +  1700

Not Used    900  +  1700

1300  +  1700

Table 5-1

MF CHARACTERISTICS

5.7.1.1  MF Sender Characteristics

The CPE MF sender shall have the following characteristics.

A.  KP digit duration shall be:                   90-120 ms.

B.  All other digit duration shall be:            58-75 ms.

C.  Inter digit duration shall be:                58-75 ms.

D.  Frequency accuracy shall be within:           + 1.5% of

nominal.

E.  The two frequencies of each code shall start and end within: 1

ms (of each other).

F.  Power output per frequency:          -7 +1 dBm at 0 dB TLP.

G.  Power difference between the two frequencies shall not exceed:

1 dB.

H.  The total power of extraneous signal components shall be

suppressed to:  at least 30 dB below the power level.

5.7.1.2  MF Receiver Characteristics

The CPE MF receiver shall have the following characteristics.

A.  KP digit signal duration shall be:            55 ms.

B.  Digit duration shall be:                      30 ms (min.).

C.   Inter digit duration shall be:               25 ms (min.).

D.  Frequency accuracy shall be within:           +(1.5% + 5 Hz) of

nominal.

E.  The two frequencies of each code shall start and end within: 4

ms (of each other).

F.  The receiver shall accept tones within the power levels of:  0

to -25 dBm.

(per frequency).

G.  Power difference between the two frequencies shall not exceed:

6 dB.

5.7.2  DTMF Characteristics

The second type of address signaling that the MCI switch will

support is the Dual Tone Multi Frequency (DTMF) type.  Conformity

with the criteria indicated in this subsection will ensure

compatibility of the signals between the MCI switch and the CPE.

The DTMF provides for 16 unique signals.  Each signal is composed

of two voice band frequencies, one from each of two mutually

exclusive frequency groups consisting of four frequencies each.

Table 5-2 identifies the signaling codes.

HIGH-GROUP FREQUENCIES (Hz)

1209      1336      1477      1633

697      1         2         3     Not Used

LOW

770      4         5         6     Not Used

GROUP

852      7         8         9     Not Used

FREQUENCIES

941      *         0         #     Not Used

Table 5-2

DTMF CHARACTERISTICS

5.7.2.1  DTMF Sender Characteristics

The CPE DTMF sender shall have the following characteristics.

A.  Digit duration shall be:                      50 ms.

B.  Inter digit duration shall be at least:       50 ms.

C.  Time period (cycle time) shall be at least:   100 ms.

D.  Frequency accuracy shall be within:           + 1.5% of

nominal.

E.  Power output per frequency:                   -7 +0.5 dBm at 0

dB TLP.

F.  The total power of extraneous signal components shall be

suppressed to:                                    at least 30 dB

below the power level.

5.7.2.2  DTMF Receiver Characteristics

The CPE DTMF receiver shall have the following characteristics.

A.  Digit duration shall be:                      50 ms.

B.  Inter digit duration shall be at least:       50 ms.

C.  Time period (cycle time) shall be at least:   100 ms.

D.  Frequency accuracy shall be within:           + 1.5% of

nominal.

E.  Power output per frequency:                   -7 +0.5 dBm at 0

dB TLP.

F.  The receiver shall accept tones within the power levels of:  0

to -25 dBm.

5.8  Audible Signals

The audible call progress tones provided by MCI will have the

following characteristics:

A.  Busy Tone:           480 Hz plus 620 Hz interrupted at 60 IPM

at a level of -24 dBmO per frequency.

B.  Reorder Tone (Fast Busy): 480 Hz plus 620 Hz interrupted at 120

IPM at a level of -24 dBmO per frequency.

C.  Ringback Tone:       440 Hz plus 480 Hz (two seconds on, four

seconds off) at a level of -19 dBmO per

frequency.

5.9  Disconnect

To disconnect a call the MCI switch will transmit an on hook

towards the CPE for at least 350 ms.  Conversely, the CPE may

disconnect a call by transmitting an on hook towards the switch for

at least 350 ms.

5.9.1  Busy Guard Time

The timed interval used to ensure trunk release before re-seizure

is called the guard time.  The trunk shall be held busy for that

300 ms interval and the station shall not accept an incoming call.

###