This is an actual document from BELL LABS and is presented by P-80

systems and is presented in its entirety!!!

CARD-READING PUBLIC STATIONS REQUIREMENTS Bell Communications Research,

Inc.

1. This document describes generic requirements for card-reading public

telephone stations. These stations will be deployed by a Bell Operating

Company (BOC) primarily to provide customers having magnetic-stripe

cards with easier access to both BOC and inter-LATA carrier (IC)

facilities.

A BOC Card-Reading Public Station (alternatively referred to as

"station") is intended to work similarly to a current Charge-a-Call

station with features added to read a magnetic-stripe card and

conveniently select an IC. The BOCs expect to issue Calling Cards for

use in making intra-LATA toll and local calls over the BOC networks, and

inter-LATA calls over any carrier capable of accepting a Calling Card

number. Similarly, calls billed via Commercial Credit Cards (CCC) would

be possible if the involved BOC or IC were able to accept them.

1.1 Outline of Document

Section 1.2 describes the scope of the document. The general operation

of the station is described in section 1.3.

Section 2 specifies the requirements for a card-reading public telephone

station. Section 2.1 sets the requirements for card presentation,

including how and when the card should be presented, and what the

station should do if the presentation of the card differs from that

required.

Section 2.2 specifies the requirements for signaling by the customer.

This includes acceptable dialed destination numbers, methods of

selection of the IC, and anti-fraud features of the station.

Section 2.3 specifies the requirements on how the station will interface

with the end (local) office. The features of the line and electrical

characteristics are described.

Section 2.4 describes the interface between the station and the IC or a

processor. This includes description of the access environment and what

the station should do to help set up calls.

Section 2.5 specifies the physical characteristics of the station.

Section 3 contains requirements pertaining to the support that the

supplier should provide for these stations.

1.2 Scope of Document

This document provides functional requirements for a card-reading,

non-coin public station. The requirements are intended to describe what

the station does in its various interactions with the customer, the

local office, and the carrier of choice or an associated processor. The

requirements also describe the physical environment in which the station

operates. This document is not intended to dictate how given functions

are specifically realized.

Card reading transactions described here are limited to magnetic-stripe

card technology and cover only those actions required to set up and

properly bill a call. These requirements are generally limited to the

station itself. Actions by other facilities to provide public calling

via credit cards are included only insofar as they relate to pertinent

station functions.

This document contains requirements primarily concerning public

stations, card reading, and carrier selection. The station should also

meet requirements specified by the FCC Rules regarding registration of

telephone sets (Part 68), FCC Rules regarding radiated emissions (Part

15), requirements concerning the normal station to network interface,

and requirements concerning abnormal conditions on the local loop.

Human factors of the station and the related service have not been, for

the most part, specifically addressed. Good human factors design,

however, is crucial to a successful product, and underlies many of the

requirements.

1.3 General Operation of the Station

This document describes a station similar to Charge-a-Call stations,

with features added to read a magnetic-stripe card and conveniently

select an IC to handle the call. This station needs to provide four

basic functions:

Voice-band transmission and reception,

Addressing the far end party,

Selection of a carrier,

Relaying appropriate billing and call status information.

The first two functions are common to all telephone stations. Carrier

selection is a relatively new function and one of increasing importance.

Relaying of billing information is a function inherent in public

stations.

The station described in this document will operate as a typical modern

telephone with respect to voice-band transmission and reception.

Addressing the far end will be done by standard Dual-Tone

Multi-Frequency (DTMF) signals.

The station will provide the customer at least one way to select an IC.

One way lets the customer dial a particular sequence of digits on the

DTMF key pad. This includes speed code dialing or 7-digit access numbers

for carriers, as well as equal access carrier codes (10XXX). A second

way allows the customer to select certain ICs by pressing a single

button on a button field distinct from the DTMF key pad. This is

sometimes referred to a "Select-a-Carrier" or Choose-a-Carrier" feature.

The station will have to translate the single button activation into an

appropriate sequence of DTMF signals, but this translation will be

largely transparent to the customer. A third potential way that could be

implemented in combination with the first two ways would have an IC code

on the magnetic stripe of a Calling Card. This Calling Card could be

issued by a BOC or by the IC. If the BOC issued the Calling Card, the

call would be directed to that carrier for inter-LATA calls unless the

customer override s. If the IC issued th e Calling Card, the carrier

selection could not be changed.

The station will provide the customer three ways of entering billing

information. The first tow ways are available on today's Charge-a-Call

stations and involve entering information by voice to an operator, or by

use of the DTMF key pad. The third way allows the customer to enter

billing information by physically presenting a magnetic-stripe credit

card to a card-reading device in the station.

In general, the station will need enough intelligence to perform the

following functions:

Accept carrier selection and billing information from the customer.

Store some of that information for later use in completing the call, if

it is not immediately needed.

Communicate billing and/or addressing information to carriers or

associated processors, possibly according to more than one protocol.

2. Station Requirements

Two types of features of the station are described in this document. The

word "should" indicates a mandatory requirement. "It is desirable"

indicates a non-mandatory feature. These requirements are dynamic and

could be influenced by time, technology, market strategy, or economics.

2.1 Card Reader

The card reader and its associated electronics needs to provide four

basic functions: accept the card from the customer, read it, and get it

safely back to the customer; perform certain checks on the data encoded

on the magnetic stripe; separate the card data into meaningful data

fields and store each field for use when and if it is needed; and notify

the customer about errors or invalid cards.

This document does not specify a type of card reader. Any type may be

used, as long as it can be easily used by the customer. Some of these

requirements pertain only to particular card reader types.

2.1.1 Card Presentation

1. When the customer presents a card to an off-hook station, the card

reader should read the card, check its validity (see Section 2.1.2), and

store the data for use during call set-up by the station.

2. Any card data should be purged from the station after the station

goes on-hook.

3. If the card reader type entails the customer releasing the card while

it is being read, the station should alert the customer to remove the

card from the reader before sitting up the call. The station should not

proceed with the call setup until the card is removed. Also, it is

desirable that part of the card always remain in sight of the customer.

4. The customer should be able to remove the card at any time, even

while it is being read.

5. The card reader should be positioned such that the customer action is

natural. If the customer moves the card in a vertical slot, the motion

should be downward. In a horizontal slot, the card should be moved from

left to right. For insertion type readers, the card should be pushed

forward and then removed.

6. If the customer must move the card past the reader head, the card

reader should successfully read the card over the range of speeds that

the customer might reasonably attempt to move the card.

7. The customer should receive tactile feedback when presenting the card

to the card reader, as well as feedback when the card is read

successfully.

2.1.2 Checks

1. The station should check the parity of each character. Each data

character will be encoded on the card with an odd parity bit.

2. The station should do a Longitudinal Redundancy Character (LRC)

check. The LRC is a check character for the remaining data on the

magnetic stripe. The first four bits in the LRC check the corresponding

bits in the other characters on the magnetic stripe. Each bit is an even

parity bit that checks the corresponding bits in the other characters on

the card. The fifth LRC bit is an odd parity bit for the LRC itself.

3. The station should check the format of Calling Cards for proper

layout of the data on the card. The format is specified in Section

2.1.3. If the four digits of the PIN are zeros, the actual PIN is not on

the card.

4. It is desired that the station not check the expiration or effective

dates on the card.

2.1.3 Card Characteristics

1. Calling Card characteristics are covered thoroughly in Reference (3).

Both Calling Cards and CCCs will adhere to standards set by the American

National Standards Institute (ANSI) and the American Bankers Association

(ABA).

2. The Calling Card data will be contained on Track 2. The first

character will be a start sentinel. The next six characters will be a

one-digit major industry identifier, and a five-digit issuer

identifier. A ten-digit billing number will be encoded in the next ten

characters, followed by a Luhn mod 10 check character and field

separator. The next four characters will contain an expiration date.

Following the expiration date, the four-digit Personal Identification

Number (PIN) will be encoded. The magnetic stripe may also contain up

to another 11 digits of data used to describe various services. Three

of the characters may be a carrier identifier, or may be unused. The

next 8 potential characters are, at this time, unused. Unused

characters will not be encoded on the magnetic stripe. The end sentinel

will follow the last data character, and the LRC will follow the end

sentinel.

3. The start and end sentinels should be transmitted as DTMF tone D. The

separator characters should be transmitted as DTMF tone C. The LRC

should not be stored or transmitted as card data. Thus, no more than 39

characters of magnetic stripe data will need to be stored by the

station.

4. If the last four characters of the 14-digit Calling Card number are

zeros, the Calling Card PIN is not on the magnetic stripe. The specific

data fields to be transmitted are specified in Sections in 2.4.

5. The station should be capable of being field modified to accommodate

changes in the format of the card. 2.1.4 Error Feedback

1. The station should provide feedback to the customer for the following

problems on card presentation and reading:

a. The magnetic stripe on card is damaged, causing errors in reading.

b. The customer presented the card in the wrong orientation. The station

should include clear user instructions, or graphics, on how to present

the card.

c. The card is not an acceptable CCC or Calling Card (i.e., issuer or

industry identification is inappropriate).

2. It is desired that the station respond to these errors by emitting

tones, or by sending inband signals to the network, as appropriate. The

inband signals should be sent using DTMF tones, using the codes defined

in Section 2.4.4.

2.2 Customer Signaling

The station is expected to accommodate three basic customer signaling

functions: carrier selection via either the DTMF key pad,

"Select-a-Carrier" buttons, or carrier pre-selection on the Calling

Card; far-end party address signaling; and possible manual dialing of

billing information. In addition, the station is expected to incorporate

anti-fraud features.

2.2.1 Carrier Selection

1. The station should implement either Select-a-Carrier buttons or allow

manual dialing of carrier access codes, or both.

2. If a method is implemented, it should be implemented according to the

requirements in Sections 2.2.1.1 and 2.2.1.2.

3. It is desirable that the station accept carrier specific Calling

Cards as described in Section 2.2.1.3.

2.2.1.1 Separate Select-a-Carrier Buttons

1. Positive feedback (an appropriate combination of aural, visual, and

tactile), should be provided when a button is pressed.

2. The number of buttons should be kept to a reasonable level. It is

suggested that no more than 12 buttons be provided.

3. When transmitting the number to the local office, that is, when using

the station as a Select-a-Carrier station, the station should substitute

an access number or a speed calling code for the button pressed. The

access number may be up to twelve digits. The speed calling code may

combine digits with the * or # signal.

4. When transmitting to a processor (dial-up or directly connected, as

described in Sections 2.4.4 and 2.4.5), the station should transmit

10XXX or *XXX for the button pressed.

2.2.1.2 Manual Dialing of Carrier Access Code

1. The station should transmit all digits dialed on the key pad as they

are entered.

2.2.1.3 Pre-Selected Carrier on Card

1. If the station reads a Calling Card with a carrier code included on

the magnetic stripe in the optional field (see Section 2.1.3), the

station should direct the call towards a processor (as explained in

Section 2.4).

2. If the issuer identifier is '8555', the card is an IC Calling Card

issued by AT&T Communications (ATT/C), even though these cards will not

have a carrier code in the optional field on the magnetic stripe. These

calls should be directed towards AT&T/C.

2.2.2 Address Signaling

1. The station should allow the customer to dial the terminating number

before or after carrier selection.

2. These dialed numbers should be expected from the stations:

a. 0+7/10 Digits

b. 01+Country Code+National Number (7 to 12 digits in CC+NN)

c. 411, 611, 911

d. (0,1)+800+7 Digits

e. 0-

f. (1)+555-XXXX

g. (1)+NPA-555-XXXX

h. 950-XXXX, if a carrier is not selected in another manner.

3. The local office can block all other dialing sequences that may be

dialed. Thus, it is normally not required for the station to screen for

improper dialing sequences. If a BOC specifies that it is necessary to

do so, an optional feature could allow the station to block all or most

calls not on the above list.

4. It is desirable that the station enable new calls to be placed

without reusing a card. The method to make sequence calls will be

determined.

2.2.3 Anti-Fraud Features

1. The station should prevent the customer from signaling via

manipulation of the switchhook.

2. For loop-start lines, the station should disable the dial until dial

tone is received. For ground-start lines, the local office can detect

DTMF tones as soon as current is provided to the station.

3. The station should mute or otherwise control the transmitter during

any time where an acoustic coupler could be used for fraud purposes. In

particular, the transmitter should be muted or controlled during the

transmission of the card number and whenever the dial is disabled.

Specific requirements on when to mute or control the transmitter are

detailed in Sections 2.4.4 and 2.4.5.

2.3 End Office Interface

Electrical and signaling characteristics of the station should meet the

requirements described in: Reference (1) concerning the normal station

to network interface; Reference (2) concerning abnormal conditions on

the local loop. The station should also meet requirements specified by

the FCC rules regarding registration of telephone sets.

2.3.1 Features of Line

1. The station should operate on a line with Charge-a-Call class of

service. This class of service allows only nonsent-paid and free calls.

An optional feature may allow the station to operate on lines without

Charge-a-Call class of service, as described in Section 2.2.2, item 3.

2 The stations should operate with any electronic end office commonly in

use by the BOCs.

3. The station should operate on either standard loop-start or

ground-start lines.

4. Answer supervision cannot be provided on these lines. Thus, the

station should not depend on answer supervision for any functions.

2.3.2 Electrical Characteristics

1. The loop current from the local office to the station will be a

minimum of 23 milliamps at 48 volts DC. It is desirable that the station

operate on this power level without a supplementary power supply. If the

station needs more than 23 milliamps current, a supplementary power

source may be used.

2. The polarity of the loop should not affect operation of the station.

2.3.3 Signaling Characteristics

1. The station should operate on lines with DTMF signaling, also know as

Touch-Tone.

2. When dialing Calling Card, CCC, or IC access numbers, the station

should transmit digits at a maximum rate of 10 per second. The tone

duration and interdigit interval should not be less than 50 ms and 45

ms, respectively, and the cycle time (sum of tone duration and

interdigital time) should not be less than 100 ms. It is desirable that

digits be transmitted at close to the maximum rate.

2.4 Carrier and Processor Interfaces

These stations should interface either with a carrier or with a

processor (designed to interface with one or more carriers), depending

on instructions programmed into the station.

It is proposed that the station be kept simpler by using identical

protocols to all ICs whenever feasible. Still, the need for different

protocols is envisioned for direct station-carrier interfaces. Carriers

using Feature Groups A or B would conform to a single protocol from

these stations (currently undefined). Feature Group D carriers, except

for ATT/C, might need to conform to another standard protocol. ATT/C,

whether Feature Group C or D, would use a third protocol. These

protocols are described in Sec tions 2.4.1 through 2.4.3. The station

would determine which protocol to use based on the customer's actions to

select the carrier.

An interface to a processor could be substituted for the carrier

arrangements. The processor would, in turn, establish and administer the

interfaces to the ICs. The processor could be dialed up by the station

or directly connected. Call sequences and station actions for dial-up

and directly connected processors are described in Sections 2.4.4 and

2.4.5, respectively.

The station should be programmable to send all calls of certain types to

a dial-up processor, and handle others by the protocols described for

the various carriers. Section 2.4.4 specifies the calls that should

cause the station to dial the processor. The dial-up processor would

then handle the call set-up for those calls. Calls where the customer

begins the call by dialing a number or selecting a carrier would be

handled using Charge-a-Call treatment or direct interfaces to the

carriers.

2.4.1 Call protocol for BOC and ATT/C (Feature Group C)

1. BOC and ATT/C (FG C) calls are set up by the customer dialing 0+ the

terminating number after receiving dial tone. The station should then

wait for the "bong tone" from the TSPS. The bong tone is defined to be

100 ms or the DTMF frequencies for the # sign, 941 and 1477 Hz, followed

by 1.4 seconds of dial tone, 350 and 440 Hz. The amplitude of the signal

starts at -7dBm0/frequency +/- 1dB at -3 TLP. The amplitude of the dial

tone portion is exponentially decayed with a time constant of 200 ms.

The amplitu des at the station can vary by loop. The range of amplitudes

will be determined.

2. The station should be able to prefix a digit (e.g., '9') before the

first customer-dialed digit. (This feature should only be used if it

cannot be avoided. This feature could cause human interface problems.)

3. The station should then transmit only the 10 (if PIN not on card) or

14 digits of the BOC or AT&T Calling Card number (as explained in

Section 2.1.3.

4. The station should begin transmitting the Calling Card number within

500 ms or recognition of the bong tone. It is desirable that the digits

be transmitted at the maximum rate.

2.4.2 Call Protocol for Feature Groups A or B IC

The call protocol for Feature Groups A or B ICs will be determined later

if any ICs are interested in being accessed directly by the stations.

2.4.3 Call protocol for Feature Group D IC

The call protocol for Feature Group D ICs will be determined later if

any ICs are interested in being accessed directly by the stations.

2.4.4 Call sequence for Dial-Up Processor

1. The station should dial the processor number, as programmed, only if

a customer presents a card to the station first. The station should be

able to store and dial a one-to-seven-digit processor number.

(EXCEPTION) ((If an AT&T Calling Card is presented to the station first,

the station should store the card number and handle the call as

described in Section 2.4.1.))

2. When a processor is connected, the station should detect a bong tone

that indicates the processor is ready to receive DTMF signals. The

station should then transmit a calling station identification (up to ten

digits, if required),DTMF tone 'D' and all the data from track 2 of the

magnetic stripe of the Calling Card or CCC, DTMF tone 'D' again, and the

carrier access code (10XXX), if a carrier is selected. The processor

will have announcements to prompt the customer, but the station should

transmit the da ta after detecting the processor tone, or as soon as the

customer provides the information, whichever is later.

3. The station should begin transmitting the calling station

identification within 500 ms of recognition of the tone. It is desirable

that the digits be transmitted at the maximum rate.

4. The station should disable the transmitter in the handset during

transmission of the processor number until the card number and DTMF tone

'D', a 3 digit code, and DTMF tone 'D' again for these messages to the

processor:

CODE      MESSAGE

011       Card not readable (failed parity check) 002       Not an

ANSI-standard Card 003       Self-diagnostic indication of card reader

trouble 08X,09X   Give announcements in specified language

Other codes and messages will be assigned when needed.

6. The station should allow the customer to dial the terminating number

or carrier codes any time after the billing data are transmitted.

7. If the customer dials a number first (could include dialing a

carrier), the station should transmit all digits dialed and monitor for

a bong tone. If it detects a bong tone, it should transmit a 14-digit

Calling Card number if one was read, or allow manual dialing; if no bong

tone is detected within five seconds after the last digit dialed, the

station should transmit any card number read after that time.

8. If the customer presses a Select-a-Carrier button first, the station

should transmit the code stored for that button. The station should then

operate the same as if a number had been dialed first. However, the card

data transmitted may be carrier specific.

9. After receiving the processor tone, the station should read out the

last card number read.

2.4.5 Call Sequence for Direct-Connect Processor

1. The station should accept inputs from the card reader, the DTMF dial

pad, or a Select-a-Carrier button pad when the handset is off-hook.

2. After a card is presented with the station off-hook, the station

should send DTMF tone 'D', the data from track 2 of the magnetic stripe

of the Calling Card or CCC, and DTMF tone 'D' again. The dial pad should

be disabled while the station is transmitting the card data.

3. The station should send the codes described in Section 2.4.4, if

appropriate, instead of the card number.

4. The transmitter should be disabled from the time the handset goes

off-hook until a card is presented and transmitted, or until a digit is

dialed. This prevents a customer from acoustically coupling DTMF tone

'D' and a stolen or made-up card number into the handset.

5. If a card is presented first, the station should transmit the carrier

access code if a carrier is selected, or allow the customer to dial.

6. If a number is dialed first, the station should not transmit any

carrier codes, and should only transmit card data after it receives a

bong tone, or waits at least 5 seconds after the last digit dialed and

then reads a card.

2.5 Physical and Other Requirements

The station will be composed of several major physical components.

2.5.1 General

1. These requirements specify a station to operate in a public, indoor

environment.

2. All components of the station should be flame resistant.

3. The station should be built to withstand the abuse expected in a

public environment.

4. It is desirable that self diagnostics be designed into the card

reader.

5. The station should not be susceptible to radiated emissions from

other sources that are within legal limits. It is desirable that the

station not be susceptible to any radiated emissions from other sources

that may reasonably be encountered.

2.5.2 Card reader

1. The card reader should be designed such that the reader head cannot

be easily damaged by misuse or abuse encountered in a public

environment.

2.The card reader should be protected to minimize the frequency of

cleaning.

3. The card reader should be easily maintained.

4. The card reader should be physically shielded from the interior of

the set such that customers cannot access or tamper with the electrical

components of the station.

2.5.3 Dial Pad

1. The dial pad should be will designed from a human factors standpoint

with respect to button spacing, shape, graphics, and activation

feedback.

2. The station should have a moisture and dirt resistant dial pad.

3. The dial should be durable. Each key should withstand at least

500,000 depressions before failure.

2.5.4 Handset and Switch-hook

1. On wall sets, the handset should be connected to the station with a

cord that can withstand a 400 pound tensile test.

2. The handset cord should have enough cutting and shear resistance to

withstand attacks by tools such as knives or scissors. Tools with

cutting ability equal to or less than 5-inch diagonal cutters should not

be able to sever the cord.

3. When destroyed while in service due to any circumstances, the cord

should not create a customer safety hazard.

4. The handset should be manufactured such that it cannot be

disassembled by customers or field craftspersons. The handset and cord

should only be replaceable by opening the station housing.

5. The handset should allow people with hearing impairments to use the

telephone with their hearing aids. The station should meet the

Electronics Industries Association's Recommended Standard, RS-504,

Magnetic Field Intensity Criteria for Telephone Compatibility with

Hearing Aids, to be considered hearing aid compatible. This capability

should be shown by use of a blue grommet on the handset cord.

6. The handset and switch-hook should be able to withstand at least

200,000 hang-ups before failing.

2.5.5 Station Housing

1. The wall station should be capable of being mounted in standard

enclosures currently used by BOCs, including 10A-type enclosures and the

178A backboard.

2. It is desired that the station be equipped with locks to discourage

theft of the set or tampering with the program codes.

3. Space should be available on the face of the set to provide

instructions for use of the set, and to place the telephone number and

station location information.

3. Support

This section defines the obligations of the supplier to support the

station. Additional requirements are to be determined.

1. The supplier should provide documentation for the installation,

maintenance, and testing of the stations.

2. The supplier should provide satisfactory means for replacement parts.

3. The supplier should have a factory repair service, or similar means

for refurbishment, for repairs that cannot be made in the field.

4. The supplier should provide data on the reliability of the station

and each major component.

5. The supplier should have an adequate quality assurance program,

including a means for handling engineering complaints on product design,

manufacturing, operation, maintenance, documentation, and other aspects

of the product.

This file has been presented by P-80