Date:  Thu, 28-Feb-85

Subject:  Dial Back isn't always secure

From: [usenet via anonymous donor]

An increasingly popular technique for protecting dial-in ports

from the ravages of hackers and other more sinister system penetrators

is dial back operation wherein a legitimate user initiates a call to the

system he desires to connect with, types in his user ID and perhaps a

password, disconnects and waits for the system to call him back at a

prearranged number.  It is assumed that a penetrator will not be able to

specify the dial back number (which is carefully protected), and so even

if he is able to guess a user-name/password pair he cannot penetrate the

system because he cannot do anything meaningful except type in a

user-name and password when he is connected to the system.  If he has a

correct pair it is assumed the worst that could happen is a spurious

call to some legitimate user which will do no harm and might even result

in a security investigation.

Many installations depend on dial-back operation of modems for

their principle protection against penetration via their dial up ports

on the incorrect presumption that there is no way a penetrator could get

connected to the modem on the call back call unless he was able to tap

directly into the line being called back.  Alas, this assumption is not

always true - compromises in the design of modems and the telephone

network unfortunately make it all too possible for a clever penetrator

to get connected to the call back call and fool the modem into thinking

that it had in fact dialed the legitimate user.

The problem areas are as follows:

Caller control central offices

Many older telephone central office switches implement caller

control in which the release of the connection from a calling telephone

to a called telephone is exclusively controlled by the originating

telephone.  This means that if the penetrator simply failed to hang up a

call to a modem on such a central office after he typed the legitimate

user's user-name and password, the modem would be unable to hang up the

connection.

Almost all modems would simply go on-hook in this situation and

not notice that the connection had not been broken.  If the same line

was used to dial out on as the call came in on, when the modem went to

dial out to call the legitimate user back the it might not notice (there

is no standard way of doing so electrically) that the penetrator was

still connected on the line.  This means that the modem might attempt to

dial and then wait for an answerback tone from the far end modem.  If

the penetrator was kind enough to supply the answerback tone from his

modem after he heard the system modem dial, he could make a connection

and penetrate the system.  Of course some modems incorporate dial tone

detectors and ringback detectors and in fact wait for dial tone before

dialing, and ringback after dialing but fooling those with a recording

of dial tone (or a dial tone generator chip) should pose little problem.

Trying to call out on a ringing line

Some modems are dumb enough to pick up a ringing line and

attempt to make a call out on it.  This fact could be used by a system

penetrator to break dial back security even on joint control or called

party control central offices.  A penetrator would merely have to dial

in on the dial-out line (which would work even if it was a separate line

as long as the penetrator was able to obtain it's number), just as the

modem was about to dial out.  The same technique of waiting for dialing

to complete and then supplying answerback tone could be used - and of

course the same technique of supplying dial tone to a modem which waited

for it would work here too.

Calling the dial-out line would work especially well in cases

where the software controlling the modem either disabled auto-answer

during the period between dial-in and dial-back (and thus allowed the

line to ring with no action being taken) or allowed the modem to answer

the line (auto-answer enabled) and paid no attention to whether the line

was already connected when it tried to dial out on it.

The ring window

However, even carefully written software can be fooled by the

ring window problem.  Many central offices actually will connect an

incoming call to a line if the line goes off hook just as the call comes

in without first having put the 20 hz.  ringing voltage on the line to

make it ring.  The ring voltage in many telephone central offices is

supplied asynchronously every 6 seconds to every line on which there is

an incoming call that has not been answered, so if an incoming call

reaches a line just an instant after the end of the ring period and the

line clairvointly responds by going off hook it may never see any ring

voltage.

This means that a modem that picks up the line to dial out just

as our penetrator dials in may not see any ring voltage and may

therefore have no way of knowing that it is connected to an incoming

call rather than the call originating circuitry of the switch.  And even

if the switch always rings before connecting an incoming call, most

modems have a window just as they are going off hook to originate a call

when they will ignore transients (such as ringing voltage) on the

assumption that they originate from the going-off-hook process.  [The

author is aware that some central offices reverse battery (the polarity

of the voltage on the line) in the answer condition to distinguish it

from the originate condition, but as this is by no means universal few

if any modems take advantage of the information so supplied]

In Summary

It is thus impossible to say with any certainty that when a

modem goes off hook and tries to dial out on a line which can accept

incoming calls it really is connected to the switch and actually making

an outgoing call.  And because it is relatively easy for a system

penetrator to fool the tone detecting circuitry in a modem into

believing that it is seeing dial tone, ringback and so forth until he

supplies answerback tone and connects and penetrates system security

should not depend on this sort of dial-back.

Some Recommendations

Dial back using the same line used to dial in is not very secure

and cannot be made completely secure with conventional modems.  Use of

dithered (random) time delays between dial in and dial back combined

with allowing the modem to answer during the wait period (with

provisions made for recognizing the fact that this wasn't the originated

call - perhaps by checking to see if the modem is in originate or answer

mode) will substantially reduce this window of vulnerability but nothing

can completely eliminate it.

Obviously if one happens to be connected to an older caller

control switch, using the same line for dial in and dial out isn't

secure at all.  It is easy to experimentally determine this, so it ought

to be possible to avoid such situations.

Dial back using a separate line (or line and modem) for dialing

out is much better, provided that either the dial out line is sterile

(not readily tracable by a penetrator to the target system) or that it

is a one way line that cannot accept incoming calls at all.

Unfortunately the later technique is far superior to the former in most

organizations as concealing the telephone number of dial out lines for

long periods involves considerable risk.  The author has not tried to

order a dial out only telephone line, so he is unaware of what special

charges might be made for this service or even if it is available.

A final word of warning

In years past it was possible to access telephone company test

and verification trunks in some areas of the country by using mf tones

from so called "blue boxes".  These test trunks connect to special ports

on telephone switches that allow a test connection to be made to a line

that doesn't disconnect when the line hangs up.  These test connections

could be used to fool a dial out modem, even one on a dial out only line

(since the telephone company needs a way to test it, they usually supply

test connections to it even if the customer can't receive calls).

Access to verification and test ports and trunks has been

tightened (they are a kind of dial-a-wiretap so it ought to be pretty

difficult) but in any as in any system there is always the danger that

someone, through stupidity or ignorance ity of the Hackers controling satelites in geo-sync

orbit are being moved out of there assigned orbits. Granted they

did not move the bird,but did gain control of the rotation control

for the satelite.

And  it was stated that the information needed to do such  things

was  found  on an underground bulletin board.  Ok,that  might  be

true,but information that is far more valuable to people on earth

is being posted on the boards. And the information comes from the

trash  can  or from insiders who have become disgruntled or  just

from plain old research looking for publicly available sources.

Some  of  these public sources are to include users manuals  and

system documentation. Others are to include users groups and just

talk.

ASPECTS OF HACKER CRIME : HIGH TECHNOLOGY TOMFOOLERY OR THEFT ?

-------------------------------------------------------------

Other  interesting facts about the boards is that they contain  a

group   of   sub-sections  that  are  to  include forwarded to line B, to avoid callers when it is trying to dial out.

Line C is some phone in the attacker's control.  The attacker forwards

line C to line A, and then calls line C from yet another phone.  The

call is forwarded only from C to A, not from C to A to B.  --

Joe Eykholt

[Opinions expressed by me are not necessarily held by any other entity.]