Dialing Plan (16 May 1990)

1. Area Codes, North America

Bell Communications Research (Bellcore) is the organisation responsible

for the maintenance of the area codes and the dialing setup for

North America since the breakup of AT&T several years back. This

group sets standards for dialing, telephone number format, etc.

Area codes (or Numbering Plan Areas (NPA)) in North America are presently

of the form NZX where N represents a number from 2 to 8, Z is 0 or 1 and

X is any number. This distinguishes area codes from central office (exchange)

codes which are usually of the form NNX (ie. the difference between an

exchange and an area code is usually in the second digit, whether or not

it is a 0 or 1).

Area codes ending in -11 are not used, as they are normally used for special

services. The special purposes generally reserved are:

411 directory assistance

611 repair

811 business office

911 emergency

The other N11 codes are sometimes used within specific localities for

special purposes (711 is used for mobile service in British Columbia,

in the form 0+711)). In some cases, services are accessed through the

long distance network (ie. 1+411 or 1+611).

Area codes ending in -10 have been TWX numbers (teletype service).

TWX area codes include 510, 610, 710, 810 and 910. Normal telephone

numbers do not (yet) use area codes ending in -10, though these codes

will be reassigned to regular telephone service as TWX is eliminated.

Upcoming examples of this are the area code splits of 213 (Los Angeles

adds area code 310) and 415 (San Francisco adds area code 510).

Area codes ending in -00 are usually reserved for special services

like 700, 800 or 900. In fact, these codes are not considered to be

area codes as such, but Service Access Codes (SACs).

(2) Dialing Codes...

Theoretical capacity of the telephone system:

The original design of telephone numbers was:

(NZX) NNX XXXX

In theory, this gives:

N   Z   X    N   N   X    X    X    X    X

8 * 2 * 10 * 8 * 8 * 10 * 10 * 10 * 10 * 10 = 1.024 billion numbers

However, as some area codes and exchanges are reserved for special

purposes (such as 411, 555 exchange, etc), the total possible number

of telephone numbers will be somewhat less.

As the exchange codes in some area codes were used up, some central offices

started using the NXX format, where the middle digit can then be a 0 or 1;

examples of these may be found in New York City or Los Angeles. The March

1990 conversion of the (416) area code around Toronto is another example,

as is a similar change to North Carolina's dialing around the same time.

The dialing is changed so that dialing 1+ a number always requires an area

code, and in the case of (416), will mean that 1+416+ will be needed for

long distance calls within (416). In New York City, however, all calls

within an area code are dialed with seven digits only (without dialing

1 first, let alone the area code).

Now, telephone numbers look like this:

(NZX) NXX XXXX

This gives a potential of:

N   Z   X    N   X    X    X    X    X    X

8 * 2 * 10 * 8 * 10 * 10 * 10 * 10 * 10 * 10 = 1.28 billion numbers

However, codes like 411 and 611 would not be assigned because they will

still be needed for services such as directory assistance and repair.

Nevertheless, some unused N11 codes like 211 may be found in some area

codes as active exchanges. It's also not a good idea to assign the home

area code (could cause confusion).

Going from NNX exchange codes to NXX only represents a 25% increase in the

total theoretical amount of telephone numbers, and not all area code regions

are expected to run out of exchanges. Also, area codes do not generally go

outside a state or provincial boundary (the only exceptions are in Canada,

where 902 serves both Nova Scotia and Prince Edward Island, area 819 which

covers the eastern Northwest Territories as well as part of Quebec, and

area 403 which covers Alberta, Yukon and the western Northwest Territories).

The ultimate goal is not only to use area codes for exchanges codes, but

to use exchanges codes for area codes also. This means that telephone

numbers will ultimately look like this:

(NXX) NXX XXXX

This gives a potential of:

N   X    X    N   X    X    X    X    X    X

8 * 10 * 10 * 8 * 10 * 10 * 10 * 10 * 10 * 10 = 6.4 billion numbers

With a five-fold increase in the number of possible area codes, there

should be plenty of room to grow for some time.

(3) Dialing Codes...

Basically, all calls within an area code will ultimately be dialed in

one of the following ways 1) dial seven digits, 2) dial 1 + home area

code + number, or 3) dial 1 + seven digit number within area code, then

wait for a few seconds to time out. One alternative not mentioned in

official documents (for touch tone phones) is to use 1 + seven digit

number in home area code + '#' with the # key terminating the dialing.

The initial set of new area codes will take the form NN0, or those numbers

ending in '0'. This means that areas codes that do not have exchanges

ending in '0' (or only a few NN0 exchanges that could be renumbered)

can still be able to tell the difference between an exchange and an

area code by looking at the first three digits.

The new NN0-type area codes will be assigned starting with these

first few codes (in order):

260, 480, 520, 590, 650, 220, 250, 490, 660, 680, 720, 730, 850, 940 ...

970 will be reserved for phone testing purposes, and is not slated to

become an area code.

All telephone systems are expected to change their systems over to allow

for the new style of area codes by 1 July 1995, or perhaps earlier. This

could mean that other area codes will be required to dial the area code

for long distance calls within area, even though the area code is not

running out of exchanges itself (eg. areas 519, 705).

Dialing Scheme

* 1 + will generally be used for direct-dialed long distance calls within

North America, especially calls outside the local area code.

* 0 + is used to dial operator-assisted or automated credit card calls within

North America. After 0 + (area code) + number are dialed, a prompt tone

(same tones as a dial tone, but for a very short duration) will be issued,

then one of the following actions will be taken:

1) wait for a few seconds, then an operator will come on line

2) dial '0' to get the operator immediately (for a collect, person to

person call, etc)

3) dial the telephone company credit card number for billing

It is unclear what will happen in the cases of automated collect calls,

as to what kinds of dialing would be standard in that case.

(4) Dialing Codes...

Dialing 0 and waiting will get the local area operator.

* 00 is used in the U.S. to get the operator for a default long distance

carrier. This is used as most long distance companies have their own

operators. A single 0 digit will call up the local operator (with the local

telephone company as opposed to the long distance company).

It's important to note that the U.S. is broken up into LATAs (Local Access

Transport Areas), and long distance calls are treated differently whether

they are made within a LATA, or if a call is placed to another LATA. Thus,

there could be an operator for within the LATA (intra-LATA calling) and an

operator for inter-LATA calling (from a long distance carrier).

* 01 is used for overseas calls. 01 + indicates an operator-assisted or

automatic credit card call, while 011 + indicates a direct-dialed overseas

call. 010 is reserved for some unspecified future use.

* 10XXX + is used in the U.S. to indicate which long distance carrier to use

in a situation known as "equal access". This allows a telephone subscriber

to select a long distance company for a particular call. For instance,

10288+ gets AT&T while 10222+ gets MCI and 10333+ US Sprint. After this

code, a 1 or 0 is dialed (to indicate direct dial or operator-assisted call)

then the number to be called.

The codes 10000 is not available for assignment

10001 - 10099 are reserved for restricted purposes

10100 - 10199 are reserved for international carriers

(note that because of possible dialing mistakes that may

confuse some codes with 01- overseas dialing codes,

10100-10119 are to be assigned after all 10120-10199 codes

are assigned.)

10200 - 10999 are assigned to standard long distance carriers

A list of the U.S. long distance carriers and their 10XXX+ codes is listed

in a separate document.

Canada doesn't have to worry about this code quite yet, depending on the

results of the CNCP/Rogers intention to provide alternate long distance

service.

* 11- or '*'- (tone dialing) is reserved for special calling services like

call waiting functions, etc. For instance, 1170 is used to disable the

call waiting, like *70 can be used on touch tone phones. The current

special calling codes on many local telephone systems are:

*57 or 1157 - call tracing request (some systems use this for call back)

*60 or 1160 - call blocking activated

*61 or 1161 - priority ring activated

*63 or 1163 - select call forwarding activated

*66 or 1166 - repeat dialing activated

*67 or 1167 - call number ID blocking (done for each call)

*69 or 1169 - call return activated

*70 or 1170 - disable call waiting

*71 or 1171 - 3 way calling according to usage

*72 or 1172 - enable call forwarding

*73 or 1173 - disable call forwarding

*74 or 1174 - modify speed calling directory entry (for 8 number service)

*75 or 1175 - modify speed calling directory entry (for 30 number service)

*76 or 1176 - call pickup

*79 or 1179 - ring again

*80 or 1180 - call blocking disabled

*81 or 1181 - priority ring disabled

*83 or 1183 - select call forwarding activated

*86 or 1186 - repeat dialing disabled

*89 or 1189 - call return disabled

(5) Dialing Codes...

2. Reserved exchanges

Each area code will have certain exchanges set aside for special purposes.

These exchanges are:

555 - directory assistance

950 - used to access other long distance services (U.S.)

958, 959 - reserved for phone tests

976 - special recordings, private services

844 and 936 have been reserved for time and weather services, but this

is not widely adhered to. In area code 416, these exchanges are for

Oakville and Tottenham respectively. These services will likely be

incorporated into 976 service, or into other exchanges.

Other Information

A file with the Tel Docs (TELDOC) package, GUIDECOD.DOC, explains in more

detail the 700, 800 and 900 area codes, with lists of exchange/carrier

assignments and other details. This file should be included in the Tel

Docs package. This is the same document as the Guide prepared by TELECOM

Digest, an electronic conference regarding telephone/telecommunication

matters.