II==================================II

][                                  ][

|| Incoming Trunk Service Observing ||

II -        -     -       -         II

][                                  ][

||            Written by            ||

II                                  II

][          The Executioner         ][

||                                  ||

II              of the              II

][                                  ][

||    [+] PhoneLine Phantoms [+]    ||

II        -    -    -               II

][==================================][

The ITSO (Incoming Trunk Service Observing) feature provides the ability

to perform service observing on incoming 2-wire trunk [per trunk signalling

or CCIS]. The ITSO feature provides a method to determine how effectively the

telefone system serves the customer. This assists the telefone co. in

maintaining network quality.

The intent of servince observing is to evaluate the completion performance

of the office's subtending networks. All calls selected for observation must

originate from offices of equal or higher rank in the toll heirarchy and must

terminate in the area served by the toll office. The ESS switch randomly

selects calls for observation. Only one call can be monitored at any given

time for the ITSO feature. Equipment malfunctions, network overloads, or other

problems preventing call completion are detected. The calls are also directly

monitored for transmission quality. Each service observing position, located in

the SES (Service Evaluation System) bureau is capable of monitoring calls.

The information required by the service observer is provided to the SES

bureau from the ESS switch via a dedicated data link and a voice monitoring

channel. A 1200 baud data link is used to transmit call progress data and

control signals between the ESS switch and the SES bureau. The dedicated

one-way voice channel is required for monitoring voice transmission. The

interface is capable of monitoring one call at a time.

The SES has complete control over the SES/ESS interface. The observing

feature is a demand system where the demand originates with the SES. There

are 5 possible control messages that the SES can send to an ESS switch.

They are:

1. ENABLE- This message is accepted when the interface is in the idle state.

It directs the ESS switch to obtain the next observable call.

2. RELEASE- This message is used to release the present call and return the

interface to the idle state or to release the present test and return to

idle state.

3. TONE TEST- This message is used ony if the interface is in the idle

state. It causes a 1-milliwatt tone to be applied to the voice channel.

4. TERMINATION TEST- This messages is only accepted if the interface is in

the idle state. It causes a quiet termination to be applied to the voice

channel for 30 seconds (unless a RELEASE order is sent).

5. DATA ECHO TEST- This message is accepted only if the interface is in the

idle state. It sends a message to the ESS switch to verify the sanity of

the data channel. The ESS switch appends a unique 2-character header to

the received message and outputs the recieved message to the data

channel.

There are 15 potential call progress messages that the ESS switch can

generate and send to the SES. They are:

1. SEIZURE- This message indicates that the ESS switch has found a candidate

call. If the SES does not recieve this signal within a period of time

after being enabled, a RELEASE message is transmitted be the SES.

2. RELEASE AKNOWLEDE- This message is transmitted whenever the ESS switch

decodes a RELEASE signal deo the SES. The interface returns to the idle

state and the SES is allowed to attempt to observe a new call.

3. TERMINATION- This message is Xmited when the interface returns to the

idle state due to an abnormal ending of the call observation in progress,

or when the tone or termination test times out.

4. INCOMING TRUNK DISCONNECT- This message is Xmitted to the SES for every

calling party on on-hook occurrence.

5. ON-HOOK- This message is Xmitted to the SES for every called party

on-hook occurrence.

6. OFF-HOOK- This message is xmitted to the SES for every called party

off-hook occurrence.

7. EMERGENCY- This message indicates an emergency announcement is being

applied to the calling party.

8. 60 IPM- This call progress message indicates that the busy tone is being

applied to the calling party.

9. 120 IPM- This call progess message indicates that recoreder tone is being

applied to the calling party.

10. NO CIRCUIT AVAILABLE- This call progress message indicates that a no

circuit available is being applied to the calling party.

11. VACANT CODE ANNOUNCEMENT- This message indicates that a vacant code

announcement is being applied to the calling party.

12. RECORDER- This message indicates that a recorder announcement is being

applied to the calling party.

13. AUDIBLE RING- This message indicates that audible ringing is being

applied to the calling party. In the case of a locally terminating call,

the observer is not bridged onto the call until after answer and is

therefore unable to detect the actual audible ringing.

14. CALLED NUMBER DIGITS- This message contains the called number of digits.

15. DATA ECHO TEST- This contains the DATA ECHO TEST message.

{

The 4 states of SES/ESS interface are as follows:

o Idle- This is the wait state of the interface. It is the stae to

which the interface returns after tests and completing observations.

o Active- This state is entered only when the ESS switch recieves an

ENABLE message frm the SES. Service observing is performed in this

state. This state is maintained for no moe than 3 minutes (2-wire

trunks) after seizure.

o Tone Test- This can only be entered from the idle state upon receipt

of a tone test order. In this state, a 1- milliwatt tone is applied

to the voice channel. The interface returns to the idle state upon

the RELEASE message or after a 30 second test.

o Termination Test- This state can only be entered from the idle state

upon the TERM. TEST message. In this state a quiet term. is applied

to the voice channel. The interface returns to the idle state upon

the RELEASE or after 30 secs.

Action to establish an observation is initiated when the SES send an

ENABLE message to the ESS switch. This message triggers the switch to begin

looking for an observable call. When the switch has found a call, the

SEIZURE and CALLED NUMBER DIGITS messages are transmitted to the SES.

From this point on, the SES observes the progress of the call using the

progress messages.

=====================================

=All sysops are allowed to put this =

=file on their BBS's.               =

=====================================

(C) 1985 Sexy-Exy & PLP

Dedicated to Christi and Linda.

Downloaded from Just Say Yes. 2 lines, More than 500 files online!

Full access on first call. 415-922-2008 CASFA