-=[ NorTel MilleniumZ - The Payphone for the next 1000 Years ]=-

Those NorTel Milleniums that are being bought by the hoards by the RBOCs and

put up everywhere are really a work of art.  At first look, they might appear

to be a bad idea in high vandalism areas, but they are actually very tough.

The bodies are built with 1/4" reinforced steel, and they employ a 2-way dual

locking system.  First of all, the coin box and the logic box are locked

seperately.  Secondly, they use a 4-pin Medeco(tm) lock with a notched T-Bit

bolt.

The 4-Pin Medeco lock for the coin box is on the lower right side of the

phone.  This must be unlocked, before the T-Bit located on the front of

the coin box can be turned to open the box.

The 4-Pin Medeco lock for the logic box is on the upper left side of the

phone.  This must be unlocked, before the T-Bit that is also located on the

upper left side of the phone can be turned.  Once this is done, the logic

box opens from top to bottom.  That is, the part of the phone that includes

the card reader, handset, keypad, display, and RBOC logo flip down.  The

hinge is located just under the card reader.

Oh, a little note about the Medeco locks.  Only 4 pins, you say?!?  Well

you can almost forget about trying to pick them.  Medeco locks are special, in

that not only must the pins be raised a certain amount, but they must also be

rotated a certain amount.  This rotation can be clockwise or anti-clockwise.

And also, both Medeco locks are keyed differently for security purposes.

How these phones operate is quite unusual.  The days of ACTS and its variants

are gone.  These new breed of phones operate on the COCOT principal.  I

sometimes call them BOCOTs.  The phone itself is responsible for billing.  Not

only for local calls, but for long distance and for overseas calling as well.

When you pick up the handset on a Millenium and hear a "dial tone", it is not

really a dial tone you are hearing.  It is mearly a fake dial tone that the

phone produces.  After dialing your number, the phone then decides on what

sort of call it is.  Is it local, long distance, or overseas.  If it is a

local call, then a synthesized voice asks for the $0.25 and the display also

prompts for the money.  Once the money is in, the phone picks up the real line

that it is connected to, and then re-dials the number that you entered into

its memory.  The call then goes on as normal.

If it is a long distance call, the phone checks it's rate table for current

rate based on time of day, day of week, and mileage to destination CO.  It

then asks for the appropriate amount of money and continues as with a local

call.  The same goes for an overseas call.

Now, stuff gets a little trickier if you plan to use a card to bill the call.

Milleniums are equipped with both a magnetic strip reader AND a smart card

reader.  However, not all RBOCs have issued smart cards.  Bell Canada

(Ontario/Quebec) has for sure, but as for the others, I don't know.  I know

that BC Tel doesn't have a smart card planned until mid '98 at the least.

Anyways, after you dial your number, you are asked to put in your money or

enter your card into the slot.  If you opt to enter your card into the slot,

then the phone reads in your card data.  It then takes the real line that it

is connected to off-hook and proceeds to call it's predetermined credit card

authorization center.  After authorizing your card, it hangs up, then

re-seizes the line and proceeds to dial the number that you have entered into

its memory.

A word about authorization.  In Canada, where Stentor owns DataPac AND the

RBOCs, it is possible to use real-time credit card authorization even for a

$0.25 call.  That means that your card better be valid, or it will fail

authorization.  However, in the USA, where de-regulation has been widespread,

an authorization costs about $0.50 for the use of a public switched network

such as telenet.  It doesn't make sense to spend this much money for an

authorization on a call that is only going to cost the customer on an average

of $2.30 for the first minute, and about $0.60 for each additional minute.

Well, at least that is the way the RBOCs in the USA think.  Because of this,

credit card authorization is not in real time.  Therefore, if you have a

magnetic strip writer, you could write a valid Visa number to an old strip,

and use it in a phone to call anywhere in the world for free, and without the

annoying prompt for more money every minute.  I have tried this in Seattle

(USWest) on one of their Milleniums and it worked fine).

However, in Canada, it won't work.  The card number that you write to the card

MUST be a REAL ISSUED number.  Of course, these aren't TOO hard to come by,

now are they?

Well, there are LOTS more things to be learned about these new Milleniums for

me and the rest of us, so get hacking!  These are THE phones that will take us

into the next millenium.  There are things to be done with the keypad, but I

don't know enuf to write about that at the present time. Codes that start with

* and #.

Anyways, my fingers hurt...

=[mastermiind: agent 777]=