The Telephone System

A Basic Overview

By:

EVIL INCARNATE

WARNING:

YOU NEED SOME ELECTRONIC EXPERIENCE TO READ THIS...BUT IF YOU KNOW ***REAL***

BASIC ELECTRONICS, YOU'LL SLIDE. BUT YOU CAN'T BE AN ULTRA-DWEEB EITHER, YOU

MUST (READ ****ABSOLUTELY MUST****) HAVE AN IQ OF GREATER THAN ROOM TEMPERATURE.

The telephone arrived as a practical instrument over a century ago in

1876 , an outgrowth of experiments on a device to send multiple telegraph

signals over a single wire.  Alexander Graham Bell, a native of Scotland,

while conducting electrical experiments spilled acid on his trousers.  His

sulphurous reaction, the now famous "Mr. Watson, come here, I want you",

brought Thomas A. Watson on the run not only because of his employer's

distress, but because the words had been carried by electricity into

Watson's room and reproduced clearly on his receiving set.  The simple

instrument being tested on Court Street in Boston on March 10, 1876  wasn't

very practical(the acid was used in the system) but improvement followed so

rapidly that putting into action Bell's concept of a public telephone

network - "this grand system", "whereby a man in  one part of the country

may communicate by word of mouth with another in a distant place" - was

well underway by January of 1878, when the first commercial exchange was

operated in New Haven.  By 1907, one hotel alone(the Waldorf Astoria in NY_

had 1,120 telephones and processed 500,000 calls per year.

That concept has grown into an industry in which one entity (AT&T) is

the largest company on earth, providing over one hundred million telephone

sets, making a profit several billion dollars per year, and employing over

one million people.

Telephone sets perform a surprising number of functions.  Here is a

list of the most important ones:

1.It requests the use of the telephone system when the handset is lifted.

2.It indicates that the system is ready for use by receiving a tone, called

the dial tone.

3.It sends the number of the telephone to be called to the system.  The

number is initiated by the caller by pressing number keys or rotating a

dial.

4.It indicates the state of a call in progress by receiving tones

indicating the status.

5.It changes speech of a calling party to electrical signals for

transmission to a distant party through the system.  It changes electrical

signals received from a distant party to speech for the called party.

7.It automatically adjusts for changes in the power supplied to it.

8.It signals the system that a call is finished when a caller "hangs-up"

the handset.

Of course for a telephone to be of any use, it must be connected to

another telephone.  In the very early days of telephones, the phones were

simply wired together with no switching.  This became impractical as the

number of phones increased and the local exchange or central office was

established to handle the switching and other functions.

Each subscriber telephone is connected to a central office that

contains switching equipment, signaling equipment and batteries that supply

direct current to operate the telephone.  Each phone is connected to the

central office through a local loop of wires called a wire pair(the other

wires on a phone line are grounds in case of over voltage)  One of theses

wires is called T(for Tip(DUH!)) and the other is called R(for ring) which

refers to the tip and ring parts of the plug used in manual switchboards.

Switches in the central office respond to the dial pulses or tones

from the telephone to connect the calling phone to the called phone.  When

this connection is established, the two telephones communicate over the

transformer coupled loops using the current supplied by the central office

batteries.

INITIATING A CALL

When the handset of the telephone is resting in its cradle, the

weight

of the handset holds the switchhook buttons down and the switches are open.

This is called the on-hook condition.  The circuit between the telephone

handset and the central office is open; however, the ringer circuit in the

telephone is always connected to the central office.  The capacitor blocks

the flow of DC from the battery(in the central office).  The ringer circuit

presents a high impedance to speech signals so it has no effect on them.

When the handset is removed from its cradle, the spring loaded

buttons come up and the switchook closes.  This completes the circuit to

the exchange and current flows in the circuit.  This is called the off-hook

condition.

The off-hook signal tells the exchange that someone wants to make a

call.  The exchange returns a dial tone to the called phone to let the

caller know that the exchange is ready to accept a telephone number.

SENDING A NUMBER

Some telephone sets send the telephone number by dial pulses while

others send it by audio tones.

DIAL PULSING

Most telephone sets that use dial pulsing have a rotary dial which

opens and closes the local loop  circuit at a timed rate.  The number od

dial pulses resulting from one operation of the dial is determined by how

far the dial is rotated before releasing it.

DUEL TONE MULTI-FREQUENCY

Some telephone systems use the newer method of using audio tones to

send the telephone number.  These can be used only off the central office

is equipped to process the tones.  Instead of a rotary dial, these

telephone sets have a push-button keypad with 12 keys, for the numbers 0

through 9 and the symbols * and the #.  Pressing one of the keys causes an

electronic circuit in the keypad to generate two output tones that

represent the number.

CONNECTING THE PHONES

The central off ice has various switches and relays that

automatically

connect the calling and called phones.  For now, assume that the connection

has been made.  The actual operation of switching systems will be covered

in more detail a little later.

If the called phone handset is off-hook when the connection is

attempted, a busy tone generated by the central office is returned to the

calling phone.  Otherwise, a ringing signal is sent to the called phone to

alert the called party that a call is waiting.  At the same time, a

ringback tone is returned to the calling phone to indicate that the called

phone is ringing(NOT TO BE CONFUSED WITH CALL WAITING WHICH WILL BE

EXPLAINED IN LATER TEXT PHILES)

RINGING THE CALLED PHONE

Early telephone circuits were point to point(not switched), and the

caller gained the attention of the party at the other end by picking up the

transmitter and shouting "Hello".  This was not very satisfactory, and

schemes based on a mechanical signaling arrangements were soon invented.

The one in common use today, called the "polarized ringer" or bell, was

patented in 1878 by Thomas A. Watson(Mr. Bell's assistant).

ANSWERING THE CALL

When the called party removes the handset in response to a ring, the

loop to that phone is complete by its closed switchook and loop current

reflows through the called telephone.  The central office then removes tex

ringing signal and the ringback tone from the circuit.

TALKING

The part of the telephone into which a person takes is called the

transmitter.  It converts speech (acoustical energy) into variations in an

electric current (electrical energy) by varying or modulating the loop

current in accordance with the speech of the talker.

The part of the telephone that converts the electric current

variations into sound that a person can hear is called the receiver.  The

signal produced by the transmitter is called by the loop current to the

receiver of the called party.  Also, a small amount of the transmitter

signal is fed back into the talker's receiver.  This is called the

sidetone.

Sidetone is necessary so that the person can hear his/her own voice

from the receiver to determine how loudly to speak.  The sidetone must be

at the proper level because too much sidetone will cause the person to

speak too softly for the good reception by the called party.  Conversely,

too little sidetone will cause the person to speak so loud that it may

sound like a yell at the receiving end.

ENDING THE CALL

The call is ended when either party hangs up the handset.  The

calling party can hang up the phone for a second, but the called party must

hang the phone up for twenty seconds for a disconnect.  The on-hook signal

tells central office to release the line connections.  In some central

offices, the connection is released when either party goes on-hook.  In

others theconnection is released when only the calling party goes on-hook.

BEYOND THE LOCAL LOOP

Thus far the discussion of connecting two telephones together has

been limited to local loops and a central office exchange.  Most central

office exchanges can handle up to 10,000 telephones.  But what if it is

required to connect more phones than 10,000 or connect phones in different

cities or in different states, or in different countries.  Over the years,

complex network of many telephone exchanges has been established to

accomplish these requirements.  Lets look next at how this network is

arranged.

THE PUBLIC SWITCHED TELEPHONE NETWORK

EXCHANGE DESIGNATIONS

Each telephone exchange in North America has two designations, office

class and name to describe its function.

Subscriber telephones are normally, but not exclusively, connected to

end offices.  Toll (long distance) switching is performed by Class 4, 3, 2,

1 offices.  The Intermediate Point or Class 4X offices may interconnect

subscriber telephones as well as other Class 5 and Class 4 exchanges.

The ten Regional Centers (Class 1 offices) in the U.S. and two in

Canada all are connected directly to each other with large-capacity trunk

groups(see ESS just stops use from 2600 Hz tones LOCALLY, but if you can

get past the local ESS, and get the 2600 Hz tone past ESS, then there's

nothing they can do, cause it would cause too much to change ALL THESE

trunks).  In 1981, there were 67 Class 2; 230 Class 3; 1,300 Class 4 and

about 19000 Class 5 exchanges.

INTERCONNECTION

The network is organized like a tree, or rather like a small grove of

trees, whose roots have grown together.  Each exchange is optimized for a

particular function.  A call requiring service which cannot be performed by

a lower class exchange is usually forwarded to the next higher exchange in

the network for further processing.

The regional Center like the base of each tree, forms the foundation

of the network.  The branch levels are the Class 2, 3, 4, 4X and Class 5

offices.  Most offices are connected to more than one other, and the

interconnections depend on the patters of the traffic arriving at and

leaving each office.

The network makes connections by attempting to find  the shortest

path from the class 5 office serving the caller to the class 5 office

serving the  called party.  The high usage interoffice trunk groups which

provide direct connection between offices of equal and lower level are used

first.  If they are busy, trunk groups at the next higher level(called

final groups) are used.  Digital logic circuits in the common control of

each exchange make decisions based on rules stored in memory that specify

which trunk groups are to be tried and in what order.  These rules, for

example, prevent more than nine connections in tandem, and prevent endless

loop connections.

STRUCTURE

The supervisory signals used to set up telephone connections and the

voice signals of the conversations are carried by transmission systems over

paths called facilities.  These systems are divided into three broad

categories, Local, Exchange Area, and Long-Haul.

THE LOCAL NETWORK

The local network is the means by which telephones in residences and

businesses are connected to central offices.  The local facilities are

almost exclusively wire pairs which fan out like branches of a tree from a

point called the wire center throughout a serving area.  Serving areas vary

greatly in size, from an average of 12 square miles in urban locations to

130 square miles for rural areas.  An average wire center in an urban area

will serve 41,000 subscriber lines and 5,000 trunks.  The urban exchange

are generally of higher call carrying capacity than the rural exchanges.

THE EXCHANGE AREA NETWORK

The exchange area network is intermediate between the local network

and the long-haul network.  Exchanges are interconnected with exchange area

transmission systems.  These systems may consist of open wire pairs on

poles, wire pairs in cables, microwave radio links, and fiber optic cables.

The exchange area network normally interconnects local exchanges and tandem

exchanges.  Tandem exchanges are those that make connections between

central offices when an interoffice trunk is not available.  A tandem

exchange is to central offices as a central office is to subscriber

telephone sets.

THE LONG HAUL NETWORK

In the long-haul network, local exchanges are interconnected with

toll(long distance) exchanges. These facilities are normally of high

capacity per circuit, and consist mostly of cable and microwave radio

links.  In some paths (called routes) which require a great many links,

such as the backbone links between Boston and Washington, very high

capacity fiber optic links, each carrying about 4000 voice channels

simultaneously.

TYPES OF TRANSMISSIONS

Spoken messages or voice signals are not the only signals that are

transmitted on a phone line.  In the previous discussion of making a

connection between the calling telephone and the called telephone, some of

these other signals were discussed, dial tone, dial pulses or key tones

used for sending a number, busy tone, and ringback tone.  These are for

control of the switching connections or to indicate the status of the call.

Such signals are called control signals or supervisory signals.  They may

be tone signals(analog) or ON-OFF(digital) signals.  Therefore if one were

to examine the signals on many local loops, one would find analog voice

signals, analog tone signaling, and digital ON-OFF signaling.  It would be

a mixture of analog and digital signals.

ANALOG VOICE TRANSMISSIONS

Signals that have continuously and smoothly varying amplitude and

frequency are called analog signals.  Speech signals are of this type.

They vary in amplitude and frequency. Voice frequencies that contribute to

speech can extend from below 100 Hz to above 6000Hz.  However, it has been

found that the major energy necessary for intelligible speech is contained

in a band of frequencies between 200Hz and 4000Hz.

VOICE CHANNEL BANDWIDTH

In order to eliminate unwanted signals that could disturb

conversations or cause errors in control signals, the circuits that carry

the telephone signals are designed to pass only certain frequencies.  The

range of frequencies that are passed are said to be in the bass band.  0 to

4000 Hz is the pass band of a telephone system voice channel.(a VF channel)

Bandwidth is the difference between the upper limit and the lower

limit of the pass band; thus. the bandwidth if the VF channel is 4000 Hz.

However not all of the VF channel is used for the transmission of speech.

The voice pass band is restricted to 300 to 3000 hertz.  Hence any signal

carried on the telephone circuit which is within the range of 300 to 3000

hertz is called an inband signal.  Any signal which is not within the 300

to 3000 Hz band but is within the VF channel is called an out of band

signal.  All speech signals are in band signals.  Some signaling

transmissions are in band and some are out of band.

VOICE CHANNEL NOISE

Transmission systems often must operate in the presence of various

unwanted signals (referred to generally as noise) that distort the

information being sent.  Lightning, thermal noise, induced signals from

nearby power lines, battery noise, corroded connections, and maintenance

activities all contribute to degradation of the signal.  Analog channel

speech quality is primarily determined by the absolute noise level on the

channel when it is idle; that is when there is not speech signal present.

Speech tends to mask any noise present, but noise in an idle channel is

quite objectionable to a listener.  Stringent standards have been set for

this idle channel noise in the US network.

Another type that originates from the voice transmission itself is an

echo.  The primary echo is the reflection of the transmitted signal back to

the receiver of the person talking.  The amount of delay in the echo

depends on the distance from the transmitter to the point of reflection.

The effect of the delay on the talker may be barely noticeable to the very

irritating to down right confusing.  Echo also affects the listener on the

far end, but to a lesser degree. Echoes are caused by mismatches in

transmission line impedances which usually occur at the hybrid interface

between a 2 wire circuit and a 4 wire transmission system.

MULTIPLEXING

A local loop can carry only one voice channel conversation at a time.

This is not economical for toll transmission and a method was devised so

that a transmission path can carry many telephone conversations at the same

time.  This is accomplished by Multiplexing.  For analog signals, frequency

division multiplexing(FDM) is used.  In simplified terms, this means that

several telephone conversations are all sent together over one transmission

channel, but are separated by their frequency.

The basic principles of this are a voice signal having frequencies

within the voice frequency channel bandwidth of from 0 to 4 kilohertz is

changing the amplitude of another frequency(8,140 Khz) which is called the

carrier frequency.  The 0to 4kHz voice frequency signal is amplitude

modulating the 8,140 Khz carrier.  The information in the voice signal is

being carried by the changing amplitude of the 8,140 Khz signal and the

voice frequencies have been translated to different frequencies.

If different voice signals are placed on different carrier

frequencies, then many conversations may be multiplexed on one transmission

path and transmitted to the receiving point.  At the receiving point, the

different conversations can be identified and separated by their unique

frequency and the original conversation can be recovered from the

carrier(demodulated) and sent to the called telephone.

Since each voice channel has a 4 Khz bandwidth, 12 channels require

48kHz bandwidth.  Since the lower frequency in the example is, 8,140 kHz,

the output multiplexed signal frequency extends from 8140 kHz to 8188 kHz.

It should be apparent that if the individual voice channel bandpass were

made larger, the spread in carrier frequency would have to be larger or if

the number of voice channels to be multiplexed together, the spread in

carrier frequencies would have to be larger.  In technical terms, in

general, as the number of voice channels to be transmitted, over a

transmission path increases, the required bandwidth of the transmission

path must increase.

SIGNALING TRANSMISSION

As stated previously, signaling refers to specific signals on the

transmission line that are used for controlling the connection from the

calling telephone to the called telephone, or that are used to indicate the

status of a call as it is being interconnected.  The first type to be

discussed is dc signaling.

DC SIGNALING

DC signaling is based on the presence or absence of circuit current or

voltage, or the presence of a given voltage polarity.  The stante of the

signal indicates on-hook, off-hook, dial pulses, or status of the

interconnection.  These signals are on-off type digital signals.

On local loops, an on-hook is indicated by an open circuit and no

current flow.  Off hook is signaled by a closed circuit and a continuous

current flow.  Dial pulses consists of a current flow interrupted at a

specified rate as discussed previously.

A type of dc signaling called reverse battery signaling is used

between central offices to indicate the status of the switched connection.

When the newer end exchange requests service, an idle trunk is seized.  A

polarity oar a given voltage exists on the trunk which indicates to the

near end that the called phone is on hook and ringing.  The far end

exchange acknowledges and indicates to the near end that the called party

has answered by reversing the voltage polarity.

E&M signaling is used for the same purpose on long interoffice and

short hail toll trunks.  This type signaling requires two extra wires in

the originating and terminating trunk circuits, one for the E lead and the

other for the M lead.  Since separate wires are used for each, the on-hook

and off-hook states can be signalled from both ends of the circuit.  This

allows signaling to be sent in both directions at the same time without

interfering with one another.  Sometimes two wires are used for each signal

to avoid noise problems caused by a common ground.

TONE SIGNALING

Various tones are used for both control and status indication. The

tones may be single frequency or combinations frequencies.  These are

analog signals that are either continuous tones or tone bursts.  The call

progress tones are sent by the exchange to the calling phone to inform the

caller about the status of the call.  For example the dial tone which has

been mentioned previously is a continuous tone made by combining the

frequencies of 350Hz and 440Hz.  The busy signal that tells the caller that

the called phone is off-hook, is a combination frequency tone that appears

in bursts of .5 seconds on time separated by an offtime of .5 seconds.  The

receiver off-hook warning signal is separated by an offtime of .5 second.

The receiver off-hook warning signal is a combination frequency tone of

four frequencies which os on for .1 seconds and off for .1 seconds.  This

signal is very loud in order to get the attention of someone to hang up the

receiver that has been left off-hook.  All of these tones as well as the

DTMF addressing tomes discussed previously are in-band signaling.

---------------------------------------------------------------------------

This has been a Digital Rodent Syndicate Production by Evil Incarnate.

Sysops may use this text file as well as long as it is not modified in any

way.  Call these ULTRA-FINE boards:

(415)524-3649     RatHead           Sysop:Ratsnatcher       Co:Myself

(415)524-9951     The Crystal Ship  Sysop:Laughing Swede    Co:Myself

(415)527-9444     IHOL              Sysop:Yellow Jacket     Co:Myself

(415)649-0416     Metropolis        Sysop:Doppy Flisk       Co:Ravenman(phew!)

All of these boards except for metro are 2400 baud and Metro is 1200 baud.

This text file is simply for informational purposes only, no illigal

activity is to be derived from this file, and if any illigal activities are

derived from this file, i am not responsible in any way.

Leave me mail on ANY of these fine boards about future text files.  Next

one will be on Switching systems..Ever notice how NOBODY talks about

Crossbar, even though some of us are still on Crossbar? I'll deal with ESS,

ANI Level 5, tracing, and Crossbar on my next text file.