From: Raymond J. Rueb

How Toll Busy Line and Emergency Interrupt Work

After reading the previous discussions, I think there is some

confusion about what the operator is capable of during BLV and EI

situations. So,

"How Toll Busy Line Verify (BLV) and Emergency Interrupt (EI) Work"

I worked on OSPS BLV & EI for AT&T, and I became familiar with how

TSPS works also. For TOPS info, ask someone else (though this

information should be close).

The best way to understand BLV and EI is to run through a customer to

operator to termination scenario.

1) After repeatedly receiving a busy signal from their intended

forward number, a customer calls the operator and asks if the operator

can help.

2) The operator enters "VERIFY mode". On TSPS this is done by

accessing a separate call loop and performing the VERIFY on that loop.

On OSPS the operator has a separate VERIFY key to press which causes

the switch to perform the following checks:

a) Is there a valid forward number?

b) Is the forward number within this operator's VERIFY network? If

not, the operator must go inward to the local operator who can

then perform the VERIFY. More about VERIFY networks later.

c) Is the forward number verifiable? ie: is it on the list of numbers

of which the operator is not allowed to perform BLV and EI. I never

understood this idea, maybe the CIA might rest easier, but it really

is a security redundancy.

3) If this operator can perform the VERIFY, then the operator presses

send.

a) The call begins to route over the VERIFY network.

b) The back party is automatically split from the connection (for

security reasons, OSPS only; in TSPS the operator is performing the

VERIFY on a separate loop and there is no back party on that loop)

c) The operator's talking path is disabled.

d) A voice scrambler circuit kicks in (in TSPS this is a physical

circuit attached to the trunk, in OSPS this is firmware in the

operator's VDT). The scrambler allows the operator to tell IF

conversation is taking place, but not WHAT is being said.

It kinda sounds like ducks talking.

4) The call arrives at the local Central Office (CO) on an incoming

BLV trunk.  This is where my knowledge is a little weak. If the number

to be verified is a line loaded on that switch, then we're home free.

The switch automatically causes a test trunk to bridge the port

associated with the number being called and the operator is now

connected with all forward parties associated with the call. If,

however, the line is on a PBX connected to the CO, the connection MAY

be verifiable. I believe all true PBX's must have incoming VERIFY

trunks and be capable of performing BLV bridging. I know that some

PBX's can do this (a 5ESS Switch (tm) can be a PBX), but I don't know

whether ALL PBX's can do this. It works on centrex.

5) The operator listens to the scrambled connection to determine if

the line is in-service or not. This method of determination has the

following drawbacks:

a) If there is a long lull in the conversation when the operator

performs the VERIFY, the operator might assume that the number is

out of service.

b) Most operators don't seem to be able to recognize data connections

through the scrambler (what's that funny noise???)

c) A pre-howler announcement, "please hang-up the phone" sounds like

conversation. There's actually a lot of debate as to what constitutes

in-service; if the telco's product is working, but the person has

recently left the phone off-hook, is it in-service?

6) The operator splits the forward connection, unsplits the back party

(TSPS operators change loops) and informs the customer of the status.

7) The customer requests an EI.

8) The OSPS operator presses the EI key.

a) The back party is split.

b) A two second long 440Hz tone is applied to the forward connection.

c) The scrambler is deactivated.

d) The operator's voice path with the forward connection is restored.

e) Every 10 seconds, a 0.5 second tone is repeated to serve as a

reminder to the forward parties that their conversation is no longer

private. This tone continues until the operator breaks the EI bridge.

9) The operator can now talk with the forward party and request that

they hang-up so that the back party can call them.

10) The operator relays the response to the back party.

At this point the operator will charge the customer for the VERIFY and EI.

If they have been taught well, the operator will ask if the back party

wants the operator to connect them with their intended forward party.

This will result in additional charges for an operator assisted call.

NOTES about BLV and EI service:

1) BLV and EI are separately tariffed services, but are always flat

rate. Some states, like Michigan, don't allow charging for BLV but do

allow EI charges.

2) During VERIFY and EI, the back party and forward party are NEVER in

contact.

3) Calls CANNOT be completed across the VERIFY network.

4) The VERIFY network USUALLY crosses LATA boundaries.

5) The VERIFY network is often BOC owned, and often only the BOCs are

allowed access to them. This means that BOCs frequently are in the

position of providing INTER-LATA BLV and EI services. Since BOCs are

forbidden from providing inter-lata services like COLLECT and CARD#,

why is inter-lata BLV allowed?

NOTES about the VERIFY network and security:

1) The VERIFY network is separate from the toll network.

2) Routing on the VERIFY network cannot occur from a customer's phone.

If you're accidently routed to a VERIFY trunk, your signaling will be

wrong.

3) Much emphasis has been placed on operators being able to break into

private phone conversations as a breach of privacy. Given the way BLV

and EI perform (with the scrambler and tones) I don't feel that this

is an issue.

HOWEVER....

4) Craft test sets have special software to prevent routing on trunks

MARKED as VERIFY trunks, but there is NOTHING to prevent a craft

person from modifying how a trunk is marked, and then routing (of

course it is still special routing signalling that must be used, but

them craft seem to know about it).  I was appalled to find out that

this is common practice when testing the BLV trunks. What this means

is that a skilled, but unscrupulous, craftsperson has the

unsupervised, unmonitored, unscrambled ability to listen in on ANY

active phone conversation anywhere their VERIFY network reaches. Guess

you gotta trust them.####################################################################################################################

Downloaded from Just Say Yes. 2 lines, More than 1500 files online!

Do you write? Give us a call!   415-922-2008 CASFA

Another file downloaded from:                     NIRVANAnet(tm)

& the Temple of the Screaming Electron              510-935-5845

Rat Head                                            510-524-3649

Burn This Flag                                      408-363-9766

realitycheck                                        415-474-2602

Lies Unlimited                                      415-583-4102

Specializing in conversations, obscure information, high explosives,

arcane knowledge, political extremism, diversive sexuality,

insane speculation, and wild rumours. ALL-TEXT BBS SYSTEMS.

Full access for first-time callers.  We don't want to know who you are,

where you live, or what your phone number is. We are not Big Brother.

"Raw Data for Raw Nerves"