3/5/92        Asynchronous Communications Eavesdropper        Page 1

Version 2.0

Copyright (c) 1991,1992  Nick Jancewicz

P.O. Box 3491, Boulder CO 80307

The Asynchronous Communications Eavesdropper (ACE) is intended for

personal use and may be freely copied and distributed as long as it

is not done for profit.  The author makes no warranties of any kind

relating to this software and assumes no liabilty regarding its use.

*************************************************************************

Requirements:

* ACE software

* IBM-compatable PC with two serial ports (configured as COM1 and COM2)

* DOS 2.1 or later with ANSI.SYS driver installed in the CONFIG.SYS file

*************************************************************************

ACE is a software utility that turns any IBM PC compatable equipped

with two serial ports into a versatile serial communications monitor.

It can display and capture to disk an both data streams between

two devices operating in full-duplex mode.  ACE "eavesdrops" on both

sides of a "conversation" between two serial devices with the aid of a

special cable which taps into the wires over which the two devices

communicate.  It thus provides an objective, "third-party" view of the

data transmitted between the two serial devices and, in the event of

communications difficulties, enables you to ascertain what went wrong

and to identify which device is at fault.

The characters transmitted by one device are displayed in regular video

(white characters on black background) while the other device's

transmissions are displayed in reverse video (black characters on

white background).  Thus, the relative character order and timing are

clearly distinguished.  As the data streams are displayed, an exact

record can also be captured to a disk file for later inspection and

analysis.  To switch between the regular and reverse video display

attributes, ACE uses special ANSI escape sequences.  This requires

that you have the standard DOS ANSI.SYS device driver installed on the

host PC. This is done by inserting the line, "DEVICE=C:\DOS\ANSI.SYS"

(or a similar line instructing DOS where on your disk the ANSI.SYS

file can be found) in your config.sys file.  Alternatively, (or in

addition) you may use the "dte" and "dce" command line parameters to

label each device's transmissions and so distinguish them in that

manner (see NOANSI.BAT under Miscellaneous Utilities below).

A simple 3-way Data Tap (or Data Tee) is often used for the same

purpose, but has the inherent problem of mixing transmissions from

both devices on the same conductor so that when both devices transmit

at the same time the results will be garbled.  In Boolean terminology

this is known as a "Wired-OR" circuit because the two signals (i.e.

the serialized characters) are logically ORed together by a

double-diode circuit.  This is inappropriate for monitoring

full-duplex communications, that is, where both serial devices can

3/5/92        Asynchronous Communications Eavesdropper        Page 2

transmit at the same time.  In most serial communication protocols,

full-duplex is the rule rather than the exeception.  In debugging full

duplex protocols, it is important to know which device transmitted

first and in what order relative to the transmissions of the other

device.  ACE, when used on a PC with two serial ports (COM1 and COM2),

keeps the two signals separate from each other by a special cable

which routes the transmissions from one device to the COM1 port and

the transmissions from the other device to the COM2 port.

Even in situations where only one COM port is available and a Data Tee

must be used, monitoring with ACE still offers some advantages over

using a modem communications utility.  Such general-purpose

communications programs were never intended for monitoring binary

communications and hence lack the ability to display (in a meaningful

way) either binary transmissions or the "non-displayable" characters

that are often intermixed with normal ASCII text.  Many modem

utilities cannot capture an accurate recording of a communications

session to disk due to the fact that they completely ignore nulls in

the data stream.  Even worse, they also ignore parity errors, framing

errors and break errors which are critical to detecting problems in

transmission line quality and/or incompatible communications parameter

settings between the serial devices under test.  When a parity,

framing and/or break error occurs, ACE beeps and displays an error

message.

All of the parameters which ACE needs in order to monitor a serial

communications session are supplied as command line options when you

invoke it at the DOS prompt.  This makes it easy to set up batch files

for monitoring various devices with different protocols and

communications parameters (see the examples under Miscellaneous

Utilities below).  Typing "ACE" by itself at the DOS command prompt

results in a brief help screen being displayed which gives a summary

of the required options and their respective order.  Most of these are

self explanatory, i.e. baud rate, parity, number of data bits, and

number of stop bits.  The command line usage is:

ace baud parity data stop format [dte=s] [dce=s] [log=n] [-t] [-f] [file]

where,

baud      is the baudrate: 50,75,110,134.5,150,300,600,1200,1800,

2000,2400,3600,4800,7200,9600,19200,38400 or 115200

parity    is N,O,E,M or S for none, odd, even, mark or space parity

data      is the number of data bits: 5,6,7 or 8

stop      is the number of stop bits: 1 or 2

format    is H to show non-displayable characters in ASCII Hex format,

or N to show all characters in native format,

or A to show all characters in ASCII Hex format

file      is filename for logging session

-t        enables timestamping of each transmission

dte=s     specifies optional label to identify which is DTE

dce=s     specifies optional label to identify which is DCE

("s" is a string which may contain backslash-prefixed

ASCII Hex values, e.g. "\0D\0A")

log=n     selects alternating log files and sets their max size to n bytes

-f        selects fast display format

The parameters shown in brackets are optional.

3/5/92        Asynchronous Communications Eavesdropper        Page 3

ACE has three possible display options for monitoring binary or mixed

binary and ASCII text communications.  With the "H" option,

non-displayable characters are converted to a readable format

consisting of a backslash character followed by the two ASCII-hex

digits representing the binary value for the character.  With the "N"

option, all characters are displayed in "native" form i.e. if

character codes below 20 hex or above 7F hex are encountered they will

appear as special IBM graphics characters, producing an effect similar

to what you see when you dump a binary file (e.g. an executable) to

the screen with DOS's TYPE command.  The third option, "A", displays

all characters in ASCII-hex.  This latter option is useful when

monitoring binary protocols.

There are several optional command line parameters which allow you to

make the session more readable, i.e. the "dte" and "dce" options.

These allow you to specify identifier strings to distinguish between

the two devices being monitored which ACE will insert into the data

stream as it is captured.  These strings may include binary values

such as Carriage Returns and Line Feeds which are specified by

"backslash escape sequences", e.g.  "\0D" and "\0A", respectively.

You may also instruct ACE to insert a timestamp each time the data

stream changes from one device to the other by including the "-t"

command line option.

The "log=n" option is useful for capturing long duration sessions to

disk.  If used, this option will cause ACE to initially start

capturing the session to a file called "log1".  When the size of the

file nearly reaches the number of bytes specified by "n", the first

file will be closed and ACE will start capturing to a second file

called "log2".  When "log2" reaches the specified size, ACE will

switch back to "log1" again.  This keeps your disk from filling up,

which can happen in a very short time if you are dealing with a high

communications throughput application.

The "-f" option is for monitoring very high speed communications (e.g.

115200 baud) or for monitoring with slower, XT class PCs. Most users

will never need to use this option, but it is described here for the

benefit of Dedicated Hacker types.  The "-f" option greatly reduces

the overhead of displaying the data stream(s), by displaying just one

character at a time from each port (by writing directly to the EGA

video RAM).  You will want to specify a filename as the last command

line parameter whenever this option is used so that ACE will dump the

contents of its communication buffer to the specified file upon

terminating.  This option is a bit of a kludge, but it does let you

save the last 28,000 or so bytes that were received.  Note that the

file is a BINARY IMAGE of ACE's communications buffer, regardless of

the display format that was selected.  Each element in the buffer is

two bytes, where the high byte encodes the UART status (refer to 8250

data sheets for details) and the low byte is the received character.

The first two bytes are the oldest character and its status and the

last two bytes are for the most recent character received.  This file

can be viewed with DOS's DEBUG or a binary file dump utility.

The last command line option may be omitted but if present, specifies

the name of a file for recording the communications session.  If the

file already exists, ACE will append the current session to it

otherwise a new file will be created.  Note that this option may not

be used if the "log=n" option is used (they are mutually exclusive).

3/5/92        Asynchronous Communications Eavesdropper        Page 4

To terminate a communications session with ACE, press the Esc (escape)

key at any time.

RS-422 or RS-485 signals can be monitored through RS-232C COM1 and

COM2 ports by connecting only the "negative" side (i.e. the one marked

"-" or "B") of each differential pair.  However, like the Data Tee's

double-diode trick, this is not guaranteed to work with all RS-232C

receivers.  In my experience I have only ever encountered one COM port

which would not work with the above technique.  Oddly enough it was an

early model IBM Asynchronous Communications Adapter Card and it did

NOT use the ubiquitous MC1489 RS-232C driver chip found on most other

serial cards.

*************************************************************************

ACE Cable

Although no special hardware is required for the host PC (aside from

the two serial port adapters configured as COM1 and COM2) a special

cable is required which allows the host PC to "tap" into the

connections between the two DUTs (Devices Under Test).  One end of

this cable connects "in-line" with the DUT while the other end

connects to the two COM ports on the host PC. See Figure 1 below.

The cable may consist of an RS-232C "patchbox" (similar in appearance

to a "null modem" adapter, but containing a printed circuit board with

a dual row of solder pads for soldering jumpers to establish custom

interconnections between the two DB-25 connectors at either end) and

two cables (or one cable with two DB connectors attached at the

opposite end).

The basic idea is to connect one cable in parallel with the DUT's TXD

line and the other cable in parallel with the DUT's RXD line of the

cable connecting the two devices to be monitored.  Each cable connects

to one of the two serial ports (COM1 and COM2) in the PC which serves

as the ACE monitor.  Thus, the RXD line of the DUT connects to the RXD

input of COM1, and the TXD line connects to the RXD input of COM2.

This approach should work with just about any device which has a fully

compliant RS-232C interface, since the DUT is only subjected to one

additional RS-232C electrical load by the parallel connection.  This

is easily handled by a true RS-232C driver.

On a typical RS-232C patchbox, you will need to solder in place

jumpers for all of the signals used by the DUT. Most often, this

requires only pins 2 (TXD), 3 (RXD) and 7 (Signal Ground) but for a

deluxe, "do all, end all" cable you will want to connect pins 2, 3, 4,

5, 6, 7, 8, 20 and 22 straight through.  The next step is to connect

the cable(s) which "tap" into TXD, RXD and SG conductors in the

patchbox and carry their respective signals to the two COM ports on

the host PC. The approach I prefer is to use one cable on the patchbox

end and then splice two shorter cables onto the end that connects to

the host.  Two DB-25 connectors (or two DB-9s) can be used for the

host end but I've found that most AT- compatable PCs typically have a

DB-9 for COM1 and a DB-25 for COM2 so I usually use that combination.

Note that the pin numbering and signal assignments for a DB-9 differs

from that of the standard DB-25 connector as shown in Table 1.

3/5/92        Asynchronous Communications Eavesdropper        Page 5

Figure 1: ACE Cable

PC:  COM1      COM2

(DB-9)    (DB-25)

2  5      3  7

^  |      ^  |

|  |      |  |

|  |      |  |

DTE                |  |      |  |                     DCE

(DB-25)              |  |      |  |                   (DB-25)

2 ----------------*--|------|--|--------------------> 2

3 <------------------|------*--|--------------------- 3

7 -------------------*---------*--------------------- 7

DTE (Data Terminal Equipment) and DCE (Data Communications Equipment)

are the EIA's terminology (somewhat archaic) for the devices that

transmit on pin 2 and pin 3, respectively.

*************************************************************************

Table 1: RS-232C Signals and Connector Pinouts

Signal  DB-9    DB-25   Function             DTE  DCE

------  ----    -----   ------------------   ---  ---

TXD     3       2       Transmit Data         O    I

RXD     2       3       Receive Data          I    O

RTS     7       4       Request To Send       O    I

CTS     8       5       Clear To Send         I    O

DSR     6       6       Data Set Ready        I    O

SG      5       7       Signal Ground         -    -

DCD     1       8       Data Carrier Detect   I    O

DTR     4       20      Data Terminal Ready   O    I

RI      9       22      Ring Indicator        I    O

Note: I = Input, O = Output

*************************************************************************

Miscellaneous Utilities

There are some additional utilities which can be used with ACE to

modify captured session files in various ways.  These are mainly

useful for converting captured session files into a printable format

(for hardcopy output on a printer) although some of them have other

uses as well.

ACE12.BAT - demonstrates how to invoke ACE from a DOS batch file to

monitor at 1200 baud, no parity, 8 data bits, 1 stop bit, with ASCII

Hex display mode and capture to a session file called "test1".

NOANSI.BAT - demonstrates how to use ACE on a PC without ANSI.SYS.  It

invokes ace.exe at 1200 baud, no parity, 8 data bits, 1 stop bit, with

ASCII Hex display mode, displays DTE data stream on a newline with

"DTE: ", displays DCE data stream on a newline with "DCE: ", and

3/5/92        Asynchronous Communications Eavesdropper        Page 6

captures to a session file called "test2".

FILT.EXE - filters a file (or the standard input device), by replacing

any arbitrary sequence of bytes into a different sequence of bytes.

This is like a global "search and replace" command except that it

works on binary files and strings as well as standard ASCII.

H2B.EXE - filters a file (or the standard input device), converting

any ACE style backslash escape sequences into their binary equivalents

(e.g. the displayable ASCII sequence "\02\04" gets converted into two

binary bytes: 02, 04 or STX, EOT).  This utility is also handy for

sending arbitrary byte streams to devices such as printers and

plotters, e.g. escape sequences for printer initialization.

ELITE.BAT - an example of how H2B.EXE can be used for initializing a

printer.

SLICE.EXE - divides a huge file into more manageble pieces.  Most text

editors can't deal very effectively with multi-megabyte files

(especially ones containing arbitrary binary sequences) so this

utility can be used to break up a very large session file into smaller

chunks which can then be processed more conveniently.

SES2DOC.BAT - uses filt.exe to convert a session file captured without

the dte and dce command line options into an editable document file.

It replaces the ANSI escape sequences for reverse and regular video

attributes with ASCII labels to identify which device transmitted

what.