Original text compiled by Ralph Brandi, (ralph@mtunq.att.com on

Internet)

Shortwave Frequency list by 72330.3515@CompuServe.com

Edited by Shel Hall (76701,103 on CompuServe, 76701.103@ CompuServe.com

on InterNet)

This posting contains answers to the following questions:

o What is shortwave radio?

o Where can I find broadcasts by Radio Foobar?

o Where can I find a list of broadcasts in the English language?

o What kind of receiver should I get?

o Where can I get a shortwave radio?

o Could you explain the frequencies used?  What's the 40 meter band? etc.

o Why can't I receive all of the broadcasts listed in Monitoring

Times/WRTH/Passport/etc.?

o What are some books or other resources that can help me get started?

o Where can I find further information?

o What can I listen to now in the Mid-East?

o What is shortwave radio?

From a purely technical point of view, shortwave radio refers to those

frequencies between 3 and 30 MHz.  Their main characteristic is their

ability to "propagate" for long distances, making possible such

worldwide communications as international broadcasting and coordination

of long-distance shipping.

From a social point of view, shortwave radio is a method of

facilitating worldwide dissemination of information and opinion, and a

way to find out what the rest of the world thinks is important.  Many

countries broadcast to the world in English, making it easy to find out

what a given country's position is on those things it finds important.

Shortwave radio can also provide a way to eavesdrop on the everyday

workings of international politics and commerce.

o Where can I find broadcasts by Radio Foobar?

The World Radio TV Handbook is the standard reference for this sort of

information.  The WRTH provides SWLs worldwide with virtually everything

they need on frequencies, schedules and addresses.  It comes out

annually, right about the first of the year, with an optional update

magazine throughout the year.  It covers virtually every shortwave

station in the world, and many of the medium wave (AM), FM, and

television stations as well.  The body of the book is a listing of

stations by country, with a cross-reference in the back by frequency.

It's available from any radio store dealing in shortwave.

World Radio TV Handbook

ISBN 0-8230-5921-9

Billboard Publications      Billboard Ltd.        WRTH

1515 Broadway               71 Beak Street        Soliljevej 44

New York, NY  10036         London W1R 3LF        DK-2650 Hvidovre

United States               United Kingdom        Denmark

The past five years have seen competition of a sort for the WRTH, in the

form of Passport to World Band Radio.  Passport's main section is a

graph/table of what's on the air, by frequency.  There are few

addresses, but the beginning of the book is filled with articles of

interest to the beginner.  There is also a comprehensive review section

of shortwave receivers currently available, one of the few places all

this information can be found in one place.  The book is more useful for

identifying a station you've already tuned in than for searching out a

particular transmission; the WRTH is useful at both, however, rendering

the purchase of this book not essential.  It can still be worthwhile,

though, especially for beginners who won't be put off by the "gee whiz,

look what we can listen to" tone of some of the articles.  The book is

unabashedly an advocate of making the hobby of "World Band Radio"

accessible to people who wouldn't have participated before the advent of

good, cheap portables.

Passport to World Band Radio

International Broadcast Services, Ltd.

Box 300

Penn's Park, PA  18943

For utility band listeners, there are a couple of books that perform

much the same function as the above two books, although due to the

nature of point-to-point communication, not with the same sense of

definitiveness.

Confidential Frequency List

Published by Gilfer Shortwave

(address elsewhere)

The Shortwave Directory

Published by Grove Enterprises

(address elsewhere)

o Where can I find a list of broadcasts in the English language?

The World Radio TV Handbook used to carry this information each year,

but this feature is not present in the 1990 edition. (It will return,

however, in future editions, and in fact, the WRTH has printed up a

supplement to the 1990 edition with the information.  You should be able

to get this at the addresses above.) Nevertheless, there are still

sources for this information.

-Monitoring Times magazine carries a listing every month, one of the

best arguments I know of for subscribing (it's what keeps me on their

rolls....)

-The North American Shortwave Association (NASWA) periodically publishes

a complete listing in their bulletin, The Journal, sent to all members

monthly; each month there are updates to the list.  NASWA can be reached

at:

NASWA

45 Wildflower Road

Levittown, PA 19057

Membership costs: $23/yr; sample issue $1

-Tom Sundstrom, sysop of the Pinelands BBS in New Jersey (609-859-1910

modem) offers a subscription service with constantly updated electronic

versions of his listing (which are also the source for the NASWA

listings).  It comes in text form, or formatted for Tom's Shortwave

Database program for MS-DOS computers.

The Shortwave Listeners Program Guide which used to be listed in this

article appears to have ceased publishing; the listing has therefore

been removed.

o What kind of receiver should I get?

That depends largely on what kind of listening you expect to do.  There

are two or three basic kinds of radios.  The first is the travel

portable.  These usually cost between $70 and $250.  Their main

characteristic is their extremely small size, making them most suitable

for the person who spends a lot of time on airplanes.  They do an

adequate job of receiving the major broadcasters, such as the BBC, the

Voice of America, Radio Nederland, etc. They are generally not capable

of receiving hams, or utility transmissions, and they do not do a good

job on weak stations.  Many of them also lack frequency coverage beyond

the major international broadcasting bands.  As such, they cannot

receive the out-of-band channels that often provide clearer reception

(due to lessened interference) of such stations as the BBC, Kol Israel,

and the Voice of Iran.

The second category overlaps with the first, and consists of slightly

larger portables.  Common among this category are radios like the

Sangean ATS-803A, a fine starter radio with many capabilities for the

inexpensive price of $200. These radios often have digital readout,

making it easier to know which frequency you are tuned to, and such

features as dual conversion (which decreases the possibility of your

radio receiving spurious signals from other frequencies), audio filters

(which allow you to decrease interference from stations on adjacent

frequencies) and beat frequency oscillators (which allow you to decode

morse code and single sideband (SSB) transmissions on the ham and

utility bands).  The top range of this kind of radio includes

technically sophisticated radios like the Sony ICF-2010 and Grundig

Satellit 500, which contain innovative circuitry to lock on to a given

signal and allow you to choose the portion of the signal you want to

listen to, depending on which part gets the least interference.  If you

follow the newsgroup for any amount of time, you're bound to notice some

discussion of the relative merit of these features versus their cost

(about double that of the Sangean radio.)  Many of these radios can be

and have been used to receive distant and weak stations from a number of

countries; they're also suitable for listening to programs from the

major broadcasters.

The third category of receivers is the tabletop receiver.  These

receivers cost from $600 upward, with a concentration of radios around

$1000.  These radios naturally contain many more features than the

portables, and are used by serious hobbyists who specialize in rare and

weak stations.  Current radios in this group include the ICOM R-71A, the

Kenwood R-5000, and the Japan Radio Corporation NRD-525.  These radios

can be very complex to operate, and are generally not recommended for

the beginner.  Radios from the first two categories can give a beginner

a very good idea of what's on the air and where their interests lie, at

which point one of these radios may be an appropriate acquisition.

There are many sources for detailed information on specific radios, most

of it provided by two groups.  Larry Magne, who publishes the Passport

to World Band Radio, includes a review of virtually all shortwave radios

currently available in that publication.  For more extensive reviews of

selected receivers, he offers detailed "white papers", which run between

ten and twenty pages or so. Magne also contributes a monthly review

column to Monitoring Times, and also appears on Radio Canada

International's "SWL Digest" program monthly with equipment reviews.

The other main source for equipment reviews is a group centered around

Radio Nederland and the WRTH in Holland.  The WRTH, as mentioned above,

has a review section covering mainly new receivers, but also contains a

table with ratings of most currently available radios.  Radio Nederland

also offers a free booklet with receiver reviews.

There are also two books published by Gilfer Shortwave in New Jersey

that cover the subject of receivers, called *Radio Receivers, Chance or

Choice*, and *More Radio Receivers, Chance or Choice*.

Here are some addresses for sources for more information and receivers

mentioned above:

RDI White Papers

same address as Passport to World Band Radio

Radio Nederland Receiver Guide

Engineering Department

PO Box 222

1200 JG Hilversum

The Netherlands

Radio Receivers, Chance or Choice

More Radio Receivers, Chance or Choice

Published by Gilfer Shortwave

(address in next section)

o Where can I get a shortwave radio?

Many stereo stores and discount chains carry the Sony and Panasonic

lines of receivers; the people there, however, generally don't know much

about shortwave, and you're not likely to find many accessories there.

Mail order stereo sources like J&R Music or 47th Street Photo in New

York generally give the cheapest prices, but have the same problem.

More knowledgeable, and falling roughly between the two in price, are

the mail order houses that specialize in ham and/or shortwave radio.

Many of them offer catalogs that contain useful tips for the beginner.

Listing all of the houses is beyond the scope of this posting, but here

are addresses for some of the better-known and respected businesses:

Electronic Equipment Bank              Gilfer Shortwave

137 Church St. N.W.                    52 Park Ave

Vienna, VA  22180                      Park Ridge, NJ  07656

(800) 368 3270 (orders)                (800) GILFER-1 (445-3371) (orders)

(703) 938-3350 (local and              (201) 391-7887 (New Jersey, business

technical information)                 and technical)

(703) 938-6911 (FAX)                   Free Catalog

Free catalog

Grove Enterprises                      Radio West

P.O. Box 98                            850 Anns Way Drive

Brasstown, NC 28902                    Vista, CA  92083

(800) 438-8155                         (619) 726-3910

(704) 837-9200                         Price list: $1

Free Catalog

Universal Radio

1280 Aida Drive

Reynoldsburg, Ohio  43068

(800) 431-3939

(614) 866-4267

SWL Catalog: $1.00

o Could you explain the frequencies used?  What's the 40 meter band? etc.

As you tune around, you'll notice certain kinds of signals tend to be

concentrated together.  Different services are allocated different

frequency ranges.  International broadcasters, for instance, are

assigned to ten frequency bands up and down the dial.  These are:

3900-4000 kHz (75 meter band)

5950-6200 kHz (49 meter band)

7100-7300 kHz (41 meter band)

9500-9900 kHz (31 meter band)

11650-12050 kHz (25 meter band)

13600-13800 kHz (22 meter band)

15100-15600 kHz (19 meter band)

17550-17900 kHz (16 meter band)

21450-21850 kHz (13 meter band)

25600-26100 kHz (11 meter band)

In general, lower frequencies (below 10000 kHz) are better received at

night and for a few hours surrounding dawn and dusk, and higher

frequencies (15000 kHz and up) are better received during the day.  The

frequencies in between are transitional, with reception being possible

most times.  In practice, these guidelines are not absolute, with

reception on high frequencies being possible at night, and lower

frequencies can provide decent medium-distance reception during the

day.

Hams (who have their own newsgroup, rec.ham-radio) and point-to-point,

or utility communications, fill most of the rest of the frequencies.

The Confidential Frequency List and The Shortwave Guide mentioned above

can provide more information on what can be heard in these areas, as can

utility loggings in magazines like Monitoring Times and Popular

Communications, and in club bulletins.

o Why can't I receive all of the broadcasts listed in Monitoring

Times/WRTH/Passport/etc.?

This is a fact of life on shortwave.  Because of propagation, antenna

headings, the kind of radio you have, your local environment, etc.,

you're never going to be able to hear all the things you find in a list.

The lists in Monitoring Times, etc., aren't lists of what's being heard

in a general location.  They're lists of everything that you could

possibly hear, from a daily powerhouse like the BBC to a once or twice a

year rarity like Bhutan.  They're listed because you *might* hear them,

depending on where you are and the given circumstances, not because

they're necessarily being heard outside of their immediate target area.

If you want lists of what is actually being heard in something roughly

analogous to "your area", the best source for these are the logging

sections of the bulletins of the SWL/DX clubs.  You might want to sample

a few club bulletins to see if they'll help.  The bulletins also offer

articles from experts on many facets of the hobby.

o What are some books or other resources that can help me get started?

There are a number of books dealing with the basics of the hobby.  One

of the best is *Shortwave Listening with the Experts*, edited by Gerry

Dexter, with contributions from many of the most experienced people in

the hobby.  The book makes an excellent introduction to a wide variety

of aspects of the hobby, from basics like how to set up your shack, to

in-depth articles on DXing the Andes. Unfortunately, this fine book has

recently gone out of print; if you happen to see a copy, you might want

to pick it up while you can.

There will soon be another posting available listing many other

worthwhile books for the hobbyist.

o Where can I find further information?

There are a number of hobby publications available.  Two glossy

magazines which cover the hobby are Monitoring Times and Popular

Communications.  They both cover a number of aspects of the hobby,

including international broadcasts, scanning, pirate radio, QSLing, and

Utility broadcasting.  Monitoring Times also contains listings of

broadcasts and programs in English, which gives it a slight edge.

PopComm, however, is the one you're more likely to find on your local

newsstand.

Monitoring Times

published by Grove Enterprises (address elsewhere)

Popular Communications

76 North Broadway

Hicksville, NY  11801

There are many clubs catering to the hobbyist, many of which publish

bulletins. Many of these groups were until recently part of an all-

encompassing group known as ANARC, the Association of North American

Radio Clubs.  However, ANARC disbanded in mid-1990 sue to a lack of

interest and a general consensus that it had fulfilled its mission.  The

WRTH contains contact addresses for the clubs that used to constitute

ANARC.

ANARC has counterpart organizations in Europe and the south Pacific.

The European organization is the European DX Council (EDXC).  More

information on their constituent clubs is available for 2 International

Reply Coupons from P.O. Box 4, St. Ives, Huntingdon, PE17 4FE, England.

In the south Pacific, the organization is the South Pacific Association

of Radio Clubs, or SPARC.  They offer information from P.O. Box 1313,

Invercargill, New Zealand.

A company called The Radio Collection offers a number of publications in

a series called "Radio 101" aimed at the beginner.  The compiler hasn't

seen any of the publications, but judging from the titles, they look

like they would be useful to anyone getting seriously interested in the

hobby aspects of shortwave radio.  A catalog is available for US$1 from

The Radio Collection, P.O. Box 149, Briarcliff Manor, NY 10510.

And, naturally, listening to the radio can provide you with excellent

information on radio.  There are a number of excellent "DX" programs on

the air for the radio hobbyist.  The WRTH contains a comprehensive list

of such shows; Tom Sundstrom also has a list as part of his Shortwave

Database subscription service, and Al Quaglieri of SPEEDX freely

distributes a list of some of the better programs electronically.

Different shows have different strengths.  DX Party Line on Ecuador's

HCJB is directed toward the beginner.  Sweden Calling DXers on Radio

Sweden is a compendium of news about shortwave and satellites, including

frequency changes, station reactivations and deactivations, and such.

Radio Nederland's Media Network is a slickly produced general-coverage

program. Radio Canada International's SWL Digest is another strong entry

along these lines.

o  Mid-East frequencies

The times given are "UTC" which is practically the same as Greenwich

Mean Time; it is 5 hours ahead of Eastern Standard Time.  0000 UTC

(Midnight) is 7:00 p.m. EST.

Broadcast       Prime        Other

Times           Freq.        Freqs

-----------     ------       ------

Radio Kuwait:

1500 - 2300     11990

0400 - 0800     13610*

1800 - 2300     13610*        11665

0900 - 1500     13620         15505

0200 - 0800     15435          9560

0200 - 2300     15495

0930 - 2300     15505*

0400 - 0600     17850

0900 - 1100     17895

0400 - 0730     17895

0800 - 1800     21675*

- - - - - - - - - - - - - - - - - - - - - - - - - -

Radio Baghdad:

0000 - 0200     11775 11810     11830 13650 15150 11720 15230 9505 7280 6010

0230 - 0430      9515  9700     11830 13650 15150 11720 15230 9505 7280 6010

2000 - 2200     13660           11830 13650 15150 11720 15230 9505 7280 6010

0200             9760            9505  7280  6010

- - - - - - - - - - - - - - - - - - - - - - - - - -

Radio Dubai, United Arab Emirates:

0330 - 0400     11940 15435 15555 17890         13675 15400 17830

0530 - 0600     15435 17775 17830 21700         15300 21515 11985

1030 - 1100     15320 15435 17865 21605         13605 11965 15985

1330 - 1400     15320 15435 17865 21605         11815 15340  9600

1600 - 1645     11790 15320 15435 21605

- - - - - - - - - - - - - - - - - - - - - - - - - -

Radio Damascus:

2000 - 2100     15095 12085 11710  9950

- - - - - - - - - - - - - - - - - - - - - - - - - -

KOL Israel:

17575 15615 21760 21790 17590 12077 15130 15095 17685 11790 15084 11605

15640 11585 11655 17630 15485 15650 21745 21780 12080 15592 15585  5900

9435  9855  9930  7410  7400

- - - - - - - - - - - - - - - - - - - - - - - - - -

Voice of the Islamic Republic of Iran:

11895 11790 15084 11620 11825 9575 9705 11715

4065   (Voice of Iranian Kurdiatan)

4224   (Voice of the Workers)

4469.8 (Communist Party of Iran)

9045   (Radio Farsi)

7190 7230 9695 9022 6035 9670 9525 9685 9770 7115

- - - - - - - - - - - - - - - - - - - - - - - - - -

Radio Cairo, Egypt:

9455 9475 9675 9805 9850 9900 11665 12050 15255 15375 17595 17675

- - - - - - - - - - - - - - - - - - - - - - - - - -

King of Hope and Radio Lebanon:

6215 6280 6550

- - - - - - - - - - - - - - - - - - - - - - - - - -

Voice of Turkey/Radio Ankara

9445 9665 9685 9795 14880 15220 15267 17760 17785 17880

- - - - - - - - - - - - - - - - - - - - - - - - - -

Radio Riyadh/BSKSA, Saudi Arabia

9705 9720 15060 9075 12085 9445 17880

- - - - - - - - - - - - - - - - - - - - - - - - - -

Radio Jordan, Amman:

9022 9560 11780 11955 13655 17710 17780

- - - - - - - - - - - - - - - - - - - - - - - - - -

Al-Qups Radio, Palestine:

5990 6030 15050

- - - - - - - - - - - - - - - - - - - - - - - - - -

Radio Jamahiriyrh Broadcasting(LJB), Libya:    15235 15415 15435

Voice of the Libyan People: 9500 15700

- - - - - - - - - - - - - - - - - - - - - - - - - -

Radio Bayrak, Cyprus:   6150

- - - - - - - - - - - - - - - - - - - - - - - - - -

Radio Bahrain:  15505

- - - - - - - - - - - - - - - - - - - - - - - - - -

Radio Oman:  9735 17735

- - - - - - - - - - - - - - - - - - - - - - - - - -

Radio Sanaa, Yemen:  9170 9779

- - - - - - - - - - - - - - - - - - - - - - - - - -

Qatar Broadcasting Service:  21525

- - - - - - - - - - - - - - - - - - - - - - - - - -

Voice of America - Middle East Service:

6040 9700 9760 11710 11760 11965 15205

Other VOA:

6030 7325 9595 9775 15250 15400 17730

- - - - - - - - - - - - - - - - - - - - - - - - - -