2 METER QUAD ANTENNA

The boom is 4 feet long 1 1/4" x 1 1/4" square dry fir. Straight

and light . Element spreaders are made from 3/8" dowels.

2 pcs. at 30 15/16" Reflector

2 pcs. at 29 3/16"  Driven element

4 pcs. at 27 3/4"   Directors

Plexiglas insulators made from 1/8" stock. Dimensions are

triangular 2" x 2"x 2 3/4". Two required, one for the feed point

and the other for the SWR adjustment jumper. The elements are

made from insulated #14 solid copper flameseal building wire. You

will need about 22 feet.

Hardware: 4 #6 nuts and bolts for the insulator posts. Wire for

lashing the insulators to the spreaders. I used a 1" pipe clamp

to fasten the antenna to a 1" plastic water pipe for

support.Waterproof glue, paint and Duxseal for weatherproofing

the connections on the insulators.

-----------------------------------------------------------------

CONSTRUCTION

First cut and drill the spreaders. Cut to the lengths as shown

above and drill the holes for the insulated #14 wire 1/2" from

the ends of the spreaders, be sure to line up the holes in the

same plane.

Next cut and drill the boom. In one plane only (like you are

making a Yagi) drill 3/8" holes for the spreaders. Center the

holes for the end elements 3" from the end of the boom. Drill the

hole for the first director 13" from the second director center

to center. You should now have 16" from the center of the

reflector to the center of the driven element.

Put one reflector, one driven and two director spreaders though

their respective holes and glue into position making sure that

the holes in the spreaders are at right angles to the length of

the boom. After the glue is dry, drill the boom for the other

spreaders drilling through the previously glued ones.Glue in the

remaining spreaders and let dry.The whole assembly should then be

painted with a good quality exterior paint to protect it from the

weather.

Run the insulated wire through the holes in the spreaders in the

reflector and the two directors baring the ends and twisting

until tight.Solder and tape each one for a good connection.

Fasten the insulators to the boom on opposite ends of the

horizontal dowel of the driven element with short pieces of wire.

Run the wire from one insulator to the opposite one securing the

bared ends to the insulator with the machine screws.

At one insulator connect a short jumper between the two screws

for SWR adjustment and at the other one connect the inner and

outer conductors of the RG58 or RG8 co-ax cable. After the

antenna is tuned seal the end of the co-ax cable and the two

insulators with Duxseal in order to prevent corrosion of the

connections.